SA WG2 Meeting #172	S2-2510432

17 – 21 Nov, 2025, Dallas, US                                                                          was S2-2510162

Source:	China Unicom, CATT, China Telecom, vivo, ZTE, OPPO

Title:	[WT#1.2, Network Sharing] WT(s) and KI(s) on 6G network sharing

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR gives the additional WT(s) and KI(s) on 6G network sharing based on the SA2#171 progress.

- Justifications

In SA2#171, the initial KI(s) on 6G Network Sharing has been approved and captured in TR 23.801-01:

5.X	 Key Issue #X: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

- However, there is still a remaining EN in the approved WT description to capture the other aspects which have not reached a consensus during the previous meetings. The approved WT description is shown as:

A.1.2.1	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- 	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

Editor's note:	It is FFS whether other aspects of network sharing in 6G can be added in this clause.

This pCR aims to capture the additional WT and KI on 6G network sharing, specifically considering the network sharing on radio access network with sensing capability to solve the EN. And based on the comments collected from the previous meetings, this pCR gives the detailed feedback from the aspects of operator requirements and technical analysis.

- 1.1 Network sharing on radio access network with sensing capability

- 1.1.1 Operator requirements

Given the multitude of use cases for services, some operators, based on their business model, may deploy one or more network capabilities (e.g., computing capability, or sensing capability) in some areas to serve their services in 6G, but not in all areas. However, through multi-operator sharing collaboration, the service coverage of specific network capability can be expanded, for instance, this could be used to provide positioning and velocity sensing services for drones, smart transportation and refrigerated transport. This particularly occurs in rural areas, in long-distance logistics and in traffic management, for example, in order to reduce the cost of deployments. The potential use case of Network Sharing on radio access network with sensing capability is shown in Figure 1 as below:

Figure 1 Network Sharing on radio access network with sensing capability

For point-to-point delivery by logistics UAVs from area A to area B, the sensing service can provide logistics customers with real-time positioning and velocity data assistance through operator sensing capability based on the operator’s policy.

As shown in Figure 1, Mobile Network Operator #A (OP#A) can provide sensing service by deploying radio access network with sensing capability (e.g., RAN node as sensing transmitter/receiver) in area A. The coverage of sensing service in OP #A is area A.

Mobile Network Operator #B (OP#B) can provide sensing service by deploying radio access network with sensing capability in area B. The coverage of sensing service in OP#B is area B.

To expand the sensing service coverage for OP#A and OP#B, OP#A and #B have the sensing network sharing agreement. This will extend their sensing service coverage to area A plus area B.

For the specific sensing service (e.g. UAV tracking), OP#A provides sensing service for logistics UAV tracking for collection of sensing data related to UAV using the radio access network with sensing capability of OP#A (e.g., UAV’ position, velocity and environment related data), when the UAV enters the sensing area A. When the UAV enters sensing area B, based on sensing network sharing agreement between operators, the OP#A can use the shared radio access network with sensing capability to collect the sensing data which satisfies the service requirement of OP#A’s customer (e.g. logistics UAV tracking) as Figure 1, while these collected data will not be used by other operators. Logistics customers do not need to be aware of which radio access network with sensing capability are used to provide the sensing service.

In the above case, the OP#A and OP#B just share the sensing functionality of RAN to each other, assuming that the communication functionality of RAN has been deployed in the area A and area B by OP#A and OP#B.

NOTE 1:  In the above case, based on the sensing network sharing agreement between the OP#A and OP#B, the shared radio access network with sensing capability of OP#B can be used as OP#A’s own radio access network with sensing capability, vice versa.

NOTE 2:  The use case and requirement of Network Sharing on radio access network with sensing capability has been captured in the clause 5.7.10 of TR 22.870.

- 1.1.2 Potential architectural impact

In 6G Day1, the network sharing on radio access network with sensing capability only focuses on the sensing mode where the sensing entity is RAN node (i.e. the shared Base Stations act as the sensing transmitter/receiver) and sharing the RAN sensing entity.

This work have the dependency with WT#4 Sensing, if the sensing mode where the sensing entity is RAN node in the sensing topic (WT#4 Sensing) has the progress (e.g. it has the interim conclusion), the network sharing related aspects can study how the network sharing architectures (e.g. MOCN in 6G and/or INS in 6G) are applicable for the network sharing on radio access network with sensing capability.

Comparison with the traditional RAN sharing, the network sharing on radio access network with sensing capability has the different service procedure, for example, the sensing consumer sends the sensing request to the core network of operator and then the sensing function of core network will select the sensing entity (i.e. RAN node) based on the sensing request and operator policy. This will cause the potential enhancement (e.g. selection of shared RAN sensing entity crossing different operators) from the network sharing aspects to support this use case.

Based on the above analysis, there may be few impact of network sharing architectures to support the network sharing on RAN with sensing capability because the configuration and interface connection related to sensing feature can be achieved based on the sensing network sharing agreement between the operators.

Q&A part:

- Question 1: Whether the network sharing on RAN with sensing capability should be in the scope of sensing topic?

- Answer 1: WT#4 sensing focuses on studying the general aspect of sensing architecture. When the sensing mode where the sensing entity is RAN node in the sensing topic (WT#4 Sensing) has the progress (e.g. it has the interim conclusion), then the network sharing related aspect can study how the network sharing architectures (e.g. MOCN in 6G and/or INS in 6G) are applicable for the network sharing on radio access network with sensing capability and this part should be in the scope of network sharing. And from the procedure perspective, in this phase, it is important to reach the consensus on the use case we need to solve and the corresponding WT&KI, then in the solution phase, the solution can be proposed to solve multiple relevant KIs.

- Question 2: What is the consideration to focus on the network sharing on RAN with sensing capability?

- Answer 2: the traditional RAN sharing just focuses on the RAN communication resources sharing to provide the connectivity service. Currently, 6G will introduce some new services, “the network sharing on RAN with sensing capability” can be a start point to explore how to support some new services through network sharing mechanism. And the current consideration of this aspect focuses on the base station based sensing sharing, because the base station resources belong to the operator resources.

Summary: If the sensing mode where the sensing entity is RAN node in the WT#4 Sensing has the progress, the 6G network sharing related aspects should study how the network sharing architectures can be applicable for the network sharing on radio access network with sensing capability.

- Proposals

This pCR solves the EN to give the additional WT. On top of the WT, the corresponding KI is proposed as well.

The following should be captured in TR 23.801-01.

**** First Change ****

A.1.2.1	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

- How to support the network sharing on radio access network with sensing capability in 6G:

  - Study how to support the sensing service in network sharing architectures (e.g. MOCN in 6G and/or INS in 6G).

NOTE :   The network sharing on radio access network with sensing capability only focuses on the sensing mode where the sensing entity is RAN node and sharing the RAN sensing entity. The support of this sensing mode depends on the progress of WT#4 (e.g. start the work of the above bullet 2 if the corresponding sensing work has the interim conclusion).

**** Second Change ****

- X	Key Issue #X: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

- How to support the network sharing on radio access network with sensing capability in 6G:

a.Study how to support the sensing service in network sharing architectures (e.g. MOCN in 6G and/or INS in 6G).

NOTE :   The network sharing on radio access network with sensing capability only focuses on the sensing mode where the sensing entity is RAN node and sharing the RAN sensing entity. The support of this sensing mode depends on the progress of WT#4 (e.g. start the work of the above bullet 2 if the corresponding sensing work has the interim conclusion).

**** End of Changes ****