SA WG2 Meeting #172	S2-2510447

17 – 21 November, 2025, Dallas, USA

Source:	Xiaomi

Title:	Interim agreements for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC/Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for KI#1 of FS_5GSAT_Ph4-ARC.

- Discussion

Until SA2#171, 14 solutions were accepted for KI#1 in TR 23.700-19. This contribution tries to give summary and analysis to the solutions, and interim conclusion is given based on the analysis.

Table 1: Solution summary

| Solution | Transmission path |  |  | Transmission type |  |  | PDN connection |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CP | UP | CP+UP | IP | NIDD | IP+NIDD | 1PDN (1bearer) | 1PDN (2 bearers) | 2 PDN connections |  |
| 1 |  | ● |  | ● |  |  | ● |  |  | Non-GBR default bearer is applied to IMS signalling and voice data |
| 2 |  | ● |  | ● |  |  |  | ● |  |  |
| 3 |  | ● |  |  |  | ● |  | ● |  | - IMS signalling over default bearer uses IP type; - Voice data over dedicated bearer uses non-IP data |
| 4 |  | ● |  |  | ● |  |  |  | ● | Selecting same PGW for serving two non-IP PDN connections for a UE |
| 5 | ● |  |  | ● | ● | ● |  | ● | ● | - IMS signalling + voice data are transported over one single PDN connection with IP/non-IP type - IMS signalling using one IP PDN connection + voice data uses another PDN connection with non-IP type |
| 6 | ● |  |  | ● |  |  |  | ● |  |  |
| 7 |  |  | ● | ● |  |  | ● |  |  | - Default bearer is used for both IMS signalling and voice data - SRB is used for signalling and can be switched to S1-U for voice transmission |
| 8 |  |  | ● | ● |  |  |  | ● |  | - IMS signalling over CP default bearer - Voice data over UP dedicated bearer - Both S11-U path and S1-U path needs to be maintained simultaneously |
| 9 |  | ● |  | ● |  |  |  | ● |  | Idle UE originally uses CP to transport IMS signalling, then uses UP to transport both signalling and voice data |
| 10/11 |  |  | ● | ● | ● |  |  |  | ● |  |
| 27 |  |  |  |  |  |  |  |  |  | Focusing on P-CSCF reselection when UE changes its access RAT |
| 28 |  | ● |  |  |  | ● |  | ● |  | - IMS signalling over default bearer uses IP type; - Voice data over dedicated bearer uses non-IP data |
| 29 |  |  | ● |  |  | ● |  | ● |  | IMS signalling uses CP default bearer in IP type, voice uses UP dedicated bearer with non-IP data |

It has been agreed the interim conclusion for KI#1 in SA2#171 meeting that the voice packets shall be transported over NB-IoT(GEO) user plane, while whether using control plane or user plane to transport IMS signalling still needs to be decided. The following first part is to discuss IMS signalling transmission path for supporting of IMS voice call over NB-IoT(GEO).

- IMS signalling over CP vs. UP

Solution 5 and 6 propose CP based solution. Both IMS signalling and voice packets are transferring over control plane. Solution 7,8,10/11 and 29 propose CP+UP based solution. The IMS signalling is transferring over control plane while voice packets are transferring over user plane.

Under the agreement that UP is applied to voice packet transmission, assuming CP is selected for IMS signalling, then both CP and UP will be used simultaneously for supporting IMS voice, which may have the following problems that needs to be considered:

- Supporting both CP and UP based bearers within an IMS PDN connection requires S-GW to maintain both S11-U path and S1-U path simultaneously during the IMS call and needs to identify the IMS signalling and voice packet. This may increase the complexity of SGW operation.

- Considering IMS signalling over CP will increase the NAS overhead and in further leads to the increase of latency. NAS header reduction by eliminating 5 bytes of NAS layer security overhead is trying to be considered to mitigate the latency. However, based on the LS reply from SA3 (S3-253797), majority were not in favour of the 5 bytes overhead elimination considering security issues. Therefore, using CP to transfer IMS signalling will intensify latency of IMS registration and call setup time.

Solution 1,2,3,4,9 and 28 propose both IMS signalling and voice packet are transferring over UP.  The PDN connection and EPS bearer establishment can follow the existing procedures without any enhancement. Regarding the 2-DRB restriction, based on the LS reply from RAN2 (R2-2507784), supporting more than 2 DRBs is feasible for a UE accessing NB-IoT(GEO).

Proposal 1: The IMS signalling shall be transported over NB-IoT(GEO) user plane as well.

- IP, non-IP or IP+NIDD combined solution

8 solutions propose IP based solution, 3 solutions propose NIDD(non-IP) based solution and 4 solutions propose IP+NIDD solution.

Compared with NIDD solution, IP based solution has larger packet overhead. But it can get a significant reduction through RoHC for IP/UDP/RTP header. Regarding RoHC state fall back due to consecutive packets lost, based on the RAN1 LS reply(R1-2508096), it can be negligible in the case of LoS condition for the duration of the call and may not be negligible in the case of LoS condition not being guaranteed for the duration of the call.

If the voice data is sent without UDP/IP (NIDD solution), some estimates or analysis show that using full RTP header may be larger than when RoHC is applied for the RTP/UDP/IP headers. Whether and how to reduce the potential protocol overhead when using NIDD solution (e.g., reducing on RTP header) is still waiting to be studied (e.g., by SA4). Another aspect is the IMS service relies on SIP for signalling exchange and the media streams are typically transmitted over IP/UDP/RTP. Both SIP signalling and voice media typically require IP address to interaction between two sides. The NIDD based solution requires IP/non-IP convert on both UE and network side. For example, the UE should support NIDD encapsulate of voice packet and S-GW should transform the NIDD packet to the IP Packet. This will increase the complexity of UE and S/P-GW operation.

Based on the above analysis, IP based solution is proposed to transport voice data between UE and P-GW. Non-IP based solution can be supported as an optional but whether and how to reduce the overhead of RTP header is up to SA4.

Proposal 2: The IMS signalling shall be transported as IP data between UE and P-GW.

Proposal 3: The voice data is transported as IP data between UE and P-GW.

Proposal 4: The voice data can be transported as non-IP (remove UDP and IP header) data between UE and P-GW.

- One PDN connection vs. two PDN connections:

2 solutions propose using one PDN connection with one default bearer for both IMS signalling and voice data, 8 solutions propose using one PDN connection with one default bearer for IMS signalling and one dedicated bearer for voice data, 3 solutions propose using a pair of PDN connections for IMS voice service.

For one PDN connection with one default bearer to carry both IMS signalling and voice data, it cannot provide QoS differentiation to the signalling and voice. For supporting IMS service, the current mechanism is using a single PDN connection with an APN for IMS service. For using a pair of PDN connections, two APNs needs to be deployed for an IMS service, besides it requires both UE and network side to associate context of the two PDN connections.

Proposal 5:  A single PDN connection with two bearers is proposed for supporting IMS voice over NB-IoT (GEO).

2 	Proposal

It is proposed to capture the following interim agreements into TR 23.700-19 v1.1.0.

**** First Change ****

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The IMS signalling shall also be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The IMS signalling shall be transported as IP data between UE and P-GW.

-	The voice data is transported as IP data between UE and P-GW.

-	The voice data can be transported as non-IP (remove UDP and IP header) data between UE and P-GW.

NOTE: 	Whether and how to reduce the overhead of RTP header when using non-IP to transmit voice data is up to SA4.

-	A single PDN connection with two bearers is used for supporting IMS voice over NB-IoT (GEO).

**** End of Changes ****