SA WG2 Meeting #S2-172	S2-2510449

17 - 21 November, 2025, Dallas, USA

Source: 	vivo

Title: 	KI#1 interim conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update interim agreement for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following changes to TR 23.700-04.

Start of Change

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

1. UP path termination entity

-	An existing 5GC NF (e.g. DCCF) is defined as the UP path termination entity to support the transfer of data over UP for UE data. The UP path termination entity will further transfer the received data to the AF for UE side model training.

-	The UP path termination entity is selected by NEF (on behalf of UE model training entity server) based on NF discovery procedure from NRF or local configuration.

-	The UP path termination entity performs the user consent check from UDM by reusing the existing use consent check purpose.

2. UP path establishment

-	A UP path is used between the UE and the 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	The UP path termination entity sends the UP information for data reporting (i.e. IP address or FQDN of the UP path termination entity) to the UE via NAS signalling, which is used by the UE to establish the UP path between the UE and the UP path termination entity.

-	The FQDN / IP address of the UP path termination entity is self-constructed by the UE according to the URSP rule or received UP information for UP path establishment.

3.	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules. The URSP includes the FQDN/target address for data reporting in TD and the dedicated DNN/S-NSSAI in RSD.

4. Data Transfer Request

-	The UP path termination entity sends the Data Transfer Request with specific requirements (e.g. data type, reporting time, area, data format and etc.) to the UE (either via the control plane along with the UP information or via the user plane), according to data request from the data consumer.

-	The UE reports the collected data to the UP path termination entity via the UP path, based on the requirements received in the Data Transfer Request.

NOTE: which standardized data can be collected by the UE will be defined by RAN1. The mechanism how UE collects data and corresponding RRC configuration depend on RAN1 and RAN2’s decision.

5.	Controllability

-	The UE is allowed to negotiate with the UP path termination entity about data transferring (e.g., accept / reject data transferring request). The UE also can cancel data transferring, e.g. due to leaving the area received from the UP path termination entity in the Data Transfer Request.

6.	Visibility

-	The UP path termination entity checks the reported UE data by comparing them with the specific requirements (e.g., data type, data format).

-	The UP path termination entity exposes the data that meet the requirements to the data consumer with the authorization of NEF.

7.	Others

-	The UP path termination entity may store the collected data from UEs in ADRF.

-	No need to support paging of UEs in IDLE or INACTIVE mode.

Next Change

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether and how to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

End of Changes