SA WG2 Meeting #S2-172	S2-2510450

17 - 21 November, 2025, Dallas, USA	(revison of xxxx)

Source: 	vivo

Title: 	KI#2 interim conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes an update on KI#2 interim conclusion.

1	Discussion

| Category | Output Parameters | Resource solution | details |
| --- | --- | --- | --- |
| Time info | Time slot start | #23, #24, #27, #29, #36 | Time slot start within the Analytics target period. |
|  | Duration | #23, #24, #27, #29, #36 | Duration of the time slot. |
|  | time period(s) | #22, | expected/observed time period(s) of abnormal user plane observations |
|  | Start time & end time | #31 | Start time / end time of the specified traffic flow. |
| Identifier info | application ID | #22, #27, #29, #31, #33, #36 |  |
|  | UPF ID | #23, #27, #29, #31, #36 |  |
|  | IP packet filter | #23, #27, #31, #33, #36 | Target Packets or packet patterns are identified by packet pattern ID (#22) |
|  | UE ID | #22, #23, #27, #29 | PDU session ID (#24) QFI (#33) PDU session List (#36) PDU sessions (SUPI(s), DNN, S-NSSAI) |
| Exception level | predicted severity UP performance level Traffic burst index traffic anomaly risk levels exception level | (#23) (#27) (#29) (#35) (#36) | For example: risk level, e.g. Informative, Warning, High, Critical. |
| Exception type | type of abnormal user plane observations | (#22) |  |
|  | UP pattern type | #23 | Predicted UP pattern type, e.g. DDoS, misbehaving server/UE, burst, malformed packets, unknown packets, duplicate packets, fragmented packets, etc. |
|  | Exception type | #24 | Indicates the type of exception, i.e. one of burst, malicious, suspicious. |
|  | cause | #27 | The cause of potential/ detection UP performance degradation, e.g. unexpected service flow, unexpected traffic volume or burst size, etc. |
|  | Abnormal packet indicator | #29 | Indicates whether unclassified, malformed, or suspicious packet flows were detected during the slot. Value: Yes / No. |
|  | abnormal detection information | #30 |  |
|  | Reason of abnormality | #31 | The reasons of abnormal packets, e.g. failure transmission, repeat transmission. |
|  | Malicious type suspicious identification | #34 | Malicious type of the identified Application ID or data flow: e.g. Phishing websites, Telecommunication fraud websites or apps, Malicious emails. Whether the data flow or Application ID input by the NF consumer is suspicious |
|  | exception ID | #36 | The identifier of the exception predicted by NWDAF Exception ID can be, e.g. DDoS, too much data, too high or too low rate…… |
|  | Flow stability score exception trend | #29 #36 | Indicates how consistent the traffic is over time: Stable / Medium / Unstable. Predicted trend (up/down/unknown/stable) |
| Traffic pattern | Traffic characteristics | #27 | The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), packet delay, loss rate, throughput, etc. within the measurement period. |
|  | Traffic prediction information | #30 #34 |  |
|  | Traffic Pattern Report | #31 | List of traffic pattern report towards the traffic over the specified UPF. |
|  | traffic characteristics | #35 |  |

Observation 1: Time info is mandatory in analytics output;

Observation 2: Identifier is mandatory in analytics output, with different granularities (UE, PDU, UPF, Application).

Observation 3: Most solutions introduced exception level to indicate severity.

Observation 4: Most solutions introduced exception type to classify anomalies.

Observation 5: Some solutions also introduced traffic pattern as part of the output.

Proposal 1: Document observation 1 to 5 in the Agreed Principles for KI#1 UC#1.

| Consumer NF | Actions | Resource solution | details |
| --- | --- | --- | --- |
| SMF | configure UPFs | #22 | block abnormal traffic |
|  | Instruct the UPF | #23 | Overload controls, e.g. reduce UPF load. Re-selections, e.g. to less loaded UPF. …… |
|  | initiate UPF reselection | #27 | distribute the load across UPF instances upon detecting abnormal traffic patterns |
|  | notifying UPF to execute traffic gating …… | #27 | e.g. notifying UPF to execute traffic gating or shaping, enforcing maximum data rate or adjusting QoS parameters. |
|  | modify FARs, reassess forwarding behaviour, or update UP path configurations | #29 | To modify FARs, reassess forwarding behaviour, or update UP path configurations in cases of burstiness, idle prediction, or QoS profile mismatch (e.g. 5QI misalignment). |
|  | select/reselect suitable UPF | #31 | SMF could select/reselect suitable UPF for new PDU sessions, adjust N4 rules, etc. |
|  | update FAR | #33 | If the NF consumer is SMF, the SMF may update FAR based on the Analytics result without instruction from PCF. |
|  | Selective traffic handling Re-selections | #36 | Selective traffic handling, via node rules in a N4 Node level procedure where SMF instructs UPF to apply actions (drop, throttle, inhibit DLDR) on data flows that match the provided packet filters. |
| PCF | apply PCC rules | #22 | block abnormal traffic, or to optimize the user plane performance optimization by adjusting the QoS |
|  | Generates or updates the PCC rules | #23 | indicate to drop packet from the abnormal traffic filter or degrade the QoS level of the abnormal traffic filter |
|  | provision policies to SMF | #27 | e.g. executing traffic gating or shaping, enforcing bandwidth parameters or adjusting QoS parameters. |
|  | Adjust QER, 5QI assignment…… | #29 | adjust QER, 5QI assignment, shaping profiles, or DLDR configuration via PCC rules |
|  | update PCC rules at the SMF | #34 | it can instruct the SMF to gate the traffic or ask the SMF to release existing PDU session or/and reject future establishment request(s) |
|  | formulates UPF function set upgrade/downgrade strategies | #35 | PCF formulates UPF function set upgrade/downgrade strategies based on NWDAF’s analytics, considering UPF load, traffic anomaly risks, and service priorities. |
|  | Generates the PCC rule(s) | #36 | indicate to drop packet from the abnormal traffic filter or degrade the QoS level of the abnormal traffic filter. |
| OAM | configure SMF and/or UPF | #22 | block abnormal traffic or to increase capacity |
| AF | take action to correct abnormal traffic | #22 | control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices. |
| UPF | adjusting message frequency or apply downlink traffic suppression | #27 | control the number of DLDR (Downlink Data Report) messages sent to SMF by e.g. adjusting message frequency |
|  | traffic handling optimization | #30 | e.g. data rate limitation, traffic filtering, etc. |
|  | sends an N4 Session Report to SMF | #30 | SMF may decide to update N4 rules or updated SM policy. |
|  | allocate more resource adjust interface thresholds | #31 | detect malicious IP packets, allocate more resource before the bursts, adjust interface thresholds, etc. based on the Analytics ID. |

Observation 6: The analytics consumer is usually PCF, NWDAF, AF, UPF or OAM.

Observation 7: SMF is the most frequent consumer, typically taking actions to configure or instruct UPF (e.g., reselection, FAR updates).

Observation 8: PCF consumes analytics mainly for policy control (e.g., apply/update PCC rules, QoS adjustments).

Observation 9: UPF as consumer may perform local optimizations (e.g., traffic filtering, allocate more resource).

Proposal 2: Document Observations 6–9 in the Agreed Principles for KI#1 UC#1.

| Category | Input Parameters | Resource solution | details |
| --- | --- | --- | --- |
| Anomaly info | Unexpected flow info Unexpected packet info | #23 | Anomaly information regarding a traffic |
|  | Abnormal packet indicator | #24 | Wrong or malformed or duplicated packets. |
|  | (Abnormal) data packet information | #27 | For example, malformed packets, duplicate packets, packet retransmission, traffic hijacking and traffic spoofing. |
|  | Packet drop cause Malformed packet count | #29 | Indicates why packets were dropped Number of packets dropped or flagged due to protocol violations |
|  | Malformed/ Duplicate/ Retransmitted/ Fragmented packets | #36 | Number of Malformed/ Duplicate/ Retransmitted/ Fragmented packets in the flow. |

And as descripted in clause 4.2 of TR 23700-04, the UPF could detect wrong or malformed or duplicated packets.

Proposal 3: Document the provision of Anomaly information in the Agreed Principles for KI#1 UC#1.

Proposal 4: Clarify the UPF reselection for distribute the load across UPF instances is based on the description in clause 4.3.5 of TS 23.502.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288 [5]) for input data filtering to UPF.

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal data packets patterns (e.g. malformed, unexpected traffic volume, burst, etc.), malicious (e.g. DDoS) or suspicious.

-	Exception level to indicate severity level.

-	Volume, rate or burst size of abnormal traffic.

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s).

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	Time information (e.g. time slot start and duration).

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

-	From UPF/SMF:

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.).

-	Traffic flow filters.

-	Identification of corresponding UEs.

-	Information of traffic characteristics.

-	Anomaly info (e.g. abnormal packet info).

-	The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501 [2] and clause 4.3.5 of TS 23.502).

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations.

-	The UPF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations).

-	adjust packet processing resources.

NOTE 1:	UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.

-	The PCF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2:	This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

End of Changes