SA WG2 Meeting #S2-172	S2-2510488

17 - 21 Nov, 2025, Dallas, US	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Update for KI#2 UC#2 Interim Agreements and Conclusions

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements and conclusion for AIML_CN KI#2 use case 2.

- Discussion

Interim agreements have been made for KI#2 use case#2 but the type/identification of traffic pattern is still not clear:

-	The need and details of type/identification of traffic pattern is FFS.

This contribution proposes to copy the interim conclusion in the conclusion. For the type/identification of traffic pattern, its purpose is for some unspecified pattern, it is proposed to reuse existing QoS reference provided by AF, which can be configured based on operator policy.

This contribution proposes to capture above changes and the agreed principles for use case#2 in the conclusion.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

****************Start of the change ****************

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-	For UC1, the input from AF, possibly via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

NOTE:	How UPF subscribes to the new analytics is FFS.

****************Next change ****************

8	Conclusions

Following conclusions for key issue#2 use case#2 has been made:

- A new analytic shall be defined to support normal traffic analysis.

- Following inputs shall be supported:

- Table 8-1: input data for normal traffic analytic

| Information | Source | Description |
| --- | --- | --- |
| traffic flow filters | UPF/SMF | Information identifying the associated application/service data flow(s), e.g. packet filter, application ID of the traffic flow |
| Identification of corresponding UEs | UPF/SMF | Identifier of the UEs receive/send the traffic. |
| traffic characteristics | UPF/SMF | Information related to the data flow characteristics that can be used by NWDAF to determine the QoS related information, including, N6 delay, congestion information, indication that UPF drops packets and packet drop rate, data volume, packet delay, data rate, periodicity, and packet interval. |

- Table 8-2: output data for normal traffic statistics/predictions

| Information | Description |
| --- | --- |
| Traffic flow filters | Corresponding information of the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s); |
| Maximum Burst Size | Maximum Burst Size as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| Maximum Data Burst Volume | MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| Maximum Bitrate | Requested Maximum Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| Guaranteed Bitrate | Requested Guaranteed Bitrate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| 5GS delay | Requested 5GS delay (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| Packet Error Rate | Requested Packet Error Rate (UL, DL) as defined in clause 6.1.3.22 of TS 23.503 [4] of the target application(s). |
| Periodicity | Periodicity as defined in clause 5.27 of TS 23.501 [2] of the traffic for target application(s) if applicable. |
| QoS reference | QoS Reference parameter that refers to pre-defined QoS information (e.g. above QoS parameters in the output), as described in clause 6.1.3.22 of 23.503[4]. |

The service consumer of the normal analytic is PCF. The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc. The PCF may take the analytic outputs into account to derive the QoS (e.g. generate or update PCC rules) for the target application(s).

****************End of the change ****************