3GPP SA WG2#172	S2-2510526

Dallas, US, 17-21 November, 2025

Source:	OPPO

Title:	Interim Agreement for SMS over NAS to Emergency Centre

Spec:	3GPP TR 23.700-65

Agenda item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution is to propose the interim agreement for KI#1 and KI#2.

1. Discussion

Based on TR 23.700-65 V0.2.0, Solutions #1-#11 are to address KI#1 and KI#2. Main issues and alternative options are summarized below:

a) Scenario 1: UE can detect SMS2EC

a1) Capability negotiation on SMS2EC

- Network (e.g., AMF/MME/S-CSCF) indicates to the UE whether network supports routing an SMS to a local Emergency Response Centre (SMS2EC) during Attach/Registration. (Solutions #2, #3, #5, #6, #7, #8, #11.)

- UE indicates support for SMS2EC during Attach/Registration. (Solutions #3, #5, #7, #11.)

We understand that network needs to indicate UE whether SMS2EC is supported, as the serving PLMN may not support routing an SMS to an Emergency Response Centre. Whether UE needs to indicate support for SMS2EC to network depends on the specific solution. For instance, if the emergency number is always provided to the UE, the UE doesn’t need to indicate its capabilities. If emergency numbers are only provided to UEs capable of supporting SMS2EC, the UE needs to indicate its capability.

a2) Provision of Emergency number, emergency service type, SMSC address

- Serving network provides local emergency numbers to the UE. (Solutions #1, #2, #3, #5, #6, #7, #8, #9, #11)

- Serving network provides emergency service types to the UE. (Solutions #3, #4, #5, #6, #8, #9, #11)

- Serving network provides local SMSC address to the UE. (Solutions #6, #7)

- When UE builds a SMS2EC, the emergency numbers for SMS and associated emergency service type for SMS is included in the SMS TPDU parameters. (Solution #6)

The UE requires the local emergency numbers and associated service types to route SMS messages to the correct PSAP. The provision of a local SMSC address by the Serving Network, however, is solution-dependent.

a3) Identification of SMS2EC

- UE detects SMS2EC based on the emergency number. (Solutions #1, #2, #3, #5, #6, #7, #8, #9, #11)

- Network (e.g., SMSC) detects SMS2EC based on the TPDU parameters within the Short Message (Solutions #1, #3, #6, #7, #8, #9, #10)

- Network (e.g., AMF/SMSF/MME/MSC) detects SMS2EC based on the indication outside the Short Message (Solutions #4, #5, #7, #10, #11)

If the emergency numbers are provided or configured in the UE, UE can detect the SMS2EC. The UE can indicate via NAS indication or TPDU parameters that the SMS needs to be routed to the Emergency Response Centre.

a4) RRC establishment cause

The UE set the RRC establishment cause to emergency if the UE is triggering to send the SMS2EC. (Solution #5)

a5) Location determination

The solutions fall into the following categories:

UE-Supplied Location:

- the UE populates the SMS TPDU parameters with UE’s location information. (Solutions #1, #6)

- the UE’s location information is within the user data. (Solution #3, #7)

- the UE’s location information is included in the NAS payload container. (Solution #4)

Network-Derived Location:

- The AMF/MME/SMSF retrieves or verifies the UE's location information via location services. (Solutions #1, #3, #5, #6, #7, #10)

- Utilize the current ULI of the UE. (Solution #8, #9)

If UE can obtain its location information, UE can provide its location information within SMS. If the network detects an emergency message or receives an instruction from the UE, the network retrieves the UE's location information via location services or utilizes the current ULI.

a6) Support for roaming users

The solutions fall into the following categories:

- Based on the indication outside the short message, AMF/SMSF/MME/MSC can be aware that this is for SMS2EC, and the local SMSC will be selected to route the SMS to the Emergency Centre. (Solution#4, #5, #7, #10)

- UE can replace the RP-DA with a local SMSC address/country code/emergency number, such that the SMS can be delivered to a local SMSC in serving PLMN, and finally be routed to the Emergency Centre. (Solution#6, #8, #9)

a7) Callback from PSAP

The callback from PSAP will be treated as a regular SMS. The response message will be submitted to the SMSC in UE’s serving PLMN (i.e., the HPLMN of the PSAP), and then be treated as a regular MT SMS.

b) Scenario 2: UE cannot detect SMS2EC

b1) How network detects SMS2EC

For SMS over NAS in the 5G System, the SMSF in the serving PLMN is always selected, regardless of whether the UE is roaming. SMSF is enhanced to detects the emergency number and determine whether SMS from roaming UE is submitted to a local SMS.

c) Security Aspects

Editor's note:	The security implication if any due to RP-DA replacement is to be studied in SA WG3.

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-30.

* * * * First Change * * * *

### 7.1.X	Agreed Principles for SMS over NAS to Emergency Centre

The following principles are agreed for KI#1 and KI#2 to support SMS over NAS to Emergency Centre.

Capability negotiation:

-	UE indicates its support for SMS over NAS in the Registration Request/Attach Request as already specified in TS 23.401 and TS 23.502.

-	AMF/MME indicates to the UE whether network supports routing a SMS to a local Emergency Response Centre in the Registration Accept/Attach Accept message.

Provision of emergency numbers, service types, and SMSC address:

-	The AMF/MME in the serving network provides local emergency numbers for SMS, associated emergency service type for SMS to the UE in the Registration Accept/Attach Accept message.

-	The AMF/MME in the serving network provides local SMSC address to the UE in the Registration Accept/Attach Accept message.

-	When UE builds a SMS2EC, the emergency numbers for SMS and associated emergency service type for SMS is included in the SMS TPDU parameters.

Delivery of SMS2EC:

-	If UE detects SMS2EC based on the emergency number, UE may

-	replace the RP-DA with a local SMSC address, or country code and emergency number.

-	indicate to AMF/MME the SMS is destined for a local Emergency Response.

- 	AMF indicates to SMSF the SMS is destined for a local Emergency Response.

-	Based on the RP-DA or the indication indicating the SMS is destined for a local Emergency Response, the SMS is submitted to a local SMSC in the serving PLMN.

-	The local SMSC delivers the SMS to a PSAP, considering the emergency numbers, associated emergency service type and UE’s location information.

Location determination:

-	The UE's location information is populated into the SMS TPDU parameters or SMS user data.

-	The AMF/MME/SMSF retrieves UE's location information via location services and then provides to the SMSC.

RRC establishment cause:

-	The UE set the RRC establishment cause to emergency if the UE is triggering to send the SMS2EC.

Callback from PSAP:

-	The response from PSAP will be treated as a regular SMS.

Support for the scenario where UE cannot detect SMS2EC:

-	If SMSC detects the receiver’s number is an emergency number, SMSC replaces the RP-DA with a local SMSC address.

* * * * End of Changes * * * *