3GPP TSG-WG SA2 Meeting #172 	S2-2510569

Dallas, U.S.A, Nov 17th – 21st, 2025	(revision of S2-2509066)

Source:	NTT DOCOMO

Title:	Solution#5 Update: Addressing ENs and Updating Procedure.

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes to update Solution #5 by addressing ENs and updating the procedure.

# 1. Discussion

In TR 23.700‑14 v1.1.0, Solution #5 (AMF-Inclusive Framework for 5GC Sensing) currently has open ENs. The purpose of this paper is to resolve these ENs, and to update the corresponding procedure to address general concerns raised regarding AMF-based sensing framework.

The ENs in place were regarding possible enhancements required for the AMF to support this solution as well as uncertainty regarding AMF’s capability to support aggregation of data collected from the RAN. The update to the procedures in this paper makes use of existing Namf_Communication service and signaling procedures as specified for TSN in clause 4.15.9.5 in TS 23.502 and for PWS in clause 9A.2.3 in TS 23.041. In this solution, like the existing procedures mentioned earlier, the SF sends the Namf_Communication NonUeN2InfoSubscribe request/response messages to subscribe for receiving sensing data. Furtehrmore, the SF sends Namf_Communication_NonUeN2MessageTransfer request to AMF and includes an N2 Container indicating to the NG-RAN to start reporting the sensing data. The AMF sends the corresponding N2 messages to all applicable NG-RAN nodes. The AMF then send to the SF a list of N2 Containers containing response messages from the NG-RAN nodes and the corresponding NG-RAN node identifier(s). In doing so, the AMF may receive sensing data from several sensing entities and aggregate the sensing data report messages it receives. The corresponding impacts on NG-RAN to support this procedure has also been updated.

# 2. Proposal

It is proposed to update the Solution #5 of TR 23.700‑14 v1.1.0.

# * * * * First change* * * *

## 6.5	Solution #5: AMF-Inclusive Framework for 5GC Sensing

### 6.5.0	High-level solution Principles

The high-level solution principles are as follows:

-	An AMF-inclusive framework where the AMF is responsible for:

-	Discovery and selection of sensing entities

-	Forwarding the sensing request to the sensing entities.

-	Introduction of a new network function for Sensing called Sensing Function (SF) which:

-	Authorizes the usage of sensing service and selects the relevant AMF(s) based on the sensing service request parameters.

-	Processes the sensing data to derive the sensing result.

-	The N2 interface is enhanced to support sensing data transmission.

### 6.5.1	Description

The solution provides a high-level overview of the procedure for sensing. The following key principles apply to the sensing procedure:

-	The Application Function (AF), acting as the service consumer, initiates the sensing service by sending a sensing service subscription request to the 5GC, e.g. to the Network Exposure Function (NEF).

-	Upon receiving the request, the NEF authorizes the request and selects an appropriate Sensing Function (SF) based on the sensing service request parameters.

-	The SF performs the following actions:

-	Authorizes the usage of sensing service.

-	Selects the relevant AMF(s) based on the sensing service request parameters.

-	The AMF(s) continues the procedure as follows:

-	Discovers and selects the corresponding sensing entities (e.g. Sensing Transmitter(s) and/or Receiver(s)).

-	Forwards the sensing request to the sensing entities.

-	The sensing entities acquire the sensing data.

-	The Sensing Data is then transmitted to the SF via the AMF, which processes the data to derive the Sensing Result.

-	The SF exposes the resulting information to the AF via the NEF.

### 6.5.2	Procedure of the framework

Figure 6.5.2-1: Procedure of the AMF-inclusive control plane-based sensing framework

1.	The AF invokes the Nnef_Sensing_Subscribe service operation toward the NEF to subscribe for the sensing information from the network.

NOTE 1:	The detailed description of the sensing service requirements, sensing service types, and associated sensing-related parameters is subject to the solution of KI#2.

2.	The NEF performs authorization of the AF for the usage of the sensing service for the received request.

NOTE 2:	The specific mechanisms for NEF authorization of the AF will be addressed in the solution to KI#2.

3.	The NEF may perform discovery and selection of candidate Sensing Function (SF).

NOTE 3:	The detailed handling of Sensing Function discovery will be addressed in the solution to KI#3.

4.	The NEF invokes the Nsf_Sensing_Subscribe service operation toward the SF, including the relevant information received from the AF in step 1.

5.	The SF may perform authorization based on the provided sensing service parameters. If authorization is not granted, the SF skips the subsequent sensing procedure (i.e. Steps 8-17) and responds to the NEF with an authorization failure in step 6. If authorization is successful, the SF responds to the NEF with an authorization success indication in step 6.

6.	The SF indicates whether the request has been authorised or rejected via Nsf_Sensing_Subscribe response.

7.	The NEF notifies the AF of the outcome of the subscription authorization-either acceptance or rejection-via the Nnef_Sensing_Subscribe Response.

8.	The SF performs the selection of the AMF(s) based on the information received from the NEF.

NOTE 4:	Based on the target of the sensing request, the SF either queries the UDM or the NRF for the selection of the AMF(s). The detailed description of the sensing service parameters, e.g. sensing service types, and associated sensing-related parameters is subject to the solution of KI#3.

9.	The SF invokes the Namf_NonUeN2InfoSubscribe request/response messages toward the selected AMF(s) and includes the information for the event reporting and the sensing-related parameters. This is done to subscribe for receiving sensing data.

10. The SF invokes an Namf_Communication_NonUeN2MessageTransfer request to the AMF. Inside this request, the SF includes an N2 Container indicating to the NG-RAN to start reporting the sensing data and containing the SF ID. Additionally, in this request the SF may specify TA(s) and/or Sensing Entities (NG-RAN node(s)). Based on local configuration and/or TA or NG-RAN node information as received from the SF, the AMF may request some or all NG-RAN nodes in the TA(s) to perform sensing.

11.	The AMF performs the discovery and selection of sensing entity/entities based on factors such as the sensing service parameters and the sensing capabilities supported by the candidate sensing entities. Alternatively, the SF may perform discovery and selection of sensing entity/entities. If the discovery and selection of sensing entity/entities is performed by SF, AMF transfers the message from SF to sensing entity/entities as in previous step 10.

NOTE 5:	The detailed procedures for discovery and selection of sensing entities will be addressed in the solution to KI#3.

12-13.	The AMF sends the corresponding N2 messages to all applicable NG-RAN nodes. Upon receiving a response(s) from the NG-RAN node(s), the AMF includes in the Namf_Communication_NonUeN2MessageTransfer response or Namf_Communication_N2InfoNotify request to the SF a list of N2 Containers containing response messages from the NG-RAN nodes and the corresponding NG-RAN node identifier(s). The AMF may aggregate the response messages it receives from the NG-RAN nodes.

NOTE 6:	Details regarding the sensing configuration parameters will be addressed in the solutions to KI#6.

14.	The sensing entity/entities carry out the sensing measurement and collect the sensing data.

15.	The sensing data is transmitted to the AMF.

NOTE 7:	The detailed handling of sensing measurement data and its reporting will be addressed in the solutions to KI#4.

16.	The AMF forwards the sensing data to the SF in the Namf_Communication_NonUeN2InfoNotify to the specific SF instance identified by the SF Instance ID as specified in step 10. The AMF may receive sensing data from several sensing entities and aggregate the sensing data report messages it receives.

17.	Upon receiving the sensing data, the SF processes the data to determine the sensing result. The SF may wait for the sensing data from several AMFs until processing the sensing data and sending the notification towards the AF.

18.	The SF sends the sensing result to the NEF via Nsf_Sensing_Notify.

19.	The NEF provides the sensing result to the AF via Nnef_Sensing_Notify.

NOTE 8:	Steps 14-19 may be performed either once, periodically, or triggered by specific events, depending on the event reporting parameters in the sensing service request from the AF.

NOTE 9:	The AF can invoke the Nnef_Sensing_Unsubscribe service operation toward the NEF to stop the sensing service in 5GS.

NOTE 10:	This solution is applicable as long as the N1/N2 interface will not be overloaded.

Editor's note:	The service message names are FFS and may be further specified as part of the detailed solution specification.

### 6.5.3	Impacts on services, entities and interfaces

NEF:

-	Supports selection of the Sensing Function.

-	Interaction with AF and Sensing Function for sensing authorization, invocation and revocation.

-	Supports exposure of the sensing results obtained from the SF to the AF.

Sensing Function (SF):

-	Supports selection of AMF(s) and sending the sensing request to the selected AMF(s).

-	Supports reception of sensing requests from the NEF.

-	Supports the reception of sensing data from the AMF(s).

-	Supports the processing of the sensing data and determination of the sensing result.

-	Supports exposure of the sensing results to the NEF.

AMF:

-	Supports interaction with the SF for the reception of sensing requests from the SF and for provisioning of the sensing data to the SF.

-	Supports selection of sensing entity/entities and interaction with the selected sensing entity/entities for requesting execution of sensing and for reception of sensing data.

AF:

-	Supports interaction with 5GC to request sensing services.

-	Supports the acquisition of sensing result from 5GC.

NG-RAN:

-	Supports Sensing Entity and the N2 messages to control the Sensing Entity and transmission of the sensing data.

# * * End of changes * * * *