3GPP TSG-SA WG2 Meeting #172	S2-2510573

Dallas, USA, November 17 – 21, 2025

Source:	Samsung

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the conclusion for KI#1.

# 1.	Discussion

Based on discussions at the SA2#171 meeting, the following two bullets need further clarficiation:

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF depending on SA5 feedback.

3)	New formulas taking idle energy consumption into account will be discussed in SA2#172.

It is proposed to clarify these points in the conclusion for KI#1:

- Support of Renewable Energy related enhancement should be decided during the normative work, based on the further confirmation with SA5 on the availability of the renewable energy related information via OAM

- Remove bullet 3), if the availablitity of idle energy consumption information is not confirmed with SA5.

# 2. Proposal

It is proposed to adopt the following changes to TR 23.700-67.

* * * Start of 1st Change  * * * *

# 8	Conclusions

### 8.1	Conclusion for KI#1

The following conclusions are agreed for normative work on KI#1.

1)	The following additional energy related information is exposed by EIF:

A)	The exposure of additional information in association to the exposure of energy consumption information at a desired granularity, specifically,

a)	This additional information, if requested by the consumers of the exposure services of the EIF, is provided together with the Energy Consumption information at the desired granularity and is the result of postprocessing of the information the EIF already has collected (i.e. no new information is required beyond what the EIF already has collected from OAM and SMF).

b)	energy consumption per bit: this is defined as the calculated energy consumption of the desired granularity divided by the measured data volume of the granularity used to calculate the energy consumption.

c)	“Energy consumption category” that is a scalar with multiple values that are determined by the EIF using PLMN-operator defined rules (e.g. 5 categories of percentile) and the rules and values associated to each category are not in scope of 3GPP. This category information is applicable to the same scope (e.g. regional, PLMN-wide) as the scope of the information that the EIF is collecting for the same granularity for other UEs that are e.g. associated to ongoing exposure subscriptions. The scope of the information an EIF collects is implementation and deployment specific.

B)	Per UE application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session with the following restriction:

a)	The application ranking related information exposed to the AF is only about the applications that the AF has permission/authorization with the applications and for which has requested the ranking. This permission/authorization is performed by the NEF. The EIF performs the ranking only of the ongoing applications subject to exposure.

NOTE 1:	The application ranking related information is exposed only to AF(s) related to those applications.

NOTE 2:	It is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

2)	The support of Renewable energy aspect for collecting, calculating and exposing energy consumption information per different granularities by EIF as defined in clause 6.8. will be decided during the normative work,  depending on SA5 confirmation of availability of node level (i.e., gNB and UPF) renewable energy information by OAM.

* * * End of Change * * * *