3GPP TSG-SA WG2 Meeting #172	S2-2510575

Dallas, USA, November 17 – 21, 2025

Source:	Samsung

Title:	Updated on interim principles for KI#3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes update on the conclusion for KI#3.

# 1.	Discussion

Based on discussions at the SA2#171 meeting, the followings are left for further discussion:

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow.

-	In case of multiple configured energy policies in network, the Energy Priority is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.2.3-1.

Table 7.2.3-1

| Information | Description |
| --- | --- |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

This contribution proposes further changes for conclusion on KI#3 issues.

# 2. Proposal

It is proposed to adopt the following changes to TR 23.700-67.

* * * Start of 1st Change  * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.1.3-1.

Table 7.1.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account Energy-related NF profile attributes parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The Energy Priority can be an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy.

* * * Start of 2nd Change  (all new text)* * * *

### 7.2.3	Topics for further consideration for KI#3

Void

* * * End of Change * * * *