3GPP TSG-WG SA2 Meeting #172	S2-2510695

17 – 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source: 	Lenovo

Title: 	KI#1: Update to the Interim Agreements

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for KI#1.

# 1	Discussion

The interim agreements to KI#3 about Sensing Entity and Sensing Function Discovery and (Re-)Selection have been documented in S2-2509659.

It has been agreed that the SF may contain the Sensing control functionality (SCF) and Sensing Processing functionality (SPF). It is assumed that the SF may contain at least one of those functionalities, i.e. either SCF, or SPF or both. In this case there can be one SF supporting SCF functionality which can use the services of another SF supporting SPF functionality. To enable such deployment this, the communication between SFs needs to be supported.

Proposal 1: it is proposed that there may be multiple SFs deployed (e.g. in various geographical areas) and one SF can use the services of another SF.

The support of roaming scenario for sensing service was not discussed in this study. Due to lack of time, it is proposed that the sensing service is not supported in roaming scenario.

Proposal 2: The sensing service is not supported in roaming scenario in this release.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.1.0.

# * * * * First change* * * *

### 7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain at least one of the Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

Principle 2: There may be multiple SFs deployed (e.g. in various geographical areas) and one SF can communicate and use one or more services of another SF.

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 3: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 4: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 6:	The sensing service is not supported in roaming scenario in this release.

# * * * * End of change* * * *