3GPP TSG-WG SA2 Meeting #172	S2-2510697

17 – 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source: 	Lenovo

Title: 	KI#3: Update to the Interim Agreements

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for KI#3.

# 1	Discussion

The interim agreements to KI#3 about Sensing Entity and Sensing Function Discovery and (Re-)Selection have been documented in S2-2509825.

Regrading the Sensing Function (SF) selection, it has been agreed that the NEF can use the NRF services to discover the SF. In addition, it is required that the SF registers with the NRF and the NF profile should store the target sensing area sensing capabilities (e.g. whether SF-GW functionality is supported).

-	The SF supported sensing area is the sum of the supported sensing area of the SEs which have established association with the SF.

-	In addition, the NF profile may include the supported sensing service type (e.g. motion monitoring, object detection and tracking).

Proposal 1: it is proposed to update the interim agreements by including the above point.

Regrading the RAN Sensing Entity (SE) section, it depends whether direct connectivity architecture between the SE and SF is assumed, or also indirect connectivity architecture via AMF needs to be supported. For any of the connectivity options, the SF can select the RAN SE using one of the following options:

-	The SF can store local configuration with available RAN SEs and their supported sensing area.

-	The RAN SE may establish an association with one or more SF(s) and during the association establishment, the RAN SE can indicate the supported sensing area.

Please note that it is up to RAN3 decision how RAN SE can select an SF to establish an association with.

Proposal 2: it is proposed to agree that the RAN SE establishes an association with one or more SFs. This is applicable for both direct and indirect connectivity. The RAN SE indicates its supported sensing area to the SF.

There may be multiple RAN SEs connected to a SF. The next question is how does the SF select the appropriate SE(s)? We consider that there may be RAN SEs with overlapping areas, e.g. RAN nodes operating in different frequency bands. When the SF receives a sensing service request, the SF may select and send the request all RAN SEs which are within the target sensing area. In order to avoid overlapping use of resources, the SF may perform a discovery procedure to discover the best suitable RAN SE for the specific sensing service request.

Proposal 3: The SF may perform a RAN SE discovery procedure for a specific sensing service request. The SF sends SE discovery request including a sensing service description and the SE replies with the confidence level to perform the sensing service.

Proposal 4: The SF selects one or more RAN SEs for a sensing service. The SF uses the following parameters for the SE selection: mapping of requested sensing area to the SE supported sensing area, confidence level of sensing service accuracy.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.1.0.

# * * * * First change* * * *

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile that includes supported sensing area and sensing capabilities:

-	The SF supported sensing area is the sum of the supported sensing area of the SEs which have established association with the SF.

-	The SF supported sensing capabilities may include the Sensing control functionality (SCF) and/or Sensing Processing functionality (SPF).

-	Sensing Function is selected based on at least its supported sensing area.

-	The SF functionality (e.g. SCF, SPF, etc. as specified in KI#1) may be geographically distributed so that one SF may select another SF for a specific sensing service.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	The Sensing Entities (SEs) establish an association with one or more SFs. During the association establishment the SE indicates its SE ID, supported sensing area and supported capabilities.

-	The SF may perform a SE discovery procedure for a specific sensing service request. The SF sends SE discovery request including a sensing service description and the SE replies with a pre-assessed confidence level to perform the sensing service.

-	The SF selects one or more SEs for a sensing service. The SF uses the following parameters for the SE selection: mapping of requested sensing area to the SE supported sensing area, pre-assessed confidence level of SE for sensing service accuracy.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# * * * * End of change* * * *