3GPP TSG-WG SA2 Meeting #172	S2-2510720

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#1.4] KI proposal for the essential services

Document for:	Approval

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: this pCR is to update the key issue proposal of WT#1.4, based on the SA2#171 discussion.

1. Introduction/Discussion

Based on the latest version of the WT#1.4 which was postponed in SA2#171 meeting (S2-2509781), it is proposed the following changes:

-	modified bullet P-3b to remove the reference to fallback but keep the goal of studying how to support voice services if the serving cell does not provide them.

-	removed NOTE 3 in key issue P as it is overlapping with NOTE 1

-	modified bullet Q-4a along the lines of bullet P-3b

-	removed part of the text in NOTE R-1 and created a new note to indicate that since location services need to be specified to support essential services, it would be more efficient to already pursue a general, unified location service architecture that could also be leveraged by other functionality discussed in other work tasks.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

# 5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in  5GS to support voice services as the starting point for discussion) – the following aspects will be studied:

1.	How a UE supporting 6G RAN selects and connects to a network for voice service.

2.	Whether and how to update the IMS specification to support an IPCAN in the 6GS.

3.	How the 6GS supports IMS based voice over the 6G access network

4.	How to support the voice continuity between 6G RAT and selected 3GPP RATs or between 6G RAT and selected non-3GPP Accesses.

P-3b.	Whether and how to support voice services in 5GS if they are not provided in the serving 6G cell.

P-3e.	How to support the compatibility between Policy Functions for 6G and the existing deployed AF with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh)

NOTE 1:	Any NTN specific aspects for voice will be studied as part of Key Issue related to WT#7

NOTE 2:	Interworking, migration and service continuity aspects need to be coordinated with WT#2.

NOTE 4: The existing IMS architecture is used as a starting point for the study.

NOTE 4a:	Coordination with RAN and SA4 will be required.

# * * * * Second change * * * *

5.Q. Key Issue #Q: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key issue P.

Additionally, this key issue will study:

1.	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

2.	The functionality required in the UE and the 6G system to establish emergency session via 6GS for emergency voice service.

3.	How to identify emergency services in 6GS in case of non UE detectable emergency call, such that the required  treatment for an emergency call is applied.

4.	How to support a UE in Limited Service state so that it can establish emergency session via 6GS.

Q-4a.	Whether and how to support emergency services in 5GS if not provided by the serving 6G cell.

# * * * * Third change * * * *

# 5.R. Key Issue #R: Location Services for 6G

This Key Issue will investigate how to support location service in 6G – the following aspects will be studied:

R-1.	The architecture to support location services in 6GS.

R-2.	How to support regulatory location services for emergency voice service in 6GS.

R-3.	How to support location service exposure, e.g. to 6GC NF, AF.

NOTE R-1:	The focus in this release of the specifications will be on the required location service capability to support emergency and other regulatory services.

NOTE R-x: A unified location service architecture should be pursued (using 5GS as the starting point for discussion), by considering the location service requirement in this WT and from other WT(s) if any.

NOTE R-1a: The scope will keep alignment with RAN.

# * * * * Fourth change * * * *

5.S. Key Issue #S: Messaging Services for 6G

For messaging service in 6G this Key Issue will study:

1.	How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [x1], TS 22.261 [x2], TS 22.105 [x3].

NOTE:	For SMS to Emergency centre support it is assumed that the outcome of 5G Advanced Rel-20 study will be adopted also for 6G.

# * * * * End of changes * * * *