SA WG2 Meeting #172	S2-2510738

17 – 21 November, 2025, Dallas, USA	(was S2-2509802)

Source:	[Xiaomi [Pen holder]], CATT

Title:	[WT#2, Migration and Interworking] Migration and Interworking WT&KI

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the WT and Key Issues relating to Migration and Interworking under WT#2 as defined in SP-250806.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

SP-250806 approved in SA#108 includes the scope of both migration and interworking as the following:

WT#2 Study migration and interworking:

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

2. Merging status

The following contributions are considered to the WT#2: Migration and Interworking for SA2#171 meeting.

| - | Study on Architecture for 6G System (FS_6G_ARC) - WT#2 | - | Docs:=15 |
| --- | --- | --- | --- |
| S2-2508219 | 23.801-01: [WT#2] Scope and key issue on Migration and Interworking for voice | NEC | WT update to include voice service supporting KI: 6GS interworking and migration for voice |
| S2-2508309 | 23.801-01: [WT#2] WT Scope Update and Key Issue on Migration and Interworking | OPPO | Add NOTE X to clarify WT#2 covers IWK aspects under WT#1. Service level interworking for AI and sensing is within WT#3, WT#4 Add KI part |
| S2-2508354 | 23.801-01: [WT#2, Migration and interworking] Migration and interworking | vivo | Update Scope of bullet2 Add NOTEX The interworking from 6GS to 5GS or from 6GS to EPS may be used to perform “fallback” procedure for IMS voice service when native IMS voice over PS session in 6GS is not support for an operator Add KI part |
| S2-2508473 | 23.801-01: [WT#2, Migration and Interworking] WT Scope and KI for migration and interworking | China Telecom | Add KI part |
| S2-2508555 | 23.801-01: [WT#2, Migration and Interworking] Scope and New Key Issue for migration and interworking | LG Electronics | Update bullet 3 to remove “whether” Add KI part |
| S2-2508806 | 23.801-01: [WT#2, 3GPP access Interworking KI part] Study of 3GPP access interworking. | [MediaTek Inc] NTT DOCOMO | Add KI part |
| S2-2508820 | 23.801-01: [WT#2] Interworking WT and Key issue proposal | ZTE | Add NOTEs: - The interworking with EPS will be coordinated and aligned with R - This key issue also considers interworking for other topics, e.g., UE–CN signalling, user plane, SBA, QoS, slicing, policy, etc. Coordination with other WTs is required. Add KI part |
| S2-2508899 | 23.801-01: [WT#2, Migration and Interworking WT scope] Work Task Scope of Migration and Interworking . | InterDigital Inc. | Add KI parts |
| S2-2508938 | 23.801-01: [WT#2] Migration and Interworking | Apple | Add NOTEs: - Among other tasks the study should identify a minimum migration path that can apply to any UE capable to support both 6GS and earlier 3GPP systems (e.g. 5GS). - The study scope covers general interworking and migration aspects for topics identified in other work tasks (e.g., UE–CN signalling, user plane, QoS, slicing, service continuity, inter-RAT mobility, FWA, localized service, cellular IoT, etc). Specific interworking requirements resulting from new 6GS feature/enhancement can be analysed and if needed further addressed in the corresponding dedicated WT. Add KI part |
| S2-2508946 | 23.801-01: WT#2, Migration and Interworking and study proposal | [Xiaomi (pen holder)], CATT | Clarify bullet 1 of WT scope: How to support migration to 6GS from 5GS |
| S2-2508988 | 23.801-01: [WT#2, Migration and Interworking] Study of Migration and Interworking | MediaTek Inc. | Update NOTE 2 to Additional migration options other than stand-alone, MRSS and inter-RAT mobility between NR and 6GR will be considered (if needed) in alignment with TSG RAN decision after September 2026 [ref: RP-252912] Remove NOTE 3 Consider bullet 1-3 as general aspects, add additional specific aspects to study: IMS voice over PS session, user plane management… Add KI part |
| S2-2509012 | 23.801-01: [WT#2, Interworking and Migration] Inter-System interworking and migration consideration | Huawei, HiSilicon | Update WT scope to include the support of interworking with non-3GPP access Add KI part |
| S2-2509076 | 23.801-01: [WT#2, Interworking] WT&KIs for Migration and Interworking | Xiaomi | Add NOTE X: Interworking aspects in other WT(s) are studied under this WT with a checkpoint to coordinate and align with the other WT(s) Add KI part |
| S2-2509159 | 23.801-01: [WT#2 Migration and Interworking] Scope and key issue on Migration and Interworking | Ofinno | Update Scope of bullet2 Update NOTE1 in WT scope Add KI part |
| S2-2509210 | 23.801-01: [WT#2] Work Task Scope clarification | Nokia, T-Mobile US, AT&T | Add NOTEs: - Only the general migration and interworking framework is within scope of this WT. Specific interworking requirements resulting from any new 6GS feature/enhancement from other WTs (e.g., Policy, QoS) will be addressed in the corresponding WTs but need to respect the overall IWK procedures as concluded in WT2 - Interworking between 5G PNI-NPN/SNPN, 5G Femto and 6G if need is within WT2 scope. - 5G-6G interworking scope does not preclude any access control mechanism used in each RAT. Add KI part |

This contribution gives proposals for WT & KIs based on S2-2508129.

**** First Change (all text new) ****

Annex A.X. Migration and Interworking Scope

Study migration and interworking, including

-	How to support migration to 6GS.

-	How to support interworking with 5GS.

-	Whether and how to support interworking with EPS.

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope and migration options will be coordinated and aligned with RAN.

NOTE 3:	It is assumed that interworking for roaming is within scope of this task.

NOTE 4:	This WT focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

**** Next Change ****

The work task will be copied here and proposed as key issue.

**** End of Changes ****