3GPP TSG-WG SA2 Meeting #172	S2-2510753

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: update conclusions

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract:

# 1. Introduction/Discussion

As previously discussed multiple times and presented in previous contributions, DNAI refers to the selection of the network behind the UPF, with the selection of the UPF being a consequence. DNAI is primarily for the edge and is not used for all UPFs, thus it is not considered valuable for the selection of the UP path.

In the previous meeting, we discussed the use of the schedule for EnergySaving and not EnergySaving based on the SA5 state. These two states are defined as follows:

- The EnergySaving state is when a cell or network function is powered down for energy-saving purposes, while the cell or NF remains controllable.

- The notEnergySaving state is when no energy-saving measures are in progress.

It was also noted that other possible information included in the NRF profile is possible. Currently, TS 29.510 defines the shutdown time in table 6.1.6.2.2-1 (Definition of type NFProfile).

| shutdownTime | DateTime | O | 0..1 | This IE may be present if the nfStatus is set to "UNDISCOVERABLE" due to scheduled shutdown. When present, it shall indicate the timestamp when the NF Instance is planned to be shut down. (NOTE 25) |  |
| --- | --- | --- | --- | --- | --- |

NOTE 25:	If the NF Instance wishes to change its nfStatus from “UNDISCOVERABLE” to another state, the NF shall remove the shutdownTime IE if this IE had been registered in the NF profile when being in the "UNDISCOVERABLE" state, by using one of the methods specified in clause 5.2.2.3.1A (NF Profile Complete replacement) and clause 5.2.2.3.1B (NF Profile Partial Update).

And the NF state in the NRF profile are defined in clause 6.1.6.3.7 as

Table 6.1.6.3.7-1: Enumeration NFStatus

| Enumeration value | Description |
| --- | --- |
| "REGISTERED" | The NF Instance is registered in NRF and can be discovered by other NFs. This status may result from new registration of an NF into NRF, or when, e.g. an NF comes out of "SUSPENDED" state after, e.g. re-issue of certificate (see clause 10.6 of 3GPP TS 33.310 [50]). |
| "SUSPENDED" | The NF Instance is registered in NRF but it is not operative and cannot be discovered by other NFs. This status may result from a NF Heart-Beat failure (see clause 5.2.2.3.2) or a NF failure, or the expiry/revocation of NF certificate (see clause 10.6 of 3GPP TS 33.310 [50]) and may trigger restoration procedures (see clause 6.2 of 3GPP TS 23.527 [27]). (NOTE 1) |
| "UNDISCOVERABLE" | The NF instance is registered in NRF, is operative but cannot be discovered by other NFs. This status may be set by the NF e.g. in shutting down scenarios where the NF is still able to process requests for existing resources or sessions but cannot accept new resource creation or session establishment. (NOTE 1) |
| "CANARY_RELEASE" | The NF instance is registered in NRF, is operative and can be discovered and selected by other NFs under certain conditions (see SelectionConditions, in clause 6.1.6.2.123). This status may be set by the NF e.g. in upgrade scenarios or during canary testing scenarios where the NF is able to process requests for new resource creation or session establishment under certain conditions (e.g. for a restricted set of users). (NOTE 2) |
| NOTE 1: An NF instance cannot be discovered by other NFs if the NF status is set to "SUSPENDED" or "UNDISCOVERABLE". NOTE 2: A discovered NF instance with NFStatus "CANARY_RELEASE" shall only be selected by an NF Service Consumer if the conditions included in the "selectionConditions" attribute of the NFProfile are evaluated to <true>; if such attribute is not included, the NF instance shall not be selected. |  |

Furthermore is defined in clause 5.2.2.6.2 (Notification from NRF in the same PLMN) that

NOTE 2:	When a subscribing entity receives a notification indicating the NF Instance (or the NF Service Instance) changes its NFStatus (or NFServiceStatus) to "UNDISCOVERABLE" together with a "shutdownTime" attribute, the subscribing entity may take this "shutdownTime" into account for corresponding actions, e.g. smoothly move the resources (e.g. PDU sessions) currently served by the to-be-shutdown NF Instance (or NF Service Instance) to any other candidate NF Instance (or NF Service Instance) within the indicated shutdown time period.

The UNDISCOVERABLE states indicates that the “NRF, is operative but cannot be discovered by other NFs”, hence it can not be selected. The states can be changed to UNDISCOVERABLE for example subject to operator's policy, if the NF Service Instance to be software-upgraded is not to be kept operative, it changes its status to "UNDISCOVERABLE", and potential consumers are notified.

For the specific energy state, the TS 28.310 defines the EnergySaving state and the NotEnergySaving State as follow:

energySaving state: state in which a cell or network function is powered-down for energy saving purposes.

Note 1: 	In energySaving state, the cell or network function is still controllable.

notEnergySaving state: state when no energy saving is in progress.

The energySaving state and the UNDISCOVERABLE state from point of view of UPF are similar since in both cases they indicate that the UPF, is operative but cannot be discovered by other NFs. However in SA5 28-serie of specifications and in CT 29-serie of specifications whether a link between the 2 states exist is not clear since SA5 specification do not includes “UNDISCOVERABLE state” and CT specification does not include “energySaving state” as searched by authors of this paper.

Option a)

Conclusion #1: Considering that :

- The UNDISCOVERABLE states in conjunction with the shutdownTime covers the same scenario of scheduling the power-down due to a EnergySaving decision and other criteria as for maintenance, load, etc.

- The current stage 3 specification in TS 23.510 addresses the scenario that when the state in NRF is “"UNDISCOVERABLE" together with a "shutdownTime" the subscribing NF “may take this "shutdownTime" into account for corresponding actions, e.g. smoothly move the resources (e.g. PDU sessions) currently served by the to-be-shutdown NF Instance (or NF Service Instance) to any other candidate NF Instance (or NF Service Instance) within the indicated shutdown time period”

- SA5 defines the EnergySaving state and the procedure for entering in Energy Saving states

- The EnergySaving state and UNDISCOVERABLE are similar for a NF.

Therefore the following conclusions are proposed:

- The following behaviour may be used to address the schenario of energy saving :

  - OAM decides to shut down an NF in the near future and then informs the NF about it. In consequence, NF is updating it's profile with UNDISCOVERABLE + shutdowntime. This blocks any future NF selection but also informs the others that the NF is still operating for ongoing services until the shutdowntime is reached. When shutdowntime is reached, the NF stops operating normally and enters the energySaving state.

  - Therefore is not need to define new states and new scheduling informaiton, but only clarify that current existing UNDISCOVERABLE and shutdownTime may be triggered by EnergySaving decision per spec

- Capture with a note that during normative phase this will be verify with SA5/CT

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.1.3-1.

Table 7.1.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority. indicates the priority relative to other NFs of the same type. from an energy standpoint. The Energy priority is Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF profile can be updated to UNDISCOVERABLE states with associated indication of shutdownTime as specified in TS 29.510 R19 [x[. The shutdowntime when is present in NRF profile indicates the timestamp when the NF Instance is planned to be shut down. As clarified in clause 5.2.2.6.2 of TS 29.510 the “subscribing entity may take this "shutdownTime" into account for corresponding actions, e.g. smoothly move the resources (e.g. PDU sessions) currently served by the to-be-shutdown NF Instance (or NF Service Instance) to any other candidate NF Instance (or NF Service Instance) within the indicated shutdown time period” (reference Note 2). Therefore the current R19 procedure for UPF reselection can be used also for energySaving and no additional specification is required. The OAM may decide to shut down an NF in the near future sinc emove to EnergySaving State starting the ES procedure define din TS 28.510. The OAM informs the NF about it. In consequence, NF is updating it's profile with UNDISCOVERABLE plys shutdowntime. This blocks any future NF selection but also informs the others that the NF is still operating for ongoing services until the shutdowntime is reached. When shutdowntime is reached, the NF stops operating normally and enters the energySaving state.

NOTE 1: The above behaviour may be already supported via Rel-19 specification, but during normative phase will be further verify with CT and SA5 whether is possible since the aboev behavious if not explicitly metioned in both SA5 28-series of specificaiton and CT 29-series of specification.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

# * * * * Second change * * * *

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	In case of multiple configured energy policies in network, the Energy Priority is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

-

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

# * * * * Third change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

The interim agreements in clause 7.1.3 are considered as conclusion for KI#3.

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *