SA WG2 Meeting #172	S2-2510758

17 - 21 November, 2025, Dallas, USA	(revision of S2-2509785)

Source:	China Telecom

Title:	[WT#1.2, Enhanced User Plane] Scope and Key Issue for Enhanced User Plane

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Enhanced User plane revision with updates to work task scope and key issue.

- Justification:

In 5GC some observation on the inefficient traffic handling in UP architecture can be found. For example, to support selective traffic routing to different services in a single PDU Session, single PDU Session with multiple PDU Session Anchors is designed. But this UP architecture is inefficient as the all traffic from the each PDU Session Anchor must be aggregated to an central UPF. This means a traffic must be handled at least twice, both by the targeting anchor UPF and aggregating UPF, when there are two PDU Session Anchors. The following is a demonstration of this issue with the usage of an UL Classifier for a PDU Session.

The red traffic is handled by PSA 1 and UL CL, and the blue traffic is handled by PSA 2 and UL CL. Even if in implementation the UL CL and PSA 2 can be merged in a single UPF, the red traffic is still handled by this UPF and PSA 1.

It is desired to solve the issue of inefficient traffic handling of 5G in UP architecture of 6G CN.

**** First Change ****

Annex A.X. WT #1.2 User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

- Whether and how to enhance user plane flexibility (by user plane function (re)selection)  and traffic handling efficiency for different service requirements, session continuity and mobility with consideration of path performance and capability between access network and data network.

- Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE z: The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

**** Second Change ****

5.X. Key Issue #X: User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

- Whether and how to enhance user plane flexibility (by user plane function (re)selection) and traffic handling efficiency for different service requirements, session continuity and mobility with consideration of path performance and capability between access network and data network.

- Whether and how to enhance resilience, scalability, and high availability of User Plane Function.

NOTE z: The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

**** End of Changes ****