3GPP TSG-SA2 Meeting #172 	S2-2510782

17 – 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

# Source:	CATT

# Title:	New WID on QoS Notification Control with Available Bit Rate

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	QoS Notification Control with Available Bit Rate

Acronym:	TEI20_QNCwABR

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

3	Justification

In Release-19, the RAN can measure and report the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles as a QoS Monitoring function as defined in TS23.501, clause 5.37.4 Network Exposure of 5GS information and clause 5.45.6 Available bitrate monitoring. However, if the 5G network does support Network Exposure of 5GS information as defined in clause 5.37.4, the RAN can still provide such UL and/or DL available bitrate for a GBR QoS Flow in the QoS Notification Control.

Therefore, it is proposed to enhance QoS Notification Control without Alternative QoS Profiles to report the UL and/or DL available bitrate for the GBR QoS Flow.

4	Objective

The objective is to enhance QoS Notification Control without Alternative QoS Profiles, stated as the following:

-	Support of providing the UL and/or DL available bitrate information in the QoS Notification Control without Alternative QoS Profiles.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Update the clause 5.7.2.4.1a to support of providing the UL and/or DL available bitrate information in the QoS Notification Control without Alternative QoS Profile. | TBD | This will be handled as “TEI20”. |
| TS 23.503 | Update clause 6.1.3.5 and clause 6.1.3.22 to include the indication to report the UL and/or DL available bitrate information in the QNC in the AF request messages and PCC rules. | TBD | This will be handled as “TEI20”. |

This TEI20_WID requires 0.5 TU.

6	Work item Rapporteur(s)

Chunshan Xiong, CATT, chunshan.xiong@cictmobile.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

CATT