3GPP TSG-WG SA2 Meeting #171	S2-2509304

# Wuhan, China, Oct 13 – 17, 2025	 	(revision of S2-2508481 was 6968)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1508 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment and correction on VFL training procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For VFL training in clause 6.2H.2.3.1, VFL server might send estimated VFL training completion time optionally in step 0, but “estimated VFL training completion time” is missing in corresponding service operations. Besides, “estimated VFL training completion time” are not aligned in step 0c and step 0d in clause 6.2H.2.3.1. - The parameter “ML model accuracy monitoring check flag” in clause 6.2H.3 is not aligned with “ML model accuracy monitoring flag” in clause 6.2H.2.3. - No “training iteration round ID” definition in VFL, thus, it should be aligned as “training iteration number” in clause 6.2H.2.3.1. - In step 5, it is described that “A client may indicate in message for a request to leave the VFL.”, but no related indication in VFL training service to request leave. - In Step 7, it is described that the “VFL correlation ID” could be provided by VFL server to AnLF directly or via MTLF then to AnLF for requesting VFL inference. But “VFL correlation ID” is missing in corresponding services. - Correction on other typos. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “estimated VFL training completion time” in clause 6.2H.4 and clause 7.5.2 and also align this parameter name in step 0c and step 0d. - Alignment on parameter “ML model accuracy monitoring check flag” in clause 6.2H.3 with the description in clause 6.2H.2.3. - Change “training iteration round ID” to “training iteration number” in clause 6.2H.2.3.1. - Add “leave request” in clause 6.2H.3. - Add “VFL correlation ID” in clause 6.2H.4 and clause 7.5.2. - Correction on other typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment and misleading content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1,6.2H.3, 6.2H.4, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.2H.2.3	Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

NOTE 1:	In the case when AF is server the clients can only be NWDAFs.

0.	[OPTIONAL] VFL training is triggered in the VFL server for the following cases:

-	Based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

-	Triggered by either Analytics consumer, AnLF via ML model provisioning or NWDAF containing MTLF (VFL Server is a trusted AF):

Step 0a (case A). The NWDAF containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe.

Step 0b. (case A). The NWDAF containing MTLF decides to use VFL for the subscription and realizes it cannot be VFL server but wants an AF to act as VFL server, the NWDAF containing MTLF discovers an AF as VFL server according to 6.2H.2.1, and may send a request to the VFL server AF using Naf_Training_Subscribe including Analytics ID, optionally Notification target address, the AF as VFL server starts VFL Training according to step 1. The NWDAF containing MTLF may either send in the service subscription response that no ML model identifier will be made available due to VFL model to be used and that training is ongoing, alternatively the NWDAF containing the MTLF just indicates that no ML model identifier will be made available and training is completed together with the NF ID of the VFL server and the NWDAF containing AnLF may request the inference output using the requested Analytics ID as described in step 1 of clause 6.2H.2.4.1. If the received Nnwdaf_MLModelProvision_Subscribe service contains the time when ML Model is needed, the MTLF may forward it in Naf_Training_Subscribe. If VFL server cannot meet the time when ML Model is needed it may not to start the training.

NOTE 2:	The AF may have registered in NRF for the particular Analytics ID that it can perform Inference for it. In this case no training will be performed.

-	Triggered by NWDAF containing AnLF (VFL Server is a trusted AF):

Step 0c. (case B). Same as step 0a. Alternatively, the NWDAF containing AnLF knows by configuration that the Analytics ID is trained using VFL, then the AnLF discovers the AF VFL Server and requests the AF VFL Server to train the ML Model.

If a ML model provisioning subscription request from an NWDAF containing AnLF is received in step 0c, the MTLF sends in the service subscription response an indication that no ML model is available due to VFL model to be used and that the subscription is terminated (i.e. as new cause code) and may also provide VFL server ID. The NWDAF containing AnLF can directly send the Analytics Subscription requests to the NWDAF containing MTLF as VFL Server or the VFL server indicated by the VFL server ID as described in step 1 of clause 6.2H.2.4.1. The NWDAF containing AnLF can also discover an AF VFL Server from NRF that supports the Analytics ID.

The AnLF then sends request to AF VFL server to train a ML Model with the Analytics ID as described in step 1 of clause 6.2H.2.4.1. The AnLF may also include the time when ML Model is needed in the training request. The VFL server AF estimates the VFL training completion time using Analytics ID and compare it with the time when ML Model is needed. If the VFL training completion time is no later than the time when ML Model is needed, the AF may start VFL training, indicate to the AnLF that training is ongoing with VFL training completion time and will subsequently indicate that the training is complete to the NWDAF containing AnLF, optionally, together with the VFL correlation ID. If not, the VFL server AF indicates to the MTLF that the time when ML Model is needed cannot be fulfilled together with estimated VFL training completion time. Based on the message sent by VFL server, the NWDAF containing AnLF can send a request to perform Inference using the requested Analytics ID and optionally the VFL correlation ID to retrieve an VFL Inference output from as described in step 1 of clause 6.2H.2.4.1. The AnLF may send VFL inference request to VFL server AF after it receives the Analytics request from consumer NF.

-	Triggered by NWDAF containing AnLF via ML model provisioning (VFL Server is a NWDAF):

Step 0d. (case C). If the NWDAF containing AnLF wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe. If the discovered NWDAF containing MTLF does not have a complete model, it sends a response to the AnLF that no complete Model is available due to VFL used and that the subscription is terminated (i.e. as new cause code), and may also provide VFL server ID and/or an estimated VFL training completion time when the training is complete. The NWDAF containing AnLF can directly send the Analytics Subscription requests to the NWDAF containing MTLF as VFL Server or the VFL server indicated by the VFL server ID as described in step 1 of clause 6.2H.2.4.1.

The NWDAF containing AnLF can also discover new VFL Server from NRF that supports the Analytics ID, then and can send Analytics Subscription requests to that VFL Server as described in step 1 of clause 6.2H.2.4.1.

-	Triggered by analytics consumer. (VFL Server is either an AF or an NWDAF):

Step 0e. (case D). NF consumer needs analytics output for a specific Analytics ID, it discovers an NWDAF from NRF to provide the analytics ID. If the discovered NWDAF is a VFL server which supports the analytics ID, then to generate the analytics output via VFL inference as defined in clause 6.2H.2.4, the VFL server may trigger VFL training if no corresponding VFL training has been performed.

For all cases, if the time when Analytics information includes in the Analytics request, the VFL server estimate the completion time of VFL training and/or VFL inference using Analytics ID received and compare it with the time when Analytics information is needed. If the completion time is no later than the time when Analytics information is needed, the VFL server starts VFL training and/or VFL inference.

NOTE 3:	If the VFL training is complete, then only the time of VFL inference will be estimated.

NOTE 4:	VFL server can also decide to initiate VFL training based on operator policy or internal configuration.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients. The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase and the VFL training iteration number set to 0, and a Notification Correlation ID. Optionally, the VFL Server includes parameters according to clause 6.2H.3.

If the VFL procedure continues in subsequent iterations, the VFL server sends an update request for a new VFL training iteration containing an incremented a VFL training iteration number and intermediate model training information to each of the VFL clients for next round of VFL training. The VFL Server, based on internal logic, may provide checkpoint information according to clause 6.2H.3.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL client, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server. VFL client(s) may also report a delta from the initial list of samples according to clause 6.2H.3. Based on internal logic, the VFL server may indicate if the training has to resume from a previous checkpoint by using its training iteration number.

NOTE 5:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server. The Notification Correlation ID and a VFL training iteration number. A client may indicate in message for a request to leave the VFL. Each client may report local ML model accuracy monitoring information if ML model accuracy monitoring flag is received in step 2.

NOTE 6:	If the VFL Server deems the training can continue, it responds back by informing the FL Client to cease the ML Model training by performing step 8 for this client.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients and/or the model of the VFL Server. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

The VFL server may locally compute contribution weights for each VFL client participating in the VFL process and store them for subsequent use, e.g. during inference. How the VFL computes the VFL contribution weights is up to implementation.

The VFL server may also decide to compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label at any time of VFL training. When determining to generate the ML model metric, the VFL server may include an indication to indicate to the VFL clients to use the sample ID of the dataset for accuracy monitoring to calculate intermediate training results in Nnwdaf_VFLTraining_Subscribe service operation. Corresponding to the sample IDs for accuracy monitoring, each VFL client collects local data as input data for local ML model trained in step 4, then the VFL client generates the intermediate training result based on the trained local ML model and the input data, and it provides the intermediate training result to VFL server. The VFL server computes the ML model metric (e.g. VFL accuracy) based on the received intermediate training result and the label, which are both corresponding to the sample IDs for accuracy monitoring. The VFL server may optionally take account the local ML model accuracy monitoring information received from VFL client(s) when computing global ML Model metric.

7.	[Optional] The VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached and/or global ML model metric is stable) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Step 7b, 7c and 7f. Based on the consumer request, the VFL server notifies VFL status report to the consumer, to update the metric value to the consumer periodically (e.g. a certain number of training rounds or at fixed periods) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).The VFL status report may include global model metric (e.g. ML model accuracy). When training is completed a VFL server will add training is completed to the Notification target address (i.e. NWDAF containing MTLF or AnLF). An AF VFL server may include VFL correlation ID.

If NWDAF including MTLF has forwarded a subscription request to VFL server AF in step 0b, and it did not add Notification target set to the first NWDAF containing AnLF, it adds the AF ID, in the Notification to the first NWDAF containing AnLF. When training is completed, it indicates training is completed, and if AF VFL server added the VFL correlation ID it may forward it to the NWDAF containing AnLF.

NOTE 7	The AnLF can later include the VFL correlation ID in inference request to VFL server AF.

Steps 7d, 7e, 7g and 7i. The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to either unsubscribe or continue the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information (including the NF ID of the VFL Client), which may be used to determine associated VFL client in the VFL inference.

Each VFL client updates local ML model based on the intermediate model training information, if received and stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 8:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

NOTE 9:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# * * * * 2rd change * * * *

### 6.2H.3	Contents of ML Model VFL Training services for Vertical Federated Learning

The consumers of the ML Model VFL training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnefe_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	At start of Training phase VFL server sends VFL Correlation ID: identifies a VFL process to be executed among the candidate VFL participants.

-	[OPTIONAL] VFL Interoperability Information that indicates the intermediate model training information that the VFL Server supports and asks the client to verify its support for (e.g. activations, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this Release.

-	[OPTIONAL] In training phase, Maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result).

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	In training, the Intermediate model training information includes what the Server sends to each client which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	[OPTIONAL] ML model accuracy monitoring flag: request local ML model accuracy monitoring information which assists VFL server to perform ML model accuracy monitoring.

-	[OPTIONAL] Feature ID(s) as described in preparation phase and at start of training phase. See definition of Feature ID below.

-	[OPTIONAL] At start of Training phase, Use case context: indicates the context of use of ML Model.

-	In training, VFL training iteration number.

-	In the preparation phase, the initial list of samples selected by the VFL Server.

-	[OPTIONAL] At start of Training phase, sample ID of the dataset for accuracy monitoring are selected from the sample ID list decided in the preparation phase, identifying the data to be used for generating intermediate training result by VFL client and supporting the VFL Server to determine the VFL ML model accuracy.

NOTE 1:	How the VFL server determine the sample IDs of dataset for accuracy monitoring is up to implementation.

-	[OPTIONAL] In later steps in training, the VFL server may provide delta from the sample list sent in first step of training procedure to indicate which samples aligned in the preparation phase will not be part of the rest of training procedure.

NOTE 2:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] Checkpoint information: indicates whether model status at the current iteration should be saved as a training checkpoint.

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	In the preparation phase, the list of samples accepted by the VFL client.

-	[OPTIONAL] In training the VFL client may provide delta from the sample list received in first step of training procedure to indicate which samples will not be part of the rest of training procedure.

NOTE 3:	The delta list can only remove samples from the initial sample list.

-	[OPTIONAL] As described in preparation phase, the Feature ID(s) that the VFL client supports. A Feature ID indicates what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

-	[OPTIONAL] VFL Interoperability Information that indicates the Intermediate model training information that the client supports in response to what the server sent in the subscription request.

-	In training, the Intermediate training result includes what the Client sends to the server which contains what has been agreed in the preparation phase. The content is not standardized in this Release.

-	[OPTIONAL] Local ML model accuracy monitoring information.

NOTE 3:	Local ML model accuracy monitoring information can be used to assist the model convergence and calculation of accuracy and is not specified in this release of the specification.

- [OPTIONAL] Leave Request to indicate the client will leave the VFL training.

# * * * * 3rd change * * * *

### 6.2H.4	Contents of ML Model Training services for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Naf_Training service or in Nnef_Training service as listed below. The NWDAF decides which of the optional parameters it forwards when it has received an ML Model provisioning subscription request.

-	Analytics ID: identifies the analytics for which the ML Model is used.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML Model.

-	[OPTIONAL] ML Model Filter Information: indicates the applicable conditions of the trained ML model and enables the consumer to select which ML Model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML Model is requested, e.g. specific UEs i.e. a list of SUPIs, a group of UEs i.e. a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UE(s) in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs i.e. a list of Internal-Group-Ids.

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML Model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by the consumer (e.g. NWDAF containing AnLF) during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 1:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used, indicated as a list of NF instance (or NF set) identifiers.

-	ML Model Reporting Information for the subscription, parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications with this subscription.

-	[OPTIONAL] Accuracy level of Interest.

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML Model(s).

-	[OPTIONAL] ML Model Monitoring Information:

-	desired ML Model metric.

NOTE 2:	In this Release, only "ML Model Accuracy" is defined as ML Model metric.

-	[OPTIONAL] For subscription ML Model monitoring reporting mode: such as Accuracy reporting interval or pre-determined status. Depending on the reporting mode, the AF reports the ML Model accuracy either periodically or when the ML Model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer (as a kind of pre-determined status). It also can be used as an indication that the AF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by AF to retrain or reprovision of the ML Model.

The AF provides to the consumer the output information as listed below:

-	For Notifications, the Notification Correlation Information.

-	For the Analytics ID requested by the service consumer, the following information:

-	Indication whether training is ongoing when e.g. accuracy report is provided on request from consumer, or that training is done, and thereby Analytics or Inference can be requested to NF identified by ML Model provider information.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 3:	Spatial validity and Validity period are determined by internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML Model representative ratio: indicating the percentage of UE(s) in the group whose data is used in the ML Model training when the Target of ML Model Reporting is a group of UE(s).

-	[OPTIONAL] ML Model Accuracy Information: indicates the accuracy of the ML Model if related ML Model Monitoring Information was provided, which includes:

-	the metric value of the ML Model.

-  [OPTIONAL] Estimated VFL training completion time: indicates when the training is estimated to be complete.

-  [OPTIONAL] VFL correlation ID.

# * * * * 4th change * * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe

Description: Subscribes to NWDAF ML Model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2 (conditional, required if the ML model is requested for analytics generation), or an indication that a model for LMF-based AI/ML Positioning is requested (conditional, required if the ML model is requested for UE location calculation), Notification Target Address (+ Notification Correlation ID).

NOTE:	Whether the indication of requesting a model for LMF-based AI/ML Positioning is implemented in a new information element is up to stage 3.

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model subscription).

-	For each Analytics ID or an indication that a model for LMF-based AI/ML Positioning is requested, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-	NF consumer information, ML Model Reporting Information, Time when model is needed, ML Model Monitoring Information (including e.g. desired ML Model metric, ML Model monitoring reporting mode, ML Model Accuracy Threshold, DataSetTag and ADRF ID, ML Model identifier), Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy). When the subscription is not accepted, an error response with cause code (e.g. no ML model is available due to VFL model to be used and that the subscription is terminated).

Outputs, Optional: Indication that no ML model identifier will be made available, Indication that training is ongoing or NF ID of a VFL server, estimated VFL training completion time, VFL correlation ID.

# * * * * End of changes * * * *