SA WG2 Meeting #171																					S2-2509711

13 – 17 October 2025, Wuhan, China	(was S2-2508123)

Source:	Ericsson (contact point)

Title:	[WT#1.2, Policy] Policy Framework architecture in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides our view on the issues that require further consideration and proposes some clarifications on those aspects that were considered more stable for the scope of the WT#1.2 Policy. It also proposes a Key Issue description.

# 1. Discussion

This document merges the content of the WT proposals for the Policy Framework architecture in 6G as listed in the table below. The discussion on the WT takes place first, when agreed we take the discussion on the KI(s).

|  | Policy Framework |  |
| --- | --- | --- |
| S2-2508348 | 23.801-01: [WT#1.2, Policy framework] 6G policy framework | vivo |
| S2-2508349 | 23.801-01: [WT#1.2, Policy framework] UE input for AM policy | vivo |
| S2-2508361 | 23.801-01: [WT#1.2, Policy] Policy Framework architecture in 6G | NTT DOCOMO |
| S2-2508367 | 23.801-01: [WT#1.2, Policy] Policy Framework architecture in 6G. | Nokia |
| S2-2508572 | 23.801-01: WT and KI for Policy Framework in 6G | Huawei, HiSilicon |
| S2-2508848 | 23.801-01: [WT#1.2, Policy] WT & KI for Policy Framework architecture in 6G | Ericsson |
| S2-2508945 | 23.801-01: [WT#1.2, Policy] Policy Framework architecture in 6G | [Ericsson (contact point)], CATT |
| S2-2509071 | 23.801-01: [WT#1.2, Policy] Policy Framework architecture in 6G | [Ericsson (contact point)], Xiaomi |
| S2-2509161 | 23.801-01: WT#1.2, Policy] Scope and key issue on Policy Framework in 6G | Ofinno |
| S2-2509200 | 23.801-01: [WT#1.2, Policy] Policy Framework architecture in 6G | Qualcomm Incorporated |
| S2-2509212 | 23.801-01: [WT#1.2, Policy] Addition of user specific policies based on user identity | Philips International B.V. |
| S2-2509257 | 23.801-01: Work Task update for WT#1.2 Policy Framework | Samsung Electronics France SA |

The following topics need further discussion:

- QC proposal to use the 5GS Policy Framework as the baseline;

  - Proposal from the contact point is to use policy framework as baseline for all topics that are considered simplifications of the existing policy framework. For other topics, it needs further discussion.

    - Support: QC, Ericsson, Xiaomi.

    - Not support: Huawei.

      - Second proposal from the contact point is not to take that discussion during solutions instead. As such, no assumptions are taking.

- Vivo, Huawei proposes to study how to support the 5GS procedures for AF influence on policy control can be requested by the UE. Ericsson proposes to add a relation with network exposure WT#1.2 that defines how to support network capabilities to the UE.

  - No consensus on what is to be studied yet. The core network exposes capabilities to influence policy control by an AF, so the question is what the goal is:

    - The network exposed capabilities to influence policies in the Core Network via an AF is used by the UE. UE->AF->CN.

    - The network exposed capabilities to influence policies in the Core Network via an AF are extended to add UE as consumer.

    - The network exposed capabilities to influence policies in the Core Network via an AF are not reused but a new procedure is defined.

      - Offline discussion indicates that goal is to study point 2 above.

- NTT DOCOMO proposal to support Rx interface for backward compatibility. There is a counter proposal in WT#1.4 by Ericsson, BT S2-2508885.

  - So far, there is an agreement not to include this proposal here.

- Philips International B.V. proposes to define user policies based on user identities that seems to have a large scope that policies.

  - No consensus yet:

    - QC, Ericsson, Nokia. it would be a system level decision on the UE/user profile management.

    - Huawei is positive to study policy aspects in this WT.

- Ericsson proposes to take the discussion on whether the user inputs are considered for a policy decision as part of WT#3.1 on AI for network, given that the user input is understood to be “intents”.

  - There is consensus that if user input refers to intents then it will be studied in WT#3.

  - There is no consensus of what other user inputs are needed:

    - Nokia, Offino propose to add user input to a policy decision, not related to intents.

    - QC, Ericsson asks for a more clear problem description

- CATT proposes to study to support the new functionalities in 6G.e.g. the new functionalities of the new 6G services. The contact point suggestion is to follow what the rapporteurs stated to cover that in each new 6G feature instead, as such NOTE 5 is also removed.

  - NOTE 5 re-instantiated.

**** First Change ****

# Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

To support the Policy Framework in 6GS, the policy and charging control Framework defined in TS 23.503 for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

- Whether and how to simplify the policy control framework for 6G, including:

a)   whether and how to reduce the number of services or interactions to efficiently provide policies to the different policy enforcement points.

b)  whether and how to harmonize the UE Policy provisioning procedures for UE Policies in 6GS.

2.	 Whether and how to consider user preferences (including local configurations of UE) when applying UE related policies in 6G.

3.	 Whether and how the 6G CN can assist the UE with the mapping from application traffic to a user plane connection in 6G via session management policy control procedures.

NOTE: user plane connection in this WT refers to the PDU Session in 5G terminology.

- Whether and how to evolve the External Parameter Provisioning Framework aspects of policy control for 6G.

NOTE: General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure.

- Whether and how policy control information is provided to UE application and/or the AF (e.g. to notify the UE Application or the AF that policy and charging control are changed).

- Whether and what parameters that the UE can provide as input for PCC decision, e.g. QoS control to the 6G CN, in case AF input for QoS is not available or no AF exists.

NOTE:  The mechanism for the UE to provide the input for PCC decisions to the 6G CN is studied in WT.1#2 Network exposure and the WT1.2 QoS. The AF input defined in 23.502/503 can be treated as start point for discussion.

- Whether, how and what to support for policy and charging control in the different roaming scenarios in 6G.

NOTE:	This WT takes energy efficiency and consumption aspects into account for policy decisions.

NOTE:  	This WT has IWK aspects that are scoped under WT#2.

NOTE: 	This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs.

NOTE:	This WT depends on the overall 6G roaming architecture.

*** End of Changes ****