SA WG2 Meeting #172	S2-2509712_draft_v05

13 – 17 October, 2025, Wuhan, China                             (Revision of S2-2509511, 8535, 8136)

Source:	InterDigital Inc., ZTE, Xiaomi

Title:	[WT#1.2, Network Exposure Framework] Network Exposure Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #1.2 on the topic of Network Exposure Framework.

# 1. Source Documents

This paper is a continuation of the merger discussion that ended with S2-2508136 at SA2 #171.

This revision considers the following submissions for SA2 #171.

- S2-2508535 (InterDigital, ZTE) (Baseline)

- S2-2508883 (Ericsson)

- S2-2508351 (vivo)

- S2-2508573 (Huawei)

- S2-2508601 (Nokia)

- S2-2508947 (CATT)

- S2-2509070 (Xiaomi)

- S2-2509213 (Qualcomm)

Items that need further discussion.

- Bullet #2 (How to support network capabilities exposure to UE)

  - At least two companies propose to delete the part of the work task that covers how to support network capabilities exposure to UE (Huawei, Nokia).

- Bullet #4 (Intent-Based Exposure)

  - At least two companies propose to delete the part of the work task that covers intent-based exposure (vivo, Qualcomm).

- Proposal from Qualcomm to delete most of the work task and add a note that says “This work task depends on all other work tasks. Work can only start once other work tasks have made sufficient progress so that the exposure requirements of other work tasks can be derived. The work is not assumed to start before SA2#175 (Q2/2026)”

- Proposal from CATT to add a bullet that says “How to support the UE and or DN exposure information to 6GS”

- A proposal from Ericsson to change the “in-band” to “on-path”.

**** First Change – All New Text ****

# Annex A.X. WT#1.2 Network Exposure Framework

This work task is to study whether and how to support a network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

- The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. What new network capabilities are exposed in 6G is dependent on other work tasks.

- The second category is for existing capabilities. Existing capabilities refer to capabilities that are exposed in the 5G network. What existing capabilities are exposed in 6G is dependent on SA2 decisions and may also be dependent on other work tasks.

Since this work task depends on other work tasks, time will not be allocated for solutions for this work task by SA2 in Q1 of 2026.

Aspects that can be studied are:

- How to support a common network exposure framework for the exposure of network capabilities to an AF.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

  - Whether and how to support the co-existence with the 5G exposure frameworks.

NOTE 0:	The Common Network Exposure Framework in SA2 does not duplicate with the SA WG6 CAPIF Framework and, depending on SA WG6 decisions, CAPIF may be reused.

NOTE 1:	Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters.

  - Whether and how to support enhanced privacy protection, user consent, data anonymization, and service authorization mechanisms.

NOTE 1a:	Authorization, Privacy protection and user consent aspects require coordination with SA WG3.

  - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

NOTE 3: 	The scope of this work task is to specify the common exposure framework, including possible impact to NEF. The exposure framework that is developed in this work task should be adaptable to and suitable for other work tasks (e.g. computing and sensing).

NOTE 4:	What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks.

- How to support network capabilities exposure to UE.

  - Whether, and how, and what network capabilities should be exposed to a consumer on the UE so that network capabilities are accessible Application Clients on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context. The network capability exposure  features that are supported in 5GC are to be treated as a starting point (e.g. QoS Monitoring Exposure, Setting up an AF session with required QoS procedure, and UE ID retrieval).

NOTE 4a:	Studying what network capabilities should be exposed means that identifying which capabilities will be beneficial for exposure to the UE. The network capabilities exposure defined in TS 23.502 and TS 23.503 are used as a starting point for what can be exposed, and this can be revisited based on the output of NWM on 5G features

NOTE 4b:	Network capabilities will not be exposed to the UE via NAS.

  - Whether and Hhow to easily authorize the UE to consumers of network capabilities when the consumer is resident on the UE.

NOTE 4d:	Authorization is in the remit of SA WG3.

- How to support on-path Capability Exposure to Application Endpoints.

  - Whether, how, and what capabilities to expose on-path between application endpoints (i.e. in the UE or AS) and the 6G network in an operator-controlled manner.

  - Whether the exposure/reporting path of an exposed capability can be optimized (e.g. on-path) and if so, how.

  - How to discover, authorize and activate on-path exposure services offered by the network and whether and how to integrate on-path exposure into the common exposure framework.

NOTE 4f:	Authorization aspects require coordination with SA WG3.

NOTE 4c:	"on-path" refers to exchanging the information via a PDU Session.

NOTE 5:	On-path information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 6:	This work task will cover how capabilities and information are exposed on-path. What capabilities and information are exposed on-path and how they will be exposed will be coordinated with other work tasks (e.g. the work that is done under the User Plane and QoS aspects of Work Task 1.2).

NOTE 7:	On-path information exposure may require coordination with Work Task 1.1.

- Whether and how to support Intent-Based Exposure.

  - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure. The scenarios for intent-based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces of the NEF can achieve any intents that are identified within work task #3.

NOTE 7:	Information exposure is based on operator policies/SLAs.

NOTE 8:	The goal of intent-base exposure is to make accessing exposed services more user friendly.

NOTE 9:	The Intent-Based Exposure aspect of work task may have dependency on Work Task #3.

NOTE 10:	This work task requires coordination with SA WG3, SA WG6, and SA WG5.

NOTE 11:	This work task has dependency on service requirements to be specified by SA WG1.

**** Next Change – All New Text ****

**** End of Changes ****