SA WG2 Meeting #171	S2-2509738

13 - 17 October, 2025, Wuhan, China	(was S2-2508409)

Source:	CATT, CUC

Title:	[WT#7]: Work task and key issue for 6G satellite access

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the scope of WT#7 and corresponding Key Issue(s) for SA2 6G SID.

1. Justifications

SA2 SID S2-2506096 proposes to study the aspects of non-terrestrial networks (NTN) in 6G. NTN topic is included in the following WTs:

-	WT#2: Study migration and interworking, including

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

-	WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

In addition, 3GPP RAN groups also agree to include NTN in the 6G Radio SI (see RP-251881) and will design harmonized 6G Radio for TN and NTN.

2. Merged Documents

The following contributions are considered to the WT#7: 6G NTN for SA2#171 meeting.

| TDoc | Title | Source | WT |
| --- | --- | --- | --- |
| S2-2508226 | [WT#7] Scope and key issue on NTN | NEC | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. |
| S2-2508409 | WT_7_Work task and key issue for 6G satellite access | CATT | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. NOTE 3: The enhancement of backhaul using 6G satellite access to provide feeder link is to be considered. NOTE 4: QoS/policy improvement for satellite backhaul serving regenerative payload mode 6G satellite access is to be considered. NOTE 5: This WT has IWK aspects that are scoped under WT#2. |
| S2-2508690 | [WT#7, NTN] Scope and New Key Issue to Support 6G RAT for NTN | LG Electronics | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. NOTE 3: Coordination with WT#2 for interworking with EPS/5GS is needed. |
| S2-2508762 | [WT#7] Scope and New Key Issues to Support 6G RAT for NTN | Nokia, T-Mobile USA, [China Telecom, TNO, Thales, Vivo, ESA, IRDM, EchoStar, Boost Mobile Network, Viasat, Sateliot, AT&T] | WT 7.Y - Support of 6G RAT and Service Continuity for NTN in 6GS - Study architecture enhancements needed for NTN, if any, based on the 6G RAT specified by 3GPP RAN. - Study whether and how 6G networks support service continuity when a UE moves to(/from) an NTN network from (/to) a TN network, as well as among satellites of NTN that are deployed at different orbits (e.g., LEO, MEO, GEO). NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. |
| S2-2508868 | [WT#7] WT and KI for 6G satellite access | Ericsson | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. NOTE 3: This WT is dependent on WT#1 for the general 6G architecture |
| S2-2508953 | Work task proposal for NTN | MediaTek Inc. | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. Editor's Note: Whether the improvement of backhaul via 6G satellite access is to be considered is FFS. Editor's Note: Whether any QoS/policy improvement is needed for satellite backhaul is FFS. |
| S2-2508982 | [WT#7, NTN] WT scope for 6G satellite access | CSCN | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The mobility mechanism study should consider multi-orbit and frequent TN-NTN/NTN-NTN mobility scenarios. NOTE 3: The improvement of backhaul using 6G satellite access for feedlink should be studied. |
| S2-2509045 | [WT#7, NTN] NTN specific WTs and KIs for 6G support of NTN | Huawei, HiSilicon | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN. NOTE 2: The gap due to multi-orbit scenario can be studied based on the agreed TN mobility mechanism for 6G. 1. - Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance. Editor's Note: Whether any QoS/policy improvement is needed for satellite backhaul is FFS. |
| S2-2509073 | [WT#07, all topics] Scope and key issue on NTN | NTT DOCOMO | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects, including scenarios involving mobility across different satellite orbits (e.g., LEO, MEO, GEO). NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G, including implications for service continuity. Editor's Note: Whether the improvement of backhaul via 6G satellite access is to be considered is FFS. Editor's Note: Whether any QoS/policy improvement is needed for satellite backhaul is FFS. |
| S2-2509077 | [WT#7, NTN] WT&KIs for 6G NTN | Xiaomi | WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G. |

3. Text proposal

It is proposed to agree the following changes to TR 23.801-01:

**** First Change (all text new)****

A.X	Work Task 7

Editor's note:	Description of Work Task X. Clauses can be used if needed.

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	Further works may be introduced based on RAN progress on 6G NTN.

**** Second Change (all text new) ****

5.X. Key Issue #X: Support of 6G NTN

Editor's note:	This clause identifies key issues to be addressed.

For the support of 6G NTN, following aspects shall be considered and studied:

-	How to support satellite access in the 6G system.

NOTE 1:	Further works may be introduced based on RAN progress on 6G NTN.

Editor's Notes:	Which satellite specific functions defined in 5G system will be reused in 6G system is FFS.

Editor's Notes:	Whether and how to re-use 4G satellite specific functions, such as store and forward, is FFS.

**** End of Changes ****