3GPP TSG-SA2 Meeting #172	S2-2510841

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025				  (revision of S2-2510227)

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0565 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Enhancements to support MANET multicast for IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V., NIST, Qualcomm Inc, FirstNet, CATT, AT&T, China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | ProSe_Ph4-ARC |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces enhancements in 5G ProSe to enable using PC5 groupcast mode for MANET multicast for IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay to reduce the amount of redundant transmissions when using unicast transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhancements to support the use of PC5 groupcast mode for MANET HELLO and TC messages. - Enhancements to support the use of PC5 groupcast mode for MANET multicast traffic delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MANET multicast for IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay cannot be supported efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.3.1, 5.1.5.1a, 5.14.3.2, 6.4.3.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	Void.

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	Void.

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[24]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	Void.

[27]	Void.

[28]	3GPP TS 38.351: "NR; Sidelink Adaptation Layer Protocol".

[29]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[30]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[32]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[33]	3GPP TS 22.268: "Public Warning System (PWS) requirements".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	IETF RFC 7181: "The Optimized Link State Routing Protocol Version 2 (OLSRv2)".

[36]	IETF RFC 6130: "Mobile Ad Hoc Network (MANET) Neighborhood Discovery Protocol (NHDP)".

[37]	IETF RFC 5444: "Generalized Mobile Ad Hoc Network (MANET) Packet/Message Format".

[38]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[39]	IETF RFC 5498: "IANA Allocations for Mobile Ad Hoc Network (MANET) Protocols".

[40]	IETF RFC 6621: "Simplified Multicast Forwarding".

* * * * Next Change * * * *

#### 5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

NOTE 2:	For MANET multicast routing message delivery, the IP multicast address is set as defined in RFC 5498 [39], and not included in the configuration.

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 3:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

NOTE 4:	Radio parameters for 5G ProSe Direct Communication when the UE is not "served by NG-RAN" in clause 5.1.5.2.1 and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe UE-to-UE Relay communication are expected to be aligned for direct and relayed UE to UE communication.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 5:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 6:	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.6.1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	The mapping of ProSe services (i.e. ProSe identifiers) to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [12] and TS 38.331 [16] for further information).

NOTE 7:	A "catch all" entry with the lowest priority in each above mapping can be used for the ProSe services that do not have explicit policy/parameters mapping above.

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2] and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configuration (see TS 38.331 [16]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by NG-RAN".

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

NOTE 8:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 7) may be provided or updated to the UE by the ProSe Application Server.

* * * * Next Change * * * *

#### 5.1.5.1a	Policy/Parameter provisioning for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator, Multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The Multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and sending/receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 UE-to-UE Relay discovery with Model A and Model B.

NOTE 2:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and for enabling the UE to connect to the 5G ProSe Layer-3 UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery parameters (Relay Service Code(s), UE-to-UE Relay Layer indicator, Multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The Multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay:

1)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 3:	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Radio parameters for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

NOTE 5:	The RSC used for Multi-hop service is different from the RSC used for single-hop service.

3)	Policy/parameters for PC5 groupcast for MANET multicast operation:

-	Includes for each RSC an indication of whether to use PC5 groupcast mode for MANET multicast operation.

NOTE 6:	All other PC5 groupcast parameters are set as defined in clause 5.1.3.1.

* * * * Next Change * * * *

#### 5.14.3.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used. The 5G ProSe Multi-hop UE-to-UE Relays supporting the same RSC establish IP based Layer-2 links with each other and form a relay cloud to provide IP connectivity for 5G ProSe End UEs connected to any of the 5G ProSe Multi-hop UE-to-UE Relays.

NOTE:	If a 5G ProSe Multi-hop UE-to-UE Relay support multiple RSCs, different Layer-2 links will be established, with each associated with a specific RSC. The 5G ProSe UE-to-UE Relay operates as separate logical entities in the separate relay clouds of different RSCs, i.e. no routing table sharing and no traffic forwarding across different relays clouds of different RSCs. The 5G ProSe End UE can have separate IP address, separate routing with different RSCs as well.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets).

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connectivity in the relay cloud to provide Multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relay acts as a MANET router. The exact MANET routing protocols to be supported, e.g. Neighborhood Discovery Protocol (NHDP) [36], OLSRv2 [35], are associated with the RSCs based on configuration. The MANET multicast protocol to be used for IP multicast handling, e.g. Simplified Multicast Forwarding (SMF) RFC 6621 [40], is the same for all relay nodes with the same RSC.

Based on configuration associated with the RSC, the 5G ProSe End UE obtains the IP address/prefix in two options:

-	Each of the 5G ProSe End UE may be configured with a routable IP address/prefix associated with the RSC. This IP address/prefix does not change when the 5G ProSe End UE changes the connections with 5G ProSe Multi-hop UE-to-UE Relays.

-	If the 5G ProSe End UE does not have an IP address/prefix configured for the RSC, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay it connects to. In this case, the 5G ProSe End UE may need to change the IP address/prefix when it changes its connection to another 5G ProSe UE-to-UE Relay.

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries (per RSC). 5G ProSe Multi-hop UE-to-UE Relay needs to support DNS operations.

Once the 5G ProSe Multi-hop UE-to-UE Relay becomes aware of the 5G ProSe End UE's IP address/prefix, the 5G ProSe Multi-hop UE-to-UE Relay propagates the related DNS information (i.e. User Info ID and associated IP address/prefix) to all other 5G ProSe Multi-hop UE-to-UE Relays connected to the same MANET (i.e. corresponding to the same RSC) using a dedicated MANET Discovery Info message. The MANET Discovery Info message is diffused throughout the MANET using Multipoint Relay flooding (MPR flooding) as defined in IETF RFC 7181 [35], if available. If MPR flooding is not available, then the MANET Discovery Info message is diffused using simple flooding i.e. by transmitting a copy of the MANET Discovery Info message on all interfaces except the one on which the message has been received.

The MANET Discovery Info message is based on the generalized MANET message format defined in IETF RFC 5444 [37]. The 5G ProSe Multi-hop UE-to-UE Relay initiating the sending of the MANET Discovery Info message controls the propagation depth of the MANET Discovery Info message based on the configured "maximum number of hops per RSC" as specified in 5.1.5.1a or based on the Hop Limit if provided by the target End UE, by using the built-in mechanism defined in IETF RFC 5444 [37].

When a 5G ProSe Multi-hop UE-to-UE Relay detects an End UE is no longer connected (e.g. the PC5 link is released), it may delete the corresponding DNS entry and initiate MANET Discovery Info message to enable other 5G ProSe Multi-hop UE-to-UE Relay(s) to delete the corresponding DNS entry. The propagation depth of the MANET Discovery Info message is controlled based on the configured hop limit.

When MANET protocol determines the route for IP forwarding, layer-2 link QoS and hop counts can be reflected, e.g. with the Link Metrics value as defined in OLSRv2 [35]. The 5G ProSe Multi-hop UE-to-UE Relay can also enforce the configured hop limit based on the link metrics when building the routing tables, e.g. ignore the route that exceeds the threshold derived from hop limit.

If 5G ProSe Multi-hop UE-to-UE Relay has traffic for its own, it can use any IP address it owns for the transmission.

If PC5 groupcast mode for MANET multicast is configured for the RSC, as defined in 5.1.5.1a, MANET multicast routing messages (e.g., HELLO, TC) and MANET IP multicast traffic shall be transmitted over the corresponding PC5 groupcast mode bearer associated with the RSC.  Upon reception of a MANET multicast routing message or a MANET IP multicast packet and only if the message or packet is from one of the neighbor nodes which has already established a Layer-2 unicast link with the relay, the relay associates the message or the IP multicast packet to the corresponding Layer-2 unicast link and delivers it to IP layer.

When PC5 groupcast mode is used for MANET multicast routing messages (e.g., HELLO, TC), the Destination Layer-2 ID is derived from the IP multicast address as defined in RFC 5498 [39] and the associated RSC. When PC5 groupcast mode is used for MANET IP multicast packet, the Destination Layer-2 ID is derived from the associated RSC and the IP multicast address of the packets. The mechanism for converting the RSC and IP multicast address into the Destination Layer-2 ID is defined in TS 24.554 [23], same as that defined in 5.8.2.3 where the combination of RSC and IP Multicast Address is used as the Application Layer Group ID. The 5G ProSe Multi-hop UE-to-UE Relay may make use of the MANET multicast protocol information to determine which PC5 groupcast Layer-2 ID to receive data from.

NOTE: Security protection of the PC5 groupcast mode for MANET multicast use is specified in TS 33.503 [29].

* * * * Next Change * * * *

##### 6.4.3.10.1	Layer-2 Link establishment and management for IP PDU type

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.5.2:

-	The Direct Communication Request message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User info (i.e. Application Layer ID) of source 5G ProSe End UE or User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the source 5G ProSe End UE or 5G ProSe Multi-hop UE-to-UE Relay requesting the connection establishment.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the target 5G ProSe Multi-hop UE-to-UE Relay provided during the 5G ProSe Multi-hop UE-to-UE Relay Discovery procedure.

-	(optional)ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe Multi-hop UE-to-UE Relay as requested.

-	Security Information: the information for the establishment of security for the PC5 link establishment (see TS 33.503 [29]).

-	(Optional) Hop-Limit: the 5G ProSe End UE may include a hop-limit value smaller than the configured "maximum number of hops per RSC", if it desires to reduce the number of hops the other End UE can communicate with it via the relays.

The Direct Communication Accept message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the 5G Prose Multi-hop UE-to-UE Relay sending the Accept message.

Other link management procedures as defined in clause 6.4.3 (i.e. Link identifier update in 6.4.3.2, Layer-2 link release in clause 6.4.3.3, Layer-2 link modification in clause 6.4.3.4, Layer-2 link maintenance in 6.4.3.5) can be used for the layer-2 link without enhancements.

When a new layer-2 link is established, or an existing layer-2 link is released, the 5G ProSe Multi-hop UE-to-UE Relay need to inform the status change internally to the MANET router stack.

If the use of PC5 groucast mode for MANET multicast for an RSC is not configured, each of the established layer-2 link with another 5G ProSe UE-to-UE Relay is treated as a MANET interface as defined in RFC 6130 [36]. The layer-2 links of the same 5G ProSe UE-to-UE Relay may share the same IP address, as they are isolated, i.e. each of the layer-2 link only allows communication with one peer 5G ProSe UE-to-UE Relay.

If the use of PC5 groupcast mode for MANET multicast for a RSC is configured, as defined in 5.1.5.1a, a single MANET interface as defined in RFC 6130 [36] represents the set of established Layer-2 links with other 5G ProSe UE-to-UE Relays. Transmission of the MANET routing messages and IP multicast packets over this MANET interface are defined in 5.14.3.2.

NOTE: 	It is up to UE implementation to manage the discovery and caching of the mapping between the IP next hop address and Layer-2 destination ID, whether by standard IPv6 Neighbor Discovery techniques or by learning the mapping via processing of received IP neighbor discovery or routing messages.

The 5G ProSe UE-to-UE Relay may based on local configuration use a large value for HELLO_INTERVAL, VALIDITY_TIME and REFRESH_INTERVAL as defined in RFC 6130 [36] to reduce the frequency of periodic HELLO messages. The 5G ProSe UE-to-UE Relay can use the information from 5G ProSe link management procedures defined in clause 6.4.3 for maintaining the information in Neighbor Information Base.

* * * * End of Changes * * * *