SA WG2 Meeting #S2-172	S2-2510926

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510896)

Source: 	vivo

Title: 	KI#1 interim conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update interim agreement for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following changes to TR 23.700-04.

Start of Change

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

1. UP path termination entity

-	An 5GC NF is defined as the UP path termination entity to support the transfer of standardized data over UP for UE data. The UP path termination entity will further transfer the received data to the UMTES

NOTE 1: The UMTES is the AF/OTT server for UE-side model training.

2. Discovery of UP path termination entity

-	(the UMTES discovers UP termination entity) The UP path termination entity may be either locally configured at the UMTES, or the UP path termination entity may be discovered by the UMTES via NRF (if UMTES is trusted) or by NEF (the UMTES is untrusted).

The following options allow the UE to learn the UP path termination entity, either a or b,

- the UE has the UP path termination entity address/FQDN locally configured

- the 5G core network sends UP Path termination entity address (i.e. IP address/FQDN) to the UE in a N1 container via NAS.

3. UP path establishment

-	The UE establishes a new PDU Session, or uses an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, by reusing existing mechanism (e.g. URSP).

4. Data transfer procedure

- 	UMTES sends the data transfer request to UP path termination entity including the information related to candidate UE(s) for data transfer (e.g. candidate UE list, TAC(s), geographical area), and the requested data id.

Editor’s Note: what information will be used to select target UE(s) for data transfer is FFS. NOTE 2: The mapping between the requested data id and the data to be transferred needs to be determined via coordination with RAN.

-	The UP path termination entity sends the request for data transfer with requested data id to the target UE(s)

NOTE: The list of target UEs is determined taking into account the candidate UE(s) provided by the UMTES and user consent

-	The UE reports the requested data to the UP path termination entity via the established UP path.

Editor’s note: how to send the data transfer request to the UE, i.e. via NAS signalling or UP is FFS.

-	 The UP path termination entity sends information (e.g. requested data) to gNB via AMF, which may assist the gNB to decide data collection configuration to UE.

NOTE 3: whether and what information to assist data collection configuration will be defined by RAN.

-	The UE reports the collected data to the UP path termination entity via the established UP path, based on the requirements received in the Data Transfer Request from the UP path termination entity.

NOTE 4: The requested data for UE sided model training will be defined in RAN WGs.

NOTE 5: The mechanism of how UE collects data depend on RAN’s decision.

5.	Traffic Differentiation with following two alternatives:

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of the collected standardized data from the UE regular traffic per PDU Session level. Or

-	A QoS Flow is used to differentiate the transfer of the collected standardized data from the UE regular traffic per QoS flow level.

6.	Controllability

-	The UP path termination entity is allowed to initiate / stop / modify data transfer procedure.

-	The UE is allowed to request / accept / reject data transfer from the UP path termination entity. The UE is also allowed to stop data transferring based on internal determination, e.g. emergency session, low battery, etc.

7. Visibility -	The UP path termination entity may send to gNB via AMF an indication that data transfer is allowed for a UE

Alternative 2: The UP path termination entity verifies/matches the data specified to be collected and the data that is being reported.

Editor’s Note: whether and how to verify/match the data specified to be collected and the data that is being reported in standardized manner is FFS.

Alternative 3: The UP path termination entity can check whether the data types and formats of reported UE data match the specified/configured data types and formats for data collection/transfer, based on the mapping between the requested data id and the data to be transferred.

Editor’s Note: Whether other aspects needed to be checked for data visibility is based on the feedback of RAN.

NOTE 6: What requirements will be used by UP path termination entity for data check will be based on requirements of RAN.

8. User consent:

-	User consent checking with UDM for data transfer is needed.

Next Change

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

The following ENs are needed to be resolved to make the Option-2 UP solution to work following the agreed principles:

Editor’s Note: 	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

Editor’s Note: 	Which 5GC NF will be defined as the UP path termination entity is FFS.

Editor’s Note: whether the UP path termination entity can be discovered by other network function is FFS.

it’s FFS which 5GC NF sends the address of UP path termination entity to the UE

Editor’s Note: how to select which option (2a or 2b or co-exist) to use is FFS.

Editor’s Note: If there are multiple UP path termination entities, whether and how to ensure that the UE and the UMTES connect to the same UP Path termination entity is FFS.

Editor’s Note: Where and how to perform user consent check is FFS.

Editor’s Note: Whether using the existing or new user consent purpose is FFS. And coordination with SA3 is required.

Editor’s Note: whether, where and how to select the target UE by CN is FFS.

Editor’s Note: how to send the data transfer request to the UE, i.e. via NAS signalling or UP is FFS.

Editor’s Note: whether and how to select the UE by RAN is FFS.

Editor’s Note: how the UP path termination entity initiate / stop / modify data transfer procedure, i.e. via CP or UP is FFS.

Editor’s Note: whether 	the UP path termination entity send to gNB via AMF an indication that data transfer is allowed for a UE is FFS.

Editor’s Note: whether and how to verify/match the data specified/configured to be collected and the data that is being reported in standardized manner is FFS.

Editor’s Note: Whether the information (e.g. requested data) is sent with the data transfer request or within a separate request is FFS.

End of Changes