3GPP TSG-SA2 Meeting #172	S2-2511034

Dallas, TX, USA, 17-21 November 2025	revision of S2-2511989

Source:	Nokia

Title:	[KI#3] Conclusions proposal for KI#3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to proceed with some conclusions for normative work for KI#3. It is a pen holder merge of a number of submitted papers hence the source company is not necessarily endorsing all the content of the paper

# 1.	Introduction

At SA2#171 some principles were agreed. This paper aims at documenting a set of conclusions for KI#3 as a basis for normative WID drafting at SA2#172.

It is a pen holder merge of the papers in the table here below, hence the source company is not necessarily endorsing all the content of the paper

| S2-2510171 | 23.700-67: [KI#3] Conclusions proposal for KI#3 | Nokia |
| --- | --- | --- |
| S2-2509950 | 23.700-67: KI#3 conclusions | CATT |
| S2-2510285 | 23.700-67: KI#3, Conclusion | InterDigital Inc. |
| S2-2510302 | 23.700-67: KI#3 Conclusion | NTT DOCOMO |
| S2-2510425 | 23.700-67: KI#3: Conclusions for KI#3 | vivo |
| S2-2510577 | 23.700-67: Conclusion for KI#3 | Samsung Electronics France SA |
| S2-2510586 | 23.700-67: Key issue#3, the conclusion proposal | ZTE |
| S2-2510753 | 23.700-67: KI#3 conclusion | Huawei, HiSilicon |
| S2-2510240 | 23.700-67: Update of the agreed principles for KI#3 |  | Motorola Mobile Com Technology |  |

# 2. Conclusion

It is proposed that the following text is agreed for TR 23.700-67

Start of changes

# 8	Conclusions

As a result of the study documented in this TR, the following are to be specified as part of rel-20 normative specifications:

For KI#3:

For NF selection, the NF profile is enhanced to include the parameters in the table below.

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses theEnergy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| Schedule of Energy State | The NF is registered in the NRF. The NF is operable by OAM. The Schedule of Energy State indicates to a consumer when the NF can be selected. The NRF can provide the consumers with the schedule and make the NF discoverable even during the time when the schedule identifies the NF to be "SAVING ENERGY". The schedule includes the value " SAVING ENERGY" or "NORMAL OPERATION" for each time interval in the schedule. A consumer shall not select a NF when it is indicated as "SAVING ENERGY" If this attribute is present, the Energy State attribute is not present in NF profile. |
| Energy State | The NF is registered in the NRF. The NF is operable by OAM. The value of the attribute may be " SAVING ENERGY ", and in this case the NF may or may not be operating and no restoration procedure shall be triggered, or "NORMAL OPERATION" and in this state the NF is fully operating. When the attribute value is "SAVING ENERGY" it is not selectable and the NRF may also make it not discoverable by consumers. If this attribute is present, the Schedule of Energy State is not present in NF profile. |

NOTE 1:	These new proposed attributes will be further confirmed and, if confirmed, detailed in normative phase in cooperation with CT4.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF and/or EIF reporting at PDU session level only for the UPFs serving the session, may be considered by the SMF as a trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The Energy State can be used to e.g. steer consumer away from NFs that need to stop serving consumers due to energy saving reasons. The NRF shall not make the NF discoverable to a consumer if the attribute is "SAVING ENERGY". If a NF updates the NRF with value of the Energy State to be "SAVING ENERGY" the consumers are updated and shall not select the NF.

-	The Schedule of Energy State information may be taken into account in the process of the selection of the NF. The criteria for taking into account the schedule information is implementation-specific. During the time when the NF is identified to be "SAVING ENERGY" a consumer shall not select the NF.

-	While the NF Energy State at the NRF (either via the presence of the Energy State attribute or the Schedule of Energy State Attribute) is "Saving Energy", the NRF may tolerate not receing heartbeats up to a configurable time interval which the PLMN configures to exceed the expected maximum times a NF can spend in "SAVING ENERGY" state. While the NF is still serving consumers in this state the NF can still provide heartbeats.

-	When a NF exists "SAVING ENERGY" state and no schedule was provided in NF profile, the NF shall either perform a NF profile update or e.g. send a heartbeat or another message with indication the NF is returning to "NORMAL OPERTION" operation (details TBD in Normative phase)

-	The NFs discovered with a Schedule of Energy State in NF profile should not be aged out of the cache unless operator policy requires to do so. In general, the aging of such NFs at consumers cache should follow a different policy than NFs without the schedule.

-	The AMF may be configured with energy related information for N3IWFs and TNGF so that the information can be considered for N3IWF or TNGF reselection based on implementation.

-	The consumer NF may consider the energy policy configured for UE (i.e. the value of Energy Saving Indicator defined in TS 23.501 clause 5.51) to determine the corresponding energy priority when multiple Energy Priority Information are configured in the NF profile per energy policy.

NOTE 2: A single Energy Priority Information in the NF profile can represent different configured priorities per policy during the NF discovery/selection process via appropriate encoding which is left to Stage 3.

End of changes