Source:	SA4 Audio SWG Chair[*] (Ericsson LM)

Title:	Report from Audio SWG AH on FS_ULBC, Oct 23 and Oct 27

Document for:	Approval

Agenda Item:	5.1

## Executive Summary

The Audio SWG met on October 23, 2025, 14:00-15:13 CEST (38 participants, see Annex B) and October 27, 2025, 22:00-23:08 CET (30 participants, see Annex B). The sessions were handled as a joint meeting and were dedicated to FS_ULBC.

The outcome of the meeting (both sessions) can be summarized as follows:

- There were several contributions on RAN simulations methodologies being discussed and some results were provided for a few transport block sizes. However, there is still no agreement on the method to use finding the configurations to obtain loss traces for. The corresponding Tdocs in S4aA250112, S4aA250115, S4aA250116, S4aA250117, S4aA250118 were noted.

- The RAN1 LS response on Block Error Rate (BLER) assumptions was further debated based on S4aA250121, on how to interpret the RAN1 statement. There was no common understanding, and although another LS may be sent it was considered likely that a response would be late given the high workload of RAN1. Offline discussions were encouraged. Tdoc S4aA250121 was noted.

- There were some updates on the proposal on audio bandwidth requirements in S4aA250113 which was noted.

- There were updates on proposals on complexity constraints in S4aA250124 and on noise suppression in S4aA250125, which both were noted.

## 1.1. Opening of the session and registration of documents

On October 23, 2025, 14:00 CEST, the Audio SWG Chairman, Mr. Tomas Toftgård (Ericsson), opened the call. The agenda with Tdoc allocation was presented and approved (see Annex A). Notes were taken by Thomas Stockhammer (with assistance of co-pilot).

On October 27, 2025, 22:00 CET, the Audio SWG Chairman, Mr. Tomas Toftgård (Ericsson), opened the call. New documents (since the previous session) were added to the agenda which was approved (see Annex A). Notes were taken by Thomas Stockhammer (with assistance of co-pilot).

## 1.2. IPR, antitrust & consensus principles reminder

The IPR, antitrust & consensus principles reminder were read at both sessions.

The attention of the delegates to the meeting of this SWG Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

- to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

- to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG/SWG meetings was important to allow for full and fair consideration of such matters.

The attention of the delegates to the meeting was drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members were encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership would conduct the present meeting in a manner whereby informal methods of reaching consensus would be encouraged, whilst ensuring that well justified concerns are taken into account.

## 1.3. Reports/Liaisons from other groups/meetings

## 1.4. Release 18 and earlier matters

## 1.5. IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2)

## 1.6. ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2)

## 1.7. DaCAS (Diverse audio CApturing system for Smartphone devices)

## 1.8. FS_ACAPI (Study on Audio Codec APIs)

## 1.9. FS_ULBC (Study on Ultra Low Bitrate Speech Codec)

S4aA250108 is withdrawn

S4aA250111 is revised to S4aA250117

| S4aA250117 | [FS_ULBC] RAN simulation methodology for ULBC | Qualcomm Innovation Center Inc | Ma Liangping |
| --- | --- | --- | --- |

Presenter: Liangping Ma (Qualcomm)

Co-Pilot Summary

1. Background

The methodology for simulating Radio Access Network (RAN) performance for ULBC is not yet finalized.

Previous papers proposed several approaches; this document builds on "Approach 3" and integrates feedback from other proposals (notably from vivo and Dolby).

The goal is to jointly optimize per-UE (User Equipment) performance (TBS and BLER) and system capacity (number of UEs serviced simultaneously), balancing individual performance with commercial viability.

2. Simulation Workload Analysis

The number of possible simulation parameter configurations is extremely large (over 117,120 for uplink alone), making brute-force generation of all loss traces infeasible.

Parameters include direction (uplink/downlink), TBS, bundling period, Doppler spread, number of RX antennas, SCS, number of tones, NPUSCH/NPDSCH length, seed, and BLER.

3. Simulation Assumptions

To simplify, the methodology assumes:

- UL and DL share the same TBS and BLER.

- This aligns with LTE QoS, where UL and DL have the same packet error loss rate.

These assumptions significantly reduce the number of required traces (e.g., 960 for one scenario instead of 39,040).

4. Analysis of Previous Proposal

The "option 3" proposal from vivo (with Dolby input) avoids brute-force simulation but has inconsistencies and lacks detail on the trade-off between transmission performance and system capacity.

Figure 1 in the document highlights issues with the criteria for selecting the best configuration and BLER thresholds.

5. Proposed Methodology

The document proposes a revised simulation methodology with two options:

- Option 3.1: For each BLER target (1%, 2%, 6%, 10%), find the configuration that maximizes system capacity while meeting BLER in both UL and DL. Generate loss traces if achievable SNRs are met.

- Option 3.2: Find the configuration where both UL and DL BLER are ≤10%, maximizing system capacity at the maximum achievable SNRs. Generate loss traces accordingly.

Companies can choose which option to simulate for step 3.

Key Takeaways

The proposed methodology aims to balance simulation complexity with practical relevance by reducing the number of required traces and focusing on commercially meaningful scenarios.

It provides actionable steps for generating loss traces and optimizing both per-UE performance and system capacity.

The approach is designed to be flexible, allowing companies to select the simulation option that best fits their needs.

Discussion:

- Stefan B: Thank you. Concept of two options, what does it mean? What does it mean that companies would go for option 1 or 2. I would go for option 2 only, option 1 seems not possible. One system with one TBS, and another system with another neighboring, would you compare at the same SNR.

  - Liangping:

    - The idea of option 2 is not as trivial, the SNRs is different. We will show that in document 118. There is an uplink constraint that is mentioned. In the downlink we will match the SNR.

    - Qualcomm will do simulation 1

  - Stefan: We can only compare different TBS, if they use the same resources. We have never proposed cases where the resource usage is different.

- Jiayi: Do you want to do both or only one? Companies can choose?

  - Liangping: Companies can choose

  - Jiayi: Can they contradict? If so, what do we do?

  - Liangping: They are both valid

- Stefan B: How can we compare on a fair basis two proposals at the same SNR that use slightly different TBS sizes? In Option 1.

  - Liangping: we will get quite a lot of data, and we could still do the comparison

  - Stefan: Do you believe that 1, 2, 6 and 10% is densily sampled

- Gerardo: What is the if statement in option 1

  - Liangping: We make it clear you can only compare if we are in the power class

  - Gerardo: We should also compare with different repetitions

  - Liangping: Yes, this should address this. It is included.

Decision:

- Parked on Oct 23

- Noted on Oct 27

S4aA250117 is noted.

| S4aA250112 | [FS_ULBC] Detail methodology for error trace generation | vivo | Wang Dong |
| --- | --- | --- | --- |

Presenter: Wang Dong (vivo)

Co-Pilot Summary

1. Introduction

- The methodology for error trace generation was not finalized at the recent SA4#133-e meeting.

- This contribution proposes a detailed step-by-step approach for generating packet loss traces.

2. Methodology Steps

Step 1: Configuration Selection

- For a bundling time of 160ms and TBS (Transport Block Size) of 208 bits, all possible NPUSCH (Uplink Shared Channel) and NPDSCH (Downlink Shared Channel) configurations are listed.

- Focus is on maximum repetition numbers for uplink, as UL is typically the coverage bottleneck.

- Tables detail the combinations of SCS (Subcarrier Spacing), Nrep (Repetition Number), IRU/ISF, ITBS, and resulting transmission durations.

Step 2: BLER vs SNR Curve Generation

- For each configuration, generate BLER (Block Error Rate) vs SNR (Signal-to-Noise Ratio) curves for both uplink and downlink.

- Performance results are visualized in figures and tables.

Step 3: Link Budget and Candidate Selection

- Calculate link budgets for different UE (User Equipment) parameters to determine achievable UL/DL CNRs (Carrier-to-Noise Ratios).

- For target BLERs (e.g., 10% and 2%), compare selected SNRs with required SNRs from BLER-SNR curves.

- If required SNR is less than selected SNR, the configuration is considered a candidate.

- Tables summarize candidate configurations for both UL and DL at different BLER targets.

Step 4: Multiplexed UE Number Calculation

- For each candidate configuration, calculate the maximum number of multiplexed UEs (users) that can be supported, considering both TDM (Time Division Multiplexing) and FDM (Frequency Division Multiplexing).

- The final supported UE number is the minimum of UL and DL capacities.

- Tables and figures show the multiplexing results for different scenarios.

Step 5: Trace Generation

- Generate packet loss patterns for selected TBS configurations and SNR values.

- Figures illustrate packet loss patterns for key configurations.

3. Proposal for 3GPP TR 26.940

- The document proposes the following methodology for inclusion in the specification:

  - Select bundling times and corresponding TBS/net bit rates.

  - Generate BLER vs SNR curves for each configuration.

  - For each TBS, calculate link budgets and select candidate configurations based on BLER targets and multiplexed UE numbers.

  - Generate packet loss traces for selected configurations.

Key Insights

- The methodology provides a systematic way to select configurations that balance error performance and system capacity.

- It emphasizes maximizing the number of supported UEs while maintaining acceptable BLER.

- The approach is data-driven, relying on detailed tables and performance curves.

Discussion:

- Stefan B: Thank you for contribution. Same question as for 1, 2, 6, 10 is this dense enough and the comparison for codec bitrates?

  - Wang Dong: it just a performance boundary. We would set a system at 2% BLER loss rate.

  - Stefan B: Why would be discard a system that would better operate at 3% BLER

  - Tomas: If I understand correctly, the question is why configurations that would correspond to 3% would be excluded.

  - Thomas: We are still confused, where there is a statement on the system optimization. Not in TR and not in PD.

  - Stefan B: It is in the LS statement.

  - Thomas: But you are interpreting this in a different way we got this after the discussion with our RAN collegues. It would be good that Dolby checks with their RAN colleagues and experts.

Decision:

- Parked on Oct 23

- Noted on Oct 27

S4aA250112 is noted

| S4aA250113 | [FS_ULBC] Discussion on Audio Bandwidth for ULBC | vivo, Samsung, MediaTek Inc., Bytedance, Nokia, Xiaomi, Spreadtrum | Wang Dong |
| --- | --- | --- | --- |

Presenter: Wang Dong (On October 27, 2025)

Purpose

This document discusses the audio bandwidth requirements for the Ultra-Low Bitrate Codec (ULBC), focusing on its application in extremely constrained environments, especially satellite (GEO) communications. The main goal is to ensure robust, efficient, and reliable voice transmission under challenging conditions.

Key Points

1. Narrowband (NB) as Essential

NB (8 kHz sampling, 50–4000 Hz) is critical for ULBC’s primary use case: voice over GEO satellites.

NB remains widely used globally, especially in developing regions and as a fallback in poor coverage areas.

Without NB support, ULBC would be inefficient for the large user base still relying on NB, wasting bandwidth when connecting to NB endpoints.

2. User Expectations in Emergency/Last Resort Scenarios

GEO calls are often a last resort (e.g., rescue missions), where reliability is paramount.

Users expect at least NB quality, as this is the fallback in terrestrial networks.

Emergency communications typically terminate on NB endpoints (e.g., PSTN), making NB support vital.

3. Technical Trade-offs

Forcing wideband (WB) or super-wideband (SWB) at ultra-low bitrates can degrade speech clarity and increase complexity and power consumption.

Modern AI-based codecs scale poorly in complexity with higher sampling rates, making NB more efficient for power-constrained devices.

4. Experimental Analysis

Experiments using the Descript Audio Codec (DAC) show that WB (16 kHz, 50–8000 Hz) achieves excellent quality at low bitrates (~2.5 kbps).

SWB and fullband offer diminishing returns in quality relative to their increased bitrate requirements.

Mandating SWB or fullband is not justified for ULBC’s target scenarios.

5. Proposal

ULBC should support both NB (8 kHz) and WB (16 kHz) operation.

SWB and fullband support should be considered only as future enhancements.

The design constraints table in TR 26.940 should be updated to reflect these requirements.

Conclusion

The document strongly recommends that NB support be mandatory for ULBC to ensure efficiency, reliability, and compatibility with global voice services, especially in critical and emergency scenarios. WB can be included for enhanced quality where feasible, but wider bandwidths should not be prioritized at the expense of core robustness and efficiency.

Discussion:

- Presenter suggests to collect comments and then note.

- Stefan has some general questions on how to proceed.

Decision:

S4aA250113 is noted.

S4aA250114 is revised to S4aA250118

| S4aA250118 | [FS_ULBC] RAN simulation results for ULBC | Qualcomm Innovation Center Inc | Ma Liangping |
| --- | --- | --- | --- |

Presenter: Liangping Ma

Purpose

This document presents simulation results for the Ultra-Low Bitrate Codec (ULBC) using an 80ms bundling period, TBS 144, and 1 UE RX, following a specific simulation methodology [1].

Key Points

1. Simulation Parameters

The study evaluates both uplink (UL) and downlink (DL) scenarios.

Parameters include: bundling period (80, 160, 320 ms), Doppler spread (1Hz, 5Hz), subcarrier spacing (SCS: 3.75, 15 kHz), number of tones, and Block Error Rate (BLER) targets (1%, 2%, 6%, 10%).

2. Optimal Configurations

For each BLER target, the document identifies the optimal DL and UL configurations that maximize system capacity while meeting the required BLER.

DL (Downlink): The resource allocation is fixed at 180 kHz, with duration determined by the number of subframes (NSF) and repetitions (Nrep).

UL (Uplink): The configuration is determined by NRU, SCS, and number of tones.

3. Results by BLER Target

1% BLER: Achievable only with 16ms NPDSCH (DL) and 48ms NPUSCH (UL); allows scheduling up to 5 UEs per cell.

2% BLER: Achievable with 12ms NPDSCH (DL) and 64ms NPUSCH (UL); up to 6 UEs per cell.

6% BLER: Achievable with 8ms NPDSCH (DL) and 64ms NPUSCH (UL); up to 10 UEs per cell.

10% BLER: Achievable with 6ms NPDSCH (DL) and 64ms NPUSCH (UL); up to 13 UEs per cell.

4. Impact of UE Power Class

At 23 dBm, only 10% BLER is achievable (system capacity: 13 UEs/cell).

At 26 dBm, 10%, 6%, and 2% BLERs are achievable.

At 31 dBm, all BLER targets (10%, 6%, 2%, 1%) are achievable.

5. Proposal

The document proposes including the simulation methodology and optimal configuration selection process in the permanent documentation, to guide future evaluations and standardization.

Conclusion

The simulation results provide a clear methodology for selecting optimal configurations for ULBC, balancing per-UE performance (BLER) and system capacity. The findings highlight the trade-offs between BLER targets, system capacity, and device power class, offering practical guidance for standardization and deployment[1].

Discussion:

- Gerardo: these are link-level for a single user. On this framework, you can check how many users you can put. But this is no system-level, only link-level

  - Liangping: We do link-level, but we look at system capacity by extrapolating the single user capacity to system.

  - Gerardo: Not saying it is wrong, just not system level

- Tomas: only a single TBS, but insightful. Can we get more examples.

- Stefan:

  - We would like to see more data.

  - Are you relying on specific BLERs?

- Liangping: the limitation is the simulation workload.

  - Stefan would like to reduce workload.

  - Thomas (on chat):

    - It is easy to reduce simulation constraints

    - Companies should contribute to simulations.

    - Until now only Qualcomm, Huawei, Xiaomi and vivo do.

- Tomas: Can we provide more results to check the details, for example more TBS.

  - Thomas: We want to conscious on requesting more data from companies that do the work from companies that do not contribute.

  - Stefan: SA4 should have a discussion on what to consider on receive antennas and receiver power classes. We can drop some configurations upfront.

  - Liangping: the receive antenna and power classes were confirmed by RAN. And a simulation like this takes 1 week on 2 accounts, distributing on 100 cloud computers.

Decision:

S4aA250118 is noted.

| S4aA250115 | [FS_ULBC] Updated proposal on methodology for error trace generation | Dolby Laboratories Inc., Nokia, Ericsson LM | Stefan Bruhn |
| --- | --- | --- | --- |

Presenter: Stefan Bruhn (Dolby)

Discussion:

- Liangping: Several Comments

  - In step 0, the configuration is TBS dependent. You can pick any IRU. The issue is that in step 0, it will not apply to all TBS value

  - Maximizing the capacity, you will pick smallest resource allocation for UL and DL. The performance will very low

  - In step 5, there is an error with subscript. I pointed to this in 117

- Stephan B:

  - It is about capacity optimization, if we optimize in UL and DL, it would not be good. We need to do it smarter. Some middle ground.

  - The main point is, the MCS table for uplink and downlink, we argue that there are several columns, and the columns are equal in terms of resource. This needs to be understood. The MCS table for DL, it looks very similar.

  - Liangping: But some entries in the uplink are not present in the DL. Analyzed in 117.

  - Stefan B: We can only use the TBS that are shared in UL and DL.

  - Liangping: You want to find a middleground, but lack details. It would be good to address this.

  - Stefan B: Different opinions. We need to check how we convergence.

  - Liangping: There are very different BLERs. There are square bracket values that are unreasonable.

- Jiayi: Did you gather some simulation results from RAN colleagues? I checked with RAN colleagues, and they have a very different understanding. Can you provide simulation results?

  - Stefan: Not in Dolby, but other companies

  - Jiayi: It would be good by Monday.

  - Stefan: That would not be feasible.

  - Jiayi: Step 0 for example not possible (add more details).

  - Stefan: SA4 needs to work on this and make decision.

  - Jiayi: SA4 can not decide anything by ourselves, needs to be done across.

  - Stefan: Whatever is included in other contributions, also looks for capacity

  - Liangping: Our approach is done much more carefully and takes this into account.

  - Stefan: please be honest – clarify what it means

  - Thomas: How to minute this?

  - Frederic: type what is said

  - Thomas: impossible

Decision:

- Parked on Oct 23

- Noted on Oct 27

S4aA250115 is noted

| S4aA250116 | Multiuser SPS Capacity Analysis and TBS/MCS suitability | Ericsson LM, Dolby Laboratories Inc. | Tomas Jansson Toftgård |
| --- | --- | --- | --- |

Presenter: Gerardo

Purpose

This document analyzes the system capacity for multi-user scenarios using semi-persistent scheduling (SPS) in NB-IoT Non-Terrestrial Networks (NTN) for voice-over-GEO applications. It also evaluates the suitability of Transport Block Size (TBS) and Modulation and Coding Scheme (MCS) selections[1].

Key Points

1. Multiuser SPS Scenario Analysis

The study examines both 3.75 kHz and 15 kHz subcarrier spacing (SCS) with an SPS periodicity of 80 ms.

Findings:

With 3.75 kHz SCS, up to 40 user equipments (UEs) can be supported per SPS period without needing multiplexing.

With 15 kHz SCS, up to 36 UEs can be supported, but with optimized scheduling (e.g., allocating 8 instead of 12 single-tones), up to 40 UEs can also be supported.

No capacity issues were identified for supporting multi-user SPS scenarios for voice-over-GEO in NB-IoT NTN.

2. Scheduling and Complexity

Proper scheduling strategies can maximize the number of supported UEs without introducing additional multiplexing schemes, which would complicate technical specifications.

If higher capacity is needed, enhancements like orthogonal cover codes (OCC) could be considered, but this would increase scheduler complexity.

3. TBS/MCS Suitability

The document recommends using only TBS/MCS entries compatible with BPSK and QPSK modulation for uplink (UL) and downlink (DL), as 16-QAM is not suitable for voice-over-GEO.

For UL, entries from ITBS = 0 to 10 are for single-tone, and ITBS = 0 to 13 for multi-tone allocations.

For DL, the usable range depends on deployment type (in-band vs. stand-alone/guard-band).

Proposed TBS values for initial evaluations: 120, 144, 256, and 328 bits.

4. Conclusions and Proposals

No need for new multiplexing schemes for multi-user SPS scenarios in the current context.

The recommended TBS/MCS values and scheduling strategies are sufficient for supporting the required capacity for voice-over-GEO.

The ultimate selection of TBS values will depend on the voice codec design.

Significance

This analysis confirms that, with proper scheduling and selection of modulation/coding schemes, NB-IoT NTN can efficiently support multi-user voice-over-GEO scenarios without capacity bottlenecks or the need for complex multiplexing. The recommendations guide future simulation and standardization work[1].

Discussion:

- Liangping: Good starting point, but some issues:

  - At the beginning of the paper, it shows that this results in very high system capacity. I did the simulation for everything. I do think it says 40 UEs, but you need to link level simulation to check what is useful.

  - It talks about a very large cell size, only the case if UE does not report TA. Thousands of beams.

  - There is also mentioning of OCC, but this does apply for SA4.

- Gerardo: Mostly observations, the figure shows from scheduling perspective what can be achieved. Link simulation needs to be done. We are not proposing OCC.

- Ruonan: We should not use this figure as we need to fit in downlink and uplink. At may only 8 UEs can be supported. For uplink transmission time is 64 ms, the rest for DL

  - Gerardo: more observation, but do not understand. You can use TBS 328 bit, so you can use 2ms in the time domain. We believe this example is correct.

  - Liangping: changing to 1ms is ok, but it correct only on paper. But we need simulation. This is also why step 0 in 115 is not working.

  - Gerardo: In 2.2 we need to select. We can also accommodate different TBS values.

Decision:

- Parked on Oct 23

- Noted on Oct 27

S4aA250116 is noted

S4aA250120 is withdrawn

| S4aA250121 | [FS_ULBC] Clarification on the Interpretation of RAN1 Liaison Statement to SA4 Regarding FS_ULBC | Dolby Laboratories Inc., Nokia, Ericsson LM | Stefan Bruhn |
| --- | --- | --- | --- |

Presenter: Stefan Bruhn

Purpose

This document clarifies how the RAN1 liaison statement to SA4 should be interpreted, specifically regarding the Block Error Rate (BLER) assumptions for the Feasibility Study on Ultra-Low Bitrate Codec (FS_ULBC)[1].

Key Points

1. Background

SA4 received a liaison statement (LS) from RAN1 in response to questions about evaluation assumptions for FS_ULBC, especially concerning BLER.

2. RAN1’s Response on BLER

RAN1 stated that in previous voice evaluations (e.g., AMR, AMR-WB over 3G), a 2% BLER was used as the target performance metric.

However, RAN1 emphasized that it is up to SA4 to decide what BLER values to use for their own evaluations.

3. Interpretation and Discussion

Some parties interpreted RAN1’s response as a recommendation for a 2% BLER target for ULBC.

The document clarifies that the 2% BLER figure was historically based on SA4’s own studies and not independently established by RAN1.

For ULBC, RAN1 has not recommended or confirmed a 2% BLER target; instead, SA4 should independently determine the appropriate BLER value.

The robustness of future ULBC codecs to packet loss is not yet characterized, and modern AI-based codecs may offer improved robustness.

4. Recommendation

SA4 should not assume or mandate a 2% BLER based solely on RAN1’s statement.

SA4 should consider the relationship between bitrate, BLER, and audio quality when developing ULBC.

RAN1’s LS does not suggest any specific radio bearer optimizations for ULBC, leaving SA4 free to define BLER assumptions for the study.

Conclusion

The document recommends that SA4 independently determine the BLER assumptions for FS_ULBC, rather than relying on historical figures or perceived recommendations from RAN1. The focus should be on the specific requirements and robustness of ULBC codecs[1].

Discussion:

- Liangping: We cannot freely define the BLERs. This is an issue of the scheduling.

  - Stefan B: You can freely interpret what RAN responded, but this is not factual.

  - Liangping: The statement on freely choosing is not correct.

  - Stefan B: Check what was written.

  - Ruanon: We provided values 1,2,6,10%.

  - Dong: There were a set of values in the LS in RAN1

  - Thomas: there are target BLERs mentioned. And we understand from discussions with RAN expert.

  - Stefan: You are wrongly interpreting and just liaise from one company to the same.

- Jiayi: We had the same interpretation after consulting with RAN experts. Encourage Dolby to speak with the RAN experts

  - Stefan: everyone can bring the contributions to 3GPP SA4 and they need to be discussed here.

  - Liangping: We have decided.

  - Gerardo: Some companies in RAN believe 2% and some companies believe it is 10%.

  - Thomas: I believe we need to send an LS to RAN to clarify this finally. It is a fundamental design choice.

  - Liangping: Not really keen to send an LS, RAN is overloaded.

  - Stefan: We are not insisting on 20%, this was a discussion.

  - Nik: We had been using 1 and 2% for speech for quite some time. Why change?

  - Stefan B: We have seen new codec designs that are robust. Hence RAN responded to this.

- Gerardo on chat:

  - Extract from an LS reply to SA4 in R1-2506541:

    - "-In previous RAN1 evaluations related to voice, RAN1 has considered 2% BLER as the target performance metric. It is up to SA4 to decide what values to use in their evaluations."

  - Extract from an LS reply to SA2 in R1-2508096:

    - "Assuming line-of-sight condition for the duration of the call, no change in large scale parameters (e.g.fixed shadowing), and using an NTN TDL-C channel model (which is a channel model previously used by RAN1 and SA4 evaluations), the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions (e.g.up to 10% packet error rate)"

  - Thomas on chat: Yes, there are periods where the loss rate goes up to 10% and the system should not break. But the nominal (target) is 2%. There is some misunderstanding. QoS works to 95% of the time. So the system needs to be robust to occasional higher loss periods, but it nominally operates at a target much lower.

Decision:

S4aA250121 is noted

| S4aA250124 | [FS_ULBC] On complexity constraints for ULBC | HUAWEI TECHNOLOGIES Co. Ltd. | JIAWEI LI |
| --- | --- | --- | --- |

Presenter: Jiawei Li

Purpose

This document discusses the importance of setting computational complexity and memory footprint requirements for the Ultra-Low Bitrate Codec (ULBC), especially for its deployment in voice communication over GEO satellite access[1].

Key Points

1. Diversity of Hardware Platforms

ULBC is intended for a wide range of user equipment (UE): mobile phones, smart watches, smart glasses, telematics boxes, customer premises equipment, vehicles, and IoT devices.

These devices use different processing units (DSP, CPU, NPU), each with unique characteristics:

DSPs are power-efficient, cost-effective, and ideal for real-time speech processing.

CPUs offer higher computing power but consume more energy and require careful management for real-time tasks.

NPUs are optimized for specific AI algorithms but may not deliver consistent performance across different codecs.

2. Power Consumption

Low power consumption is critical, especially for wearable devices and emergency communication scenarios.

Satellite-enabled emergency communication is highlighted as a vital use case, requiring ULBC to be highly power-efficient to ensure reliability and long battery life.

3. Complexity and Memory Requirements

ULBC should be implementable on DSP-enabled devices, as DSPs are preferred for their efficiency in low-power, real-time applications.

The computational budget for ULBC on DSPs is limited (a few hundred MIPS), considering concurrent processing needs (e.g., echo cancellation, noise suppression).

AI-based codecs may have higher complexity and storage demands than traditional codecs, so specific limits for ULBC are still to be determined.

4. Complexity Measurement

The document recommends maintaining the traditional method of measuring codec complexity using fixed-point C reference code and WMOPS (Weighted Million Operations Per Second), as done for previous codecs like AMR, EVS, and IVAS.

5. Proposal

ULBC solutions should be implementable on DSP-enabled devices, even if other processing units are available.

Complexity and memory limits are to be defined, but the measurement method should remain consistent with established standards.

Conclusion

The document emphasizes that for ULBC to be widely deployable and effective—especially in emergency and low-power scenarios—it must have well-defined, low complexity and memory requirements, favoring implementation on DSPs. The exact limits are yet to be determined, but the approach should align with established practices in codec standardization[1].

Discussion:

- Rishabh:

  - WMOPS would not be enough, also need model sizes and so on.

  - Power consumption on NPUs, etc. would need some data.

Decision:

- October 27. 2025 – noted. More discussion expected at SA4#134.

S4aA250124 is noted

| S4aA250125 | [FS_ULBC] On noise suppression for ULBC | Huawei Technologies Co., Ltd. | Lei Li |
| --- | --- | --- | --- |

Presenter: Lei Li

Purpose

This document discusses different approaches to integrating noise suppression (NS) in the Ultra-Low Bitrate Codec (ULBC) standard, analyzing their advantages and drawbacks for mobile and wearable devices[1].

Key Points

1. Background

Noise suppression is now a default feature in most mobile devices, often using multiple microphones and advanced AI-based techniques.

Clean speech input improves the performance of generative AI codecs, making NS relevant for ULBC applications.

2. Scenarios for Noise Suppression Integration

The document evaluates four possible configurations:

Case 1: Only External NS

NS is handled outside the ULBC codec (e.g., by device hardware/software).

Pros: Can use advanced, device-specific algorithms (e.g., multi-mic, AI-based).

Cons: Complicates testing and risks mismatches between ULBC and various external NS algorithms.

Case 2: Only Internal NS

NS is built into the ULBC codec; external NS is disabled during calls.

Pros: Ensures compatibility and robustness, as the codec and NS are trained together; can be optimized for low power.

Cons: Cannot leverage more powerful external NS solutions; internal NS complexity and memory must be included in ULBC constraints.

Case 3: Both Internal and External NS On

Both NS systems are active.

Cons: Impractical due to high power consumption, complexity, and potential compatibility issues.

Case 4: Switchable Internal/External NS

Internal NS can be switched on/off; external NS may also be present.

Pros: Flexible, allowing products to choose the best configuration.

Cons: May lead to inconsistent service quality across devices and networks.

3. Proposal

The Audio SWG should discuss these scenarios and reach consensus on whether ULBC should include a built-in NS function, and if so, how it should be standardized.

If internal NS is included, its complexity and memory usage must be counted within the overall ULBC constraints.

Conclusion

The document recommends a thorough discussion of the pros and cons of each noise suppression scenario for ULBC, emphasizing the need for clear decisions on standardization and resource constraints[1].

If you need a more detailed breakdown of any scenario or want to discuss implications for specific device types, let me know!

Discussion:

- Rishab:

  - No issue with the second proposal

  - On the first one, not sure how to take external NS into account. Is done on various aspects. Codec should be able to handle any type of signals, does not have to be considered.

- No other comments

Decision:

- Given the extensive discussions on this topic in the previous meeting, it seems not agreeable at this point.

S4aA250125 is noted.

## 1.10. Other Rel-19 matters including TEI

## 1.11. Close of the session

The chair thanked the delegated for their contributions and the discussions. The call on Oct 23 was closed 15:13 CEST. The call on Oct 27 was closed 23:08 CET.

## Annex A: Agenda with Tdoc Allocation

| 1 | Audio SWG |  |
| --- | --- | --- |
| 1.1 | Opening of the session and registration of documents |  |
| 1.2 | IPR, antitrust and consensus principles reminder | IPR/Anti-trust/consensus reminder |
| 1.3 | Reports/Liaisons from other groups/meetings |  |
| 1.4 | Release 18 and earlier matters |  |
| 1.5 | IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2) |  |
| 1.6 | ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2) |  |
| 1.7 | DaCAS (Diverse audio CApturing system for Smartphone devices) |  |
| 1.8 | FS_ACAPI (Study on Audio Codec APIs) |  |
| 1.9 | FS_ULBC (Study on Ultra Low Bitrate Speech Codec) | 108 111->117n 112n 115n 116n 114->118n 113n New inputs Oct 27th 120 124n 125n 121n |
| 1.10 | Other Rel-19 matters including TEI |  |
| 1.11 | Close of the session |  |

Legend for Tdocs:

- Color: not-yet processed, processed, late, withdrawn, moved to a different A.I., under email agreement

- a agreed, app approved, e endorsed, m missing, n noted np, not pursued, pp postponed, r revised

Annex B: List of Participants

Table 1: Participants on Oct 23rd.

| NAME | Organization Represented |
| --- | --- |
| Tomas Toftgård | Ericsson |
| Elmira Ramazanirend | Vodafone |
| Jaffar, Munira | EchoStar |
| Lei Li | Huawei |
| Wang Bin | Xiaomi |
| Ruonan Yang | Vivo |
| Jiayi | CMCC |
| Schnell, Markus | Fraunhofer IIS |
| Szczerba, Marek | Philips |
| Peter Kroon | Apple |
| Zhe Wang | Huawei |
| Milan Jelinek | VoiceAge |
| Fotopoulou, Eleni | Fraunhofer IIS |
| Thomas Stockhammer | Qualcomm |
| Stephane Villette | Qualcomm |
| lijiawei | Huawei |
| Gerardo Agni Medina Acosta | Ericsson |
| Ehara Hiroyuki | Panasonic |
| DONG | Vivo |
| Anssi Ramo | Nokia |
| Liangping Ma | Qualcomm |
| Tyagi, Rishabh | Dolby |
| Waqar Zia | Apple |
| Ryan Lee | Samsung |
| Peng Tan | Skylo |
| Bruhn, Stefan | Dolby |
| RAGOT Stéphane | Orange |
| Mike Ye | Bytedance |
| Das, Biswajit | Philips |
| Gabin, Frederic | Dolby |
| Döhla, Stefan | Fraunhofer IIS |
| Lasse J. Laaksonen | Nokia |
| Huan-yu Su | Huawei |
| Shilin Ding | Qualcomm |
| Vaclav Eksler | VoiceAge |
| Zakaria, Gaguk | EchoStar |
| Noboru Harada | NTT |
| Srikanth Nagisetty | Panasonic |

Table 2: Participants on Oct 27th.

| NAME | Organization Represented |
| --- | --- |
| Tomas Toftgård | Ericsson |
| Das, Biswajit | Philips |
| Dong | vivo |
| Bruhn, Stefan | Dolby |
| Huan-yu Su | Huawei |
| Lei Li | Huawei |
| RAGOT Stéphane | Orange |
| Anssi Ramo | Nokia |
| Gerardo Agni Medina Acosta | Ericsson |
| Szczerba, Marek | Philips |
| Jiayi | CMCC |
| lijiawei | Huawei |
| Schnell, Markus | Fraunhofer |
| Ehara Hiroyuki | Panasonic |
| Razvan Andrei Stoica | Lenovo |
| Lasse J. Laaksonen | Nokia |
| Thomas Stockhammer | Qualcomm |
| Liangping Ma | Qualcomm |
| Döhla, Stefan | Fraunhofer IIS |
| Stephane Villette | Qualcomm |
| Ryan Lee | Samsung |
| Ruonan Yang | vivo |
| Wang Bin | Xiaomi |
| V. Atti | Apple |
| Tyagi, Rishabh | Dolby |
| June Sun | Bytedance |
| Fotopoulou, Eleni | Fraunhofer IIS |
| Nikolai Leung | Qualcomm |
| Jaffar, Munira | EchoStar |
| Gilles Teniou | Tencent |