Source:	Ericsson LM

Title:	SPS Link Level Evaluations

Document for:	Discussion and Agreement

Agenda Item:	7.9

# 1	Introduction

As part of FS_ULBC, there is an objective to study channel characteristics and service-related dependencies, determining e.g. bitrates and loss/delay/jitter profiles [1]. In this contribution, we provide initial SPS link level evaluations.

# 2	Semipersistent Scheduling: Link-level evaluations

The Rel-20 Work Item (WI) entitled “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4” [RP-251867], explicitly mentions as part of its objectives “Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1].” In relation with it, initial discussions have been held based on the semipersistent scheduling (SPS) schemes illustrated in R1-2505140 and S4aA250277 which depict a single-user scenario.

In this contribution we provide initial link level evaluations for the UL and DL configurations shown in Table 1.

Table 1: SPS UL and DL configurations for transmitting a TBS of 144 bits

|  | SPS Parameters | SPS scheme – No TA dependent |
| --- | --- | --- |
| UL | SCS | 3.75 kHz |
|  | Number of tones | 1 |
|  | RU length | 32 ms |
|  | TBS | 144 bits |
|  | Number of allocated RUs | 2 |
|  | Number of NPUSCH repetitions | 1 |
|  | UL-to-DL switching | 1 ms |
| DL | SCS | 15 kHz |
|  | TBS | 144 bits |
|  | Number of tones | 12 |
|  | Number of NPDSCH subframes | 2 |
|  | Number of NPDSCH repetitions | 1 |
|  | Processing time required for TBS decoding and for performing a DL-to-UL switching * For the moment, the minimum quantity has been assumed, which corresponds to the DL-to-UL switching (If the processing time required for TBS decoding were to be considered, then the required SPS periodicity may be impacted). | 1 ms* |
|  | Max Differential Delay | 10.3 ms |
| Total utilized resources |  | 78.3 ms |
| SPS periodicity that can fit the utilized resources |  | 80 ms |
| Estimated UE date rate |  | 1.8 kbps |

According with our link budget analysis for UL, the SNR at the satellite receiver is around 6.74 dB. The results from the link-level evaluations associated to Table 1 are shown in the figure below as an initial reference assuming perfect knowledge of the channel:

Figure 1: Link-level evaluations of transmitting a TBS of 144 bits.

The results from Figure 1 show that it is possible reaching 10% BLER @SNR ~3dB, 1% BLER @SNR ~3.8 dB and even the 0. 1% BLER @ ~4.7 dB, with SNRs that are below the SNR at the satellite receiver according to link budget analysis. The curve from Figure 1 and a variant of it assuming no perfect knowledge of the channel (i.e., non-genie channel estimation) are plotted together with respect to the results from other companies (some of them using “DMRS bundling” to enhance the performance):

Figure 2: Link-level evaluations corresponding to transmitting a TBS of 144 bits, including 
an overview of the results shared by other Companies as part of the SA4 discussions.

One important aspect to highlight is that Companies should share the same understanding about the outcome from the link-budget as to have an aligned view on the expected SNR at the satellite receiver, which in our current understanding is ~ 6.74 dB but this number can easily vary depending on the link-budget assumptions.

The table below shows UL and DL configurations assuming larger transport block sizes i.e., a TBS = 208 bits, 256 bits, and 328 bits.

Table 2: SPS UL and DL configurations for transmitting a TBS of 208 bits, 256 bits, and 328 bits.

|  | SPS Parameters | SPS scheme – No TA dependent | SPS scheme – No TA dependent | SPS scheme – No TA dependent |
| --- | --- | --- | --- | --- |
| UL | SCS | 3.75 kHz | 3.75 kHz | 3.75 kHz |
|  | Number of tones | 1 | 1 | 1 |
|  | RU length | 32 ms | 32 ms | 32 ms |
|  | TBS | 208 bits | 256 bits | 328 bits |
|  | Number of allocated RUs | 3 | 3 | 3 |
|  | Number of NPUSCH repetitions | 1 | 1 | 1 |
|  | UL-to-DL switching | 1 ms | 1 ms | 1 ms |
| DL | SCS | 15 kHz | 15 kHz | 15 kHz |
|  | TBS | 208 bits | 256 bits | 328 bits |
|  | Number of tones | 12 | 12 | 12 |
|  | Number of NPDSCH subframes | 3 | 3 | 3 |
|  | Number of NPDSCH repetitions | 1 | 1 | 1 |
|  | Processing time required for TBS decoding and for performing a DL-to-UL switching * For the moment, the minimum quantity has been assumed, which corresponds to the DL-to-UL switching (If the processing time required for TBS decoding were to be considered, then the required SPS periodicity may be impacted). | 1 ms* | 1 ms* | 1 ms* |
|  | Max Differential Delay | 10.3 ms | 10.3 ms | 10.3 ms |
| Total utilized resources |  | 111.3 ms | 111.3 ms | 111.3 ms |
| SPS periodicity that can fit the utilized resources |  | 120 ms | 120 ms | 120 ms |
| Estimated UE date rate |  | 1.73 kbps | 2.13 kbps | 2.73 kbps |

The initial results from the link-level evaluations associated to Table 2 are shown in the figure below:

Figure 3: Link-level evaluations corresponding to transmitting a TBS of 208 bits, 256 bits, and 328 bits.

Based on the above analysis, we have the following observations.

One important aspect to highlight is that Companies should share the same understanding on the link-budget analysis as to have an aligned view on the expected SNR at the satellite receiver.

In our understanding, the SNR at the satellite receiver is ~ 6.74 dB but this number can easily vary depending on the link-budget assumptions.

Based on the analysis we have the following proposals:

SA4 to align link-budget assumptions including: Satellite elevation angle, Satellite altitude, UE transmit power, Carrier frequency, UE antenna gain, Receive antenna gain, noise figure, obtained G/T, Atmospheric loss, Scintillation loss, Polarization loss, Additional losses.

SA4 to align link-level simulation assumptions including: Power Delay profile and channel model, UE transmit power, Number Rx antennas (satellite), Subcarrier Spacing, Modulation scheme, Frequency-domain allocation (e.g., single-tone), Transport Block Size, Time-domain allocation (Number of RUs, Number of repetitions, Number of NPDSCH subframes).

SA4 to include in the Permanent Document (PD), the link-level simulation results presented in this document aiming at comparing them with those presented by other Companies on SPS scenarios for the support of voice-over-GEO in NB-IoT NTN.

The support of “data transmission/reception of voice packets” over GEO in NB-IoT NTN should not be conditioned to the support of other features such as “TA report UE capability,” whereas in our understanding “DMRS bundling” is up to implementation (i.e., optional enhancement).

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	One important aspect to highlight is that Companies should share the same understanding on the link-budget analysis as to have an aligned view on the expected SNR at the satellite receiver.

Observation 2	In our understanding, the SNR at the satellite receiver is ~ 6.74 dB but this number can easily vary depending on the link-budget assumptions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SA4 to align link-budget assumptions including: Satellite elevation angle, Satellite altitude, UE transmit power, Carrier frequency, UE antenna gain, Receive antenna gain, noise figure, obtained G/T, Atmospheric loss, Scintillation loss, Polarization loss, Additional losses.

Proposal 2	SA4 to align link-level simulation assumptions including: Power Delay profile and channel model, UE transmit power, Number Rx antennas (satellite), Subcarrier Spacing, Modulation scheme, Frequency-domain allocation (e.g., single-tone), Transport Block Size, Time-domain allocation (Number of RUs, Number of repetitions, Number of NPDSCH subframes).

Proposal 3	SA4 to include in the Permanent Document (PD), the link-level simulation results presented in this document aiming at comparing them with those presented by other Companies on SPS scenarios for the support of voice-over-GEO in NB-IoT NTN.

Proposal 4	The support of “data transmission/reception of voice packets” over GEO in NB-IoT NTN should not be conditioned to the support of other features such as “TA report UE capability,” whereas in our understanding “DMRS bundling” is up to implementation (i.e., optional enhancement).

# 4	References

SP-250635: Revised SID on Ultra Low Bitrate Speech Codec.

RP-251867: New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4.