3GPP TSG-SA WG4 Meeting #134	S4-251962

Dallas, US, 17 – 21 November 2025	revision of S4-251668

Source:	Global Analysis Lab (HEAD acoustics GmbH)

Title:	IVAS Characterization Tests – Global Analysis Lab Report

Agenda item:	7.5 (IVAS_Codec_2)

Document for:	APPROVAL

# Introduction

This report presents the collected results and statistical analysis of the listening tests for the IVAS codec family in the IVAS Characterization Phase as specified in IVAS-8b [1]. For this purpose, HEAD acoustics was contracted as the Global Analysis Lab (GAL). The results are intended to characterize the IVAS codec family based on the floating-point and fixed-point C-codes and are to be documented into 3GPP TR 26.997, as indicated in the IVAS_Codec_Ph2 work item description [2].

The subjective evaluation is at large scale. It involved 13 listening laboratories worldwide, about 1070 listeners, and a total of about 189520 votes.

The GAL has received the raw voting data from all listening labs within the required time frame.

The report presents the results grouped by test method. The following experiments have been conducted:

- 23x DCR experiments according to P.800 [3] with 30 naïve listeners per experiment and a total of about 149040 votes,

- 20x MUSHRA experiments according to BS.1534 [4] with 14 trained listeners per experiment with a total of 26544 votes,

- 1x ACR experiment according to P.800 [3] with 30 naïve listeners and a total of 10800 votes, and

- 4x room acoustics experiments as described in [1] with 12 to 21 naïve listeners per experiment with a total of 3136 votes.

In addition to the results below, the report contains an electronic attachment with raw data and statistical analysis for each experiment. The listening labs, their respective report document references, and experiment allocations are listed in Table 1.1.

Key acronyms used in the report are:

ACR		Absolute Category Rating
CuT		Codec under Test
DCR		Degradation Category Rating
DTX		Discontinuous transmission
ESDRU		Energy-based Spatial Distortion Reference Unit
EVS		Enhanced Voice Services
FB		Full Band
FE		Frame Erasure
FL		Floating-point implementation
FX		Fixed-point implementation
FER		Frame Error Rate
FOA		First-Order Ambisonics
GAL		Global Analysis Laboratory
HOA2		Higher-Order Ambisonics, 2nd order
HOA3		Higher-Order Ambisonics, 3rd order
IVAS		Immersive Voice and Audio Services
ISM		Independent Streams with Metadata (IVAS Objects)
JBM		Jitter Buffer Management
MASA		Metadata-Assisted Spatial Audio
MNRU		Modulated Noise Reference Unit
MUSHRA	Multi Stimulus test with Hidden Reference and Anchor
RD		Rate-Distortion curve 
SWB		Super Wide Band

Table 1.1: listening labs, experiment allocations, and report document references

| Listening Lab | Experiments | Report TDoc |
| --- | --- | --- |
| Mesaqin.com | P800-6, P800-8, P800-9, P800-12, P800-21 | S4-251613 |
| FORCE Technology | P800-7, P800-11, P800-13, P800-22, P800-23, BS1534-1 | S4-251620 |
| Dolby Laboratories, Inc. | P800-5, P800-15, BS1534-5, BS1534-7, BS1534-11, BS1534-14, BS1534-17 | S4-251913, S4-251915 |
| Ericsson LM | P800-16, P800-18, BS1534-6, BS1534-20 | S4-251873 |
| Fraunhofer IIS | P800-17, P800-19, BS1534-9, BS1534-16 | S4-251723 |
| Huawei Technologies Co Ltd. | BS1534-2, BS1534-3 | S4-251749 |
| Nokia | ACR-1, P800-14, P800-20, BS1534-8, BS1534-15, BS1534-18 | S4-251691 |
| NTT/Panasonic Holdings Corporation | P800-2, BS1534-4 | S4-251688 |
| Orange | P800-3, P800-4, BS1534-19 | S4-251692 |
| Philips International B.V. | BS1534-12, ROOM-1, ROOM-2, ROOM-3, ROOM-4 | S4-251741 |
| Qualcomm Incorporated | BS1534-10, BS1534-13 | S4-251627 |
| VoiceAge Corporation | P800-1, P800-10 | S4-251647 |

# Analysis Methods

Statistical tests are used to compare the subjective scores of the codecs under test (CuTs) against the scores for specified reference conditions. Due to the uncertainty of the auditory data, it is necessary to consider not only the averaged per-condition results, but also the variance and the number of votes. The tests are conduced as specified in clause 3.3.6.2 of IVAS-8b [1]. Each subjective experiment contains a number of tests to be computed by the GAL.

For the P-800 and BS.1534 experiments, single-sided Student’t t-tests at 95% confidence level are conducted. For P.800 DCR experiments, independent groups t-tests are used. The t-statistic (referred as "T-Stat" in the following) is calculated without the assumption that the variances of test and reference are equal (Welch's t-test). For BS.1534 experiments, dependent group t-tests are used. T-Stat is compared to t(M), the single-sided 95% confidence level of the inverse t-distribution, which depends on the degree of freedom M (related to the number of votes per condition). The check result R is then determined as follows:

R=BT                     T-Stat >t(M)NWT-tM≤T-Stat ≤t(M)WT                T-Stat <-t(M)	(1)

Note that this approach, which considers both lower and higher means, is equivalent to a two-sided statistical test at 90% confidence interval.

Result R indicates the following:

- NWT: CuT "not worse than" Reference applies if the statistical test does not identify a significant difference between score means.

- BT: CuT "better than" Reference applies if the statistical test indicates a significant higher mean score in the CuT.

- WT: CuT "worse than" Reference applies if the statistical test indicates a significant lower mean score in the CuT.

# Results for P.800 DCR experiments

## Overview

The P.800 DRC listening experiments target multiple aspects. Most experience aims to investigate subjective performance over varying bitrates. All experiments aim to compare floating point (FL) and fixed point (FX) reference implementations of the IVAS codec. The experiments were conducted by nine different listening labs with source content in nine different languages, as specified in Table 9 in IVAS-8b [1]. For these experiments, naïve listeners have been selected that are native speakers of the respective language.

In the following, for each of the 23 experiments, the aggregated results for each condition are reported, comprising the number of votes (COUNT), mean opinion scores (MOS), standard deviations (STD) and the confidence interval of the MOS (CI95). Moreover, statistical test results are shown which compare the subjective scores of the CuT against the scores for specified reference conditions.

Further results for each experiment are available as an electronic attachment to this document. They are provided in Excel files “P800-X.xlsx”, where X is the experiment number. Each file contains the following sheets:

- Raw: The individual votes for each item as provided by the respective listening lab.

- Condition+Category+Sample+Level: Aggregated results for each condition, category, and sample, i.e., raw results aggregated over the five votes per sample. Moreover, the respective input level from Table 5 in IVAS-8b [1] is stated (5 votes).

- Condition+Category: Aggregated results for each condition and category, i.e., raw results aggregated over votes and samples (30 votes).

- Condition: Aggregated results for each condition, i.e., raw results aggregated over categories, samples, and votes (180 votes).

- Condition+Level: Aggregated results for each condition and input level contained in the categories as stated in Table 5 in IVAS-8b [1], i.e., raw results aggregated over categories and samples with same input level and votes (60 votes).

- CodecsAggregated: Aggregated results over all FL/FX variants (number of votes depends on experiment and condition).

## P800-1: Stereo (FX, RD)

Experiment P800-1 was conducted by VoiceAge Corporation (language: FR).

The aggregated results per condition are listed in Table 3.1-1 and are shown in Figure 3.1-1. The results aggregated over FX/FL are shown in Figure 3.1-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.1-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.1-1: results per condition for experiment P800-1

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.63 | 0.60 | 0.09 |
| c02 | MNRU Q = 27dB | 180 | 3.33 | 1.24 | 0.18 |
| c03 | MNRU Q = 22 dB | 180 | 2.51 | 1.36 | 0.20 |
| c04 | MNRU Q = 17 dB | 180 | 1.56 | 0.98 | 0.14 |
| c05 | MNRU Q = 12 dB | 180 | 1.16 | 0.51 | 0.07 |
| c06 | ESDRU α=0.7 | 180 | 4.02 | 0.81 | 0.12 |
| c07 | ESDRU α=0.5 | 180 | 3.03 | 1.09 | 0.16 |
| c08 | ESDRU α=0.3 | 180 | 2.64 | 1.11 | 0.16 |
| c09 | ESDRU α=0.1 | 180 | 2.02 | 0.98 | 0.14 |
| c10 | IVAS FL enc / FX dec, DTX off, 13.2 kbps | 180 | 2.73 | 1.18 | 0.17 |
| c11 | IVAS FX enc / FL dec, DTX off, 16.4 kbps | 180 | 3.03 | 1.07 | 0.16 |
| c12 | IVAS FL enc / FX dec, DTX off, 24.4 kbps | 180 | 3.61 | 0.95 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX off, 32 kbps | 180 | 3.92 | 0.94 | 0.14 |
| c14 | IVAS FL enc / FX dec, DTX off, 48 kbps | 180 | 4.35 | 0.80 | 0.12 |
| c15 | IVAS FX enc / FL dec, DTX off, 64 kbps | 180 | 4.57 | 0.64 | 0.09 |
| c16 | IVAS FL enc / FX dec, DTX off, 80 kbps | 180 | 4.61 | 0.60 | 0.09 |
| c17 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.56 | 0.66 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.62 | 0.60 | 0.09 |
| c19 | IVAS FL, DTX off, 13.2 kbps | 180 | 2.66 | 1.09 | 0.16 |
| c20 | IVAS FL, DTX off, 16.4 kbps | 180 | 3.23 | 1.11 | 0.16 |
| c21 | IVAS FL, DTX off, 24.4 kbps | 180 | 3.73 | 0.94 | 0.14 |
| c22 | IVAS FL, DTX off, 32 kbps | 180 | 3.82 | 0.97 | 0.14 |
| c23 | IVAS FL, DTX off, 48 kbps | 180 | 4.28 | 0.83 | 0.12 |
| c24 | IVAS FL, DTX off, 64 kbps | 180 | 4.48 | 0.66 | 0.10 |
| c25 | IVAS FL, DTX off, 80 kbps | 180 | 4.54 | 0.68 | 0.10 |
| c26 | IVAS FL, DTX off, 96 kbps | 180 | 4.61 | 0.57 | 0.08 |
| c27 | IVAS FL, DTX off, 128 kbps | 180 | 4.62 | 0.61 | 0.09 |
| c28 | IVAS FX, DTX off, 13.2 kbps | 180 | 2.60 | 1.09 | 0.16 |
| c29 | IVAS FX, DTX off, 16.4 kbps | 180 | 3.19 | 1.08 | 0.16 |
| c30 | IVAS FX, DTX off, 24.4 kbps | 180 | 3.65 | 0.99 | 0.14 |
| c31 | IVAS FX, DTX off, 32 kbps | 180 | 3.78 | 1.02 | 0.15 |
| c32 | IVAS FX, DTX off, 48 kbps | 180 | 4.33 | 0.72 | 0.11 |
| c33 | IVAS FX, DTX off, 64 kbps | 180 | 4.45 | 0.63 | 0.09 |
| c34 | IVAS FX, DTX off, 80 kbps | 180 | 4.59 | 0.60 | 0.09 |
| c35 | IVAS FX, DTX off, 96 kbps | 180 | 4.54 | 0.68 | 0.10 |
| c36 | IVAS FX, DTX off, 128 kbps | 180 | 4.59 | 0.58 | 0.09 |

Figure 3.1-1: results per condition for experiment P800-1

Figure 3.1-2: results aggregated over FX/FL for experiment P800-1

Table 3.1-2: statistical significance analysis for experiment P800-1

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 2.73 | 1.18 | c19 | 2.66 | 1.09 | 0.07 | 0.558 | 0.288 | NWT |
| c11 | 3.03 | 1.07 | c20 | 3.23 | 1.11 | -0.20 | <0.001 | 0.954 | WT |
| c12 | 3.61 | 0.95 | c21 | 3.73 | 0.94 | -0.12 | <0.001 | 0.879 | NWT |
| c13 | 3.92 | 0.94 | c22 | 3.82 | 0.97 | 0.10 | 0.993 | 0.161 | NWT |
| c14 | 4.35 | 0.80 | c23 | 4.28 | 0.83 | 0.07 | 0.777 | 0.219 | NWT |
| c15 | 4.57 | 0.64 | c24 | 4.48 | 0.66 | 0.08 | 1.227 | 0.110 | NWT |
| c16 | 4.61 | 0.60 | c25 | 4.54 | 0.68 | 0.07 | 1.064 | 0.144 | NWT |
| c17 | 4.56 | 0.66 | c26 | 4.61 | 0.57 | -0.05 | <0.001 | 0.778 | NWT |
| c18 | 4.62 | 0.60 | c27 | 4.62 | 0.61 | 0.00 | <0.001 | 0.500 | NWT |
| c28 | 2.60 | 1.09 | c19 | 2.66 | 1.09 | -0.06 | <0.001 | 0.702 | NWT |
| c29 | 3.19 | 1.08 | c20 | 3.23 | 1.11 | -0.04 | <0.001 | 0.632 | NWT |
| c30 | 3.65 | 0.99 | c21 | 3.73 | 0.94 | -0.08 | <0.001 | 0.778 | NWT |
| c31 | 3.78 | 1.02 | c22 | 3.82 | 0.97 | -0.04 | <0.001 | 0.645 | NWT |
| c32 | 4.33 | 0.72 | c23 | 4.28 | 0.83 | 0.05 | 0.607 | 0.272 | NWT |
| c33 | 4.45 | 0.63 | c24 | 4.48 | 0.66 | -0.03 | <0.001 | 0.686 | NWT |
| c34 | 4.59 | 0.60 | c25 | 4.54 | 0.68 | 0.05 | 0.742 | 0.229 | NWT |
| c35 | 4.54 | 0.68 | c26 | 4.61 | 0.57 | -0.07 | <0.001 | 0.861 | NWT |
| c36 | 4.59 | 0.58 | c27 | 4.62 | 0.61 | -0.03 | <0.001 | 0.701 | NWT |

## P800-2: Stereo (DTX, FE)

Experiment P800-2 was conducted by NTT (language: JAP).

The aggregated results per condition are listed in Table 3.2-1 and are shown in Figure 3.2-1. The results aggregated over FX/FL are shown in Figure 3.2-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.2-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.2-1: results per condition for experiment P800-2

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.85 | 0.40 | 0.06 |
| c02 | MNRU Q = 27 dB | 180 | 3.96 | 1.01 | 0.15 |
| c03 | MNRU Q = 22 dB | 180 | 3.02 | 1.22 | 0.18 |
| c04 | MNRU Q = 17 dB | 180 | 1.91 | 0.99 | 0.14 |
| c05 | MNRU Q = 12 dB | 180 | 1.24 | 0.52 | 0.08 |
| c06 | ESDRU α=0.7 | 180 | 4.44 | 0.75 | 0.11 |
| c07 | ESDRU α=0.5 | 180 | 3.16 | 1.13 | 0.17 |
| c08 | ESDRU α=0.3 | 180 | 3.02 | 1.10 | 0.16 |
| c09 | ESDRU α=0.1 | 180 | 2.28 | 0.95 | 0.14 |
| c10 | IVAS FL enc / FX dec, DTX on, FER 5%, 13.2 kbps | 180 | 2.61 | 1.03 | 0.15 |
| c11 | IVAS FX enc / FL dec, DTX on, FER 5%, 16.4 kbps | 180 | 3.04 | 0.93 | 0.14 |
| c12 | IVAS FL enc / FX dec, DTX on, FER 5%, 24.4 kbps | 180 | 3.33 | 0.93 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX on, FER 5%, 32 kbps | 180 | 3.66 | 0.77 | 0.11 |
| c14 | IVAS FL enc / FX dec, DTX on, FER 5%, 48 kbps | 180 | 3.76 | 0.84 | 0.12 |
| c15 | IVAS FX enc / FL dec, DTX on, FER 5%, 64 kbps | 180 | 3.89 | 0.78 | 0.12 |
| c16 | IVAS FL enc / FX dec, DTX on, FER 5%, 80 kbps | 180 | 3.89 | 0.80 | 0.12 |
| c17 | IVAS FX enc / FL dec, DTX on, FER 5%, 96 kbps | 180 | 3.91 | 0.77 | 0.11 |
| c18 | IVAS FL enc / FX dec, DTX on, FER 5%, 128 kbps | 180 | 3.82 | 0.76 | 0.11 |
| c19 | IVAS FL, DTX on, FER 5%, 13.2 kbps | 180 | 2.64 | 1.01 | 0.15 |
| c20 | IVAS FL, DTX on, FER 5%, 16.4 kbps | 180 | 3.08 | 1.03 | 0.15 |
| c21 | IVAS FL, DTX on, FER 5%, 24.4 kbps | 180 | 3.38 | 1.00 | 0.15 |
| c22 | IVAS FL, DTX on, FER 5%, 32 kbps | 180 | 3.68 | 0.84 | 0.12 |
| c23 | IVAS FL, DTX on, FER 5%, 48 kbps | 180 | 3.77 | 0.83 | 0.12 |
| c24 | IVAS FL, DTX on, FER 5%, 64 kbps | 180 | 3.84 | 0.78 | 0.11 |
| c25 | IVAS FL, DTX on, FER 5%, 80 kbps | 180 | 3.97 | 0.75 | 0.11 |
| c26 | IVAS FL, DTX on, FER 5%, 96 kbps | 180 | 3.94 | 0.74 | 0.11 |
| c27 | IVAS FL, DTX on, FER 5%, 128 kbps | 180 | 3.92 | 0.78 | 0.12 |
| c28 | IVAS FX, DTX on, FER 5%, 13.2 kbps | 180 | 2.62 | 1.02 | 0.15 |
| c29 | IVAS FX, DTX on, FER 5%, 16.4 kbps | 180 | 3.12 | 0.95 | 0.14 |
| c30 | IVAS FX, DTX on, FER 5%, 24.4 kbps | 180 | 3.33 | 0.99 | 0.14 |
| c31 | IVAS FX, DTX on, FER 5%, 32 kbps | 180 | 3.58 | 0.84 | 0.12 |
| c32 | IVAS FX, DTX on, FER 5%, 48 kbps | 180 | 3.78 | 0.76 | 0.11 |
| c33 | IVAS FX, DTX on, FER 5%, 64 kbps | 180 | 3.82 | 0.76 | 0.11 |
| c34 | IVAS FX, DTX on, FER 5%, 80 kbps | 180 | 3.91 | 0.76 | 0.11 |
| c35 | IVAS FX, DTX on, FER 5%, 96 kbps | 180 | 3.88 | 0.75 | 0.11 |
| c36 | IVAS FX, DTX on, FER 5%, 128 kbps | 180 | 3.87 | 0.81 | 0.12 |

Figure 3.2-1: results per condition for experiment P800-2

Figure 3.2-2: results aggregated over FX/FL for experiment P800-2

Table 3.2-2: statistical significance analysis for experiment P800-2

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 2.61 | 1.03 | c19 | 2.64 | 1.01 | -0.03 | <0.001 | 0.603 | NWT |
| c11 | 3.04 | 0.93 | c20 | 3.08 | 1.03 | -0.04 | <0.001 | 0.647 | NWT |
| c12 | 3.33 | 0.93 | c21 | 3.38 | 1.00 | -0.06 | <0.001 | 0.705 | NWT |
| c13 | 3.66 | 0.77 | c22 | 3.68 | 0.84 | -0.02 | <0.001 | 0.602 | NWT |
| c14 | 3.76 | 0.84 | c23 | 3.77 | 0.83 | -0.01 | <0.001 | 0.550 | NWT |
| c15 | 3.89 | 0.78 | c24 | 3.84 | 0.78 | 0.06 | 0.670 | 0.252 | NWT |
| c16 | 3.89 | 0.80 | c25 | 3.97 | 0.75 | -0.07 | <0.001 | 0.815 | NWT |
| c17 | 3.91 | 0.77 | c26 | 3.94 | 0.74 | -0.03 | <0.001 | 0.661 | NWT |
| c18 | 3.82 | 0.76 | c27 | 3.92 | 0.78 | -0.10 | <0.001 | 0.891 | NWT |
| c28 | 2.62 | 1.02 | c19 | 2.64 | 1.01 | -0.02 | <0.001 | 0.581 | NWT |
| c29 | 3.12 | 0.95 | c20 | 3.08 | 1.03 | 0.04 | 0.372 | 0.355 | NWT |
| c30 | 3.33 | 0.99 | c21 | 3.38 | 1.00 | -0.05 | <0.001 | 0.683 | NWT |
| c31 | 3.58 | 0.84 | c22 | 3.68 | 0.84 | -0.10 | <0.001 | 0.869 | NWT |
| c32 | 3.78 | 0.76 | c23 | 3.77 | 0.83 | 0.02 | 0.191 | 0.424 | NWT |
| c33 | 3.82 | 0.76 | c24 | 3.84 | 0.78 | -0.02 | <0.001 | 0.583 | NWT |
| c34 | 3.91 | 0.76 | c25 | 3.97 | 0.75 | -0.06 | <0.001 | 0.760 | NWT |
| c35 | 3.88 | 0.75 | c26 | 3.94 | 0.74 | -0.06 | <0.001 | 0.782 | NWT |
| c36 | 3.87 | 0.81 | c27 | 3.92 | 0.78 | -0.05 | <0.001 | 0.724 | NWT |

## P800-3: FOA (FX, RD)

Experiment P800-3 was conducted by Orange (language: FR).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations. See listening lab report in S4-251692.

The aggregated results per condition are listed in Table 3.3-1 and are shown in Figure 3.3-1. The results aggregated over FX/FL are shown in Figure 3.3-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.3-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.3-1: results per condition for experiment P800-3

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.64 | 0.60 | 0.09 |
| c02 | MNRU Q = 32 dB | 180 | 4.17 | 0.88 | 0.13 |
| c03 | MNRU Q = 27 dB | 180 | 3.18 | 1.20 | 0.18 |
| c04 | MNRU Q = 22 dB | 180 | 2.19 | 1.25 | 0.18 |
| c05 | MNRU Q = 17 dB | 180 | 1.62 | 0.93 | 0.14 |
| c06 | ESDRU α = 0.8 | 180 | 4.62 | 0.64 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 3.58 | 0.93 | 0.14 |
| c08 | ESDRU α = 0.4 | 180 | 2.73 | 1.11 | 0.16 |
| c09 | ESDRU α = 0.2 | 180 | 2.34 | 1.15 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX off, 13.2 kbps | 179 | 3.13 | 1.10 | 0.16 |
| c11 | IVAS FL enc / FX dec, DTX off, 16.4 kbps | 180 | 3.53 | 1.05 | 0.15 |
| c12 | IVAS FX enc / FL dec, DTX off, 24.4 kbps | 180 | 3.87 | 0.89 | 0.13 |
| c13 | IVAS FL enc / FX dec, DTX off, 32 kbps | 179 | 4.06 | 0.79 | 0.12 |
| c14 | IVAS FX enc / FL dec, DTX off, 48 kbps | 180 | 4.36 | 0.77 | 0.11 |
| c15 | IVAS FL enc / FX dec, DTX off, 64 kbps | 176 | 4.49 | 0.68 | 0.10 |
| c16 | IVAS FL enc / FX dec, DTX off, 96 kbps | 180 | 4.64 | 0.60 | 0.09 |
| c17 | IVAS FX enc / FL dec, DTX off, 128 kbps | 180 | 4.64 | 0.57 | 0.08 |
| c18 | IVAS FL enc / FX dec, DTX off, 256 kbps | 180 | 4.64 | 0.60 | 0.09 |
| c19 | IVAS FL, DTX off, 13.2 kbps | 180 | 3.22 | 1.11 | 0.16 |
| c20 | IVAS FL, DTX off, 16.4 kbps | 180 | 3.64 | 0.95 | 0.14 |
| c21 | IVAS FL, DTX off, 24.4 kbps | 180 | 3.96 | 0.89 | 0.13 |
| c22 | IVAS FL, DTX off, 32 kbps | 180 | 4.12 | 0.83 | 0.12 |
| c23 | IVAS FL, DTX off, 48 kbps | 180 | 4.30 | 0.73 | 0.11 |
| c24 | IVAS FL, DTX off, 64 kbps | 180 | 4.43 | 0.68 | 0.10 |
| c25 | IVAS FL, DTX off, 96 kbps | 180 | 4.66 | 0.54 | 0.08 |
| c26 | IVAS FL, DTX off, 128 kbps | 180 | 4.68 | 0.56 | 0.08 |
| c27 | IVAS FL, DTX off, 256 kbps | 180 | 4.62 | 0.60 | 0.09 |
| c28 | IVAS FX, DTX off, 13.2 kbps | 180 | 3.18 | 1.11 | 0.16 |
| c29 | IVAS FX, DTX off, 16.4 kbps | 180 | 3.62 | 0.96 | 0.14 |
| c30 | IVAS FX, DTX off, 24.4 kbps | 180 | 3.81 | 0.90 | 0.13 |
| c31 | IVAS FX, DTX off, 32 kbps | 180 | 4.13 | 0.78 | 0.12 |
| c32 | IVAS FX, DTX off, 48 kbps | 180 | 4.40 | 0.71 | 0.10 |
| c33 | IVAS FX, DTX off, 64 kbps | 180 | 4.38 | 0.76 | 0.11 |
| c34 | IVAS FX, DTX off, 96 kbps | 180 | 4.56 | 0.63 | 0.09 |
| c35 | IVAS FX, DTX off, 128 kbps | 180 | 4.64 | 0.58 | 0.09 |
| c36 | IVAS FX, DTX off, 256 kbps | 180 | 4.72 | 0.48 | 0.07 |

Figure 3.3-1: results per condition for experiment P800-3

Figure 3.3-2: results aggregated over FX/FL for experiment P800-3

Table 3.3-2: statistical significance analysis for experiment P800-3

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.13 | 1.10 | c19 | 3.22 | 1.11 | -0.09 | <0.001 | 0.774 | NWT |
| c11 | 3.53 | 1.05 | c20 | 3.64 | 0.95 | -0.12 | <0.001 | 0.863 | NWT |
| c12 | 3.87 | 0.89 | c21 | 3.96 | 0.89 | -0.09 | <0.001 | 0.842 | NWT |
| c13 | 4.06 | 0.79 | c22 | 4.12 | 0.83 | -0.06 | <0.001 | 0.743 | NWT |
| c14 | 4.36 | 0.77 | c23 | 4.30 | 0.73 | 0.06 | 0.771 | 0.220 | NWT |
| c15 | 4.49 | 0.68 | c24 | 4.43 | 0.68 | 0.06 | 0.776 | 0.219 | NWT |
| c16 | 4.64 | 0.60 | c25 | 4.66 | 0.54 | -0.02 | <0.001 | 0.643 | NWT |
| c17 | 4.64 | 0.57 | c26 | 4.68 | 0.56 | -0.04 | <0.001 | 0.770 | NWT |
| c18 | 4.64 | 0.60 | c27 | 4.62 | 0.60 | 0.02 | 0.346 | 0.365 | NWT |
| c28 | 3.18 | 1.11 | c19 | 3.22 | 1.11 | -0.04 | <0.001 | 0.630 | NWT |
| c29 | 3.62 | 0.96 | c20 | 3.64 | 0.95 | -0.02 | <0.001 | 0.586 | NWT |
| c30 | 3.81 | 0.90 | c21 | 3.96 | 0.89 | -0.15 | <0.001 | 0.949 | NWT |
| c31 | 4.13 | 0.78 | c22 | 4.12 | 0.83 | 0.01 | 0.129 | 0.449 | NWT |
| c32 | 4.40 | 0.71 | c23 | 4.30 | 0.73 | 0.10 | 1.312 | 0.095 | NWT |
| c33 | 4.38 | 0.76 | c24 | 4.43 | 0.68 | -0.05 | <0.001 | 0.766 | NWT |
| c34 | 4.56 | 0.63 | c25 | 4.66 | 0.54 | -0.10 | <0.001 | 0.947 | NWT |
| c35 | 4.64 | 0.58 | c26 | 4.68 | 0.56 | -0.04 | <0.001 | 0.768 | NWT |
| c36 | 4.72 | 0.48 | c27 | 4.62 | 0.60 | 0.11 | 1.824 | 0.035 | BT |

## P800-4: HOA2 (DTX, FE)

Experiment P800-4 was conducted by Orange (language: FR).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations. See listening lab report in S4-251692.

The aggregated results per condition are listed in Table 3.4-1 and are shown in Figure 3.4-1. The results aggregated over FX/FL are shown in Figure 3.4-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.4-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.4-1: results per condition for experiment P800-4

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.69 | 0.55 | 0.08 |
| c02 | MNRU Q = 32 dB | 180 | 3.65 | 1.05 | 0.15 |
| c03 | MNRU Q = 27 dB | 180 | 2.73 | 1.22 | 0.18 |
| c04 | MNRU Q = 22 dB | 179 | 1.98 | 1.15 | 0.17 |
| c05 | MNRU Q = 17 dB | 179 | 1.51 | 0.90 | 0.13 |
| c06 | ESDRU α = 0.8 | 180 | 4.58 | 0.63 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 3.63 | 1.01 | 0.15 |
| c08 | ESDRU α = 0.4 | 179 | 2.96 | 1.14 | 0.17 |
| c09 | ESDRU α = 0.2 | 180 | 2.58 | 1.17 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX On, FER 5%, 16.4 kbps | 180 | 3.23 | 1.15 | 0.17 |
| c11 | IVAS FX enc / FL dec, DTX On, FER 5%, 24.4 kbps | 180 | 3.36 | 1.06 | 0.16 |
| c12 | IVAS FL enc / FX dec, DTX On, FER 5%, 32 kbps | 180 | 3.62 | 1.00 | 0.15 |
| c13 | IVAS FX enc / FL dec, DTX On, FER 5%, 48 kbps | 179 | 4.11 | 0.90 | 0.13 |
| c14 | IVAS FL enc / FX dec, DTX On, FER 5%, 64 kbps | 180 | 4.14 | 0.85 | 0.12 |
| c15 | IVAS FX enc / FL dec, DTX Off, FER 5%, 96 kbps | 180 | 4.29 | 0.79 | 0.12 |
| c16 | IVAS FL enc / FX dec, DTX Off, FER 5%, 128 kbps | 179 | 4.28 | 0.81 | 0.12 |
| c17 | IVAS FX enc / FL dec, DTX Off, FER 5%, 256 kbps | 180 | 4.43 | 0.65 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX Off, FER 5%, 384 kbps | 180 | 4.31 | 0.80 | 0.12 |
| c19 | IVAS FL, DTX On, FER 5%, 16.4 kbps | 180 | 3.39 | 1.09 | 0.16 |
| c20 | IVAS FL, DTX On, FER 5%, 24.4 kbps | 180 | 3.29 | 1.05 | 0.15 |
| c21 | IVAS FL, DTX On, FER 5%, 32 kbps | 180 | 3.56 | 1.10 | 0.16 |
| c22 | IVAS FL, DTX On, FER 5%, 48 kbps | 180 | 3.97 | 0.87 | 0.13 |
| c23 | IVAS FL, DTX On, FER 5%, 64 kbps | 180 | 4.17 | 0.82 | 0.12 |
| c24 | IVAS FL, DTX Off, FER 5%, 96 kbps | 180 | 4.34 | 0.71 | 0.10 |
| c25 | IVAS FL, DTX Off, FER 5%, 128 kbps | 180 | 4.36 | 0.67 | 0.10 |
| c26 | IVAS FL, DTX Off, FER 5%, 256 kbps | 180 | 4.32 | 0.78 | 0.12 |
| c27 | IVAS FL, DTX Off, FER 5%, 384 kbps | 180 | 4.32 | 0.76 | 0.11 |
| c28 | IVAS FX, DTX On, FER 5%, 16.4 kbps | 180 | 3.22 | 1.15 | 0.17 |
| c29 | IVAS FX, DTX On, FER 5%, 24.4 kbps | 180 | 3.41 | 1.06 | 0.15 |
| c30 | IVAS FX, DTX On, FER 5%, 32 kbps | 180 | 3.59 | 0.95 | 0.14 |
| c31 | IVAS FX, DTX On, FER 5%, 48 kbps | 180 | 3.98 | 0.90 | 0.13 |
| c32 | IVAS FX, DTX On, FER 5%, 64 kbps | 180 | 4.18 | 0.84 | 0.12 |
| c33 | IVAS FX, DTX Off, FER 5%, 96 kbps | 180 | 4.27 | 0.77 | 0.11 |
| c34 | IVAS FX, DTX Off, FER 5%, 128 kbps | 180 | 4.37 | 0.80 | 0.12 |
| c35 | IVAS FX, DTX Off, FER 5%, 256 kbps | 180 | 4.40 | 0.65 | 0.10 |
| c36 | IVAS FX, DTX Off, FER 5%, 384 kbps | 179 | 4.41 | 0.67 | 0.10 |

Figure 3.4-1: results per condition for experiment P800-4

Figure 3.4-2: results aggregated over FX/FL for experiment P800-4

Table 3.4-2: statistical significance analysis for experiment P800-4

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.23 | 1.15 | c19 | 3.39 | 1.09 | -0.16 | <0.001 | 0.913 | NWT |
| c11 | 3.36 | 1.06 | c20 | 3.29 | 1.05 | 0.07 | 0.602 | 0.274 | NWT |
| c12 | 3.62 | 1.00 | c21 | 3.56 | 1.10 | 0.06 | 0.550 | 0.291 | NWT |
| c13 | 4.11 | 0.90 | c22 | 3.97 | 0.87 | 0.13 | 1.433 | 0.076 | NWT |
| c14 | 4.14 | 0.85 | c23 | 4.17 | 0.82 | -0.03 | <0.001 | 0.625 | NWT |
| c15 | 4.29 | 0.79 | c24 | 4.34 | 0.71 | -0.06 | <0.001 | 0.755 | NWT |
| c16 | 4.28 | 0.81 | c25 | 4.36 | 0.67 | -0.08 | <0.001 | 0.836 | NWT |
| c17 | 4.43 | 0.65 | c26 | 4.32 | 0.78 | 0.11 | 1.464 | 0.072 | NWT |
| c18 | 4.31 | 0.80 | c27 | 4.32 | 0.76 | -0.01 | <0.001 | 0.553 | NWT |
| c28 | 3.22 | 1.15 | c19 | 3.39 | 1.09 | -0.18 | <0.001 | 0.932 | NWT |
| c29 | 3.41 | 1.06 | c20 | 3.29 | 1.05 | 0.12 | 1.054 | 0.146 | NWT |
| c30 | 3.59 | 0.95 | c21 | 3.56 | 1.10 | 0.03 | 0.259 | 0.398 | NWT |
| c31 | 3.98 | 0.90 | c22 | 3.97 | 0.87 | 0.01 | 0.118 | 0.453 | NWT |
| c32 | 4.18 | 0.84 | c23 | 4.17 | 0.82 | 0.01 | 0.068 | 0.473 | NWT |
| c33 | 4.27 | 0.77 | c24 | 4.34 | 0.71 | -0.08 | <0.001 | 0.838 | NWT |
| c34 | 4.37 | 0.80 | c25 | 4.36 | 0.67 | 0.01 | 0.141 | 0.444 | NWT |
| c35 | 4.40 | 0.65 | c26 | 4.32 | 0.78 | 0.08 | 1.031 | 0.152 | NWT |
| c36 | 4.41 | 0.67 | c27 | 4.32 | 0.76 | 0.09 | 1.205 | 0.114 | NWT |

## P800-5: HOA3 (FX, RD)

Experiment P800-5 was conducted by Dolby Laboratories, Inc. (language: ENG).

The aggregated results per condition are listed in Table 3.5-1 and are shown in Figure 3.5-1. The results aggregated over FX/FL are shown in Figure 3.5-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.5-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.5-1: results per condition for experiment P800-5

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.72 | 0.51 | 0.07 |
| c02 | MNRU Q = 32 dB | 180 | 3.69 | 1.09 | 0.16 |
| c03 | MNRU Q = 27 dB | 180 | 2.83 | 1.17 | 0.17 |
| c04 | MNRU Q = 22 dB | 180 | 2.07 | 1.19 | 0.17 |
| c05 | MNRU Q = 17 dB | 180 | 1.68 | 1.03 | 0.15 |
| c06 | ESDRU α = 0.8 | 180 | 4.51 | 0.63 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 3.70 | 1.02 | 0.15 |
| c08 | ESDRU α = 0.4 | 180 | 3.11 | 1.23 | 0.18 |
| c09 | ESDRU α = 0.2 | 180 | 2.49 | 1.15 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX off, 32 kbps | 180 | 4.26 | 0.81 | 0.12 |
| c11 | IVAS FX enc / FL dec, DTX off, 48 kbps | 180 | 4.41 | 0.70 | 0.10 |
| c12 | IVAS FL enc / FX dec, DTX off, 64 kbps | 180 | 4.45 | 0.69 | 0.10 |
| c13 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.60 | 0.68 | 0.10 |
| c14 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.65 | 0.57 | 0.08 |
| c15 | IVAS FX enc / FL dec, DTX off, 160 kbps | 180 | 4.67 | 0.58 | 0.09 |
| c16 | IVAS FL enc / FX dec, DTX off, 256 kbps | 180 | 4.67 | 0.60 | 0.09 |
| c17 | IVAS FX enc / FL dec, DTX off, 384 kbps | 180 | 4.67 | 0.53 | 0.08 |
| c18 | IVAS FL enc / FX dec, DTX off, 512 kbps | 180 | 4.66 | 0.61 | 0.09 |
| c19 | IVAS FL, DTX off, 32 kbps | 180 | 4.16 | 0.81 | 0.12 |
| c20 | IVAS FL, DTX off, 48 kbps | 180 | 4.35 | 0.74 | 0.11 |
| c21 | IVAS FL, DTX off, 64 kbps | 180 | 4.47 | 0.66 | 0.10 |
| c22 | IVAS FL, DTX off, 96 kbps | 180 | 4.56 | 0.68 | 0.10 |
| c23 | IVAS FL, DTX off, 128 kbps | 180 | 4.60 | 0.65 | 0.10 |
| c24 | IVAS FL, DTX off, 160 kbps | 180 | 4.67 | 0.58 | 0.09 |
| c25 | IVAS FL, DTX off, 256 kbps | 180 | 4.72 | 0.56 | 0.08 |
| c26 | IVAS FL, DTX off, 384 kbps | 180 | 4.66 | 0.58 | 0.09 |
| c27 | IVAS FL, DTX off, 512 kbps | 180 | 4.59 | 0.61 | 0.09 |
| c28 | IVAS FX, DTX off, 32 kbps | 180 | 4.16 | 0.80 | 0.12 |
| c29 | IVAS FX, DTX off, 48 kbps | 180 | 4.36 | 0.81 | 0.12 |
| c30 | IVAS FX, DTX off, 64 kbps | 180 | 4.46 | 0.72 | 0.11 |
| c31 | IVAS FX, DTX off, 96 kbps | 180 | 4.66 | 0.53 | 0.08 |
| c32 | IVAS FX, DTX off, 128 kbps | 180 | 4.72 | 0.48 | 0.07 |
| c33 | IVAS FX, DTX off, 160 kbps | 180 | 4.69 | 0.52 | 0.08 |
| c34 | IVAS FX, DTX off, 256 kbps | 180 | 4.59 | 0.65 | 0.10 |
| c35 | IVAS FX, DTX off, 384 kbps | 180 | 4.58 | 0.66 | 0.10 |
| c36 | IVAS FX, DTX off, 512 kbps | 180 | 4.70 | 0.55 | 0.08 |

Figure 3.5-1: results per condition for experiment P800-5

Figure 3.5-2: results aggregated over FX/FL for experiment P800-5

Table 3.5-2: statistical significance analysis for experiment P800-5

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 4.26 | 0.81 | c19 | 4.16 | 0.81 | 0.10 | 1.176 | 0.120 | NWT |
| c11 | 4.41 | 0.70 | c20 | 4.35 | 0.74 | 0.06 | 0.806 | 0.210 | NWT |
| c12 | 4.45 | 0.69 | c21 | 4.47 | 0.66 | -0.02 | <0.001 | 0.594 | NWT |
| c13 | 4.60 | 0.68 | c22 | 4.56 | 0.68 | 0.04 | 0.544 | 0.293 | NWT |
| c14 | 4.65 | 0.57 | c23 | 4.60 | 0.65 | 0.05 | 0.775 | 0.219 | NWT |
| c15 | 4.67 | 0.58 | c24 | 4.67 | 0.58 | -0.00 | <0.001 | 0.533 | NWT |
| c16 | 4.67 | 0.60 | c25 | 4.72 | 0.56 | -0.06 | <0.001 | 0.816 | NWT |
| c17 | 4.67 | 0.53 | c26 | 4.66 | 0.58 | 0.01 | 0.188 | 0.426 | NWT |
| c18 | 4.66 | 0.61 | c27 | 4.59 | 0.61 | 0.07 | 1.039 | 0.150 | NWT |
| c28 | 4.16 | 0.80 | c19 | 4.16 | 0.81 | 0.00 | <0.001 | 0.500 | NWT |
| c29 | 4.36 | 0.81 | c20 | 4.35 | 0.74 | 0.01 | 0.074 | 0.471 | NWT |
| c30 | 4.46 | 0.72 | c21 | 4.47 | 0.66 | -0.01 | <0.001 | 0.533 | NWT |
| c31 | 4.66 | 0.53 | c22 | 4.56 | 0.68 | 0.09 | 1.479 | 0.070 | NWT |
| c32 | 4.72 | 0.48 | c23 | 4.60 | 0.65 | 0.12 | 2.022 | 0.022 | BT |
| c33 | 4.69 | 0.52 | c24 | 4.67 | 0.58 | 0.02 | 0.380 | 0.352 | NWT |
| c34 | 4.59 | 0.65 | c25 | 4.72 | 0.56 | -0.13 | <0.001 | 0.981 | WT |
| c35 | 4.58 | 0.66 | c26 | 4.66 | 0.58 | -0.08 | <0.001 | 0.882 | NWT |
| c36 | 4.70 | 0.55 | c27 | 4.59 | 0.61 | 0.11 | 1.728 | 0.042 | BT |

## P800-6: MC 5.1, 7.1 (FX, RD)

Experiment P800-6 was conducted by Mesaqin.com (language: MAN).

The listening lab has reported that for each panel, up to seven instead of five participants participated. The five subjects with the highest PCC were selected for result submission.

The aggregated results per condition are listed in Table 3.6-1 and are shown in Figure 3.6-1. The results aggregated over FX/FL are shown in Figure 3.6-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.6-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.6-1: results per condition for experiment P800-6

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.73 | 0.52 | 0.08 |
| c02 | MNRU Q = 32 dB | 180 | 3.82 | 1.11 | 0.16 |
| c03 | MNRU Q = 27 dB | 180 | 3.14 | 1.25 | 0.18 |
| c04 | MNRU Q = 22 dB | 180 | 2.22 | 1.18 | 0.17 |
| c05 | MNRU Q = 17 dB | 180 | 1.54 | 0.83 | 0.12 |
| c06 | ESDRU α = 0.8 | 180 | 4.32 | 0.83 | 0.12 |
| c07 | ESDRU α = 0.6 | 180 | 2.95 | 1.17 | 0.17 |
| c08 | ESDRU α = 0.4 | 180 | 2.08 | 1.10 | 0.16 |
| c09 | ESDRU α = 0.2 | 180 | 1.63 | 0.86 | 0.13 |
| c10 | IVAS FL enc / FX dec, DTX off, 24.4 kbps | 180 | 3.86 | 1.01 | 0.15 |
| c11 | IVAS FX enc / FL dec, DTX off, 32 kbps | 180 | 4.09 | 0.95 | 0.14 |
| c12 | IVAS FL enc / FX dec, DTX off, 48 kbps | 180 | 4.34 | 0.83 | 0.12 |
| c13 | IVAS FX enc / FL dec, DTX off, 64 kbps | 180 | 4.46 | 0.77 | 0.11 |
| c14 | IVAS FL enc / FX dec, DTX off, 80 kbps | 180 | 4.51 | 0.70 | 0.10 |
| c15 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.43 | 0.78 | 0.11 |
| c16 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.47 | 0.73 | 0.11 |
| c17 | IVAS FX enc / FL dec, DTX off, 160 kbps | 180 | 4.48 | 0.70 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX off, 256 kbps | 180 | 4.53 | 0.69 | 0.10 |
| c19 | IVAS FL, DTX off, 24.4 kbps | 180 | 3.95 | 1.00 | 0.15 |
| c20 | IVAS FL, DTX off, 32 kbps | 180 | 4.02 | 0.95 | 0.14 |
| c21 | IVAS FL, DTX off, 48 kbps | 180 | 4.43 | 0.83 | 0.12 |
| c22 | IVAS FL, DTX off, 64 kbps | 180 | 4.42 | 0.75 | 0.11 |
| c23 | IVAS FL, DTX off, 80 kbps | 180 | 4.49 | 0.72 | 0.11 |
| c24 | IVAS FL, DTX off, 96 kbps | 180 | 4.53 | 0.68 | 0.10 |
| c25 | IVAS FL, DTX off, 128 kbps | 180 | 4.52 | 0.72 | 0.11 |
| c26 | IVAS FL, DTX off, 160 kbps | 180 | 4.54 | 0.69 | 0.10 |
| c27 | IVAS FL, DTX off, 256 kbps | 180 | 4.53 | 0.71 | 0.10 |
| c28 | IVAS FX, DTX off, 24.4 kbps | 180 | 3.87 | 1.02 | 0.15 |
| c29 | IVAS FX, DTX off, 32 kbps | 180 | 4.00 | 1.05 | 0.15 |
| c30 | IVAS FX, DTX off, 48 kbps | 180 | 4.42 | 0.77 | 0.11 |
| c31 | IVAS FX, DTX off, 64 kbps | 180 | 4.44 | 0.76 | 0.11 |
| c32 | IVAS FX, DTX off, 80 kbps | 180 | 4.46 | 0.69 | 0.10 |
| c33 | IVAS FX, DTX off, 96 kbps | 180 | 4.50 | 0.68 | 0.10 |
| c34 | IVAS FX, DTX off, 128 kbps | 180 | 4.43 | 0.80 | 0.12 |
| c35 | IVAS FX, DTX off, 160 kbps | 180 | 4.48 | 0.67 | 0.10 |
| c36 | IVAS FX, DTX off, 256 kbps | 180 | 4.49 | 0.71 | 0.10 |

Figure 3.6-1: results per condition for experiment P800-6

Figure 3.6-2: results aggregated over FX/FL for experiment P800-6

Table 3.6-2: statistical significance analysis for experiment P800-6

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.86 | 1.01 | c19 | 3.95 | 1.00 | -0.09 | <0.001 | 0.811 | NWT |
| c11 | 4.09 | 0.95 | c20 | 4.02 | 0.95 | 0.07 | 0.719 | 0.236 | NWT |
| c12 | 4.34 | 0.83 | c21 | 4.43 | 0.83 | -0.09 | <0.001 | 0.858 | NWT |
| c13 | 4.46 | 0.77 | c22 | 4.42 | 0.75 | 0.03 | 0.425 | 0.336 | NWT |
| c14 | 4.51 | 0.70 | c23 | 4.49 | 0.72 | 0.02 | 0.226 | 0.411 | NWT |
| c15 | 4.43 | 0.78 | c24 | 4.53 | 0.68 | -0.10 | <0.001 | 0.903 | NWT |
| c16 | 4.47 | 0.73 | c25 | 4.52 | 0.72 | -0.04 | <0.001 | 0.722 | NWT |
| c17 | 4.48 | 0.70 | c26 | 4.54 | 0.69 | -0.07 | <0.001 | 0.815 | NWT |
| c18 | 4.53 | 0.69 | c27 | 4.53 | 0.71 | 0.00 | <0.001 | 0.500 | NWT |
| c28 | 3.87 | 1.02 | c19 | 3.95 | 1.00 | -0.08 | <0.001 | 0.781 | NWT |
| c29 | 4.00 | 1.05 | c20 | 4.02 | 0.95 | -0.02 | <0.001 | 0.564 | NWT |
| c30 | 4.42 | 0.77 | c21 | 4.43 | 0.83 | -0.01 | <0.001 | 0.552 | NWT |
| c31 | 4.44 | 0.76 | c22 | 4.42 | 0.75 | 0.02 | 0.215 | 0.415 | NWT |
| c32 | 4.46 | 0.69 | c23 | 4.49 | 0.72 | -0.03 | <0.001 | 0.671 | NWT |
| c33 | 4.50 | 0.68 | c24 | 4.53 | 0.68 | -0.03 | <0.001 | 0.652 | NWT |
| c34 | 4.43 | 0.80 | c25 | 4.52 | 0.72 | -0.08 | <0.001 | 0.852 | NWT |
| c35 | 4.48 | 0.67 | c26 | 4.54 | 0.69 | -0.06 | <0.001 | 0.802 | NWT |
| c36 | 4.49 | 0.71 | c27 | 4.53 | 0.71 | -0.04 | <0.001 | 0.698 | NWT |

## P800-7: MC 5.1+4, 7.1+4 (FX, RD)

Experiment P800-7 was conducted by FORCE Technology (language: DAN).

The aggregated results per condition are listed in Table 3.7-1 and are shown in Figure 3.7-1. The results aggregated over FX/FL are shown in Figure 3.7-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.7-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.7-1: results per condition for experiment P800-7

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.60 | 0.56 | 0.08 |
| c02 | MNRU Q = 32 dB | 180 | 3.61 | 1.07 | 0.16 |
| c03 | MNRU Q = 27 dB | 180 | 2.66 | 1.13 | 0.17 |
| c04 | MNRU Q = 22 dB | 180 | 1.90 | 0.95 | 0.14 |
| c05 | MNRU Q = 17 dB | 180 | 1.35 | 0.63 | 0.09 |
| c06 | ESDRU α = 0.8 | 180 | 4.45 | 0.69 | 0.10 |
| c07 | ESDRU α = 0.6 | 180 | 3.43 | 1.02 | 0.15 |
| c08 | ESDRU α = 0.4 | 180 | 2.80 | 1.04 | 0.15 |
| c09 | ESDRU α = 0.2 | 180 | 2.32 | 1.02 | 0.15 |
| c10 | IVAS FL enc / FX dec, DTX off, 32 kbps | 180 | 3.67 | 0.97 | 0.14 |
| c11 | IVAS FX enc / FL dec, DTX off, 48 kbps | 180 | 3.92 | 0.93 | 0.14 |
| c12 | IVAS FL enc / FX dec, DTX off, 64 kbps | 180 | 4.16 | 0.87 | 0.13 |
| c13 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.33 | 0.80 | 0.12 |
| c14 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.49 | 0.69 | 0.10 |
| c15 | IVAS FX enc / FL dec, DTX off, 160 kbps | 180 | 4.48 | 0.66 | 0.10 |
| c16 | IVAS FL enc / FX dec, DTX off, 192 kbps | 180 | 4.47 | 0.74 | 0.11 |
| c17 | IVAS FX enc / FL dec, DTX off, 256 kbps | 180 | 4.54 | 0.77 | 0.11 |
| c18 | IVAS FL enc / FX dec, DTX off, 384 kbps | 180 | 4.63 | 0.59 | 0.09 |
| c19 | IVAS FL, DTX off, 32 kbps | 180 | 3.69 | 1.04 | 0.15 |
| c20 | IVAS FL, DTX off, 48 kbps | 180 | 4.00 | 0.88 | 0.13 |
| c21 | IVAS FL, DTX off, 64 kbps | 180 | 4.11 | 0.82 | 0.12 |
| c22 | IVAS FL, DTX off, 96 kbps | 180 | 4.33 | 0.78 | 0.11 |
| c23 | IVAS FL, DTX off, 128 kbps | 180 | 4.47 | 0.77 | 0.11 |
| c24 | IVAS FL, DTX off, 160 kbps | 180 | 4.55 | 0.70 | 0.10 |
| c25 | IVAS FL, DTX off, 192 kbps | 180 | 4.42 | 0.69 | 0.10 |
| c26 | IVAS FL, DTX off, 256 kbps | 180 | 4.52 | 0.72 | 0.11 |
| c27 | IVAS FL, DTX off, 384 kbps | 180 | 4.66 | 0.63 | 0.09 |
| c28 | IVAS FX, DTX off, 32 kbps | 180 | 3.51 | 0.97 | 0.14 |
| c29 | IVAS FX, DTX off, 48 kbps | 180 | 4.09 | 0.85 | 0.13 |
| c30 | IVAS FX, DTX off, 64 kbps | 180 | 4.02 | 0.86 | 0.13 |
| c31 | IVAS FX, DTX off, 96 kbps | 180 | 4.33 | 0.75 | 0.11 |
| c32 | IVAS FX, DTX off, 128 kbps | 180 | 4.42 | 0.77 | 0.11 |
| c33 | IVAS FX, DTX off, 160 kbps | 180 | 4.46 | 0.71 | 0.10 |
| c34 | IVAS FX, DTX off, 192 kbps | 180 | 4.43 | 0.76 | 0.11 |
| c35 | IVAS FX, DTX off, 256 kbps | 180 | 4.54 | 0.69 | 0.10 |
| c36 | IVAS FX, DTX off, 384 kbps | 180 | 4.58 | 0.68 | 0.10 |

Figure 3.7-1: results per condition for experiment P800-7

Figure 3.7-2: results aggregated over FX/FL for experiment P800-7

Table 3.7-2: statistical significance analysis for experiment P800-7

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.67 | 0.97 | c19 | 3.69 | 1.04 | -0.02 | <0.001 | 0.582 | NWT |
| c11 | 3.92 | 0.93 | c20 | 4.00 | 0.88 | -0.08 | <0.001 | 0.807 | NWT |
| c12 | 4.16 | 0.87 | c21 | 4.11 | 0.82 | 0.04 | 0.505 | 0.307 | NWT |
| c13 | 4.33 | 0.80 | c22 | 4.33 | 0.78 | 0.00 | <0.001 | 0.500 | NWT |
| c14 | 4.49 | 0.69 | c23 | 4.47 | 0.77 | 0.03 | 0.352 | 0.363 | NWT |
| c15 | 4.48 | 0.66 | c24 | 4.55 | 0.70 | -0.07 | <0.001 | 0.823 | NWT |
| c16 | 4.47 | 0.74 | c25 | 4.42 | 0.69 | 0.05 | 0.660 | 0.255 | NWT |
| c17 | 4.54 | 0.77 | c26 | 4.52 | 0.72 | 0.02 | 0.217 | 0.414 | NWT |
| c18 | 4.63 | 0.59 | c27 | 4.66 | 0.63 | -0.03 | <0.001 | 0.669 | NWT |
| c28 | 3.51 | 0.97 | c19 | 3.69 | 1.04 | -0.18 | <0.001 | 0.958 | WT |
| c29 | 4.09 | 0.85 | c20 | 4.00 | 0.88 | 0.09 | 0.975 | 0.165 | NWT |
| c30 | 4.02 | 0.86 | c21 | 4.11 | 0.82 | -0.09 | <0.001 | 0.855 | NWT |
| c31 | 4.33 | 0.75 | c22 | 4.33 | 0.78 | 0.00 | <0.001 | 0.500 | NWT |
| c32 | 4.42 | 0.77 | c23 | 4.47 | 0.77 | -0.04 | <0.001 | 0.711 | NWT |
| c33 | 4.46 | 0.71 | c24 | 4.55 | 0.70 | -0.09 | <0.001 | 0.896 | NWT |
| c34 | 4.43 | 0.76 | c25 | 4.42 | 0.69 | 0.01 | 0.078 | 0.469 | NWT |
| c35 | 4.54 | 0.69 | c26 | 4.52 | 0.72 | 0.02 | 0.296 | 0.384 | NWT |
| c36 | 4.58 | 0.68 | c27 | 4.66 | 0.63 | -0.07 | <0.001 | 0.855 | NWT |

## P800-8: MC (mixed CICP) (FE)

Experiment P800-8 was conducted by Mesaqin.com (language: CZ).

The listening lab has reported that for each panel, up to seven instead of five participants participated. The five subjects with the highest PCC were selected for result submission.

The aggregated results per condition are listed in Table 3.8-1 and are shown in Figure 3.8-1. The results aggregated over FX/FL are shown in Figure 3.8-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.8-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.8-1: results per condition for experiment P800-8

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.81 | 0.41 | 0.06 |
| c02 | MNRU Q = 32 dB | 180 | 4.02 | 0.96 | 0.14 |
| c03 | MNRU Q = 27 dB | 180 | 3.34 | 1.01 | 0.15 |
| c04 | MNRU Q = 22 dB | 180 | 2.37 | 1.08 | 0.16 |
| c05 | MNRU Q = 17 dB | 180 | 1.60 | 0.80 | 0.12 |
| c06 | ESDRU α = 0.8 | 180 | 4.59 | 0.64 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 3.82 | 1.05 | 0.15 |
| c08 | ESDRU α = 0.4 | 180 | 3.17 | 1.26 | 0.19 |
| c09 | ESDRU α = 0.2 | 180 | 2.70 | 1.27 | 0.19 |
| c10 | IVAS FL enc / FX dec, FER 5%, 16.4 kbps | 180 | 3.21 | 1.05 | 0.15 |
| c11 | IVAS FX enc / FL dec, FER 5%, 24.4 kbps | 180 | 3.52 | 0.94 | 0.14 |
| c12 | IVAS FL enc / FX dec, FER 5%, 48 kbps | 180 | 3.97 | 0.82 | 0.12 |
| c13 | IVAS FX enc / FL dec, FER 5%, 64 kbps | 180 | 4.08 | 0.76 | 0.11 |
| c14 | IVAS FL enc / FX dec, FER 5%, 80 kbps | 180 | 4.12 | 0.85 | 0.13 |
| c15 | IVAS FX enc / FL dec, FER 5%, 128 kbps | 180 | 4.13 | 0.90 | 0.13 |
| c16 | IVAS FL enc / FX dec, FER 5%, 160 kbps | 180 | 4.23 | 0.78 | 0.11 |
| c17 | IVAS FX enc / FL dec, FER 5%, 256 kbps | 180 | 4.29 | 0.75 | 0.11 |
| c18 | IVAS FL enc / FX dec, FER 5%, 384 kbps | 180 | 4.28 | 0.73 | 0.11 |
| c19 | IVAS FL, FER 5%, 16.4 kbps | 180 | 3.23 | 1.12 | 0.17 |
| c20 | IVAS FL, FER 5%, 24.4 kbps | 180 | 3.61 | 1.02 | 0.15 |
| c21 | IVAS FL, FER 5%, 48 kbps | 180 | 3.88 | 0.95 | 0.14 |
| c22 | IVAS FL, FER 5%, 64 kbps | 180 | 4.06 | 0.88 | 0.13 |
| c23 | IVAS FL, FER 5%, 80 kbps | 180 | 4.13 | 0.85 | 0.12 |
| c24 | IVAS FL, FER 5%, 128 kbps | 180 | 4.21 | 0.74 | 0.11 |
| c25 | IVAS FL, FER 5%, 160 kbps | 180 | 4.17 | 0.75 | 0.11 |
| c26 | IVAS FL, FER 5%, 256 kbps | 180 | 4.27 | 0.74 | 0.11 |
| c27 | IVAS FL, FER 5%, 384 kbps | 180 | 4.30 | 0.75 | 0.11 |
| c28 | IVAS FX, FER 5%, 16.4 kbps | 180 | 3.32 | 1.04 | 0.15 |
| c29 | IVAS FX, FER 5%, 24.4 kbps | 180 | 3.65 | 0.99 | 0.15 |
| c30 | IVAS FX, FER 5%, 48 kbps | 180 | 3.87 | 0.96 | 0.14 |
| c31 | IVAS FX, FER 5%, 64 kbps | 180 | 3.99 | 0.88 | 0.13 |
| c32 | IVAS FX, FER 5%, 80 kbps | 180 | 4.02 | 0.90 | 0.13 |
| c33 | IVAS FX, FER 5%, 128 kbps | 180 | 4.24 | 0.76 | 0.11 |
| c34 | IVAS FX, FER 5%, 160 kbps | 180 | 4.20 | 0.79 | 0.12 |
| c35 | IVAS FX, FER 5%, 256 kbps | 180 | 4.27 | 0.71 | 0.10 |
| c36 | IVAS FX, FER 5%, 384 kbps | 180 | 4.33 | 0.76 | 0.11 |

Figure 3.8-1: results per condition for experiment P800-8

Figure 3.8-2: results aggregated over FX/FL for experiment P800-8

Table 3.8-2: statistical significance analysis for experiment P800-8

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.21 | 1.05 | c19 | 3.23 | 1.12 | -0.02 | <0.001 | 0.576 | NWT |
| c11 | 3.52 | 0.94 | c20 | 3.61 | 1.02 | -0.09 | <0.001 | 0.805 | NWT |
| c12 | 3.97 | 0.82 | c21 | 3.88 | 0.95 | 0.08 | 0.897 | 0.185 | NWT |
| c13 | 4.08 | 0.76 | c22 | 4.06 | 0.88 | 0.03 | 0.311 | 0.378 | NWT |
| c14 | 4.12 | 0.85 | c23 | 4.13 | 0.85 | -0.01 | <0.001 | 0.549 | NWT |
| c15 | 4.13 | 0.90 | c24 | 4.21 | 0.74 | -0.08 | <0.001 | 0.816 | NWT |
| c16 | 4.23 | 0.78 | c25 | 4.17 | 0.75 | 0.06 | 0.756 | 0.225 | NWT |
| c17 | 4.29 | 0.75 | c26 | 4.27 | 0.74 | 0.03 | 0.342 | 0.366 | NWT |
| c18 | 4.28 | 0.73 | c27 | 4.30 | 0.75 | -0.02 | <0.001 | 0.586 | NWT |
| c28 | 3.32 | 1.04 | c19 | 3.23 | 1.12 | 0.09 | 0.781 | 0.218 | NWT |
| c29 | 3.65 | 0.99 | c20 | 3.61 | 1.02 | 0.04 | 0.414 | 0.340 | NWT |
| c30 | 3.87 | 0.96 | c21 | 3.88 | 0.95 | -0.01 | <0.001 | 0.543 | NWT |
| c31 | 3.99 | 0.88 | c22 | 4.06 | 0.88 | -0.06 | <0.001 | 0.748 | NWT |
| c32 | 4.02 | 0.90 | c23 | 4.13 | 0.85 | -0.11 | <0.001 | 0.874 | NWT |
| c33 | 4.24 | 0.76 | c24 | 4.21 | 0.74 | 0.03 | 0.418 | 0.338 | NWT |
| c34 | 4.20 | 0.79 | c25 | 4.17 | 0.75 | 0.03 | 0.343 | 0.366 | NWT |
| c35 | 4.27 | 0.71 | c26 | 4.27 | 0.74 | 0.00 | <0.001 | 0.500 | NWT |
| c36 | 4.33 | 0.76 | c27 | 4.30 | 0.75 | 0.03 | 0.351 | 0.363 | NWT |

## P800-9: 1-2 Objects (FX, RD)

Experiment P800-9 was conducted by Mesaqin.com (language: MAN).

The listening lab has reported that for each panel, up to seven instead of five participants participated. The five subjects with the highest PCC were selected for result submission.

The aggregated results per condition are listed in Table 3.9-1 and are shown in Figure 3.9-1. The results aggregated over FX/FL are shown in Figure 3.9-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.9-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.9-1: results per condition for experiment P800-9

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.79 | 0.41 | 0.06 |
| c02 | MNRU Q = 36 dB | 180 | 4.20 | 0.93 | 0.14 |
| c03 | MNRU Q = 30 dB | 180 | 2.92 | 1.27 | 0.19 |
| c04 | MNRU Q = 24 dB | 180 | 2.07 | 1.14 | 0.17 |
| c05 | MNRU Q = 18 dB | 180 | 1.46 | 0.79 | 0.12 |
| c06 | ESDRU α=0.7 | 180 | 3.78 | 1.07 | 0.16 |
| c07 | ESDRU α=0.5 | 180 | 2.91 | 1.14 | 0.17 |
| c08 | ESDRU α=0.3 | 180 | 2.06 | 1.07 | 0.16 |
| c09 | ESDRU α=0.1 | 180 | 1.53 | 0.77 | 0.11 |
| c10 (Cat. 1,2,5) | IVAS FL enc / FX dec, DTX off, 13.2 kbps, 1 ISM | 90 | 4.38 | 0.79 | 0.17 |
| c10 (Cat. 3,4,6) | IVAS FL enc / FX dec, DTX off, 160 kbps, 2 ISMs | 90 | 4.61 | 0.67 | 0.14 |
| c11 (Cat. 1,2,5) | IVAS FX enc / FL dec, DTX off, 16.4 kbps, 1 ISM | 90 | 4.40 | 0.82 | 0.17 |
| c11 (Cat. 3,4,6) | IVAS FX enc / FL dec, DTX off, 16.4 kbps, 2 ISMs | 90 | 3.51 | 1.18 | 0.25 |
| c12 (Cat. 1,2,5) | IVAS FL enc / FX dec, DTX off, 24.4 kbps, 1 ISM | 90 | 4.56 | 0.67 | 0.14 |
| c12 (Cat. 3,4,6) | IVAS FL enc / FX dec, DTX off, 24.4 kbps, 2 ISMs | 90 | 4.52 | 0.75 | 0.16 |
| c13 (Cat. 1,2,5) | IVAS FX enc / FL dec, DTX off, 32 kbps, 1 ISM | 90 | 4.47 | 0.74 | 0.15 |
| c13 (Cat. 3,4,6) | IVAS FX enc / FL dec, DTX off, 32 kbps, 2 ISMs | 90 | 4.46 | 0.80 | 0.17 |
| c14 (Cat. 1,2,5) | IVAS FL enc / FX dec, DTX off, 48 kbps, 1 ISM | 90 | 4.53 | 0.67 | 0.14 |
| c14 (Cat. 3,4,6) | IVAS FL enc / FX dec, DTX off, 48 kbps, 2 ISMs | 90 | 4.52 | 0.71 | 0.15 |
| c15 (Cat. 1,2,5) | IVAS FX enc / FL dec, DTX off, 64 kbps, 1 ISM | 90 | 4.59 | 0.69 | 0.14 |
| c15 (Cat. 3,4,6) | IVAS FX enc / FL dec, DTX off, 64 kbps, 2 ISMs | 90 | 4.38 | 0.76 | 0.16 |
| c16 (Cat. 1,2,5) | IVAS FL enc / FX dec, DTX off, 80 kbps, 1 ISM | 90 | 4.46 | 0.75 | 0.16 |
| c16 (Cat. 3,4,6) | IVAS FL enc / FX dec, DTX off, 80 kbps, 2 ISMs | 90 | 4.58 | 0.73 | 0.15 |
| c17 (Cat. 1,2,5) | IVAS FX enc / FL dec, DTX off, 96 kbps, 1 ISM | 90 | 4.57 | 0.73 | 0.15 |
| c17 (Cat. 3,4,6) | IVAS FX enc / FL dec, DTX off, 96 kbps, 2 ISMs | 90 | 4.61 | 0.61 | 0.13 |
| c18 (Cat. 1,2,5) | IVAS FL enc / FX dec, DTX off, 128 kbps, 1 ISM | 90 | 4.52 | 0.67 | 0.14 |
| c18 (Cat. 3,4,6) | IVAS FL enc / FX dec, DTX off, 128 kbps, 2 ISMs | 90 | 4.46 | 0.75 | 0.16 |
| c19 (Cat. 1,2,5) | IVAS FL, DTX off, 13.2 kbps, 1 ISM | 90 | 4.40 | 0.76 | 0.16 |
| c19 (Cat. 3,4,6) | IVAS FL, DTX off, 160 kbps, 2 ISMs | 90 | 4.70 | 0.55 | 0.12 |
| c20 (Cat. 1,2,5) | IVAS FL, DTX off, 16.4 kbps, 1 ISM | 90 | 4.43 | 0.75 | 0.16 |
| c20 (Cat. 3,4,6) | IVAS FL, DTX off, 16.4 kbps, 2 ISMs | 90 | 3.60 | 1.11 | 0.23 |
| c21 (Cat. 1,2,5) | IVAS FL, DTX off, 24.4 kbps, 1 ISM | 90 | 4.58 | 0.69 | 0.14 |
| c21 (Cat. 3,4,6) | IVAS FL, DTX off, 24.4 kbps, 2 ISMs | 90 | 4.36 | 0.86 | 0.18 |
| c22 (Cat. 1,2,5) | IVAS FL, DTX off, 32 kbps, 1 ISM | 90 | 4.53 | 0.66 | 0.14 |
| c22 (Cat. 3,4,6) | IVAS FL, DTX off, 32 kbps, 2 ISMs | 90 | 4.44 | 0.77 | 0.16 |
| c23 (Cat. 1,2,5) | IVAS FL, DTX off, 48 kbps, 1 ISM | 90 | 4.63 | 0.63 | 0.13 |
| c23 (Cat. 3,4,6) | IVAS FL, DTX off, 48 kbps, 2 ISMs | 90 | 4.52 | 0.71 | 0.15 |
| c24 (Cat. 1,2,5) | IVAS FL, DTX off, 64 kbps, 1 ISM | 90 | 4.62 | 0.63 | 0.13 |
| c24 (Cat. 3,4,6) | IVAS FL, DTX off, 64 kbps, 2 ISMs | 90 | 4.58 | 0.72 | 0.15 |
| c25 (Cat. 1,2,5) | IVAS FL, DTX off, 80 kbps, 1 ISM | 90 | 4.62 | 0.65 | 0.14 |
| c25 (Cat. 3,4,6) | IVAS FL, DTX off, 80 kbps, 2 ISMs | 90 | 4.49 | 0.72 | 0.15 |
| c26 (Cat. 1,2,5) | IVAS FL, DTX off, 96 kbps, 1 ISM | 90 | 4.57 | 0.65 | 0.14 |
| c26 (Cat. 3,4,6) | IVAS FL, DTX off, 96 kbps, 2 ISMs | 90 | 4.66 | 0.54 | 0.11 |
| c27 (Cat. 1,2,5) | IVAS FL, DTX off, 128 kbps, 1 ISM | 90 | 4.64 | 0.64 | 0.13 |
| c27 (Cat. 3,4,6) | IVAS FL, DTX off, 128 kbps, 2 ISMs | 90 | 4.53 | 0.67 | 0.14 |
| c28 (Cat. 1,2,5) | IVAS FX, DTX off, 13.2 kbps, 1 ISM | 90 | 4.19 | 0.87 | 0.18 |
| c28 (Cat. 3,4,6) | IVAS FX, DTX off, 160 kbps, 2 ISMs | 90 | 4.47 | 0.74 | 0.15 |
| c29 (Cat. 1,2,5) | IVAS FX, DTX off, 16.4 kbps, 1 ISM | 90 | 4.41 | 0.83 | 0.17 |
| c29 (Cat. 3,4,6) | IVAS FX, DTX off, 16.4 kbps, 2 ISMs | 90 | 3.82 | 0.98 | 0.20 |
| c30 (Cat. 1,2,5) | IVAS FX, DTX off, 24.4 kbps, 1 ISM | 90 | 4.48 | 0.78 | 0.16 |
| c30 (Cat. 3,4,6) | IVAS FX, DTX off, 24.4 kbps, 2 ISMs | 90 | 4.44 | 0.69 | 0.14 |
| c31 (Cat. 1,2,5) | IVAS FX, DTX off, 32 kbps, 1 ISM | 90 | 4.61 | 0.65 | 0.14 |
| c31 (Cat. 3,4,6) | IVAS FX, DTX off, 32 kbps, 2 ISMs | 90 | 4.54 | 0.69 | 0.14 |
| c32 (Cat. 1,2,5) | IVAS FX, DTX off, 48 kbps, 1 ISM | 90 | 4.64 | 0.53 | 0.11 |
| c32 (Cat. 3,4,6) | IVAS FX, DTX off, 48 kbps, 2 ISMs | 90 | 4.43 | 0.67 | 0.14 |
| c33 (Cat. 1,2,5) | IVAS FX, DTX off, 64 kbps, 1 ISM | 90 | 4.70 | 0.59 | 0.12 |
| c33 (Cat. 3,4,6) | IVAS FX, DTX off, 64 kbps, 2 ISMs | 90 | 4.50 | 0.71 | 0.15 |
| c34 (Cat. 1,2,5) | IVAS FX, DTX off, 80 kbps, 1 ISM | 90 | 4.59 | 0.65 | 0.14 |
| c34 (Cat. 3,4,6) | IVAS FX, DTX off, 80 kbps, 2 ISMs | 90 | 4.52 | 0.66 | 0.14 |
| c35 (Cat. 1,2,5) | IVAS FX, DTX off, 96 kbps, 1 ISM | 90 | 4.61 | 0.59 | 0.12 |
| c35 (Cat. 3,4,6) | IVAS FX, DTX off, 96 kbps, 2 ISMs | 90 | 4.64 | 0.57 | 0.12 |
| c36 (Cat. 1,2,5) | IVAS FX, DTX off, 128 kbps, 1 ISM | 90 | 4.59 | 0.67 | 0.14 |
| c36 (Cat. 3,4,6) | IVAS FX, DTX off, 128 kbps, 2 ISMs | 90 | 4.54 | 0.69 | 0.14 |

(a)

(b)

Figure 3.9-1: results per condition for experiment P800-9

Figure 3.9-2: results aggregated over FX/FL for experiment P800-9

Table 3.9-2: statistical significance analysis for experiment P800-9

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 (Cat. 1,2,5) | 4.38 | 0.79 | c19 (Cat. 1,2,5) | 4.40 | 0.76 | -0.02 | <0.001 | 0.575 | NWT |
| c10 (Cat. 3,4,6) | 4.61 | 0.67 | c19 (Cat. 3,4,6) | 4.70 | 0.55 | -0.09 | <0.001 | 0.835 | NWT |
| c11 (Cat. 1,2,5) | 4.40 | 0.82 | c20 (Cat. 1,2,5) | 4.43 | 0.75 | -0.03 | <0.001 | 0.611 | NWT |
| c11 (Cat. 3,4,6) | 3.51 | 1.18 | c20 (Cat. 3,4,6) | 3.60 | 1.11 | -0.09 | <0.001 | 0.698 | NWT |
| c12 (Cat. 1,2,5) | 4.56 | 0.67 | c21 (Cat. 1,2,5) | 4.58 | 0.69 | -0.02 | <0.001 | 0.586 | NWT |
| c12 (Cat. 3,4,6) | 4.52 | 0.75 | c21 (Cat. 3,4,6) | 4.36 | 0.86 | 0.17 | 1.373 | 0.086 | NWT |
| c13 (Cat. 1,2,5) | 4.47 | 0.74 | c22 (Cat. 1,2,5) | 4.53 | 0.66 | -0.07 | <0.001 | 0.737 | NWT |
| c13 (Cat. 3,4,6) | 4.46 | 0.80 | c22 (Cat. 3,4,6) | 4.44 | 0.77 | 0.01 | 0.103 | 0.459 | NWT |
| c14 (Cat. 1,2,5) | 4.53 | 0.67 | c23 (Cat. 1,2,5) | 4.63 | 0.63 | -0.10 | <0.001 | 0.848 | NWT |
| c14 (Cat. 3,4,6) | 4.52 | 0.71 | c23 (Cat. 3,4,6) | 4.52 | 0.71 | 0.00 | <0.001 | 0.500 | NWT |
| c15 (Cat. 1,2,5) | 4.59 | 0.69 | c24 (Cat. 1,2,5) | 4.62 | 0.63 | -0.03 | <0.001 | 0.632 | NWT |
| c15 (Cat. 3,4,6) | 4.38 | 0.76 | c24 (Cat. 3,4,6) | 4.58 | 0.72 | -0.20 | <0.001 | 0.964 | WT |
| c16 (Cat. 1,2,5) | 4.46 | 0.75 | c25 (Cat. 1,2,5) | 4.62 | 0.65 | -0.17 | <0.001 | 0.943 | NWT |
| c16 (Cat. 3,4,6) | 4.58 | 0.73 | c25 (Cat. 3,4,6) | 4.49 | 0.72 | 0.09 | 0.820 | 0.207 | NWT |
| c17 (Cat. 1,2,5) | 4.57 | 0.73 | c26 (Cat. 1,2,5) | 4.57 | 0.65 | 0.00 | <0.001 | 0.500 | NWT |
| c17 (Cat. 3,4,6) | 4.61 | 0.61 | c26 (Cat. 3,4,6) | 4.66 | 0.54 | -0.04 | <0.001 | 0.699 | NWT |
| c18 (Cat. 1,2,5) | 4.52 | 0.67 | c27 (Cat. 1,2,5) | 4.64 | 0.64 | -0.12 | <0.001 | 0.892 | NWT |
| c18 (Cat. 3,4,6) | 4.46 | 0.75 | c27 (Cat. 3,4,6) | 4.53 | 0.67 | -0.08 | <0.001 | 0.765 | NWT |
| c28 (Cat. 1,2,5) | 4.19 | 0.87 | c19 (Cat. 1,2,5) | 4.40 | 0.76 | -0.21 | <0.001 | 0.957 | WT |
| c28 (Cat. 3,4,6) | 4.47 | 0.74 | c19 (Cat. 3,4,6) | 4.70 | 0.55 | -0.23 | <0.001 | 0.991 | WT |
| c29 (Cat. 1,2,5) | 4.41 | 0.83 | c20 (Cat. 1,2,5) | 4.43 | 0.75 | -0.02 | <0.001 | 0.574 | NWT |
| c29 (Cat. 3,4,6) | 3.82 | 0.98 | c20 (Cat. 3,4,6) | 3.60 | 1.11 | 0.22 | 1.424 | 0.078 | NWT |
| c30 (Cat. 1,2,5) | 4.48 | 0.78 | c21 (Cat. 1,2,5) | 4.58 | 0.69 | -0.10 | <0.001 | 0.818 | NWT |
| c30 (Cat. 3,4,6) | 4.44 | 0.69 | c21 (Cat. 3,4,6) | 4.36 | 0.86 | 0.09 | 0.755 | 0.226 | NWT |
| c31 (Cat. 1,2,5) | 4.61 | 0.65 | c22 (Cat. 1,2,5) | 4.53 | 0.66 | 0.08 | 0.802 | 0.212 | NWT |
| c31 (Cat. 3,4,6) | 4.54 | 0.69 | c22 (Cat. 3,4,6) | 4.44 | 0.77 | 0.10 | 0.920 | 0.179 | NWT |
| c32 (Cat. 1,2,5) | 4.64 | 0.53 | c23 (Cat. 1,2,5) | 4.63 | 0.63 | 0.01 | 0.128 | 0.449 | NWT |
| c32 (Cat. 3,4,6) | 4.43 | 0.67 | c23 (Cat. 3,4,6) | 4.52 | 0.71 | -0.09 | <0.001 | 0.806 | NWT |
| c33 (Cat. 1,2,5) | 4.70 | 0.59 | c24 (Cat. 1,2,5) | 4.62 | 0.63 | 0.08 | 0.859 | 0.196 | NWT |
| c33 (Cat. 3,4,6) | 4.50 | 0.71 | c24 (Cat. 3,4,6) | 4.58 | 0.72 | -0.08 | <0.001 | 0.768 | NWT |
| c34 (Cat. 1,2,5) | 4.59 | 0.65 | c25 (Cat. 1,2,5) | 4.62 | 0.65 | -0.03 | <0.001 | 0.633 | NWT |
| c34 (Cat. 3,4,6) | 4.52 | 0.66 | c25 (Cat. 3,4,6) | 4.49 | 0.72 | 0.03 | 0.320 | 0.374 | NWT |
| c35 (Cat. 1,2,5) | 4.61 | 0.59 | c26 (Cat. 1,2,5) | 4.57 | 0.65 | 0.04 | 0.472 | 0.319 | NWT |
| c35 (Cat. 3,4,6) | 4.64 | 0.57 | c26 (Cat. 3,4,6) | 4.66 | 0.54 | -0.01 | <0.001 | 0.558 | NWT |
| c36 (Cat. 1,2,5) | 4.59 | 0.67 | c27 (Cat. 1,2,5) | 4.64 | 0.64 | -0.05 | <0.001 | 0.713 | NWT |
| c36 (Cat. 3,4,6) | 4.54 | 0.69 | c27 (Cat. 3,4,6) | 4.53 | 0.67 | 0.01 | 0.108 | 0.457 | NWT |

## P800-10: 3-4 Objects (FX, RD)

Experiment P800-10 was conducted by VoiceAge Corporation (language: FR).

The aggregated results per condition are listed in Table 3.10-1 and are shown in Figure 3.10-1. The results aggregated over FX/FL are shown in Figure 3.10-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.10-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.10-1: results per condition for experiment P800-10

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.57 | 0.71 | 0.10 |
| c02 | MNRU Q = 36 dB | 180 | 4.35 | 0.83 | 0.12 |
| c03 | MNRU Q = 30 dB | 180 | 3.72 | 1.15 | 0.17 |
| c04 | MNRU Q = 24 dB | 180 | 2.26 | 1.24 | 0.18 |
| c05 | MNRU Q = 18 dB | 180 | 1.31 | 0.69 | 0.10 |
| c06 | ESDRU α=0.7 | 180 | 3.86 | 1.02 | 0.15 |
| c07 | ESDRU α=0.5 | 180 | 2.69 | 1.12 | 0.17 |
| c08 | ESDRU α=0.3 | 180 | 2.25 | 1.16 | 0.17 |
| c09 | ESDRU α=0.1 | 180 | 1.58 | 0.90 | 0.13 |
| c10 | IVAS FL enc / FX dec, DTX off, 24.4 kbps | 180 | 2.29 | 1.10 | 0.16 |
| c11 | IVAS FX enc / FL dec, DTX off, 32 kbps | 180 | 3.14 | 1.12 | 0.17 |
| c12 | IVAS FL enc / FX dec, DTX off, 48 kbps | 180 | 4.14 | 0.90 | 0.13 |
| c13 | IVAS FX enc / FL dec, DTX off, 64 kbps | 180 | 4.24 | 0.82 | 0.12 |
| c14 | IVAS FL enc / FX dec, DTX off, 80 kbps | 180 | 4.36 | 0.74 | 0.11 |
| c15 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.16 | 0.93 | 0.14 |
| c16 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.57 | 0.69 | 0.10 |
| c17 | IVAS FX enc / FL dec, DTX off, 160 kbps | 180 | 4.66 | 0.58 | 0.09 |
| c18 | IVAS FL enc / FX dec, DTX off, 192 kbps | 180 | 4.56 | 0.68 | 0.10 |
| c19 | IVAS FL, DTX off, 24.4 kbps | 180 | 2.27 | 1.14 | 0.17 |
| c20 | IVAS FL, DTX off, 32 kbps | 180 | 3.08 | 1.12 | 0.16 |
| c21 | IVAS FL, DTX off, 48 kbps | 180 | 4.17 | 0.85 | 0.12 |
| c22 | IVAS FL, DTX off, 64 kbps | 180 | 4.35 | 0.79 | 0.12 |
| c23 | IVAS FL, DTX off, 80 kbps | 180 | 4.44 | 0.66 | 0.10 |
| c24 | IVAS FL, DTX off, 96 kbps | 180 | 4.35 | 0.76 | 0.11 |
| c25 | IVAS FL, DTX off, 128 kbps | 180 | 4.48 | 0.74 | 0.11 |
| c26 | IVAS FL, DTX off, 160 kbps | 180 | 4.48 | 0.73 | 0.11 |
| c27 | IVAS FL, DTX off, 192 kbps | 180 | 4.52 | 0.69 | 0.10 |
| c28 | IVAS FX, DTX off, 24.4 kbps | 180 | 2.23 | 1.09 | 0.16 |
| c29 | IVAS FX, DTX off, 32 kbps | 180 | 3.07 | 1.13 | 0.17 |
| c30 | IVAS FX, DTX off, 48 kbps | 180 | 4.20 | 0.80 | 0.12 |
| c31 | IVAS FX, DTX off, 64 kbps | 180 | 4.29 | 0.87 | 0.13 |
| c32 | IVAS FX, DTX off, 80 kbps | 180 | 4.21 | 0.79 | 0.12 |
| c33 | IVAS FX, DTX off, 96 kbps | 180 | 4.20 | 0.87 | 0.13 |
| c34 | IVAS FX, DTX off, 128 kbps | 180 | 4.40 | 0.71 | 0.10 |
| c35 | IVAS FX, DTX off, 160 kbps | 180 | 4.58 | 0.65 | 0.10 |
| c36 | IVAS FX, DTX off, 192 kbps | 180 | 4.62 | 0.59 | 0.09 |

Figure 3.10-1: results per condition for experiment P800-10

Figure 3.10-2: results aggregated over FX/FL for experiment P800-10

Table 3.10-2: statistical significance analysis for experiment P800-10

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 2.29 | 1.10 | c19 | 2.27 | 1.14 | 0.02 | 0.186 | 0.426 | NWT |
| c11 | 3.14 | 1.12 | c20 | 3.08 | 1.12 | 0.06 | 0.517 | 0.303 | NWT |
| c12 | 4.14 | 0.90 | c21 | 4.17 | 0.85 | -0.03 | <0.001 | 0.620 | NWT |
| c13 | 4.24 | 0.82 | c22 | 4.35 | 0.79 | -0.11 | <0.001 | 0.893 | NWT |
| c14 | 4.36 | 0.74 | c23 | 4.44 | 0.66 | -0.08 | <0.001 | 0.869 | NWT |
| c15 | 4.16 | 0.93 | c24 | 4.35 | 0.76 | -0.19 | <0.001 | 0.982 | WT |
| c16 | 4.57 | 0.69 | c25 | 4.48 | 0.74 | 0.09 | 1.174 | 0.121 | NWT |
| c17 | 4.66 | 0.58 | c26 | 4.48 | 0.73 | 0.17 | 2.490 | 0.007 | BT |
| c18 | 4.56 | 0.68 | c27 | 4.52 | 0.69 | 0.04 | 0.611 | 0.271 | NWT |
| c28 | 2.23 | 1.09 | c19 | 2.27 | 1.14 | -0.04 | <0.001 | 0.646 | NWT |
| c29 | 3.07 | 1.13 | c20 | 3.08 | 1.12 | -0.01 | <0.001 | 0.537 | NWT |
| c30 | 4.20 | 0.80 | c21 | 4.17 | 0.85 | 0.03 | 0.379 | 0.352 | NWT |
| c31 | 4.29 | 0.87 | c22 | 4.35 | 0.79 | -0.06 | <0.001 | 0.756 | NWT |
| c32 | 4.21 | 0.79 | c23 | 4.44 | 0.66 | -0.24 | <0.001 | 0.999 | WT |
| c33 | 4.20 | 0.87 | c24 | 4.35 | 0.76 | -0.15 | <0.001 | 0.959 | WT |
| c34 | 4.40 | 0.71 | c25 | 4.48 | 0.74 | -0.08 | <0.001 | 0.846 | NWT |
| c35 | 4.58 | 0.65 | c26 | 4.48 | 0.73 | 0.10 | 1.305 | 0.096 | NWT |
| c36 | 4.62 | 0.59 | c27 | 4.52 | 0.69 | 0.10 | 1.553 | 0.061 | NWT |

## P800-11: 1-4 Objects (DTX, FE)

Experiment P800-11 was conducted by FORCE Technology (language: DAN).

The aggregated results per condition are listed in Table 3.11-1 and are shown in Figure 3.11-1. The results aggregated over FX/FL are shown in Figure 3.11-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.11-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.11-1: results per condition for experiment P800-11

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.76 | 0.49 | 0.07 |
| c02 | MNRU Q = 36 dB | 180 | 4.06 | 0.97 | 0.14 |
| c03 | MNRU Q = 30 dB | 180 | 2.74 | 1.15 | 0.17 |
| c04 | MNRU Q = 24 dB | 180 | 1.83 | 0.86 | 0.13 |
| c05 | MNRU Q = 18 dB | 180 | 1.18 | 0.42 | 0.06 |
| c06 | ESDRU α=0.7 | 180 | 4.29 | 0.74 | 0.11 |
| c07 | ESDRU α=0.5 | 180 | 3.92 | 1.04 | 0.15 |
| c08 | ESDRU α=0.3 | 180 | 3.50 | 1.18 | 0.17 |
| c09 | ESDRU α=0.1 | 180 | 2.90 | 1.21 | 0.18 |
| c10 (Cat. 1,2) | IVAS FL enc / FX dec, DTX on, FER 5%, 13.2 kbps, 1 ISM | 60 | 3.28 | 1.01 | 0.26 |
| c10 (Cat. 3,4) | IVAS FL enc / FX dec, DTX on, FER 5%, 16.4 kbps, 2 ISMs | 60 | 2.30 | 0.96 | 0.25 |
| c10 (Cat. 5,6) | IVAS FL enc / FX dec, DTX on, FER 5%, 24.4 kbps, 3-4 ISMs | 60 | 1.83 | 0.87 | 0.22 |
| c11 (Cat. 1,2) | IVAS FX enc / FL dec, DTX on, FER 5%, 16.4 kbps, 1 ISM | 60 | 3.38 | 0.78 | 0.20 |
| c11 (Cat. 3,4) | IVAS FX enc / FL dec, DTX on, FER 5%, 24.4 kbps, 2 ISMs | 60 | 4.13 | 0.81 | 0.21 |
| c11 (Cat. 5,6) | IVAS FX enc / FL dec, DTX on, FER 5%, 32 kbps, 3-4 ISMs | 60 | 2.35 | 1.07 | 0.28 |
| c12 (Cat. 1,2) | IVAS FL enc / FX dec, DTX on, FER 5%, 24.4 kbps, 1 ISM | 60 | 3.45 | 0.98 | 0.25 |
| c12 (Cat. 3,4) | IVAS FL enc / FX dec, DTX on, FER 5%, 32 kbps, 2 ISMs | 60 | 3.93 | 0.84 | 0.22 |
| c12 (Cat. 5,6) | IVAS FL enc / FX dec, DTX on, FER 5%, 48 kbps, 3-4 ISMs | 60 | 3.55 | 0.96 | 0.25 |
| c13 (Cat. 1,2) | IVAS FX enc / FL dec, DTX on, FER 5%, 32 kbps, 1 ISM | 60 | 3.35 | 0.84 | 0.22 |
| c13 (Cat. 3,4) | IVAS FX enc / FL dec, DTX on, FER 5%, 48 kbps, 2 ISMs | 60 | 4.02 | 0.91 | 0.23 |
| c13 (Cat. 5,6) | IVAS FX enc / FL dec, DTX on, FER 5%, 64 kbps, 3-4 ISMs | 60 | 3.60 | 0.98 | 0.25 |
| c14 (Cat. 1,2) | IVAS FL enc / FX dec, DTX on, FER 5%, 48 kbps, 1 ISM | 60 | 3.63 | 1.07 | 0.28 |
| c14 (Cat. 3,4) | IVAS FL enc / FX dec, DTX on, FER 5%, 64 kbps, 2 ISMs | 60 | 4.02 | 0.85 | 0.22 |
| c14 (Cat. 5,6) | IVAS FL enc / FX dec, DTX on, FER 5%, 80 kbps, 3-4 ISMs | 60 | 3.48 | 1.02 | 0.26 |
| c15 (Cat. 1,2) | IVAS FX enc / FL dec, DTX on, FER 5%, 64 kbps, 1 ISM | 60 | 3.72 | 0.94 | 0.24 |
| c15 (Cat. 3,4) | IVAS FX enc / FL dec, DTX on, FER 5%, 80 kbps, 2 ISMs | 60 | 3.93 | 0.99 | 0.26 |
| c15 (Cat. 5,6) | IVAS FX enc / FL dec, DTX on, FER 5%, 96 kbps, 3-4 ISMs | 60 | 3.57 | 0.98 | 0.25 |
| c16 (Cat. 1,2) | IVAS FL enc / FX dec, DTX on, FER 5%, 80 kbps, 1 ISM | 60 | 3.88 | 0.90 | 0.23 |
| c16 (Cat. 3,4) | IVAS FL enc / FX dec, DTX on, FER 5%, 96 kbps, 2 ISMs | 60 | 4.27 | 0.76 | 0.20 |
| c16 (Cat. 5,6) | IVAS FL enc / FX dec, DTX on, FER 5%, 128 kbps, 3-4 ISMs | 60 | 3.78 | 0.92 | 0.24 |
| c17 (Cat. 1,2) | IVAS FX enc / FL dec, DTX on, FER 5%, 96 kbps, 1 ISM | 60 | 3.75 | 0.95 | 0.24 |
| c17 (Cat. 3,4) | IVAS FX enc / FL dec, DTX on, FER 5%, 128 kbps, 2 ISMs | 60 | 4.25 | 0.93 | 0.24 |
| c17 (Cat. 5,6) | IVAS FX enc / FL dec, DTX on, FER 5%, 160 kbps, 3-4 ISMs | 60 | 3.80 | 0.76 | 0.20 |
| c18 (Cat. 1,2) | IVAS FL enc / FX dec, DTX on, FER 5%, 128 kbps, 1 ISM | 60 | 3.83 | 0.89 | 0.23 |
| c18 (Cat. 3,4) | IVAS FL enc / FX dec, DTX on, FER 5%, 160 kbps, 2 ISMs | 60 | 4.38 | 0.67 | 0.17 |
| c18 (Cat. 5,6) | IVAS FL enc / FX dec, DTX on, FER 5%, 192 kbps, 3-4 ISMs | 60 | 3.90 | 1.05 | 0.27 |
| c19 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 13.2 kbps, 1 ISM | 60 | 3.27 | 1.18 | 0.30 |
| c19 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 16.4 kbps, 2 ISMs | 60 | 2.40 | 1.08 | 0.28 |
| c19 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 24.4 kbps, 3-4 ISMs | 60 | 1.92 | 0.85 | 0.22 |
| c20 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 16.4 kbps, 1 ISM | 60 | 3.18 | 1.10 | 0.28 |
| c20 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 24.4 kbps, 2 ISMs | 60 | 3.78 | 0.96 | 0.25 |
| c20 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 32 kbps, 3-4 ISMs | 60 | 2.35 | 0.92 | 0.24 |
| c21 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 24.4 kbps, 1 ISM | 60 | 3.63 | 0.88 | 0.23 |
| c21 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 32 kbps, 2 ISMs | 60 | 4.00 | 0.92 | 0.24 |
| c21 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 48 kbps, 3-4 ISMs | 60 | 3.53 | 0.98 | 0.25 |
| c22 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 32 kbps, 1 ISM | 60 | 3.62 | 0.86 | 0.22 |
| c22 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 48 kbps, 2 ISMs | 60 | 3.90 | 0.99 | 0.26 |
| c22 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 64 kbps, 3-4 ISMs | 60 | 3.62 | 1.01 | 0.26 |
| c23 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 48 kbps, 1 ISM | 60 | 3.90 | 0.88 | 0.23 |
| c23 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 64 kbps, 2 ISMs | 60 | 4.15 | 0.88 | 0.23 |
| c23 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 80 kbps, 3-4 ISMs | 60 | 3.40 | 1.04 | 0.27 |
| c24 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 64 kbps, 1 ISM | 60 | 3.70 | 0.93 | 0.24 |
| c24 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 80 kbps, 2 ISMs | 60 | 3.95 | 1.13 | 0.29 |
| c24 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 96 kbps, 3-4 ISMs | 60 | 3.48 | 1.08 | 0.28 |
| c25 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 80 kbps, 1 ISM | 60 | 3.60 | 1.11 | 0.29 |
| c25 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 96 kbps, 2 ISMs | 60 | 4.17 | 0.91 | 0.23 |
| c25 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 128 kbps, 3-4 ISMs | 60 | 3.83 | 0.85 | 0.22 |
| c26 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 96 kbps, 1 ISM | 60 | 3.75 | 0.98 | 0.25 |
| c26 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 128 kbps, 2 ISMs | 60 | 4.32 | 0.83 | 0.21 |
| c26 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 160 kbps, 3-4 ISMs | 60 | 3.93 | 0.92 | 0.24 |
| c27 (Cat. 1,2) | IVAS FL, DTX on, FER 5%, 128 kbps, 1 ISM | 60 | 3.90 | 0.95 | 0.25 |
| c27 (Cat. 3,4) | IVAS FL, DTX on, FER 5%, 160 kbps, 2 ISMs | 60 | 4.15 | 0.99 | 0.26 |
| c27 (Cat. 5,6) | IVAS FL, DTX on, FER 5%, 192 kbps, 3-4 ISMs | 60 | 3.93 | 0.80 | 0.21 |
| c28 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 13.2 kbps, 1 ISM | 60 | 3.30 | 0.91 | 0.23 |
| c28 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 16.4 kbps, 2 ISMs | 60 | 2.30 | 1.06 | 0.27 |
| c28 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 24.4 kbps, 3-4 ISMs | 60 | 1.93 | 0.92 | 0.24 |
| c29 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 16.4 kbps, 1 ISM | 60 | 3.43 | 0.91 | 0.23 |
| c29 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 24.4 kbps, 2 ISMs | 60 | 3.95 | 0.91 | 0.23 |
| c29 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 32 kbps, 3-4 ISMs | 60 | 2.43 | 1.03 | 0.27 |
| c30 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 24.4 kbps, 1 ISM | 60 | 3.73 | 0.80 | 0.21 |
| c30 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 32 kbps, 2 ISMs | 60 | 4.17 | 0.72 | 0.18 |
| c30 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 48 kbps, 3-4 ISMs | 60 | 3.38 | 1.03 | 0.27 |
| c31 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 32 kbps, 1 ISM | 60 | 3.23 | 0.87 | 0.23 |
| c31 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 48 kbps, 2 ISMs | 60 | 3.87 | 0.79 | 0.20 |
| c31 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 64 kbps, 3-4 ISMs | 60 | 3.65 | 0.90 | 0.23 |
| c32 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 48 kbps, 1 ISM | 60 | 3.48 | 1.08 | 0.28 |
| c32 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 64 kbps, 2 ISMs | 60 | 3.90 | 1.10 | 0.28 |
| c32 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 80 kbps, 3-4 ISMs | 60 | 3.32 | 1.00 | 0.26 |
| c33 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 64 kbps, 1 ISM | 60 | 3.77 | 0.83 | 0.21 |
| c33 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 80 kbps, 2 ISMs | 60 | 4.00 | 0.90 | 0.23 |
| c33 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 96 kbps, 3-4 ISMs | 60 | 3.48 | 1.02 | 0.26 |
| c34 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 80 kbps, 1 ISM | 60 | 3.75 | 0.88 | 0.23 |
| c34 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 96 kbps, 2 ISMs | 60 | 4.18 | 1.02 | 0.26 |
| c34 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 128 kbps, 3-4 ISMs | 60 | 3.60 | 1.01 | 0.26 |
| c35 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 96 kbps, 1 ISM | 60 | 3.93 | 0.92 | 0.24 |
| c35 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 128 kbps, 2 ISMs | 60 | 4.27 | 0.69 | 0.18 |
| c35 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 160 kbps, 3-4 ISMs | 60 | 3.77 | 1.05 | 0.27 |
| c36 (Cat. 1,2) | IVAS FX, DTX on, FER 5%, 128 kbps, 1 ISM | 60 | 3.77 | 0.96 | 0.25 |
| c36 (Cat. 3,4) | IVAS FX, DTX on, FER 5%, 160 kbps, 2 ISMs | 60 | 4.22 | 1.04 | 0.27 |
| c36 (Cat. 5,6) | IVAS FX, DTX on, FER 5%, 192 kbps, 3-4 ISMs | 60 | 4.03 | 1.01 | 0.26 |

(a)

(b)

(c)

Figure 3.11-1: results per condition for experiment P800-11

Figure 3.11-2: results aggregated over FX/FL for experiment P800-11

Table 3.11-2: statistical significance analysis for experiment P800-11

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 (Cat. 1,2) | 3.28 | 1.01 | c19 (Cat. 1,2) | 3.27 | 1.18 | 0.02 | 0.080 | 0.468 | NWT |
| c10 (Cat. 3,4) | 2.30 | 0.96 | c19 (Cat. 3,4) | 2.40 | 1.08 | -0.10 | <0.001 | 0.704 | NWT |
| c10 (Cat. 5,6) | 1.83 | 0.87 | c19 (Cat. 5,6) | 1.92 | 0.85 | -0.08 | <0.001 | 0.703 | NWT |
| c11 (Cat. 1,2) | 3.38 | 0.78 | c20 (Cat. 1,2) | 3.18 | 1.10 | 0.20 | 1.149 | 0.126 | NWT |
| c11 (Cat. 3,4) | 4.13 | 0.81 | c20 (Cat. 3,4) | 3.78 | 0.96 | 0.35 | 2.159 | 0.016 | BT |
| c11 (Cat. 5,6) | 2.35 | 1.07 | c20 (Cat. 5,6) | 2.35 | 0.92 | 0.00 | <0.001 | 0.500 | NWT |
| c12 (Cat. 1,2) | 3.45 | 0.98 | c21 (Cat. 1,2) | 3.63 | 0.88 | -0.18 | <0.001 | 0.857 | NWT |
| c12 (Cat. 3,4) | 3.93 | 0.84 | c21 (Cat. 3,4) | 4.00 | 0.92 | -0.07 | <0.001 | 0.661 | NWT |
| c12 (Cat. 5,6) | 3.55 | 0.96 | c21 (Cat. 5,6) | 3.53 | 0.98 | 0.02 | 0.096 | 0.462 | NWT |
| c13 (Cat. 1,2) | 3.35 | 0.84 | c22 (Cat. 1,2) | 3.62 | 0.86 | -0.27 | <0.001 | 0.956 | WT |
| c13 (Cat. 3,4) | 4.02 | 0.91 | c22 (Cat. 3,4) | 3.90 | 0.99 | 0.12 | 0.675 | 0.250 | NWT |
| c13 (Cat. 5,6) | 3.60 | 0.98 | c22 (Cat. 5,6) | 3.62 | 1.01 | -0.02 | <0.001 | 0.537 | NWT |
| c14 (Cat. 1,2) | 3.63 | 1.07 | c23 (Cat. 1,2) | 3.90 | 0.88 | -0.27 | <0.001 | 0.931 | NWT |
| c14 (Cat. 3,4) | 4.02 | 0.85 | c23 (Cat. 3,4) | 4.15 | 0.88 | -0.13 | <0.001 | 0.799 | NWT |
| c14 (Cat. 5,6) | 3.48 | 1.02 | c23 (Cat. 5,6) | 3.40 | 1.04 | 0.08 | 0.441 | 0.330 | NWT |
| c15 (Cat. 1,2) | 3.72 | 0.94 | c24 (Cat. 1,2) | 3.70 | 0.93 | 0.02 | 0.100 | 0.460 | NWT |
| c15 (Cat. 3,4) | 3.93 | 0.99 | c24 (Cat. 3,4) | 3.95 | 1.13 | -0.02 | <0.001 | 0.535 | NWT |
| c15 (Cat. 5,6) | 3.57 | 0.98 | c24 (Cat. 5,6) | 3.48 | 1.08 | 0.08 | 0.446 | 0.328 | NWT |
| c16 (Cat. 1,2) | 3.88 | 0.90 | c25 (Cat. 1,2) | 3.60 | 1.11 | 0.28 | 1.533 | 0.064 | NWT |
| c16 (Cat. 3,4) | 4.27 | 0.76 | c25 (Cat. 3,4) | 4.17 | 0.91 | 0.10 | 0.657 | 0.256 | NWT |
| c16 (Cat. 5,6) | 3.78 | 0.92 | c25 (Cat. 5,6) | 3.83 | 0.85 | -0.05 | <0.001 | 0.621 | NWT |
| c17 (Cat. 1,2) | 3.75 | 0.95 | c26 (Cat. 1,2) | 3.75 | 0.98 | 0.00 | <0.001 | 0.500 | NWT |
| c17 (Cat. 3,4) | 4.25 | 0.93 | c26 (Cat. 3,4) | 4.32 | 0.83 | -0.07 | <0.001 | 0.661 | NWT |
| c17 (Cat. 5,6) | 3.80 | 0.76 | c26 (Cat. 5,6) | 3.93 | 0.92 | -0.13 | <0.001 | 0.806 | NWT |
| c18 (Cat. 1,2) | 3.83 | 0.89 | c27 (Cat. 1,2) | 3.90 | 0.95 | -0.07 | <0.001 | 0.655 | NWT |
| c18 (Cat. 3,4) | 4.38 | 0.67 | c27 (Cat. 3,4) | 4.15 | 0.99 | 0.23 | 1.515 | 0.066 | NWT |
| c18 (Cat. 5,6) | 3.90 | 1.05 | c27 (Cat. 5,6) | 3.93 | 0.80 | -0.03 | <0.001 | 0.576 | NWT |
| c28 (Cat. 1,2) | 3.30 | 0.91 | c19 (Cat. 1,2) | 3.27 | 1.18 | 0.03 | 0.172 | 0.432 | NWT |
| c28 (Cat. 3,4) | 2.30 | 1.06 | c19 (Cat. 3,4) | 2.40 | 1.08 | -0.10 | <0.001 | 0.695 | NWT |
| c28 (Cat. 5,6) | 1.93 | 0.92 | c19 (Cat. 5,6) | 1.92 | 0.85 | 0.02 | 0.099 | 0.461 | NWT |
| c29 (Cat. 1,2) | 3.43 | 0.91 | c20 (Cat. 1,2) | 3.18 | 1.10 | 0.25 | 1.359 | 0.088 | NWT |
| c29 (Cat. 3,4) | 3.95 | 0.91 | c20 (Cat. 3,4) | 3.78 | 0.96 | 0.17 | 0.979 | 0.165 | NWT |
| c29 (Cat. 5,6) | 2.43 | 1.03 | c20 (Cat. 5,6) | 2.35 | 0.92 | 0.08 | 0.466 | 0.321 | NWT |
| c30 (Cat. 1,2) | 3.73 | 0.80 | c21 (Cat. 1,2) | 3.63 | 0.88 | 0.10 | 0.651 | 0.258 | NWT |
| c30 (Cat. 3,4) | 4.17 | 0.72 | c21 (Cat. 3,4) | 4.00 | 0.92 | 0.17 | 1.108 | 0.135 | NWT |
| c30 (Cat. 5,6) | 3.38 | 1.03 | c21 (Cat. 5,6) | 3.53 | 0.98 | -0.15 | <0.001 | 0.792 | NWT |
| c31 (Cat. 1,2) | 3.23 | 0.87 | c22 (Cat. 1,2) | 3.62 | 0.86 | -0.38 | <0.001 | 0.992 | WT |
| c31 (Cat. 3,4) | 3.87 | 0.79 | c22 (Cat. 3,4) | 3.90 | 0.99 | -0.03 | <0.001 | 0.580 | NWT |
| c31 (Cat. 5,6) | 3.65 | 0.90 | c22 (Cat. 5,6) | 3.62 | 1.01 | 0.03 | 0.189 | 0.425 | NWT |
| c32 (Cat. 1,2) | 3.48 | 1.08 | c23 (Cat. 1,2) | 3.90 | 0.88 | -0.42 | <0.001 | 0.989 | WT |
| c32 (Cat. 3,4) | 3.90 | 1.10 | c23 (Cat. 3,4) | 4.15 | 0.88 | -0.25 | <0.001 | 0.914 | NWT |
| c32 (Cat. 5,6) | 3.32 | 1.00 | c23 (Cat. 5,6) | 3.40 | 1.04 | -0.08 | <0.001 | 0.671 | NWT |
| c33 (Cat. 1,2) | 3.77 | 0.83 | c24 (Cat. 1,2) | 3.70 | 0.93 | 0.07 | 0.417 | 0.339 | NWT |
| c33 (Cat. 3,4) | 4.00 | 0.90 | c24 (Cat. 3,4) | 3.95 | 1.13 | 0.05 | 0.268 | 0.394 | NWT |
| c33 (Cat. 5,6) | 3.48 | 1.02 | c24 (Cat. 5,6) | 3.48 | 1.08 | 0.00 | <0.001 | 0.500 | NWT |
| c34 (Cat. 1,2) | 3.75 | 0.88 | c25 (Cat. 1,2) | 3.60 | 1.11 | 0.15 | 0.823 | 0.206 | NWT |
| c34 (Cat. 3,4) | 4.18 | 1.02 | c25 (Cat. 3,4) | 4.17 | 0.91 | 0.02 | 0.091 | 0.464 | NWT |
| c34 (Cat. 5,6) | 3.60 | 1.01 | c25 (Cat. 5,6) | 3.83 | 0.85 | -0.23 | <0.001 | 0.913 | NWT |
| c35 (Cat. 1,2) | 3.93 | 0.92 | c26 (Cat. 1,2) | 3.75 | 0.98 | 0.18 | 1.053 | 0.147 | NWT |
| c35 (Cat. 3,4) | 4.27 | 0.69 | c26 (Cat. 3,4) | 4.32 | 0.83 | -0.05 | <0.001 | 0.640 | NWT |
| c35 (Cat. 5,6) | 3.77 | 1.05 | c26 (Cat. 5,6) | 3.93 | 0.92 | -0.17 | <0.001 | 0.821 | NWT |
| c36 (Cat. 1,2) | 3.77 | 0.96 | c27 (Cat. 1,2) | 3.90 | 0.95 | -0.13 | <0.001 | 0.776 | NWT |
| c36 (Cat. 3,4) | 4.22 | 1.04 | c27 (Cat. 3,4) | 4.15 | 0.99 | 0.07 | 0.361 | 0.359 | NWT |
| c36 (Cat. 5,6) | 4.03 | 1.01 | c27 (Cat. 5,6) | 3.93 | 0.80 | 0.10 | 0.602 | 0.274 | NWT |

## P800-12: MASA 1 TC (FX, RD)

Experiment P800-12 was conducted by Mesaqin.com (language: MAN).

The listening lab has reported that for each panel, up to seven instead of five participants participated. The five subjects with the highest PCC were selected for result submission.

The aggregated results per condition are listed in Table 3.12-1 and are shown in Figure 3.12-1. The results aggregated over FX/FL are shown in Figure 3.12-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.12-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.12-1: results per condition for experiment P800-12

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.79 | 0.42 | 0.06 |
| c02 | MNRU Q = 32 dB | 180 | 4.24 | 0.88 | 0.13 |
| c03 | MNRU Q = 27 dB | 180 | 3.53 | 1.27 | 0.19 |
| c04 | MNRU Q = 22 dB | 180 | 2.37 | 1.18 | 0.17 |
| c05 | MNRU Q = 17 dB | 180 | 1.57 | 0.75 | 0.11 |
| c06 | ESDRU α = 0.8 | 180 | 4.52 | 0.74 | 0.11 |
| c07 | ESDRU α = 0.6 | 180 | 3.86 | 1.11 | 0.16 |
| c08 | ESDRU α = 0.4 | 180 | 2.89 | 1.18 | 0.17 |
| c09 | ESDRU α = 0.2 | 180 | 2.25 | 1.17 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX off, 13.2 kbps | 180 | 3.52 | 1.22 | 0.18 |
| c11 | IVAS FX enc / FL dec, DTX off, 16.4 kbps | 180 | 3.96 | 1.04 | 0.15 |
| c12 | IVAS FL enc / FX dec, DTX off, 24.4 kbps | 180 | 4.19 | 0.95 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX off, 32 kbps | 180 | 4.25 | 0.88 | 0.13 |
| c14 | IVAS FL enc / FX dec, DTX off, 48 kbps | 180 | 4.43 | 0.73 | 0.11 |
| c15 | IVAS FX enc / FL dec, DTX off, 64 kbps | 180 | 4.46 | 0.80 | 0.12 |
| c16 | IVAS FL enc / FX dec, DTX off, 80 kbps | 180 | 4.59 | 0.69 | 0.10 |
| c17 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.56 | 0.68 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.56 | 0.69 | 0.10 |
| c19 | IVAS FL, DTX off, 13.2 kbps | 180 | 3.51 | 1.18 | 0.17 |
| c20 | IVAS FL, DTX off, 16.4 kbps | 180 | 4.18 | 1.00 | 0.15 |
| c21 | IVAS FL, DTX off, 24.4 kbps | 180 | 4.18 | 0.92 | 0.14 |
| c22 | IVAS FL, DTX off, 32 kbps | 180 | 4.26 | 0.89 | 0.13 |
| c23 | IVAS FL, DTX off, 48 kbps | 180 | 4.40 | 0.84 | 0.12 |
| c24 | IVAS FL, DTX off, 64 kbps | 180 | 4.53 | 0.73 | 0.11 |
| c25 | IVAS FL, DTX off, 80 kbps | 180 | 4.48 | 0.77 | 0.11 |
| c26 | IVAS FL, DTX off, 96 kbps | 180 | 4.54 | 0.73 | 0.11 |
| c27 | IVAS FL, DTX off, 128 kbps | 180 | 4.57 | 0.72 | 0.11 |
| c28 | IVAS FX, DTX off, 13.2 kbps | 180 | 3.79 | 1.18 | 0.17 |
| c29 | IVAS FX, DTX off, 16.4 kbps | 180 | 3.97 | 1.07 | 0.16 |
| c30 | IVAS FX, DTX off, 24.4 kbps | 180 | 4.24 | 0.89 | 0.13 |
| c31 | IVAS FX, DTX off, 32 kbps | 180 | 4.28 | 0.92 | 0.14 |
| c32 | IVAS FX, DTX off, 48 kbps | 180 | 4.28 | 0.93 | 0.14 |
| c33 | IVAS FX, DTX off, 64 kbps | 180 | 4.47 | 0.78 | 0.12 |
| c34 | IVAS FX, DTX off, 80 kbps | 180 | 4.56 | 0.69 | 0.10 |
| c35 | IVAS FX, DTX off, 96 kbps | 180 | 4.47 | 0.77 | 0.11 |
| c36 | IVAS FX, DTX off, 128 kbps | 180 | 4.48 | 0.75 | 0.11 |

Figure 3.12-1: results per condition for experiment P800-12

Figure 3.12-2: results aggregated over FX/FL for experiment P800-12

Table 3.12-2: statistical significance analysis for experiment P800-12

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.52 | 1.22 | c19 | 3.51 | 1.18 | 0.02 | 0.126 | 0.450 | NWT |
| c11 | 3.96 | 1.04 | c20 | 4.18 | 1.00 | -0.23 | <0.001 | 0.982 | WT |
| c12 | 4.19 | 0.95 | c21 | 4.18 | 0.92 | 0.01 | 0.111 | 0.456 | NWT |
| c13 | 4.25 | 0.88 | c22 | 4.26 | 0.89 | -0.01 | <0.001 | 0.526 | NWT |
| c14 | 4.43 | 0.73 | c23 | 4.40 | 0.84 | 0.03 | 0.396 | 0.346 | NWT |
| c15 | 4.46 | 0.80 | c24 | 4.53 | 0.73 | -0.07 | <0.001 | 0.814 | NWT |
| c16 | 4.59 | 0.69 | c25 | 4.48 | 0.77 | 0.12 | 1.509 | 0.066 | NWT |
| c17 | 4.56 | 0.68 | c26 | 4.54 | 0.73 | 0.02 | 0.295 | 0.384 | NWT |
| c18 | 4.56 | 0.69 | c27 | 4.57 | 0.72 | -0.01 | <0.001 | 0.558 | NWT |
| c28 | 3.79 | 1.18 | c19 | 3.51 | 1.18 | 0.29 | 2.320 | 0.010 | BT |
| c29 | 3.97 | 1.07 | c20 | 4.18 | 1.00 | -0.22 | <0.001 | 0.975 | WT |
| c30 | 4.24 | 0.89 | c21 | 4.18 | 0.92 | 0.06 | 0.639 | 0.262 | NWT |
| c31 | 4.28 | 0.92 | c22 | 4.26 | 0.89 | 0.03 | 0.283 | 0.389 | NWT |
| c32 | 4.28 | 0.93 | c23 | 4.40 | 0.84 | -0.12 | <0.001 | 0.903 | NWT |
| c33 | 4.47 | 0.78 | c24 | 4.53 | 0.73 | -0.06 | <0.001 | 0.778 | NWT |
| c34 | 4.56 | 0.69 | c25 | 4.48 | 0.77 | 0.08 | 1.077 | 0.141 | NWT |
| c35 | 4.47 | 0.77 | c26 | 4.54 | 0.73 | -0.07 | <0.001 | 0.818 | NWT |
| c36 | 4.48 | 0.75 | c27 | 4.57 | 0.72 | -0.08 | <0.001 | 0.859 | NWT |

## P800-13: MASA 2 TC (FX, RD)

Experiment P800-13 was conducted by FORCE Technology (language: DAN).

The aggregated results per condition are listed in Table 3.13-1 and are shown in Figure 3.13-1. The results aggregated over FX/FL are shown in Figure 3.13-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.13-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.13-1: results per condition for experiment P800-13

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.78 | 0.45 | 0.07 |
| c02 | MNRU Q = 32 dB | 180 | 3.79 | 1.07 | 0.16 |
| c03 | MNRU Q = 27 dB | 180 | 2.85 | 1.16 | 0.17 |
| c04 | MNRU Q = 22 dB | 180 | 1.92 | 0.94 | 0.14 |
| c05 | MNRU Q = 17 dB | 180 | 1.25 | 0.51 | 0.07 |
| c06 | ESDRU α = 0.8 | 180 | 4.66 | 0.59 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 4.11 | 0.86 | 0.13 |
| c08 | ESDRU α = 0.4 | 180 | 3.39 | 1.07 | 0.16 |
| c09 | ESDRU α = 0.2 | 180 | 3.00 | 1.17 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX off, 16.4 kbps | 180 | 3.67 | 1.11 | 0.16 |
| c11 | IVAS FX enc / FL dec, DTX off, 24.4 kbps | 180 | 3.59 | 1.00 | 0.15 |
| c12 | IVAS FL enc / FX dec, DTX off, 32 kbps | 180 | 3.88 | 0.95 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX off, 48 kbps | 180 | 4.27 | 0.74 | 0.11 |
| c14 | IVAS FL enc / FX dec, DTX off, 64 kbps | 180 | 4.35 | 0.74 | 0.11 |
| c15 | IVAS FX enc / FL dec, DTX off, 96 kbps | 180 | 4.59 | 0.63 | 0.09 |
| c16 | IVAS FL enc / FX dec, DTX off, 128 kbps | 180 | 4.54 | 0.64 | 0.09 |
| c17 | IVAS FX enc / FL dec, DTX off, 192 kbps | 180 | 4.59 | 0.65 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX off, 384 kbps | 180 | 4.58 | 0.62 | 0.09 |
| c19 | IVAS FL, DTX off, 16.4 kbps | 180 | 3.46 | 1.14 | 0.17 |
| c20 | IVAS FL, DTX off, 24.4 kbps | 180 | 3.69 | 0.93 | 0.14 |
| c21 | IVAS FL, DTX off, 32 kbps | 180 | 3.81 | 0.99 | 0.15 |
| c22 | IVAS FL, DTX off, 48 kbps | 180 | 4.17 | 0.83 | 0.12 |
| c23 | IVAS FL, DTX off, 64 kbps | 180 | 4.35 | 0.70 | 0.10 |
| c24 | IVAS FL, DTX off, 96 kbps | 180 | 4.54 | 0.66 | 0.10 |
| c25 | IVAS FL, DTX off, 128 kbps | 180 | 4.51 | 0.61 | 0.09 |
| c26 | IVAS FL, DTX off, 192 kbps | 180 | 4.55 | 0.67 | 0.10 |
| c27 | IVAS FL, DTX off, 384 kbps | 180 | 4.56 | 0.63 | 0.09 |
| c28 | IVAS FX, DTX off, 16.4 kbps | 180 | 3.64 | 1.08 | 0.16 |
| c29 | IVAS FX, DTX off, 24.4 kbps | 180 | 3.66 | 1.06 | 0.16 |
| c30 | IVAS FX, DTX off, 32 kbps | 180 | 3.71 | 1.07 | 0.16 |
| c31 | IVAS FX, DTX off, 48 kbps | 180 | 4.12 | 0.85 | 0.12 |
| c32 | IVAS FX, DTX off, 64 kbps | 180 | 4.39 | 0.71 | 0.10 |
| c33 | IVAS FX, DTX off, 96 kbps | 180 | 4.46 | 0.73 | 0.11 |
| c34 | IVAS FX, DTX off, 128 kbps | 180 | 4.59 | 0.57 | 0.08 |
| c35 | IVAS FX, DTX off, 192 kbps | 180 | 4.62 | 0.59 | 0.09 |
| c36 | IVAS FX, DTX off, 384 kbps | 180 | 4.63 | 0.60 | 0.09 |

Figure 3.13-1: results per condition for experiment P800-13

Figure 3.13-2: results aggregated over FX/FL for experiment P800-13

Table 3.13-2: statistical significance analysis for experiment P800-13

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.67 | 1.11 | c19 | 3.46 | 1.14 | 0.21 | 1.780 | 0.038 | BT |
| c11 | 3.59 | 1.00 | c20 | 3.69 | 0.93 | -0.10 | <0.001 | 0.836 | NWT |
| c12 | 3.88 | 0.95 | c21 | 3.81 | 0.99 | 0.07 | 0.703 | 0.241 | NWT |
| c13 | 4.27 | 0.74 | c22 | 4.17 | 0.83 | 0.10 | 1.210 | 0.114 | NWT |
| c14 | 4.35 | 0.74 | c23 | 4.35 | 0.70 | 0.00 | <0.001 | 0.500 | NWT |
| c15 | 4.59 | 0.63 | c24 | 4.54 | 0.66 | 0.05 | 0.732 | 0.232 | NWT |
| c16 | 4.54 | 0.64 | c25 | 4.51 | 0.61 | 0.04 | 0.577 | 0.282 | NWT |
| c17 | 4.59 | 0.65 | c26 | 4.55 | 0.67 | 0.04 | 0.560 | 0.288 | NWT |
| c18 | 4.58 | 0.62 | c27 | 4.56 | 0.63 | 0.02 | 0.334 | 0.369 | NWT |
| c28 | 3.64 | 1.08 | c19 | 3.46 | 1.14 | 0.18 | 1.569 | 0.059 | NWT |
| c29 | 3.66 | 1.06 | c20 | 3.69 | 0.93 | -0.03 | <0.001 | 0.623 | NWT |
| c30 | 3.71 | 1.07 | c21 | 3.81 | 0.99 | -0.10 | <0.001 | 0.833 | NWT |
| c31 | 4.12 | 0.85 | c22 | 4.17 | 0.83 | -0.05 | <0.001 | 0.714 | NWT |
| c32 | 4.39 | 0.71 | c23 | 4.35 | 0.70 | 0.04 | 0.589 | 0.278 | NWT |
| c33 | 4.46 | 0.73 | c24 | 4.54 | 0.66 | -0.08 | <0.001 | 0.869 | NWT |
| c34 | 4.59 | 0.57 | c25 | 4.51 | 0.61 | 0.09 | 1.416 | 0.079 | NWT |
| c35 | 4.62 | 0.59 | c26 | 4.55 | 0.67 | 0.07 | 1.005 | 0.158 | NWT |
| c36 | 4.63 | 0.60 | c27 | 4.56 | 0.63 | 0.07 | 1.037 | 0.150 | NWT |

## P800-14: MASA 1,2 TC (DTX, FE)

Experiment P800-14 was conducted by Nokia (language: FIN).

The listening lab has reported that the presentation order of the samples to the listeners was not according to the original randomization tables due to a configuration issue in the listening test software. See listening lab report in S4-251691.

The aggregated results per condition are listed in Table 3.14-1 and are shown in Figure 3.14-1. The results aggregated over FX/FL are shown in Figure 3.14-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.14-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.14-1: results per condition for experiment P800-14

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference 1 TC, 1 TC | 180 | 4.67 | 0.64 | 0.09 |
| c02 | Reference 2 TC, 2 TC | 180 | 4.74 | 0.52 | 0.08 |
| c03 | MNRU Q = 32 dB | 180 | 3.98 | 1.03 | 0.15 |
| c04 | MNRU Q = 25 dB | 180 | 2.56 | 1.19 | 0.17 |
| c05 | MNRU Q = 18 dB | 180 | 1.27 | 0.54 | 0.08 |
| c06 | ESDRU α = 0.8 | 180 | 4.64 | 0.59 | 0.09 |
| c07 | ESDRU α = 0.6 | 180 | 4.01 | 0.91 | 0.13 |
| c08 | ESDRU α = 0.4 | 180 | 2.41 | 1.17 | 0.17 |
| c09 | IVAS FL enc / FX dec, 1 TC, DTX on, FER 4%, 13.2 kbps | 180 | 3.28 | 1.10 | 0.16 |
| c10 | IVAS FX enc / FL dec, 1 TC, DTX on, FER 4%, 16.4 kbps | 180 | 3.63 | 1.06 | 0.15 |
| c11 | IVAS FL enc / FL dec, 1 TC, DTX on, FER 4%, 24.4 kbps | 180 | 3.67 | 1.01 | 0.15 |
| c12 | IVAS FX enc / FX dec, 1 TC, DTX on, FER 4%, 32 kbps | 180 | 3.72 | 1.07 | 0.16 |
| c13 | IVAS FL enc / FX dec, 1 TC, DTX on, FER 4%, 48 kbps | 180 | 3.93 | 1.02 | 0.15 |
| c14 | IVAS FX enc / FL dec, 1 TC, DTX on, FER 4%, 64 kbps | 180 | 4.02 | 0.97 | 0.14 |
| c15 | IVAS FL enc / FL dec, 1 TC, DTX on, FER 4%, 80 kbps | 180 | 4.04 | 1.00 | 0.15 |
| c16 | IVAS FX enc / FX dec, 2 TC, DTX on, FER 4%, 13.2 kbps | 180 | 3.03 | 1.21 | 0.18 |
| c17 | IVAS FL enc / FX dec, 2 TC, DTX on, FER 4%, 16.4 kbps | 180 | 3.36 | 1.10 | 0.16 |
| c18 | IVAS FX enc / FL dec, 2 TC, DTX on, FER 4%, 24.4 kbps | 180 | 3.25 | 1.11 | 0.16 |
| c19 | IVAS FL enc / FL dec, 2 TC, DTX on, FER 4%, 32 kbps | 180 | 3.54 | 1.05 | 0.15 |
| c20 | IVAS FX enc / FX dec, 2 TC, DTX on, FER 4%, 48 kbps | 180 | 3.87 | 1.02 | 0.15 |
| c21 | IVAS FL enc / FX dec, 2 TC, DTX on, FER 4%, 64 kbps | 180 | 4.03 | 0.97 | 0.14 |
| c22 | IVAS FX enc / FL dec, 2 TC, DTX on, FER 4%, 80 kbps | 180 | 4.12 | 0.96 | 0.14 |
| c23 | IVAS FL enc / FL dec, 1 TC, DTX on, FER 8%, 13.2 kbps | 180 | 2.98 | 1.18 | 0.17 |
| c24 | IVAS FX enc / FX dec, 1 TC, DTX on, FER 8%, 16.4 kbps | 180 | 3.14 | 1.16 | 0.17 |
| c25 | IVAS FL enc / FX dec, 1 TC, DTX on, FER 8%, 24.4 kbps | 180 | 3.12 | 1.25 | 0.18 |
| c26 | IVAS FX enc / FL dec, 1 TC, DTX on, FER 8%, 32 kbps | 180 | 3.27 | 1.21 | 0.18 |
| c27 | IVAS FL enc / FL dec, 1 TC, DTX on, FER 8%, 48 kbps | 180 | 3.33 | 1.16 | 0.17 |
| c28 | IVAS FX enc / FX dec, 1 TC, DTX on, FER 8%, 64 kbps | 180 | 3.18 | 1.14 | 0.17 |
| c29 | IVAS FL enc / FX dec, 1 TC, DTX on, FER 8%, 80 kbps | 180 | 3.37 | 1.10 | 0.16 |
| c30 | IVAS FX enc / FL dec, 2 TC, DTX on, FER 8%, 13.2 kbps | 180 | 2.72 | 1.14 | 0.17 |
| c31 | IVAS FL enc / FL dec, 2 TC, DTX on, FER 8%, 16.4 kbps | 180 | 2.98 | 1.16 | 0.17 |
| c32 | IVAS FX enc / FX dec, 2 TC, DTX on, FER 8%, 24.4 kbps | 180 | 2.72 | 1.18 | 0.17 |
| c33 | IVAS FL enc / FX dec, 2 TC, DTX on, FER 8%, 32 kbps | 180 | 3.06 | 1.12 | 0.17 |
| c34 | IVAS FX enc / FL dec, 2 TC, DTX on, FER 8%, 48 kbps | 180 | 3.24 | 1.11 | 0.16 |
| c35 | IVAS FL enc / FL dec, 2 TC, DTX on, FER 8%, 64 kbps | 180 | 3.42 | 1.14 | 0.17 |
| c36 | IVAS FX enc / FX dec, 2 TC, DTX on, FER 8%, 80 kbps | 180 | 3.54 | 0.96 | 0.14 |

(a)

(b)

Figure 3.14-1: results per condition for experiment P800-14

Figure 3.14-2: results aggregated over FX/FL for experiment P800-14

Table 3.14-2: statistical significance analysis for experiment P800-14

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| No terms of references mandated for experiment . |  |  |  |  |  |  |  |  |  |

## P800-15: OSBA, 1-2 Objects (FX, RD)

Experiment P800-15 was conducted by Dolby Laboratories, Inc. (language: ENG).

The aggregated results per condition are listed in Table 3.15-1 and are shown in Figure 3.15-1. The results aggregated over FX/FL are shown in Figure 3.15-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.15-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.15-1: results per condition for experiment P800-15

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.79 | 0.43 | 0.06 |
| c02 | MNRU Q = 32 dB | 180 | 4.32 | 0.93 | 0.14 |
| c03 | MNRU Q = 27 dB | 180 | 3.78 | 1.22 | 0.18 |
| c04 | MNRU Q = 22 dB | 180 | 2.78 | 1.34 | 0.20 |
| c05 | MNRU Q = 17 dB | 180 | 1.91 | 1.22 | 0.18 |
| c06 | ESDRU α = 0.8 | 180 | 4.48 | 0.70 | 0.10 |
| c07 | ESDRU α = 0.6 | 180 | 3.57 | 1.15 | 0.17 |
| c08 | ESDRU α = 0.4 | 180 | 2.71 | 1.19 | 0.17 |
| c09 | ESDRU α = 0.2 | 180 | 2.09 | 1.10 | 0.16 |
| c10 | IVAS FL enc / FX dec, DTX Off, 32 kbps | 180 | 3.33 | 1.22 | 0.18 |
| c11 | IVAS FX enc / FL dec, DTX Off, 48 kbps | 180 | 3.82 | 1.07 | 0.16 |
| c12 | IVAS FL enc / FX dec, DTX Off, 64 kbps | 180 | 4.03 | 0.98 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX Off, 96 kbps | 180 | 4.32 | 0.73 | 0.11 |
| c14 | IVAS FL enc / FX dec, DTX Off, 128 kbps | 180 | 4.42 | 0.74 | 0.11 |
| c15 | IVAS FX enc / FL dec, DTX Off, 192 kbps | 180 | 4.44 | 0.72 | 0.11 |
| c16 | IVAS FL enc / FX dec, DTX Off, 256 kbps | 180 | 4.45 | 0.77 | 0.11 |
| c17 | IVAS FX enc / FL dec, DTX Off, 384 kbps | 180 | 4.38 | 0.74 | 0.11 |
| c18 | IVAS FL enc / FX dec, DTX Off, 512 kbps | 180 | 4.35 | 0.77 | 0.11 |
| c19 | IVAS FL, DTX Off, 32 kbps | 180 | 3.41 | 1.19 | 0.17 |
| c20 | IVAS FL, DTX Off, 48 kbps | 180 | 3.82 | 1.04 | 0.15 |
| c21 | IVAS FL, DTX Off, 64 kbps | 180 | 3.99 | 0.96 | 0.14 |
| c22 | IVAS FL, DTX Off, 96 kbps | 180 | 4.36 | 0.70 | 0.10 |
| c23 | IVAS FL, DTX Off, 128 kbps | 180 | 4.37 | 0.78 | 0.11 |
| c24 | IVAS FL, DTX Off, 192 kbps | 180 | 4.39 | 0.80 | 0.12 |
| c25 | IVAS FL, DTX Off, 256 kbps | 180 | 4.44 | 0.68 | 0.10 |
| c26 | IVAS FL, DTX Off, 384 kbps | 180 | 4.46 | 0.72 | 0.11 |
| c27 | IVAS FL, DTX Off, 512 kbps | 180 | 4.44 | 0.72 | 0.11 |
| c28 | IVAS FX, DTX Off, 32 kbps | 180 | 3.31 | 1.21 | 0.18 |
| c29 | IVAS FX, DTX Off, 48 kbps | 180 | 3.76 | 1.09 | 0.16 |
| c30 | IVAS FX, DTX Off, 64 kbps | 180 | 3.99 | 1.08 | 0.16 |
| c31 | IVAS FX, DTX Off, 96 kbps | 180 | 4.33 | 0.84 | 0.12 |
| c32 | IVAS FX, DTX Off, 128 kbps | 180 | 4.41 | 0.72 | 0.11 |
| c33 | IVAS FX, DTX Off, 192 kbps | 180 | 4.39 | 0.77 | 0.11 |
| c34 | IVAS FX, DTX Off, 256 kbps | 180 | 4.38 | 0.73 | 0.11 |
| c35 | IVAS FX, DTX Off, 384 kbps | 180 | 4.44 | 0.65 | 0.10 |
| c36 | IVAS FX, DTX Off, 512 kbps | 180 | 4.49 | 0.72 | 0.11 |

Figure 3.15-1: results per condition for experiment P800-15

Figure 3.15-2: results aggregated over FX/FL for experiment P800-15

Table 3.15-2: statistical significance analysis for experiment P800-15

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.33 | 1.22 | c19 | 3.41 | 1.19 | -0.08 | <0.001 | 0.743 | NWT |
| c11 | 3.82 | 1.07 | c20 | 3.82 | 1.04 | 0.00 | 0.045 | 0.482 | NWT |
| c12 | 4.03 | 0.98 | c21 | 3.99 | 0.96 | 0.03 | 0.332 | 0.370 | NWT |
| c13 | 4.32 | 0.73 | c22 | 4.36 | 0.70 | -0.04 | <0.001 | 0.721 | NWT |
| c14 | 4.42 | 0.74 | c23 | 4.37 | 0.78 | 0.05 | 0.625 | 0.266 | NWT |
| c15 | 4.44 | 0.72 | c24 | 4.39 | 0.80 | 0.05 | 0.623 | 0.267 | NWT |
| c16 | 4.45 | 0.77 | c25 | 4.44 | 0.68 | 0.01 | 0.144 | 0.443 | NWT |
| c17 | 4.38 | 0.74 | c26 | 4.46 | 0.72 | -0.08 | <0.001 | 0.859 | NWT |
| c18 | 4.35 | 0.77 | c27 | 4.44 | 0.72 | -0.09 | <0.001 | 0.872 | NWT |
| c28 | 3.31 | 1.21 | c19 | 3.41 | 1.19 | -0.10 | <0.001 | 0.796 | NWT |
| c29 | 3.76 | 1.09 | c20 | 3.82 | 1.04 | -0.06 | <0.001 | 0.707 | NWT |
| c30 | 3.99 | 1.08 | c21 | 3.99 | 0.96 | 0.00 | <0.001 | 0.500 | NWT |
| c31 | 4.33 | 0.84 | c22 | 4.36 | 0.70 | -0.03 | <0.001 | 0.657 | NWT |
| c32 | 4.41 | 0.72 | c23 | 4.37 | 0.78 | 0.03 | 0.430 | 0.334 | NWT |
| c33 | 4.39 | 0.77 | c24 | 4.39 | 0.80 | -0.00 | <0.001 | 0.524 | NWT |
| c34 | 4.38 | 0.73 | c25 | 4.44 | 0.68 | -0.06 | <0.001 | 0.795 | NWT |
| c35 | 4.44 | 0.65 | c26 | 4.46 | 0.72 | -0.02 | <0.001 | 0.619 | NWT |
| c36 | 4.49 | 0.72 | c27 | 4.44 | 0.72 | 0.05 | 0.659 | 0.255 | NWT |

## P800-16: OSBA, 3-4 Objects (FX, RD)

Experiment P800-16 was conducted by Ericsson LM (language: SWE).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations.  See listening lab report in S4-251873.

The aggregated results per condition are listed in Table 3.16-1 and are shown in Figure 3.16-1. The results aggregated over FX/FL are shown in Figure 3.16-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.16-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.16-1: results per condition for experiment P800-16

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 179 | 4.60 | 0.59 | 0.09 |
| c02 | MNRU Q = 32 dB | 180 | 4.39 | 0.73 | 0.11 |
| c03 | MNRU Q = 27 dB | 180 | 3.59 | 1.30 | 0.19 |
| c04 | MNRU Q = 22 dB | 179 | 2.42 | 1.22 | 0.18 |
| c05 | MNRU Q = 17 dB | 179 | 1.52 | 0.91 | 0.13 |
| c06 | ESDRU α = 0.8 | 179 | 4.43 | 0.73 | 0.11 |
| c07 | ESDRU α = 0.6 | 180 | 3.84 | 1.03 | 0.15 |
| c08 | ESDRU α = 0.4 | 179 | 3.17 | 1.14 | 0.17 |
| c09 | ESDRU α = 0.2 | 179 | 2.71 | 1.22 | 0.18 |
| c10 | IVAS FL enc / FX dec, DTX Off, 32 kbps | 180 | 3.42 | 1.13 | 0.17 |
| c11 | IVAS FX enc / FL dec, DTX Off, 48 kbps | 180 | 3.82 | 0.97 | 0.14 |
| c12 | IVAS FL enc / FX dec, DTX Off, 64 kbps | 179 | 4.23 | 0.76 | 0.11 |
| c13 | IVAS FX enc / FL dec, DTX Off, 96 kbps | 179 | 4.35 | 0.74 | 0.11 |
| c14 | IVAS FL enc / FX dec, DTX Off, 128 kbps | 180 | 4.59 | 0.65 | 0.10 |
| c15 | IVAS FX enc / FL dec, DTX Off, 192 kbps | 180 | 4.55 | 0.68 | 0.10 |
| c16 | IVAS FL enc / FX dec, DTX Off, 256 kbps | 179 | 4.62 | 0.66 | 0.10 |
| c17 | IVAS FX enc / FL dec, DTX Off, 384 kbps | 180 | 4.63 | 0.63 | 0.09 |
| c18 | IVAS FL enc / FX dec, DTX Off, 512 kbps | 180 | 4.63 | 0.58 | 0.09 |
| c19 | IVAS FL, DTX Off, 32 kbps | 180 | 3.35 | 1.16 | 0.17 |
| c20 | IVAS FL, DTX Off, 48 kbps | 180 | 3.92 | 0.92 | 0.14 |
| c21 | IVAS FL, DTX Off, 64 kbps | 180 | 4.04 | 0.85 | 0.12 |
| c22 | IVAS FL, DTX Off, 96 kbps | 178 | 4.45 | 0.70 | 0.10 |
| c23 | IVAS FL, DTX Off, 128 kbps | 180 | 4.49 | 0.66 | 0.10 |
| c24 | IVAS FL, DTX Off, 192 kbps | 179 | 4.60 | 0.56 | 0.08 |
| c25 | IVAS FL, DTX Off, 256 kbps | 180 | 4.59 | 0.70 | 0.10 |
| c26 | IVAS FL, DTX Off, 384 kbps | 180 | 4.58 | 0.62 | 0.09 |
| c27 | IVAS FL, DTX Off, 512 kbps | 180 | 4.56 | 0.65 | 0.10 |
| c28 | IVAS FX, DTX Off, 32 kbps | 180 | 3.38 | 1.07 | 0.16 |
| c29 | IVAS FX, DTX Off, 48 kbps | 179 | 3.92 | 1.00 | 0.15 |
| c30 | IVAS FX, DTX Off, 64 kbps | 178 | 4.18 | 0.83 | 0.12 |
| c31 | IVAS FX, DTX Off, 96 kbps | 180 | 4.46 | 0.72 | 0.11 |
| c32 | IVAS FX, DTX Off, 128 kbps | 180 | 4.58 | 0.60 | 0.09 |
| c33 | IVAS FX, DTX Off, 192 kbps | 180 | 4.60 | 0.61 | 0.09 |
| c34 | IVAS FX, DTX Off, 256 kbps | 180 | 4.68 | 0.54 | 0.08 |
| c35 | IVAS FX, DTX Off, 384 kbps | 180 | 4.61 | 0.62 | 0.09 |
| c36 | IVAS FX, DTX Off, 512 kbps | 179 | 4.60 | 0.69 | 0.10 |

Figure 3.16-1: results per condition for experiment P800-16

Figure 3.16-2: results aggregated over FX/FL for experiment P800-16

Table 3.16-2: statistical significance analysis for experiment P800-16

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.42 | 1.13 | c19 | 3.35 | 1.16 | 0.07 | 0.596 | 0.276 | NWT |
| c11 | 3.82 | 0.97 | c20 | 3.92 | 0.92 | -0.10 | <0.001 | 0.841 | NWT |
| c12 | 4.23 | 0.76 | c21 | 4.04 | 0.85 | 0.19 | 2.169 | 0.015 | BT |
| c13 | 4.35 | 0.74 | c22 | 4.45 | 0.70 | -0.10 | <0.001 | 0.911 | NWT |
| c14 | 4.59 | 0.65 | c23 | 4.49 | 0.66 | 0.11 | 1.527 | 0.064 | NWT |
| c15 | 4.55 | 0.68 | c24 | 4.60 | 0.56 | -0.05 | <0.001 | 0.768 | NWT |
| c16 | 4.62 | 0.66 | c25 | 4.59 | 0.70 | 0.03 | 0.362 | 0.359 | NWT |
| c17 | 4.63 | 0.63 | c26 | 4.58 | 0.62 | 0.05 | 0.764 | 0.223 | NWT |
| c18 | 4.63 | 0.58 | c27 | 4.56 | 0.65 | 0.07 | 1.030 | 0.152 | NWT |
| c28 | 3.38 | 1.07 | c19 | 3.35 | 1.16 | 0.03 | 0.238 | 0.406 | NWT |
| c29 | 3.92 | 1.00 | c20 | 3.92 | 0.92 | 0.00 | <0.001 | 0.500 | NWT |
| c30 | 4.18 | 0.83 | c21 | 4.04 | 0.85 | 0.14 | 1.584 | 0.057 | NWT |
| c31 | 4.46 | 0.72 | c22 | 4.45 | 0.70 | 0.01 | 0.160 | 0.436 | NWT |
| c32 | 4.58 | 0.60 | c23 | 4.49 | 0.66 | 0.09 | 1.345 | 0.090 | NWT |
| c33 | 4.60 | 0.61 | c24 | 4.60 | 0.56 | 0.00 | 0.032 | 0.487 | NWT |
| c34 | 4.68 | 0.54 | c25 | 4.59 | 0.70 | 0.08 | 1.280 | 0.101 | NWT |
| c35 | 4.61 | 0.62 | c26 | 4.58 | 0.62 | 0.03 | 0.507 | 0.306 | NWT |
| c36 | 4.60 | 0.69 | c27 | 4.56 | 0.65 | 0.04 | 0.521 | 0.301 | NWT |

## P800-17: OSBA, 1-4 Objects (RD, FE)

Experiment P800-17 was conducted by Fraunhofer IIS (language: GER).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations. See listening lab report in S4-251723.

The aggregated results per condition are listed in Table 3.17-1 and are shown in Figure 3.17-1. The results aggregated over FX/FL are shown in Figure 3.17-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.17-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.17-1: results per condition for experiment P800-17

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 150 | 4.77 | 0.47 | 0.07 |
| c02 | MNRU Q = 34 dB | 150 | 4.69 | 0.57 | 0.09 |
| c03 | MNRU Q = 30 dB | 150 | 4.73 | 0.54 | 0.09 |
| c04 | MNRU Q = 26 dB | 150 | 4.43 | 0.70 | 0.11 |
| c05 | MNRU Q = 22 dB | 150 | 3.62 | 1.15 | 0.19 |
| c06 | ESDRU α = 0.8 | 150 | 4.56 | 0.61 | 0.10 |
| c07 | ESDRU α = 0.6 | 150 | 3.37 | 1.11 | 0.18 |
| c08 | ESDRU α = 0.4 | 150 | 2.54 | 1.11 | 0.18 |
| c09 | ESDRU α = 0.2 | 150 | 2.16 | 1.10 | 0.18 |
| c10 | IVAS FL, DTX off, FER 0%, 32 kbps, OSBA | 150 | 3.40 | 0.95 | 0.15 |
| c11 | IVAS FL, DTX off, FER 0%, 64 kbps, OSBA | 150 | 4.13 | 0.84 | 0.14 |
| c12 | IVAS FL, DTX off, FER 0%, 128 kbps, OSBA | 150 | 4.70 | 0.53 | 0.09 |
| c13 | IVAS FL, DTX off, FER 0%, 256 kbps, OSBA | 150 | 4.67 | 0.55 | 0.09 |
| c14 | IVAS FX, DTX off, FER 5%, 32 kbps, OSBA | 150 | 2.80 | 1.07 | 0.17 |
| c15 | IVAS FX, DTX off, FER 5%, 64 kbps, OSBA | 150 | 3.39 | 1.01 | 0.16 |
| c16 | IVAS FX, DTX off, FER 5%, 256 kbps, OSBA | 150 | 4.21 | 0.81 | 0.13 |
| c17 | IVAS FX, DTX off, FER 0%, 13.2 kbps, OSBA | 150 | 1.83 | 1.00 | 0.16 |
| c18 | IVAS FX, DTX off, FER 0%, 16.4 kbps, OSBA | 150 | 2.53 | 1.07 | 0.17 |
| c19 | IVAS FX, DTX off, FER 0%, 24.4 kbps, OSBA | 150 | 3.23 | 1.02 | 0.16 |
| c20 | IVAS FX, DTX off, FER 0%, 32 kbps, OSBA | 150 | 3.37 | 1.04 | 0.17 |
| c21 | IVAS FX, DTX off, FER 0%, 48 kbps, OSBA | 150 | 3.85 | 0.92 | 0.15 |
| c22 | IVAS FX, DTX off, FER 0%, 64 kbps, OSBA | 150 | 4.14 | 0.84 | 0.14 |
| c23 | IVAS FX, DTX off, FER 0%, 80 kbps, OSBA | 150 | 4.10 | 0.86 | 0.14 |
| c24 | IVAS FX, DTX off, FER 0%, 96 kbps, OSBA | 150 | 4.57 | 0.56 | 0.09 |
| c25 | IVAS FX, DTX off, FER 0%, 128 kbps, OSBA | 150 | 4.62 | 0.55 | 0.09 |
| c26 | IVAS FX, DTX off, FER 0%, 160 kbps, OSBA | 150 | 4.61 | 0.63 | 0.10 |
| c27 | IVAS FX, DTX off, FER 0%, 192 kbps, OSBA | 150 | 4.65 | 0.67 | 0.11 |
| c28 | IVAS FX, DTX off, FER 0%, 256 kbps, OSBA | 150 | 4.65 | 0.59 | 0.10 |
| c29 | IVAS FX, DTX off, FER 0%, 384 kbps, OSBA | 150 | 4.77 | 0.50 | 0.08 |
| c30 | IVAS FX, DTX off, FER 0%, 512 kbps, OSBA | 150 | 4.75 | 0.48 | 0.08 |
| c31 | IVAS FX, DTX off, FER 0%, 48.8 kbps, ISM + HOA3 | 150 | 4.03 | 0.82 | 0.13 |
| c32 | IVAS FX, DTX off, FER 0%, 64 kbps, ISM + HOA3 | 150 | 4.20 | 0.74 | 0.12 |
| c33 | IVAS FX, DTX off, FER 0%, 96 kbps, ISM + HOA3 | 150 | 4.37 | 0.70 | 0.11 |
| c34 | IVAS FX, DTX off, FER 0%, 128 kbps, ISM + HOA3 | 150 | 4.45 | 0.77 | 0.12 |
| c35 | IVAS FX, DTX off, FER 0%, 192 kbps, ISM + HOA3 | 150 | 4.55 | 0.64 | 0.10 |
| c36 | IVAS FX, DTX off, FER 0%, 256 kbps, ISM + HOA3 | 150 | 4.70 | 0.61 | 0.10 |

Figure 3.17-1: results per condition for experiment P800-17

Figure 3.17-2: results aggregated over FX/FL for experiment P800-17

Table 3.17-2: statistical significance analysis for experiment P800-17

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c20 | 3.37 | 1.04 | c10 | 3.40 | 0.95 | -0.03 | <0.001 | 0.593 | NWT |
| c21 | 3.85 | 0.92 | c31 | 4.03 | 0.82 | -0.18 | <0.001 | 0.963 | WT |
| c22 | 4.14 | 0.84 | c11 | 4.13 | 0.84 | 0.01 | 0.135 | 0.447 | NWT |
| c22 | 4.14 | 0.84 | c32 | 4.20 | 0.74 | -0.06 | <0.001 | 0.744 | NWT |
| c24 | 4.57 | 0.56 | c33 | 4.37 | 0.70 | 0.20 | 2.732 | 0.003 | BT |
| c25 | 4.62 | 0.55 | c12 | 4.70 | 0.53 | -0.08 | <0.001 | 0.900 | NWT |
| c25 | 4.62 | 0.55 | c34 | 4.45 | 0.77 | 0.17 | 2.151 | 0.016 | BT |
| c27 | 4.65 | 0.67 | c35 | 4.55 | 0.64 | 0.09 | 1.245 | 0.107 | NWT |
| c28 | 4.65 | 0.59 | c13 | 4.67 | 0.55 | -0.02 | <0.001 | 0.619 | NWT |
| c28 | 4.65 | 0.59 | c36 | 4.70 | 0.61 | -0.05 | <0.001 | 0.751 | NWT |

## P800-18: OMASA, 1-2 Objects (FX, RD)

Experiment P800-18 was conducted by Ericsson LM (language: SWE).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations.  See listening lab report in S4-251873.

The aggregated results per condition are listed in Table 3.18-1 and are shown in Figure 3.18-1. The results aggregated over FX/FL are shown in Figure 3.18-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.18-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.18-1: results per condition for experiment P800-18

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 179 | 4.60 | 0.58 | 0.09 |
| c02 | MNRU Q = 32 dB | 180 | 4.11 | 0.98 | 0.14 |
| c03 | MNRU Q = 27 dB | 180 | 2.96 | 1.20 | 0.18 |
| c04 | MNRU Q = 22 dB | 179 | 1.84 | 1.01 | 0.15 |
| c05 | MNRU Q = 17 dB | 180 | 1.22 | 0.50 | 0.07 |
| c06 | ESDRU α = 0.8 | 179 | 4.52 | 0.65 | 0.10 |
| c07 | ESDRU α = 0.6 | 180 | 3.63 | 0.94 | 0.14 |
| c08 | ESDRU α = 0.4 | 180 | 2.73 | 1.04 | 0.15 |
| c09 | ESDRU α = 0.2 | 180 | 2.21 | 1.02 | 0.15 |
| c10 | IVAS FL enc / FX dec, DTX off, 13.2 kbps | 180 | 2.82 | 0.95 | 0.14 |
| c11 | IVAS FX enc / FL dec, DTX off, 16.4 kbps | 180 | 3.20 | 1.00 | 0.15 |
| c12 | IVAS FL enc / FX dec, DTX off, 24.4 kbps | 180 | 3.53 | 0.90 | 0.13 |
| c13 | IVAS FX enc / FL dec, DTX off, 32 kbps | 180 | 3.91 | 0.88 | 0.13 |
| c14 | IVAS FL enc / FX dec, DTX off, 48 kbps | 179 | 4.43 | 0.64 | 0.10 |
| c15 | IVAS FX enc / FL dec, DTX off, 64 kbps | 180 | 4.48 | 0.66 | 0.10 |
| c16 | IVAS FL enc / FX dec, DTX off, 80 kbps | 179 | 4.50 | 0.62 | 0.09 |
| c17 | IVAS FX enc / FL dec, DTX off, 128 kbps | 179 | 4.56 | 0.59 | 0.09 |
| c18 | IVAS FL enc / FX dec, DTX off, 256 kbps | 180 | 4.63 | 0.57 | 0.08 |
| c19 | IVAS FL, DTX off, 13.2 kbps | 180 | 2.92 | 0.99 | 0.14 |
| c20 | IVAS FL, DTX off, 16.4 kbps | 180 | 3.21 | 1.01 | 0.15 |
| c21 | IVAS FL, DTX off, 24.4 kbps | 179 | 3.56 | 1.03 | 0.15 |
| c22 | IVAS FL, DTX off, 32 kbps | 180 | 3.92 | 0.89 | 0.13 |
| c23 | IVAS FL, DTX off, 48 kbps | 179 | 4.41 | 0.72 | 0.11 |
| c24 | IVAS FL, DTX off, 64 kbps | 180 | 4.53 | 0.65 | 0.10 |
| c25 | IVAS FL, DTX off, 80 kbps | 179 | 4.52 | 0.63 | 0.09 |
| c26 | IVAS FL, DTX off, 128 kbps | 179 | 4.60 | 0.59 | 0.09 |
| c27 | IVAS FL, DTX off, 256 kbps | 180 | 4.64 | 0.57 | 0.08 |
| c28 | IVAS FX, DTX off, 13.2 kbps | 179 | 2.86 | 0.96 | 0.14 |
| c29 | IVAS FX, DTX off, 16.4 kbps | 180 | 3.19 | 1.02 | 0.15 |
| c30 | IVAS FX, DTX off, 24.4 kbps | 180 | 3.52 | 1.02 | 0.15 |
| c31 | IVAS FX, DTX off, 32 kbps | 180 | 3.86 | 0.91 | 0.13 |
| c32 | IVAS FX, DTX off, 48 kbps | 179 | 4.40 | 0.68 | 0.10 |
| c33 | IVAS FX, DTX off, 64 kbps | 180 | 4.56 | 0.61 | 0.09 |
| c34 | IVAS FX, DTX off, 80 kbps | 180 | 4.53 | 0.63 | 0.09 |
| c35 | IVAS FX, DTX off, 128 kbps | 179 | 4.65 | 0.55 | 0.08 |
| c36 | IVAS FX, DTX off, 256 kbps | 180 | 4.64 | 0.60 | 0.09 |

Figure 3.18-1: results per condition for experiment P800-18

Figure 3.18-2: results aggregated over FX/FL for experiment P800-18

Table 3.18-2: statistical significance analysis for experiment P800-18

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 2.82 | 0.95 | c19 | 2.92 | 0.99 | -0.10 | <0.001 | 0.835 | NWT |
| c11 | 3.20 | 1.00 | c20 | 3.21 | 1.01 | -0.01 | <0.001 | 0.522 | NWT |
| c12 | 3.53 | 0.90 | c21 | 3.56 | 1.03 | -0.04 | <0.001 | 0.638 | NWT |
| c13 | 3.91 | 0.88 | c22 | 3.92 | 0.89 | -0.02 | <0.001 | 0.568 | NWT |
| c14 | 4.43 | 0.64 | c23 | 4.41 | 0.72 | 0.02 | 0.306 | 0.380 | NWT |
| c15 | 4.48 | 0.66 | c24 | 4.53 | 0.65 | -0.04 | <0.001 | 0.744 | NWT |
| c16 | 4.50 | 0.62 | c25 | 4.52 | 0.63 | -0.02 | <0.001 | 0.601 | NWT |
| c17 | 4.56 | 0.59 | c26 | 4.60 | 0.59 | -0.04 | <0.001 | 0.734 | NWT |
| c18 | 4.63 | 0.57 | c27 | 4.64 | 0.57 | -0.01 | <0.001 | 0.573 | NWT |
| c28 | 2.86 | 0.96 | c19 | 2.92 | 0.99 | -0.06 | <0.001 | 0.726 | NWT |
| c29 | 3.19 | 1.02 | c20 | 3.21 | 1.01 | -0.01 | <0.001 | 0.545 | NWT |
| c30 | 3.52 | 1.02 | c21 | 3.56 | 1.03 | -0.05 | <0.001 | 0.668 | NWT |
| c31 | 3.86 | 0.91 | c22 | 3.92 | 0.89 | -0.07 | <0.001 | 0.756 | NWT |
| c32 | 4.40 | 0.68 | c23 | 4.41 | 0.72 | -0.01 | <0.001 | 0.559 | NWT |
| c33 | 4.56 | 0.61 | c24 | 4.53 | 0.65 | 0.03 | 0.423 | 0.336 | NWT |
| c34 | 4.53 | 0.63 | c25 | 4.52 | 0.63 | 0.01 | 0.196 | 0.423 | NWT |
| c35 | 4.65 | 0.55 | c26 | 4.60 | 0.59 | 0.06 | 0.930 | 0.177 | NWT |
| c36 | 4.64 | 0.60 | c27 | 4.64 | 0.57 | 0.00 | <0.001 | 0.500 | NWT |

## P800-19: OMASA, 3-4 Objects (FX, RD)

Experiment P800-19 was conducted by Fraunhofer IIS (language: GER).

The listening lab has reported that some votes are missing. This was taken into account for result aggregation and statistical calculations. See listening lab report in S4-251723.

The aggregated results per condition are listed in Table 3.19-1 and are shown in Figure 3.19-1. The results aggregated over FX/FL are shown in Figure 3.19-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.19-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.19-1: results per condition for experiment P800-19

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 150 | 4.68 | 0.58 | 0.09 |
| c02 | MNRU Q = 32 dB | 150 | 4.59 | 0.61 | 0.10 |
| c03 | MNRU Q = 27 dB | 150 | 4.21 | 0.86 | 0.14 |
| c04 | MNRU Q = 22 dB | 150 | 3.15 | 1.21 | 0.20 |
| c05 | MNRU Q = 17 dB | 150 | 1.83 | 0.90 | 0.14 |
| c06 | ESDRU α = 0.8 | 150 | 4.54 | 0.59 | 0.10 |
| c07 | ESDRU α = 0.6 | 150 | 3.55 | 0.95 | 0.15 |
| c08 | ESDRU α = 0.4 | 150 | 2.69 | 1.09 | 0.18 |
| c09 | ESDRU α = 0.2 | 150 | 2.10 | 1.02 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX off, 16.4 kbps | 150 | 2.71 | 1.11 | 0.18 |
| c11 | IVAS FX enc / FL dec, DTX off, 24.4 kbps | 150 | 2.69 | 1.04 | 0.17 |
| c12 | IVAS FL enc / FX dec, DTX off, 48 kbps | 150 | 4.15 | 0.90 | 0.14 |
| c13 | IVAS FX enc / FL dec, DTX off, 64 kbps | 150 | 4.28 | 0.80 | 0.13 |
| c14 | IVAS FL enc / FX dec, DTX off, 80 kbps | 150 | 4.31 | 0.75 | 0.12 |
| c15 | IVAS FX enc / FL dec, DTX off, 96 kbps | 150 | 4.33 | 0.76 | 0.12 |
| c16 | IVAS FL enc / FX dec, DTX off, 128 kbps | 150 | 4.51 | 0.73 | 0.12 |
| c17 | IVAS FX enc / FL dec, DTX off, 256 kbps | 150 | 4.53 | 0.62 | 0.10 |
| c18 | IVAS FL enc / FX dec, DTX off, 512 kbps | 150 | 4.58 | 0.64 | 0.10 |
| c19 | IVAS FL, DTX off, 16.4 kbps | 150 | 2.76 | 1.15 | 0.18 |
| c20 | IVAS FL, DTX off, 24.4 kbps | 150 | 2.90 | 1.04 | 0.17 |
| c21 | IVAS FL, DTX off, 48 kbps | 150 | 4.10 | 0.87 | 0.14 |
| c22 | IVAS FL, DTX off, 64 kbps | 150 | 4.12 | 0.82 | 0.13 |
| c23 | IVAS FL, DTX off, 80 kbps | 150 | 4.31 | 0.74 | 0.12 |
| c24 | IVAS FL, DTX off, 96 kbps | 150 | 4.43 | 0.70 | 0.11 |
| c25 | IVAS FL, DTX off, 128 kbps | 150 | 4.40 | 0.71 | 0.12 |
| c26 | IVAS FL, DTX off, 256 kbps | 150 | 4.56 | 0.62 | 0.10 |
| c27 | IVAS FL, DTX off, 512 kbps | 150 | 4.66 | 0.59 | 0.10 |
| c28 | IVAS FX, DTX off, 16.4 kbps | 150 | 2.66 | 1.14 | 0.18 |
| c29 | IVAS FX, DTX off, 24.4 kbps | 150 | 2.78 | 1.07 | 0.17 |
| c30 | IVAS FX, DTX off, 48 kbps | 150 | 4.14 | 0.77 | 0.12 |
| c31 | IVAS FX, DTX off, 64 kbps | 150 | 4.21 | 0.73 | 0.12 |
| c32 | IVAS FX, DTX off, 80 kbps | 150 | 4.34 | 0.72 | 0.12 |
| c33 | IVAS FX, DTX off, 96 kbps | 150 | 4.31 | 0.74 | 0.12 |
| c34 | IVAS FX, DTX off, 128 kbps | 150 | 4.43 | 0.68 | 0.11 |
| c35 | IVAS FX, DTX off, 256 kbps | 150 | 4.53 | 0.65 | 0.10 |
| c36 | IVAS FX, DTX off, 512 kbps | 150 | 4.61 | 0.61 | 0.10 |

Figure 3.19-1: results per condition for experiment P800-19

Figure 3.19-2: results aggregated over FX/FL for experiment P800-19

Table 3.19-2: statistical significance analysis for experiment P800-19

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 2.71 | 1.11 | c19 | 2.76 | 1.15 | -0.05 | <0.001 | 0.658 | NWT |
| c11 | 2.69 | 1.04 | c20 | 2.90 | 1.04 | -0.21 | <0.001 | 0.957 | WT |
| c12 | 4.15 | 0.90 | c21 | 4.10 | 0.87 | 0.05 | 0.519 | 0.302 | NWT |
| c13 | 4.28 | 0.80 | c22 | 4.12 | 0.82 | 0.16 | 1.707 | 0.044 | BT |
| c14 | 4.31 | 0.75 | c23 | 4.31 | 0.74 | 0.00 | <0.001 | 0.500 | NWT |
| c15 | 4.33 | 0.76 | c24 | 4.43 | 0.70 | -0.09 | <0.001 | 0.868 | NWT |
| c16 | 4.51 | 0.73 | c25 | 4.40 | 0.71 | 0.11 | 1.283 | 0.100 | NWT |
| c17 | 4.53 | 0.62 | c26 | 4.56 | 0.62 | -0.03 | <0.001 | 0.647 | NWT |
| c18 | 4.58 | 0.64 | c27 | 4.66 | 0.59 | -0.08 | <0.001 | 0.870 | NWT |
| c28 | 2.66 | 1.14 | c19 | 2.76 | 1.15 | -0.10 | <0.001 | 0.775 | NWT |
| c29 | 2.78 | 1.07 | c20 | 2.90 | 1.04 | -0.12 | <0.001 | 0.837 | NWT |
| c30 | 4.14 | 0.77 | c21 | 4.10 | 0.87 | 0.04 | 0.421 | 0.337 | NWT |
| c31 | 4.21 | 0.73 | c22 | 4.12 | 0.82 | 0.09 | 0.974 | 0.166 | NWT |
| c32 | 4.34 | 0.72 | c23 | 4.31 | 0.74 | 0.03 | 0.319 | 0.375 | NWT |
| c33 | 4.31 | 0.74 | c24 | 4.43 | 0.70 | -0.11 | <0.001 | 0.914 | NWT |
| c34 | 4.43 | 0.68 | c25 | 4.40 | 0.71 | 0.03 | 0.410 | 0.341 | NWT |
| c35 | 4.53 | 0.65 | c26 | 4.56 | 0.62 | -0.03 | <0.001 | 0.643 | NWT |
| c36 | 4.61 | 0.61 | c27 | 4.66 | 0.59 | -0.05 | <0.001 | 0.751 | NWT |

## P800-20: OMASA, 1-4 Objects (RD, FE)

Experiment P800-20 was conducted by Nokia (language: FIN).

The listening lab has reported that the presentation order of the samples to the listeners was not according to the original randomization tables due to a configuration issue in the listening test software. See listening lab report in S4-251691.

The aggregated results per condition are listed in Table 3.20-1 and are shown in Figure 3.20-1. The results aggregated over FX/FL are shown in Figure 3.20-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.20-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.20-1: results per condition for experiment P800-20

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.59 | 0.59 | 0.09 |
| c02 | MNRU Q = 34 dB | 180 | 4.58 | 0.62 | 0.09 |
| c03 | MNRU Q = 30 dB | 180 | 4.47 | 0.68 | 0.10 |
| c04 | MNRU Q = 26 dB | 180 | 3.81 | 1.08 | 0.16 |
| c05 | MNRU Q = 22 dB | 180 | 2.44 | 1.27 | 0.19 |
| c06 | ESDRU α = 0.8 | 180 | 4.34 | 0.73 | 0.11 |
| c07 | ESDRU α = 0.6 | 180 | 3.04 | 1.03 | 0.15 |
| c08 | ESDRU α = 0.4 | 180 | 2.11 | 0.97 | 0.14 |
| c09 | ESDRU α = 0.2 | 180 | 1.67 | 0.91 | 0.13 |
| c10 | IVAS FL, DTX off, FER 0%, 32 kbps, OMASA 2TC | 180 | 2.52 | 1.23 | 0.18 |
| c11 | IVAS FL, DTX off, FER 0%, 64 kbps, OMASA 2TC | 180 | 3.79 | 0.97 | 0.14 |
| c12 | IVAS FL, DTX off, FER 0%, 128 kbps, OMASA 2TC | 180 | 4.24 | 0.83 | 0.12 |
| c13 | IVAS FL, DTX off, FER 0%, 256 kbps, OMASA 2TC | 180 | 4.49 | 0.65 | 0.10 |
| c14 | IVAS FX, DTX off, FER 5%, 32 kbps, OMASA 2TC | 180 | 2.02 | 0.91 | 0.13 |
| c15 | IVAS FX, DTX off, FER 5%, 64 kbps, OMASA 2TC | 180 | 3.12 | 1.04 | 0.15 |
| c16 | IVAS FX, DTX off, FER 5%, 256 kbps, OMASA 2TC | 180 | 3.59 | 0.96 | 0.14 |
| c17 | IVAS FX, DTX off, FER 0%, 13.2 kbps, OMASA 2TC | 180 | 1.51 | 0.69 | 0.10 |
| c18 | IVAS FX, DTX off, FER 0%, 16.4 kbps, OMASA 2TC | 180 | 1.92 | 0.91 | 0.13 |
| c19 | IVAS FX, DTX off, FER 0%, 24.4 kbps, OMASA 2TC | 180 | 2.14 | 0.89 | 0.13 |
| c20 | IVAS FX, DTX off, FER 0%, 32 kbps, OMASA 2TC | 180 | 2.54 | 1.19 | 0.17 |
| c21 | IVAS FX, DTX off, FER 0%, 48 kbps, OMASA 2TC | 180 | 3.73 | 0.97 | 0.14 |
| c22 | IVAS FX, DTX off, FER 0%, 64 kbps, OMASA 2TC | 180 | 3.82 | 0.99 | 0.15 |
| c23 | IVAS FX, DTX off, FER 0%, 80 kbps, OMASA 2TC | 180 | 3.88 | 0.90 | 0.13 |
| c24 | IVAS FX, DTX off, FER 0%, 96 kbps, OMASA 2TC | 180 | 4.08 | 0.83 | 0.12 |
| c25 | IVAS FX, DTX off, FER 0%, 128 kbps, OMASA 2TC | 180 | 4.31 | 0.76 | 0.11 |
| c26 | IVAS FX, DTX off, FER 0%, 160 kbps, OMASA 2TC | 180 | 4.26 | 0.84 | 0.12 |
| c27 | IVAS FX, DTX off, FER 0%, 192 kbps, OMASA 2TC | 180 | 4.36 | 0.69 | 0.10 |
| c28 | IVAS FX, DTX off, FER 0%, 256 kbps, OMASA 2TC | 180 | 4.51 | 0.61 | 0.09 |
| c29 | IVAS FX, DTX off, FER 0%, 384 kbps, OMASA 2TC | 180 | 4.50 | 0.66 | 0.10 |
| c30 | IVAS FX, DTX off, FER 0%, 512 kbps, OMASA 2TC | 180 | 4.52 | 0.66 | 0.10 |
| c31 | IVAS FX, DTX off, FER 0%, 48.8 kbps, ISM + MASA 2TC | 180 | 3.64 | 0.96 | 0.14 |
| c32 | IVAS FX, DTX off, FER 0%, 64 kbps, ISM + MASA 2TC | 180 | 3.76 | 0.93 | 0.14 |
| c33 | IVAS FX, DTX off, FER 0%, 96 kbps, ISM + MASA 2TC | 180 | 4.06 | 0.77 | 0.11 |
| c34 | IVAS FX, DTX off, FER 0%, 128 kbps, ISM + MASA 2TC | 180 | 4.25 | 0.72 | 0.11 |
| c35 | IVAS FX, DTX off, FER 0%, 192 kbps, ISM + MASA 2TC | 180 | 4.38 | 0.67 | 0.10 |
| c36 | IVAS FX, DTX off, FER 0%, 256 kbps, ISM + MASA 2TC | 180 | 4.54 | 0.60 | 0.09 |

Figure 3.20-1: results per condition for experiment P800-20

Figure 3.20-2: results aggregated over FX/FL for experiment P800-20

Table 3.20-2: statistical significance analysis for experiment P800-20

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c20 | 2.54 | 1.19 | c10 | 2.52 | 1.23 | 0.02 | 0.172 | 0.432 | NWT |
| c21 | 3.73 | 0.97 | c31 | 3.64 | 0.96 | 0.09 | 0.872 | 0.192 | NWT |
| c22 | 3.82 | 0.99 | c11 | 3.79 | 0.97 | 0.03 | 0.319 | 0.375 | NWT |
| c22 | 3.82 | 0.99 | c32 | 3.76 | 0.93 | 0.06 | 0.602 | 0.274 | NWT |
| c24 | 4.08 | 0.83 | c33 | 4.06 | 0.77 | 0.03 | 0.320 | 0.375 | NWT |
| c25 | 4.31 | 0.76 | c12 | 4.24 | 0.83 | 0.07 | 0.798 | 0.213 | NWT |
| c25 | 4.31 | 0.76 | c34 | 4.25 | 0.72 | 0.06 | 0.718 | 0.237 | NWT |
| c27 | 4.36 | 0.69 | c35 | 4.38 | 0.67 | -0.02 | <0.001 | 0.620 | NWT |
| c28 | 4.51 | 0.61 | c13 | 4.49 | 0.65 | 0.02 | 0.331 | 0.370 | NWT |
| c28 | 4.51 | 0.61 | c36 | 4.54 | 0.60 | -0.03 | <0.001 | 0.669 | NWT |

## P800-21: Stereo (JBM/FE)

Experiment P800-21 was conducted by Mesaqin.com (language: CZ).

The listening lab has reported that for each panel, up to seven instead of five participants participated. The five subjects with the highest PCC were selected for result submission.

The aggregated results per condition are listed in Table 3.21-1 and are shown in Figure 3.21-1. The results aggregated over FX/FL are shown in Figure 3.21-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.21-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.21-1: results per condition for experiment P800-21

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.77 | 0.43 | 0.06 |
| c02 | MNRU Q = 27 dB | 180 | 3.92 | 0.99 | 0.15 |
| c03 | MNRU Q = 22 dB | 180 | 3.16 | 1.16 | 0.17 |
| c04 | MNRU Q = 17 dB | 180 | 2.07 | 1.05 | 0.15 |
| c05 | MNRU Q = 12 dB | 180 | 1.28 | 0.56 | 0.08 |
| c06 | ESDRU α=0.7 | 180 | 4.54 | 0.70 | 0.10 |
| c07 | ESDRU α=0.5 | 180 | 3.95 | 1.05 | 0.15 |
| c08 | ESDRU α=0.3 | 180 | 3.73 | 1.13 | 0.17 |
| c09 | ESDRU α=0.1 | 180 | 2.97 | 1.17 | 0.17 |
| c10 | IVAS FL enc / FX dec, DTX on, 24.4 kbps, No error | 180 | 4.32 | 0.74 | 0.11 |
| c11 | IVAS FL enc / FX dec, DTX on, 48 kbps, No error | 180 | 4.59 | 0.59 | 0.09 |
| c12 | IVAS FL enc / FX dec, DTX on, 96 kbps, No error | 180 | 4.75 | 0.48 | 0.07 |
| c13 | IVAS FL enc / FX dec, DTX on, 24.4 kbps, Error I1. O1 | 180 | 3.27 | 1.02 | 0.15 |
| c14 | IVAS FL enc / FX dec, DTX on, 48 kbps, Error I1. O1 | 180 | 3.42 | 1.05 | 0.15 |
| c15 | IVAS FL enc / FX dec, DTX on, 24.4 kbps, Error I1. O2 | 180 | 4.03 | 1.04 | 0.15 |
| c16 | IVAS FL enc / FX dec, DTX on, 48 kbps, Error I1. O2 | 180 | 4.30 | 0.89 | 0.13 |
| c17 | IVAS FL enc / FX dec, DTX on, 24.4 kbps, I1. O1 | 180 | 3.36 | 1.15 | 0.17 |
| c18 | IVAS FL enc / FX dec, DTX on, 48 kbps, I1. O1 | 180 | 3.58 | 0.93 | 0.14 |
| c19 | IVAS FL enc / FX dec, DTX on, 96 kbps, I1. O1 | 180 | 3.71 | 0.96 | 0.14 |
| c20 | IVAS FL enc / FX dec, DTX on, 24.4 kbps, I1. O2 | 180 | 3.84 | 1.01 | 0.15 |
| c21 | IVAS FL enc / FX dec, DTX on, 48 kbps, I1. O2 | 180 | 4.17 | 1.02 | 0.15 |
| c22 | IVAS FL enc / FX dec, DTX on, 96 kbps, I1. O2 | 180 | 4.24 | 1.04 | 0.15 |
| c23 | IVAS FL enc / FL dec, DTX on, 24.4 kbps, No error | 180 | 4.28 | 0.82 | 0.12 |
| c24 | IVAS FL enc / FL dec, DTX on, 48 kbps, No error | 180 | 4.60 | 0.59 | 0.09 |
| c25 | IVAS FL enc / FL dec, DTX on, 96 kbps, No error | 180 | 4.67 | 0.53 | 0.08 |
| c26 | IVAS FL enc / FL dec, DTX on, 24.4 kbps, Error I1. O1 | 180 | 3.35 | 1.00 | 0.15 |
| c27 | IVAS FL enc / FL dec, DTX on, 48 kbps, Error I1. O1 | 180 | 3.51 | 0.99 | 0.14 |
| c28 | IVAS FL enc / FL dec, DTX on, 24.4 kbps, Error I1. O2 | 180 | 3.97 | 0.98 | 0.14 |
| c29 | IVAS FL enc / FL dec, DTX on, 48 kbps, Error I1. O2 | 180 | 4.32 | 0.94 | 0.14 |
| c30 | IVAS FL enc / FL dec, DTX on, 96 kbps, Error I1. O2 | 180 | 4.47 | 0.85 | 0.13 |
| c31 | IVAS FL enc / FL dec, DTX on, 24.4 kbps, I1. O1 | 180 | 3.38 | 1.03 | 0.15 |
| c32 | IVAS FL enc / FL dec, DTX on, 48 kbps, I1. O1 | 180 | 3.56 | 0.96 | 0.14 |
| c33 | IVAS FL enc / FL dec, DTX on, 96 kbps, I1. O1 | 180 | 3.68 | 0.92 | 0.14 |
| c34 | IVAS FL enc / FL dec, DTX on, 24.4 kbps, I1. O2 | 180 | 3.89 | 1.00 | 0.15 |
| c35 | IVAS FL enc / FL dec, DTX on, 48 kbps, I1. O2 | 180 | 4.15 | 1.05 | 0.15 |
| c36 | IVAS FL enc / FL dec, DTX on, 96 kbps, I1. O2 | 180 | 4.22 | 1.01 | 0.15 |

Figure 3.21-1: results per condition for experiment P800-21

Figure 3.21-2: results aggregated over FX/FL for experiment P800-21

Table 3.21-2: statistical significance analysis for experiment P800-21

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 4.32 | 0.74 | c23 | 4.28 | 0.82 | 0.04 | 0.473 | 0.318 | NWT |
| c11 | 4.59 | 0.59 | c24 | 4.60 | 0.59 | -0.01 | <0.001 | 0.538 | NWT |
| c12 | 4.75 | 0.48 | c25 | 4.67 | 0.53 | 0.08 | 1.555 | 0.060 | NWT |
| c13 | 3.27 | 1.02 | c26 | 3.35 | 1.00 | -0.08 | <0.001 | 0.782 | NWT |
| c14 | 3.42 | 1.05 | c27 | 3.51 | 0.99 | -0.09 | <0.001 | 0.796 | NWT |
| c15 | 4.03 | 1.04 | c28 | 3.97 | 0.98 | 0.06 | 0.572 | 0.284 | NWT |
| c16 | 4.30 | 0.89 | c29 | 4.32 | 0.94 | -0.02 | <0.001 | 0.570 | NWT |
| c17 | 3.36 | 1.15 | c31 | 3.38 | 1.03 | -0.02 | <0.001 | 0.576 | NWT |
| c18 | 3.58 | 0.93 | c32 | 3.56 | 0.96 | 0.02 | 0.220 | 0.413 | NWT |
| c19 | 3.71 | 0.96 | c33 | 3.68 | 0.92 | 0.03 | 0.283 | 0.389 | NWT |
| c20 | 3.84 | 1.01 | c34 | 3.89 | 1.00 | -0.05 | <0.001 | 0.682 | NWT |
| c21 | 4.17 | 1.02 | c35 | 4.15 | 1.05 | 0.02 | 0.155 | 0.438 | NWT |
| c22 | 4.24 | 1.04 | c36 | 4.22 | 1.01 | 0.02 | 0.157 | 0.438 | NWT |

## P800-22: 1-2 Objects (JBM/FE)

Experiment P800-22 was conducted by FORCE Technology (language: DAN).

The aggregated results per condition are listed in Table 3.22-1 and are shown in Figure 3.22-1. The results aggregated over FX/FL are shown in Figure 3.22-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.22-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.22-1: results per condition for experiment P800-22

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.69 | 0.54 | 0.08 |
| c02 | MNRU Q = 36 dB | 180 | 4.11 | 0.87 | 0.13 |
| c03 | MNRU Q = 30 dB | 180 | 3.06 | 1.15 | 0.17 |
| c04 | MNRU Q = 24 dB | 180 | 2.03 | 0.94 | 0.14 |
| c05 | MNRU Q = 18 dB | 180 | 1.40 | 0.65 | 0.10 |
| c06 | ESDRU α=0.7 | 180 | 4.38 | 0.69 | 0.10 |
| c07 | ESDRU α=0.5 | 180 | 4.04 | 0.87 | 0.13 |
| c08 | ESDRU α=0.3 | 180 | 3.50 | 1.11 | 0.16 |
| c09 | ESDRU α=0.1 | 180 | 2.99 | 1.22 | 0.18 |
| c10 | IVAS FL enc / FX dec, 24.4 kbps, No error | 180 | 4.36 | 0.77 | 0.11 |
| c11 | IVAS FL enc / FX dec, 48 kbps, No error | 180 | 4.51 | 0.68 | 0.10 |
| c12 | IVAS FL enc / FX dec, 96 kbps, No error | 180 | 4.68 | 0.55 | 0.08 |
| c13 | IVAS FL enc / FX dec, 24.4 kbps, Error I1. O1 | 180 | 3.28 | 1.06 | 0.16 |
| c14 | IVAS FL enc / FX dec, 48 kbps, Error I1. O1 | 180 | 3.33 | 1.09 | 0.16 |
| c15 | IVAS FL enc / FX dec, 24.4 kbps, Error I1. O2 | 180 | 4.19 | 1.01 | 0.15 |
| c16 | IVAS FL enc / FX dec, 48 kbps, Error I1. O2 | 180 | 4.42 | 0.88 | 0.13 |
| c17 | IVAS FL enc / FX dec, 24.4 kbps, I1. O1 | 180 | 3.09 | 1.22 | 0.18 |
| c18 | IVAS FL enc / FX dec, 48 kbps, I1. O1 | 180 | 3.44 | 1.06 | 0.16 |
| c19 | IVAS FL enc / FX dec, 96 kbps, I1. O1 | 180 | 3.53 | 0.99 | 0.14 |
| c20 | IVAS FL enc / FX dec, 24.4 kbps, I1. O2 | 180 | 4.26 | 0.96 | 0.14 |
| c21 | IVAS FL enc / FX dec, 48 kbps, I1. O2 | 180 | 4.37 | 0.86 | 0.13 |
| c22 | IVAS FL enc / FX dec, 96 kbps, I1. O2 | 180 | 4.46 | 0.85 | 0.13 |
| c23 | IVAS FL enc / FL dec, 24.4 kbps, No error | 180 | 4.32 | 0.84 | 0.12 |
| c24 | IVAS FL enc / FL dec, 48 kbps, No error | 180 | 4.59 | 0.61 | 0.09 |
| c25 | IVAS FL enc / FL dec, 96 kbps, No error | 180 | 4.59 | 0.59 | 0.09 |
| c26 | IVAS FL enc / FL dec, 24.4 kbps, Error I1. O1 | 180 | 3.37 | 1.13 | 0.17 |
| c27 | IVAS FL enc / FL dec, 48 kbps, Error I1. O1 | 180 | 3.54 | 1.07 | 0.16 |
| c28 | IVAS FL enc / FL dec, 24.4 kbps, Error I1. O2 | 180 | 4.14 | 0.96 | 0.14 |
| c29 | IVAS FL enc / FL dec, 48 kbps, Error I1. O2 | 180 | 4.37 | 0.91 | 0.13 |
| c30 | IVAS FL enc / FL dec, 96 kbps, Error I1. O2 | 180 | 4.49 | 0.90 | 0.13 |
| c31 | IVAS FL enc / FL dec, 24.4 kbps, I1. O1 | 180 | 3.26 | 1.08 | 0.16 |
| c32 | IVAS FL enc / FL dec, 48 kbps, I1. O1 | 180 | 3.42 | 1.08 | 0.16 |
| c33 | IVAS FL enc / FL dec, 96 kbps, I1. O1 | 180 | 3.45 | 1.06 | 0.16 |
| c34 | IVAS FL enc / FL dec, 24.4 kbps, I1. O2 | 180 | 4.21 | 0.91 | 0.13 |
| c35 | IVAS FL enc / FL dec, 48 kbps, I1. O2 | 180 | 4.34 | 0.94 | 0.14 |
| c36 | IVAS FL enc / FL dec, 96 kbps, I1. O2 | 180 | 4.52 | 0.82 | 0.12 |

Figure 3.22-1: results per condition for experiment P800-22

Figure 3.22-2: results aggregated over FX/FL for experiment P800-22

Table 3.22-2: statistical significance analysis for experiment P800-22

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 4.36 | 0.77 | c23 | 4.32 | 0.84 | 0.04 | 0.461 | 0.323 | NWT |
| c11 | 4.51 | 0.68 | c24 | 4.59 | 0.61 | -0.08 | <0.001 | 0.887 | NWT |
| c12 | 4.68 | 0.55 | c25 | 4.59 | 0.59 | 0.08 | 1.396 | 0.082 | NWT |
| c13 | 3.28 | 1.06 | c26 | 3.37 | 1.13 | -0.08 | <0.001 | 0.767 | NWT |
| c14 | 3.33 | 1.09 | c27 | 3.54 | 1.07 | -0.21 | <0.001 | 0.964 | WT |
| c15 | 4.19 | 1.01 | c28 | 4.14 | 0.96 | 0.05 | 0.528 | 0.299 | NWT |
| c16 | 4.42 | 0.88 | c29 | 4.37 | 0.91 | 0.05 | 0.529 | 0.299 | NWT |
| c17 | 3.09 | 1.22 | c31 | 3.26 | 1.08 | -0.17 | <0.001 | 0.921 | NWT |
| c18 | 3.44 | 1.06 | c32 | 3.42 | 1.08 | 0.03 | 0.239 | 0.406 | NWT |
| c19 | 3.53 | 0.99 | c33 | 3.45 | 1.06 | 0.08 | 0.769 | 0.221 | NWT |
| c20 | 4.26 | 0.96 | c34 | 4.21 | 0.91 | 0.04 | 0.456 | 0.324 | NWT |
| c21 | 4.37 | 0.86 | c35 | 4.34 | 0.94 | 0.03 | 0.348 | 0.364 | NWT |
| c22 | 4.46 | 0.85 | c36 | 4.52 | 0.82 | -0.06 | <0.001 | 0.737 | NWT |

## P800-23: FOA (JBM/FE)

Experiment P800-23 was conducted by FORCE Technology (language: DAN).

The aggregated results per condition are listed in Table 3.23-1 and are shown in Figure 3.23-1. The results aggregated over FX/FL are shown in Figure 3.23-2. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 3.23-2. Significant different MOS values (BT or WT) are highlighted in red.

Table 3.23-1: results per condition for experiment P800-23

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 180 | 4.55 | 0.68 | 0.10 |
| c02 | MNRU Q = 32 dB | 180 | 4.09 | 0.87 | 0.13 |
| c03 | MNRU Q = 27 dB | 180 | 3.22 | 1.12 | 0.17 |
| c04 | MNRU Q = 22 dB | 180 | 2.41 | 1.18 | 0.17 |
| c05 | MNRU Q = 17 dB | 180 | 1.64 | 0.90 | 0.13 |
| c06 | ESDRU α = 0.8 | 180 | 4.55 | 0.69 | 0.10 |
| c07 | ESDRU α = 0.6 | 180 | 4.05 | 0.87 | 0.13 |
| c08 | ESDRU α = 0.4 | 180 | 3.36 | 1.07 | 0.16 |
| c09 | ESDRU α = 0.2 | 180 | 2.97 | 1.18 | 0.17 |
| c10 | IVAS FL enc / FX dec, 24.4 kbps, No error | 180 | 3.84 | 0.85 | 0.12 |
| c11 | IVAS FL enc / FX dec, 48 kbps, No error | 180 | 4.12 | 0.88 | 0.13 |
| c12 | IVAS FL enc / FX dec, 96 kbps, No error | 180 | 4.44 | 0.76 | 0.11 |
| c13 | IVAS FL enc / FX dec, 24.4 kbps, Error I1. O1 | 180 | 2.69 | 1.10 | 0.16 |
| c14 | IVAS FL enc / FX dec, 48 kbps, Error I1. O1 | 180 | 2.97 | 1.10 | 0.16 |
| c15 | IVAS FL enc / FX dec, 24.4 kbps, Error I1. O2 | 180 | 3.47 | 1.08 | 0.16 |
| c16 | IVAS FL enc / FX dec, 48 kbps, Error I1. O2 | 180 | 3.87 | 1.14 | 0.17 |
| c17 | IVAS FL enc / FX dec, 24.4 kbps, I1. O1 | 180 | 2.65 | 1.14 | 0.17 |
| c18 | IVAS FL enc / FX dec, 48 kbps, I1. O1 | 180 | 2.93 | 1.05 | 0.15 |
| c19 | IVAS FL enc / FX dec, 96 kbps, I1. O1 | 180 | 3.30 | 1.05 | 0.15 |
| c20 | IVAS FL enc / FX dec, 24.4 kbps, I1. O2 | 180 | 3.38 | 1.10 | 0.16 |
| c21 | IVAS FL enc / FX dec, 48 kbps, I1. O2 | 180 | 3.86 | 1.12 | 0.17 |
| c22 | IVAS FL enc / FX dec, 96 kbps, I1. O2 | 180 | 3.95 | 1.15 | 0.17 |
| c23 | IVAS FL, 24.4 kbps, No error | 180 | 3.79 | 0.82 | 0.12 |
| c24 | IVAS FL, 48 kbps, No error | 180 | 4.14 | 0.93 | 0.14 |
| c25 | IVAS FL, 96 kbps, No error | 180 | 4.46 | 0.74 | 0.11 |
| c26 | IVAS FL, 24.4 kbps, Error I1. O1 | 180 | 2.62 | 1.10 | 0.16 |
| c27 | IVAS FL, 48 kbps, Error I1. O1 | 180 | 2.99 | 1.15 | 0.17 |
| c28 | IVAS FL, 24.4 kbps, Error I1. O2 | 180 | 3.51 | 1.07 | 0.16 |
| c29 | IVAS FL, 48 kbps, Error I1. O2 | 180 | 3.80 | 1.16 | 0.17 |
| c30 | IVAS FL, 96 kbps, Error I1. O2 | 180 | 4.13 | 1.12 | 0.16 |
| c31 | IVAS FL, 24.4 kbps, I1. O1 | 180 | 2.51 | 1.04 | 0.15 |
| c32 | IVAS FL, 48 kbps, I1. O1 | 180 | 2.91 | 1.09 | 0.16 |
| c33 | IVAS FL, 96 kbps, I1. O1 | 180 | 3.26 | 1.05 | 0.15 |
| c34 | IVAS FL, 24.4 kbps, I1. O2 | 180 | 3.35 | 0.98 | 0.14 |
| c35 | IVAS FL, 48 kbps, I1. O2 | 180 | 3.79 | 1.13 | 0.17 |
| c36 | IVAS FL, 96 kbps, I1. O2 | 180 | 4.06 | 1.08 | 0.16 |

Figure 3.23-1: results per condition for experiment P800-23

Figure 3.23-2: results aggregated over FX/FL for experiment P800-23

Table 3.23-2: statistical significance analysis for experiment P800-23

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | MOS | STD | Condition | MOS | STD | ΔMOS | T-Stat | p-value | Result |
| c10 | 3.84 | 0.85 | c23 | 3.79 | 0.82 | 0.05 | 0.625 | 0.266 | NWT |
| c11 | 4.12 | 0.88 | c24 | 4.14 | 0.93 | -0.02 | <0.001 | 0.571 | NWT |
| c12 | 4.44 | 0.76 | c25 | 4.46 | 0.74 | -0.01 | <0.001 | 0.560 | NWT |
| c13 | 2.69 | 1.10 | c26 | 2.62 | 1.10 | 0.07 | 0.577 | 0.282 | NWT |
| c14 | 2.97 | 1.10 | c27 | 2.99 | 1.15 | -0.02 | <0.001 | 0.573 | NWT |
| c15 | 3.47 | 1.08 | c28 | 3.51 | 1.07 | -0.04 | <0.001 | 0.634 | NWT |
| c16 | 3.87 | 1.14 | c29 | 3.80 | 1.16 | 0.07 | 0.554 | 0.290 | NWT |
| c17 | 2.65 | 1.14 | c31 | 2.51 | 1.04 | 0.14 | 1.255 | 0.105 | NWT |
| c18 | 2.93 | 1.05 | c32 | 2.91 | 1.09 | 0.02 | 0.195 | 0.423 | NWT |
| c19 | 3.30 | 1.05 | c33 | 3.26 | 1.05 | 0.04 | 0.398 | 0.345 | NWT |
| c20 | 3.38 | 1.10 | c34 | 3.35 | 0.98 | 0.03 | 0.255 | 0.399 | NWT |
| c21 | 3.86 | 1.12 | c35 | 3.79 | 1.13 | 0.06 | 0.524 | 0.300 | NWT |
| c22 | 3.95 | 1.15 | c36 | 4.06 | 1.08 | -0.11 | <0.001 | 0.827 | NWT |

# Results for BS.1534 MUSHRA experiments

## Overview

The BS.1534 MUSHRA listening experiments were conducted by ten different listening labs with generic audio source content, as specified in Table 10 in IVAS-8b [1]. For these experiments, trained listeners have been selected.

In the following, for each of the 20 experiments, the aggregated results for each condition are reported, comprising the number of votes (COUNT), mean scores (Score), standard deviations (STD) and the confidence interval of the mean scores (CI95). Moreover, statistical test results are shown which compare the subjective scores of the CuT against the scores for specified reference conditions.

Further results for each experiment are available as an electronic attachment to this document. They are provided in Excel files “BS1534-X.xlsx”, where X is the experiment number. Each file contains the following sheets:

- Raw: The individual votes for each item as provided by the respective listening lab.

- Condition+Sample: Aggregated results for each condition and sample, i.e., raw results aggregated over listeners (14 votes).

- Condition: Aggregated results for each condition, i.e., raw results aggregated over samples and listeners (168 votes).

## BS1534-1: Stereo

Experiment BS1534-1 was conducted by FORCE Technology.

The aggregated results per condition are listed in Table 4.1-1 and are shown in Figure 4.1-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.1-2. Scores with result WT are highlighted in red.

Table 4.1-1: results per condition for experiment BS1534-1

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.67 | 2.17 | 0.33 |
| c02 | LP 3.5 kHz | 168 | 7.69 | 12.34 | 1.88 |
| c03 | Mono EVS, 16.4 kbps | 168 | 33.09 | 19.83 | 3.02 |
| c04 | Mono EVS, 32 kbps | 168 | 41.30 | 20.94 | 3.19 |
| c05 | IVAS, 16.4 kbps | 168 | 50.09 | 22.54 | 3.43 |
| c06 | IVAS, 24.4 kbps | 168 | 65.46 | 20.24 | 3.08 |
| c07 | IVAS, 32 kbps | 168 | 72.36 | 19.59 | 2.98 |
| c08 | IVAS, 48 kbps | 168 | 88.07 | 12.01 | 1.83 |

Figure 4.1-1: results per condition for experiment BS1534-1

Table 4.1-2: statistical significance analysis for experiment BS1534-1

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 50.09 | 22.54 | c03 | 33.09 | 19.83 | 17.00 | 10.085 | <0.001 | BT |
| c07 | 72.36 | 19.59 | c04 | 41.30 | 20.94 | 31.07 | 18.319 | <0.001 | BT |

## BS1534-2: Stereo

Experiment BS1534-2 was conducted by Huawei Technologies Co Ltd..

The aggregated results per condition are listed in Table 4.2-1 and are shown in Figure 4.2-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.2-2. Scores with result WT are highlighted in red.

Table 4.2-1: results per condition for experiment BS1534-2

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 98.60 | 4.40 | 0.67 |
| c02 | LP 3.5 kHz | 168 | 42.26 | 19.17 | 2.92 |
| c03 | Mono EVS, 64 kbps | 168 | 75.81 | 15.30 | 2.33 |
| c04 | Mono EVS, 128 kbps | 168 | 76.19 | 16.72 | 2.55 |
| c05 | IVAS, 64 kbps | 168 | 95.26 | 8.50 | 1.29 |
| c06 | IVAS, 96 kbps | 168 | 97.82 | 4.49 | 0.68 |
| c07 | IVAS, 128 kbps | 168 | 97.19 | 5.67 | 0.86 |
| c08 | IVAS, 256 kbps | 168 | 98.05 | 5.02 | 0.77 |

Figure 4.2-1: results per condition for experiment BS1534-2

Table 4.2-2: statistical significance analysis for experiment BS1534-2

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 95.26 | 8.50 | c03 | 75.81 | 15.30 | 19.44 | 17.289 | <0.001 | BT |
| c07 | 97.19 | 5.67 | c04 | 76.19 | 16.72 | 21.00 | 17.098 | <0.001 | BT |

## BS1534-3: FOA

Experiment BS1534-3 was conducted by Huawei Technologies Co Ltd..

The aggregated results per condition are listed in Table 4.3-1 and are shown in Figure 4.3-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.3-2. Scores with result WT are highlighted in red.

Table 4.3-1: results per condition for experiment BS1534-3

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.29 | 3.72 | 0.57 |
| c02 | LP 3.5 kHz | 168 | 48.10 | 20.95 | 3.19 |
| c03 | Mono EVS, 16.4 kbps | 168 | 68.49 | 17.83 | 2.72 |
| c04 | Mono EVS, 32 kbps | 168 | 70.41 | 17.73 | 2.70 |
| c05 | IVAS, 16.4 kbps | 168 | 76.88 | 19.76 | 3.01 |
| c06 | IVAS, 24.4 kbps | 168 | 82.06 | 16.66 | 2.54 |
| c07 | IVAS, 32 kbps | 168 | 84.23 | 15.03 | 2.29 |
| c08 | IVAS, 48 kbps | 168 | 89.92 | 11.70 | 1.78 |

Figure 4.3-1: results per condition for experiment BS1534-3

Table 4.3-2: statistical significance analysis for experiment BS1534-3

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 76.88 | 19.76 | c03 | 68.49 | 17.83 | 8.39 | 6.273 | <0.001 | BT |
| c07 | 84.23 | 15.03 | c04 | 70.41 | 17.73 | 13.82 | 10.457 | <0.001 | BT |

## BS1534-4: FOA

Experiment BS1534-4 was conducted by Panasonic Holdings Corporation.

The aggregated results per condition are listed in Table 4.4-1 and are shown in Figure 4.4-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.4-2. Scores with result WT are highlighted in red.

Table 4.4-1: results per condition for experiment BS1534-4

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 96.23 | 12.30 | 1.87 |
| c02 | LP 3.5 kHz | 168 | 30.53 | 24.66 | 3.76 |
| c03 | Mono EVS, 64 kbps | 168 | 55.52 | 23.86 | 3.63 |
| c04 | Mono EVS, 128 kbps | 168 | 55.44 | 22.89 | 3.49 |
| c05 | IVAS, 64 kbps | 168 | 79.27 | 17.92 | 2.73 |
| c06 | IVAS, 96 kbps | 168 | 85.74 | 17.37 | 2.65 |
| c07 | IVAS, 128 kbps | 168 | 88.51 | 15.37 | 2.34 |
| c08 | IVAS, 256 kbps | 168 | 88.51 | 16.24 | 2.47 |

Figure 4.4-1: results per condition for experiment BS1534-4

Table 4.4-2: statistical significance analysis for experiment BS1534-4

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 79.27 | 17.92 | c03 | 55.52 | 23.86 | 23.74 | 12.408 | <0.001 | BT |
| c07 | 88.51 | 15.37 | c04 | 55.44 | 22.89 | 33.07 | 17.972 | <0.001 | BT |

## BS1534-5: HOA3

Experiment BS1534-5 was conducted by Dolby Laboratories, Inc..

The aggregated results per condition are listed in Table 4.5-1 and are shown in Figure 4.5-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.5-2. Scores with result WT are highlighted in red.

Table 4.5-1: results per condition for experiment BS1534-5

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.89 | 0.84 | 0.13 |
| c02 | LP 3.5 kHz | 168 | 35.19 | 9.75 | 1.49 |
| c03 | Mono EVS, 64 kbps | 168 | 58.90 | 10.00 | 1.52 |
| c04 | Mono EVS, 128 kbps | 168 | 58.79 | 9.62 | 1.47 |
| c05 | IVAS, 64 kbps | 168 | 68.99 | 9.25 | 1.41 |
| c06 | IVAS, 96 kbps | 168 | 70.70 | 9.01 | 1.37 |
| c07 | IVAS, 128 kbps | 168 | 72.16 | 8.06 | 1.23 |
| c08 | IVAS, 256 kbps | 168 | 76.05 | 7.87 | 1.20 |

Figure 4.5-1: results per condition for experiment BS1534-5

Table 4.5-2: statistical significance analysis for experiment BS1534-5

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 68.99 | 9.25 | c03 | 58.90 | 10.00 | 10.10 | 19.312 | <0.001 | BT |
| c07 | 72.16 | 8.06 | c04 | 58.79 | 9.62 | 13.36 | 24.389 | <0.001 | BT |

## BS1534-6: MC 5.1

Experiment BS1534-6 was conducted by Ericsson LM.

The aggregated results per condition are listed in Table 4.6-1 and are shown in Figure 4.6-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.6-2. Scores with result WT are highlighted in red.

Table 4.6-1: results per condition for experiment BS1534-6

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.56 | 2.54 | 0.39 |
| c02 | LP 3.5 kHz | 168 | 28.34 | 17.20 | 2.62 |
| c03 | Mono EVS, 16.4 kbps | 168 | 46.65 | 19.08 | 2.91 |
| c04 | Mono EVS, 32 kbps | 168 | 50.80 | 17.76 | 2.71 |
| c05 | IVAS, 16.4 kbps | 168 | 52.53 | 23.89 | 3.64 |
| c06 | IVAS, 24.4 kbps | 168 | 65.76 | 18.12 | 2.76 |
| c07 | IVAS, 32 kbps | 168 | 67.57 | 16.85 | 2.57 |
| c08 | IVAS, 48 kbps | 168 | 84.62 | 15.46 | 2.35 |

Figure 4.6-1: results per condition for experiment BS1534-6

Table 4.6-2: statistical significance analysis for experiment BS1534-6

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 52.53 | 23.89 | c03 | 46.65 | 19.08 | 5.88 | 2.988 | 0.003 | BT |
| c07 | 67.57 | 16.85 | c04 | 50.80 | 17.76 | 16.77 | 9.400 | <0.001 | BT |

## BS1534-7: MC 5.1, 7.1

Experiment BS1534-7 was conducted by Dolby Laboratories, Inc..

The aggregated results per condition are listed in Table 4.7-1 and are shown in Figure 4.7-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.7-2. Scores with result WT are highlighted in red.

Table 4.7-1: results per condition for experiment BS1534-7

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.62 | 1.60 | 0.24 |
| c02 | LP 3.5 kHz | 168 | 36.35 | 9.88 | 1.50 |
| c03 | Mono EVS, 64 kbps | 168 | 59.30 | 11.26 | 1.72 |
| c04 | Mono EVS, 128 kbps | 168 | 59.41 | 11.19 | 1.70 |
| c05 | IVAS, 64 kbps | 168 | 71.76 | 9.49 | 1.45 |
| c06 | IVAS, 96 kbps | 168 | 76.34 | 8.41 | 1.28 |
| c07 | IVAS, 128 kbps | 168 | 77.06 | 8.25 | 1.26 |
| c08 | IVAS, 256 kbps | 168 | 84.17 | 8.80 | 1.34 |

Figure 4.7-1: results per condition for experiment BS1534-7

Table 4.7-2: statistical significance analysis for experiment BS1534-7

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 71.76 | 9.49 | c03 | 59.30 | 11.26 | 12.46 | 17.378 | <0.001 | BT |
| c07 | 77.06 | 8.25 | c04 | 59.41 | 11.19 | 17.66 | 22.739 | <0.001 | BT |

## BS1534-8: MC 5.1+2, 5.1+4

Experiment BS1534-8 was conducted by Nokia.

The aggregated results per condition are listed in Table 4.8-1 and are shown in Figure 4.8-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.8-2. Scores with result WT are highlighted in red.

Table 4.8-1: results per condition for experiment BS1534-8

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 96.57 | 8.35 | 1.27 |
| c02 | LP 3.5 kHz | 168 | 10.40 | 10.74 | 1.64 |
| c03 | Mono EVS, 64 kbps | 168 | 35.50 | 16.55 | 2.52 |
| c04 | Mono EVS, 128 kbps | 168 | 36.59 | 16.57 | 2.52 |
| c05 | IVAS, 64 kbps | 168 | 55.45 | 19.78 | 3.01 |
| c06 | IVAS, 96 kbps | 168 | 75.67 | 15.20 | 2.31 |
| c07 | IVAS, 128 kbps | 168 | 78.81 | 16.41 | 2.50 |
| c08 | IVAS, 256 kbps | 168 | 88.88 | 12.15 | 1.85 |

Figure 4.8-1: results per condition for experiment BS1534-8

Table 4.8-2: statistical significance analysis for experiment BS1534-8

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 55.45 | 19.78 | c03 | 35.50 | 16.55 | 19.95 | 12.595 | <0.001 | BT |
| c07 | 78.81 | 16.41 | c04 | 36.59 | 16.57 | 42.23 | 28.310 | <0.001 | BT |

## BS1534-9: MC 7.1+4

Experiment BS1534-9 was conducted by Fraunhofer IIS.

The aggregated results per condition are listed in Table 4.9-1 and are shown in Figure 4.9-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.9-2. Scores with result WT are highlighted in red.

Table 4.9-1: results per condition for experiment BS1534-9

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.95 | 0.49 | 0.07 |
| c02 | LP 3.5 kHz | 168 | 22.51 | 6.89 | 1.05 |
| c03 | Mono EVS, 64 kbps | 168 | 33.32 | 10.93 | 1.67 |
| c04 | Mono EVS, 128 kbps | 168 | 33.59 | 11.18 | 1.70 |
| c05 | IVAS, 64 kbps | 168 | 60.83 | 17.66 | 2.69 |
| c06 | IVAS, 96 kbps | 168 | 64.09 | 16.71 | 2.54 |
| c07 | IVAS, 128 kbps | 168 | 75.30 | 15.21 | 2.32 |
| c08 | IVAS, 256 kbps | 168 | 87.23 | 12.10 | 1.84 |

Figure 4.9-1: results per condition for experiment BS1534-9

Table 4.9-2: statistical significance analysis for experiment BS1534-9

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 60.83 | 17.66 | c03 | 33.32 | 10.93 | 27.51 | 19.608 | <0.001 | BT |
| c07 | 75.30 | 15.21 | c04 | 33.59 | 11.18 | 41.71 | 30.271 | <0.001 | BT |

## BS1534-10: ISM 1-2 Objects

Experiment BS1534-10 was conducted by Qualcomm Incorporated.

The aggregated results per condition are listed in Table 4.10-1 and are shown in Figure 4.10-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.10-2. Scores with result WT are highlighted in red.

Table 4.10-1: results per condition for experiment BS1534-10

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 97.54 | 5.29 | 0.81 |
| c02 | LP 3.5 kHz | 168 | 28.43 | 12.56 | 1.91 |
| c03 | Mono EVS, 48 kbps | 168 | 58.14 | 14.03 | 2.14 |
| c04 | Mono EVS, 96 kbps | 168 | 58.32 | 13.85 | 2.11 |
| c05 | IVAS, 48 kbps | 168 | 90.23 | 11.86 | 1.81 |
| c06 | IVAS, 64 kbps | 168 | 92.47 | 10.65 | 1.62 |
| c07 | IVAS, 96 kbps | 168 | 95.03 | 8.40 | 1.28 |
| c08 | IVAS, 128 kbps | 168 | 97.59 | 4.97 | 0.76 |

Figure 4.10-1: results per condition for experiment BS1534-10

Table 4.10-2: statistical significance analysis for experiment BS1534-10

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 90.23 | 11.86 | c03 | 58.14 | 14.03 | 32.09 | 23.951 | <0.001 | BT |
| c07 | 95.03 | 8.40 | c04 | 58.32 | 13.85 | 36.71 | 29.071 | <0.001 | BT |

## BS1534-11: ISM 3-4 Objects

Experiment BS1534-11 was conducted by Dolby Laboratories, Inc..

The aggregated results per condition are listed in Table 4.11-1 and are shown in Figure 4.11-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.11-2. Scores with result WT are highlighted in red.

Table 4.11-1: results per condition for experiment BS1534-11

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 100.00 | 0.00 | 0.00 |
| c02 | LP 3.5 kHz | 168 | 33.83 | 6.77 | 1.03 |
| c03 | Mono EVS, 24.4 kbps | 168 | 61.81 | 10.79 | 1.64 |
| c04 | Mono EVS, 48 kbps | 168 | 62.39 | 10.14 | 1.54 |
| c05 | IVAS, 24.4 kbps | 168 | 60.78 | 12.26 | 1.87 |
| c06 | IVAS, 32 kbps | 168 | 64.11 | 12.07 | 1.84 |
| c07 | IVAS, 48 kbps | 168 | 78.29 | 7.25 | 1.10 |
| c08 | IVAS, 64 kbps | 168 | 79.31 | 7.58 | 1.15 |

Figure 4.11-1: results per condition for experiment BS1534-11

Table 4.11-2: statistical significance analysis for experiment BS1534-11

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 60.78 | 12.26 | c03 | 61.81 | 10.79 | -1.04 | <0.001 | 0.172 | NWT |
| c07 | 78.29 | 7.25 | c04 | 62.39 | 10.14 | 15.90 | 20.547 | <0.001 | BT |

## BS1534-12: ISM 3-4 Objects

Experiment BS1534-12 was conducted by Philips International B.V..

The aggregated results per condition are listed in Table 4.12-1 and are shown in Figure 4.12-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.12-2. Scores with result WT are highlighted in red.

Table 4.12-1: results per condition for experiment BS1534-12

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 97.99 | 4.83 | 0.74 |
| c02 | LP 3.5 kHz | 168 | 17.28 | 17.11 | 2.61 |
| c03 | Mono EVS, 64 kbps | 168 | 41.79 | 23.57 | 3.59 |
| c04 | Mono EVS, 128 kbps | 168 | 42.76 | 24.35 | 3.71 |
| c05 | IVAS, 64 kbps | 168 | 90.99 | 13.98 | 2.13 |
| c06 | IVAS, 96 kbps | 168 | 91.35 | 13.40 | 2.04 |
| c07 | IVAS, 128 kbps | 168 | 92.04 | 13.04 | 1.99 |
| c08 | IVAS, 256 kbps | 168 | 94.05 | 11.16 | 1.70 |

Figure 4.12-1: results per condition for experiment BS1534-12

Table 4.12-2: statistical significance analysis for experiment BS1534-12

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 90.99 | 13.98 | c03 | 41.79 | 23.57 | 49.21 | 26.034 | <0.001 | BT |
| c07 | 92.04 | 13.04 | c04 | 42.76 | 24.35 | 49.28 | 23.804 | <0.001 | BT |

## BS1534-13: MASA 1 TC

Experiment BS1534-13 was conducted by Qualcomm Incorporated.

The aggregated results per condition are listed in Table 4.13-1 and are shown in Figure 4.13-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.13-2. Scores with result WT are highlighted in red.

Table 4.13-1: results per condition for experiment BS1534-13

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 98.84 | 4.19 | 0.64 |
| c02 | LP 3.5 kHz | 168 | 31.63 | 11.22 | 1.71 |
| c03 | Mono EVS, 16.4 kbps | 168 | 56.34 | 14.76 | 2.25 |
| c04 | Mono EVS, 32 kbps | 168 | 58.44 | 14.43 | 2.20 |
| c05 | IVAS, 16.4 kbps | 168 | 72.13 | 18.82 | 2.87 |
| c06 | IVAS, 24.4 kbps | 168 | 80.51 | 15.57 | 2.37 |
| c07 | IVAS, 32 kbps | 168 | 83.29 | 14.49 | 2.21 |
| c08 | IVAS, 48 kbps | 168 | 87.05 | 16.39 | 2.50 |

Figure 4.13-1: results per condition for experiment BS1534-13

Table 4.13-2: statistical significance analysis for experiment BS1534-13

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 72.13 | 18.82 | c03 | 56.34 | 14.76 | 15.79 | 10.297 | <0.001 | BT |
| c07 | 83.29 | 14.49 | c04 | 58.44 | 14.43 | 24.85 | 17.690 | <0.001 | BT |

## BS1534-14: MASA 1 TC

Experiment BS1534-14 was conducted by Dolby Laboratories, Inc..

The aggregated results per condition are listed in Table 4.14-1 and are shown in Figure 4.14-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.14-2. Scores with result WT are highlighted in red.

Table 4.14-1: results per condition for experiment BS1534-14

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.79 | 1.25 | 0.19 |
| c02 | LP 3.5 kHz | 168 | 32.71 | 6.44 | 0.98 |
| c03 | Mono EVS, 64 kbps | 168 | 65.31 | 8.78 | 1.34 |
| c04 | Mono EVS, 128 kbps | 168 | 65.82 | 8.69 | 1.32 |
| c05 | IVAS, 64 kbps | 168 | 75.21 | 8.68 | 1.32 |
| c06 | IVAS, 96 kbps | 168 | 77.26 | 8.80 | 1.34 |
| c07 | IVAS, 128 kbps | 168 | 78.49 | 8.82 | 1.34 |
| c08 | IVAS, 256 kbps | 168 | 79.63 | 8.96 | 1.36 |

Figure 4.14-1: results per condition for experiment BS1534-14

Table 4.14-2: statistical significance analysis for experiment BS1534-14

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 75.21 | 8.68 | c03 | 65.31 | 8.78 | 9.90 | 15.721 | <0.001 | BT |
| c07 | 78.49 | 8.82 | c04 | 65.82 | 8.69 | 12.67 | 20.767 | <0.001 | BT |

## BS1534-15: MASA 2 TC

Experiment BS1534-15 was conducted by Nokia.

The aggregated results per condition are listed in Table 4.15-1 and are shown in Figure 4.15-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.15-2. Scores with result WT are highlighted in red.

Table 4.15-1: results per condition for experiment BS1534-15

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 97.34 | 5.71 | 0.87 |
| c02 | LP 3.5 kHz | 168 | 12.12 | 10.53 | 1.60 |
| c03 | Mono EVS, 64 kbps | 168 | 35.27 | 13.52 | 2.06 |
| c04 | Mono EVS, 128 kbps | 168 | 37.52 | 15.59 | 2.37 |
| c05 | IVAS, 64 kbps | 168 | 66.74 | 17.32 | 2.64 |
| c06 | IVAS, 96 kbps | 168 | 74.52 | 14.56 | 2.22 |
| c07 | IVAS, 128 kbps | 168 | 83.95 | 11.72 | 1.79 |
| c08 | IVAS, 256 kbps | 168 | 89.06 | 10.46 | 1.59 |

Figure 4.15-1: results per condition for experiment BS1534-15

Table 4.15-2: statistical significance analysis for experiment BS1534-15

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 66.74 | 17.32 | c03 | 35.27 | 13.52 | 31.47 | 25.802 | <0.001 | BT |
| c07 | 83.95 | 11.72 | c04 | 37.52 | 15.59 | 46.43 | 33.814 | <0.001 | BT |

## BS1534-16: OSBA 1-4 Objects

Experiment BS1534-16 was conducted by Fraunhofer IIS.

The aggregated results per condition are listed in Table 4.16-1 and are shown in Figure 4.16-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.16-2. Scores with result WT are highlighted in red.

Table 4.16-1: results per condition for experiment BS1534-16

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 100.00 | 0.00 | 0.00 |
| c02 | LP 3.5 kHz | 168 | 23.49 | 7.91 | 1.20 |
| c03 | Mono EVS, 16.4 kbps | 168 | 39.31 | 14.83 | 2.26 |
| c04 | Mono EVS, 32 kbps | 168 | 43.12 | 14.71 | 2.24 |
| c05 | IVAS, 16.4 kbps | 168 | 43.53 | 15.76 | 2.40 |
| c06 | IVAS, 24.4 kbps | 168 | 53.04 | 17.02 | 2.59 |
| c07 | IVAS, 32 kbps | 168 | 58.44 | 15.89 | 2.42 |
| c08 | IVAS, 48 kbps | 168 | 63.59 | 14.47 | 2.20 |

Figure 4.16-1: results per condition for experiment BS1534-16

Table 4.16-2: statistical significance analysis for experiment BS1534-16

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 43.53 | 15.76 | c03 | 39.31 | 14.83 | 4.21 | 4.075 | <0.001 | BT |
| c07 | 58.44 | 15.89 | c04 | 43.12 | 14.71 | 15.32 | 13.407 | <0.001 | BT |

## BS1534-17: OSBA 1-4 Objects

Experiment BS1534-17 was conducted by Dolby Laboratories, Inc..

The aggregated results per condition are listed in Table 4.17-1 and are shown in Figure 4.17-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.17-2. Scores with result WT are highlighted in red.

Table 4.17-1: results per condition for experiment BS1534-17

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.80 | 1.18 | 0.18 |
| c02 | LP 3.5 kHz | 168 | 34.17 | 8.16 | 1.24 |
| c03 | Mono EVS, 64 kbps | 168 | 60.43 | 9.41 | 1.43 |
| c04 | Mono EVS, 128 kbps | 168 | 60.95 | 9.28 | 1.41 |
| c05 | IVAS, 64 kbps | 168 | 68.75 | 9.14 | 1.39 |
| c06 | IVAS, 96 kbps | 168 | 72.02 | 9.20 | 1.40 |
| c07 | IVAS, 128 kbps | 168 | 75.72 | 9.14 | 1.39 |
| c08 | IVAS, 256 kbps | 168 | 80.41 | 10.22 | 1.56 |

Figure 4.17-1: results per condition for experiment BS1534-17

Table 4.17-2: statistical significance analysis for experiment BS1534-17

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 68.75 | 9.14 | c03 | 60.43 | 9.41 | 8.32 | 14.550 | <0.001 | BT |
| c07 | 75.72 | 9.14 | c04 | 60.95 | 9.28 | 14.77 | 22.462 | <0.001 | BT |

## BS1534-18: OMASA 1-4 Objects

Experiment BS1534-18 was conducted by Nokia.

The aggregated results per condition are listed in Table 4.18-1 and are shown in Figure 4.18-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.18-2. Scores with result WT are highlighted in red.

Table 4.18-1: results per condition for experiment BS1534-18

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 96.42 | 8.97 | 1.37 |
| c02 | LP 3.5 kHz | 168 | 14.42 | 11.44 | 1.74 |
| c03 | Mono EVS, 64 kbps | 168 | 45.88 | 19.69 | 3.00 |
| c04 | Mono EVS, 128 kbps | 168 | 47.82 | 21.70 | 3.31 |
| c05 | IVAS, 64 kbps | 168 | 62.60 | 22.20 | 3.38 |
| c06 | IVAS, 96 kbps | 168 | 75.94 | 16.48 | 2.51 |
| c07 | IVAS, 128 kbps | 168 | 80.87 | 16.74 | 2.55 |
| c08 | IVAS, 256 kbps | 168 | 89.73 | 12.01 | 1.83 |

Figure 4.18-1: results per condition for experiment BS1534-18

Table 4.18-2: statistical significance analysis for experiment BS1534-18

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 62.60 | 22.20 | c03 | 45.88 | 19.69 | 16.72 | 10.353 | <0.001 | BT |
| c07 | 80.87 | 16.74 | c04 | 47.82 | 21.70 | 33.05 | 16.421 | <0.001 | BT |

## BS1534-19: Stereo downmix for EVS

Experiment BS1534-19 was conducted by Orange.

The aggregated results per condition are listed in Table 4.19-1 and are shown in Figure 4.19-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.19-2. Scores with result WT are highlighted in red.

Table 4.19-1: results per condition for experiment BS1534-19

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.94 | 0.85 | 0.13 |
| c02 | LP 3.5 kHz | 168 | 19.21 | 16.62 | 2.53 |
| c03 | Dual EVS + static dmx, 14.4 kbps | 168 | 25.22 | 18.74 | 2.85 |
| c04 | Dual EVS + static dmx, 26.4 kbps | 168 | 51.17 | 20.62 | 3.14 |
| c05 | Static dmx +EVS, 13.2 kbps | 168 | 45.77 | 22.23 | 3.38 |
| c06 | Static dmx +EVS, 24.4 kbps | 168 | 57.13 | 20.44 | 3.11 |
| c07 | CuT (dynamic dmx +EVS), 13.2 kbps | 168 | 46.39 | 22.84 | 3.48 |
| c08 | CuT (dynamic dmx +EVS), 24.4 kbps | 168 | 59.24 | 20.35 | 3.10 |

Figure 4.19-1: results per condition for experiment BS1534-19

Table 4.19-2: statistical significance analysis for experiment BS1534-19

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c07 | 46.39 | 22.84 | c03 | 25.22 | 18.74 | 21.17 | 16.108 | <0.001 | BT |
| c07 | 46.39 | 22.84 | c05 | 45.77 | 22.23 | 0.62 | 0.509 | 0.611 | NWT |
| c08 | 59.24 | 20.35 | c04 | 51.17 | 20.62 | 8.08 | 7.448 | <0.001 | BT |
| c08 | 59.24 | 20.35 | c06 | 57.13 | 20.44 | 2.11 | 2.668 | 0.008 | BT |

## BS1534-20: ISM 6-DoF and directivity

Experiment BS1534-20 was conducted by Ericsson LM.

The aggregated results per condition are listed in Table 4.20-1 and are shown in Figure 4.20-1. The plot shows means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The statistical significance analysis is shown in Table 4.20-2. Scores with result WT are highlighted in red.

Table 4.20-1: results per condition for experiment BS1534-20

| Designator | Condition | COUNT | Score | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | 168 | 99.64 | 1.46 | 0.22 |
| c02 | LP 3.5 kHz | 168 | 25.48 | 13.37 | 2.04 |
| c03 | IVAS, no ext. metadata, 64 kbps | 168 | 54.60 | 20.27 | 3.09 |
| c04 | IVAS, no ext. metadata, 512 kbps | 168 | 56.83 | 18.42 | 2.81 |
| c05 | IVAS, extended metadata, 64 kbps | 168 | 91.61 | 11.19 | 1.71 |
| c06 | IVAS, extended metadata, 512 kbps | 168 | 99.13 | 3.51 | 0.53 |

Figure 4.20-1: results per condition for experiment BS1534-20

Table 4.20-2: statistical significance analysis for experiment BS1534-20

| CuT |  |  | Reference |  |  | Evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Score | STD | Condition | Score | STD | ΔScore | T-Stat | p-value | Result |
| c05 | 91.61 | 11.19 | c03 | 54.60 | 20.27 | 37.01 | 23.490 | <0.001 | BT |
| c06 | 99.13 | 3.51 | c04 | 56.83 | 18.42 | 42.30 | 28.701 | <0.001 | BT |

# Results for P.800 ACR experiment

## Overview

A single P.800 ACR experiment was conducted to test the CuT at different bandwidths (WB, SWB, FB), different input formats and different bitrates.

In the following, the aggregated results for each condition are reported, comprising the number of votes (COUNT), mean opinion scores (MOS), standard deviations (STD) and the confidence interval of the MOS (CI95).

Further results for each experiment are available as an electronic attachment to this document. They are provided in the Excel file “ACR-1.xlsx”. The file contains the following sheets:

- Raw: The individual votes for each item as provided by the respective listening lab.

- Condition+Category+Sample: Aggregated results for each condition, category, and sample, i.e., raw results aggregated over the five votes per item (5 votes).

- Condition+Category: Aggregated results for each condition and category, i.e., raw results aggregated over votes and samples (30 votes).

- Condition: Aggregated results for each condition, i.e., raw results aggregated over categories, samples, and votes (180 votes).

## ACR-1: Stereo, FOA, HOA3, MASA, ISM2 (16, 32, 48 kHz)

Experiment ACR-1 was conducted by Nokia (language: FIN).

The listening lab has reported that the presentation order of the samples to the listeners was not according to the original randomization tables due to a configuration issue in the listening test software. See listening lab report in S4-251691.

The aggregated results for all condition are listed in Table 5.1-1.

The experiment contains multiple references at different bandwidths from narrowband to fullband. The scores of the references are shown in Figure 5.1-1. The results for the test conditions are shown in the different subfigures of Figure 5.1-2 separated by bandwidth (WB, SWB, and FB). In addition, the spatial reference for the respective bandwidth is shown as a horizontal, dashed line. All plots show means and 95% confidence intervals calculated according to ITU-T Recommendation P.1401 [5].

The additional plot in Figure 5.1-3 presents the data for FER 0% and DTX off in a different way by showing the results for all bandwidths WB, SWB, and FB in a single plot.

Table 5.1-1: results per condition for experiment ACR-1

| Designator | Condition | COUNT | MOS | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | Spatial FB | 180 | 4.37 | 0.70 | 0.10 |
| c02 | Spatial SWB (16 kHz) | 180 | 4.31 | 0.76 | 0.11 |
| c03 | Spatial (14 kHz) | 180 | 4.33 | 0.71 | 0.10 |
| c04 | Spatial SSWB (12 kHz) | 180 | 4.31 | 0.84 | 0.12 |
| c05 | Spatial (10 kHz) | 180 | 4.14 | 0.83 | 0.12 |
| c06 | Spatial WB (8 kHz) | 180 | 3.89 | 0.85 | 0.12 |
| c07 | Spatial MB (6 kHz) | 180 | 3.32 | 0.96 | 0.14 |
| c08 | Spatial NB (4 kHz) | 180 | 2.66 | 1.06 | 0.16 |
| c09 | MNRU Q = 30 dB | 180 | 2.92 | 1.28 | 0.19 |
| c10 | MNRU Q = 26 dB | 180 | 2.02 | 1.17 | 0.17 |
| c11 | MNRU Q = 22 dB | 180 | 1.59 | 0.98 | 0.14 |
| c12 | MNRU Q = 18 dB | 180 | 1.24 | 0.68 | 0.10 |
| c13 | ESDRU α = 0.8 | 180 | 4.26 | 0.78 | 0.11 |
| c14 | ESDRU α = 0.6 | 180 | 3.47 | 0.99 | 0.15 |
| c15 | ESDRU α = 0.4 | 180 | 2.92 | 0.98 | 0.14 |
| c16 | ESDRU α = 0.2 | 180 | 2.53 | 1.05 | 0.15 |
| c17 | IVAS-WB, DTX off, FER 0%, 13.2 kbps | 180 | 2.81 | 0.97 | 0.14 |
| c18 | IVAS-WB, DTX off, FER 0%, 16.4 kbps | 180 | 3.16 | 0.91 | 0.13 |
| c19 | IVAS-WB, DTX off, FER 0%, 24.4 kbps | 180 | 3.33 | 0.94 | 0.14 |
| c20 | IVAS-WB, DTX off, FER 0%, 32 kbps | 180 | 3.36 | 0.92 | 0.14 |
| c21 | IVAS-WB, DTX off, FER 0%, 48 kbps | 180 | 3.57 | 0.87 | 0.13 |
| c22 | IVAS-WB, DTX off, FER 0%, 64 kbps | 180 | 3.70 | 0.84 | 0.12 |
| c23 | IVAS-WB, DTX off, FER 0%, 80 kbps | 180 | 3.77 | 0.83 | 0.12 |
| c24 | IVAS-WB, DTX off, FER 0%, 96 kbps | 180 | 3.79 | 0.83 | 0.12 |
| c25 | IVAS-SWB, DTX off, FER 0%, 13.2 kbps | 180 | 2.66 | 1.03 | 0.15 |
| c26 | IVAS-SWB, DTX off, FER 0%, 16.4 kbps | 180 | 3.10 | 0.95 | 0.14 |
| c27 | IVAS-SWB, DTX off, FER 0%, 24.4 kbps | 180 | 3.58 | 0.95 | 0.14 |
| c28 | IVAS-SWB, DTX off, FER 0%, 32 kbps | 180 | 3.63 | 0.87 | 0.13 |
| c29 | IVAS-SWB, DTX off, FER 0%, 48 kbps | 180 | 3.81 | 0.94 | 0.14 |
| c30 | IVAS-SWB, DTX off, FER 0%, 64 kbps | 180 | 3.98 | 0.81 | 0.12 |
| c31 | IVAS-SWB, DTX off, FER 0%, 80 kbps | 180 | 4.03 | 0.82 | 0.12 |
| c32 | IVAS-SWB, DTX off, FER 0%, 96 kbps | 180 | 4.16 | 0.82 | 0.12 |
| c33 | IVAS-FB, DTX off, FER 0%, 24.4 kbps | 180 | 3.56 | 0.95 | 0.14 |
| c34 | IVAS-FB, DTX off, FER 0%, 32 kbps | 180 | 3.68 | 0.96 | 0.14 |
| c35 | IVAS-FB, DTX off, FER 0%, 48 kbps | 180 | 3.72 | 0.88 | 0.13 |
| c36 | IVAS-FB, DTX off, FER 0%, 64 kbps | 180 | 3.97 | 0.81 | 0.12 |
| c37 | IVAS-FB, DTX off, FER 0%, 80 kbps | 180 | 4.11 | 0.85 | 0.12 |
| c38 | IVAS-FB, DTX off, FER 0%, 96 kbps | 180 | 4.13 | 0.79 | 0.12 |
| c39 | IVAS-FB, DTX off, FER 0%, 128 kbps | 180 | 4.26 | 0.83 | 0.12 |
| c40 | IVAS-FB, DTX off, FER 0%, 160 kbps | 180 | 4.28 | 0.75 | 0.11 |
| c41 | IVAS-FB, DTX off, FER 0%, 192 kbps | 180 | 4.14 | 0.81 | 0.12 |
| c42 | IVAS-FB, DTX off, FER 0%, 256 kbps | 180 | 4.28 | 0.76 | 0.11 |
| c43 | IVAS-WB, DTX on, FER 0%, 13.2 kbps | 180 | 2.81 | 1.01 | 0.15 |
| c44 | IVAS-WB, DTX on, FER 0%, 16.4 kbps | 180 | 3.09 | 0.91 | 0.13 |
| c45 | IVAS-SWB, DTX on, FER 0%, 16.4 kbps | 180 | 3.10 | 0.84 | 0.12 |
| c46 | IVAS-SWB, DTX on, FER 0%, 24.4 kbps | 180 | 3.42 | 0.94 | 0.14 |
| c47 | IVAS-FB, DTX on, FER 0%, 24.4 kbps | 180 | 3.42 | 0.85 | 0.12 |
| c48 | IVAS-FB, DTX on, FER 0%, 32 kbps | 180 | 3.69 | 0.91 | 0.13 |
| c49 | IVAS-WB, DTX off, FER 5%, 24.4 kbps | 180 | 2.63 | 0.86 | 0.13 |
| c50 | IVAS-WB, DTX off, FER 5%, 32 kbps | 180 | 2.89 | 0.96 | 0.14 |
| c51 | IVAS-SWB, DTX off, FER 5%, 32 kbps | 180 | 2.91 | 0.94 | 0.14 |
| c52 | IVAS-SWB, DTX off, FER 5%, 48 kbps | 180 | 2.85 | 0.94 | 0.14 |
| c53 | IVAS-FB, DTX off, FER 5%, 64 kbps | 180 | 3.09 | 0.89 | 0.13 |
| c54 | IVAS-FB, DTX off, FER 5%, 80 kbps | 180 | 3.14 | 0.93 | 0.14 |
| c55 | IVAS-WB, DTX on, FER 5%, 16.4 kbps | 180 | 2.53 | 0.83 | 0.12 |
| c56 | IVAS-WB, DTX on, FER 5%, 24.4 kbps | 180 | 2.63 | 0.91 | 0.13 |
| c57 | IVAS-SWB, DTX on, FER 5%, 24.4 kbps | 180 | 2.58 | 0.84 | 0.12 |
| c58 | IVAS-SWB, DTX on, FER 5%, 32 kbps | 180 | 2.86 | 0.79 | 0.12 |
| c59 | IVAS-FB, DTX on, FER 5%, 48 kbps | 180 | 2.87 | 0.90 | 0.13 |
| c60 | IVAS-FB, DTX on, FER 5%, 64 kbps | 180 | 3.01 | 0.95 | 0.14 |

Figure 5.1-1: spatial reference conditions for experiment ACR-1

(a)

(b)

(c)

Figure 5.1-2: results per condition for experiment ACR-1

Figure 5.1-3: test conditions for all bands under test, FER 0%, DTX off for experiment ACR-1

# Results for ROOM experiments

## Overview

Four room acoustics testing experiments (ROOM) were conducted as specified in IVAS-8b [1].

For experiments, the same naming convention as for P800, BS1534 and ACR tests are used in the following subclauses (i.e., "condition", "category" and "sample") to perform statistical analysis. The specific labels per attribute are provided in Table 6.0-1, Table 6.0-2, and Table 6.0-3.

Table 6.0-1: Conditions (renderer)

| Condition | Renderer |
| --- | --- |
| c01 | Reference (anechoic) |
| c02 | Reference (Python) |
| c03 | IVAS – BINAURAL_ROOM_IR |
| c04 | IVAS – BINAURAL_ROOM_REVERB |

Table 6.0-2: Categories for ROOM experiments (room environments)

| Category | Room |
| --- | --- |
| a1 | Auditorium |
| a2 | Kitchen corner |
| a3 | Lounge |
| a4 | Listening lab |

Table 6.0-3: Sample mapping for ROOM experiments Conditions (stimuli)

| Sample | Signal type | Number of sources |
| --- | --- | --- |
| s01 | Speech | 1 |
| s02 |  | 2 |
| s03 | Music | 1 |
| s04 |  | 2 |

As specified in IVAS-8b, the results are analyzed and illustrated based on two main statistical evaluations:

ANOVA: Evaluates which of the independent variables (condition, category, sample) and interaction terms have an actual impact on the results.

Tukey's HSD: Analyses statistical significance of differences between conditions (renderers) across all categories and samples (rooms/stimuli).

The reader is also referred to the listening lab's report [5].

Further results for each experiment are available as an electronic attachment to this document. They are provided in Excel files “ROOM-X.xlsx”, where X is the experiment number. Each file contains the following sheets:

- Raw: The individual votes for each item as provided by the listening lab.

- Condition+Category+Sample: Aggregated results for each condition, category, and sample, i.e., raw results aggregated over listeners (12 votes).

- Condition+Sample: Aggregated results for each condition and sample, i.e., raw results aggregated over categories and listeners (48 votes).

- Condition: Aggregated results for each condition, i.e., raw results aggregated over samples, categories, and listeners (192 votes).

## ROOM-1: Parametric binaural renderer (FOA, 80 kbps)

Experiment ROOM-1 was conducted by Philips International B.V..

The aggregated results per condition are listed in Table 6.1-1 and shown in Figure 6.1-1. The results separated by category and separated by sample are shown and Figure 6.1-2. All boxplots illustrate the distribution of votes by means of average ("x" in black), median (horizontal lines in red), inter-quartile-range (IQR, lower/upper box limit) and outliers (indicated by whiskers at ±1.5x IQR).

The ANOVA and Tukey's HSD analyses are provided in Table 6.1-2 and Table 6.1-3, respectively.

Table 6.1-1: results per condition for experiment ROOM-1

| Designator | Condition | COUNT | SCORE | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | IVAS Binaural (default) | 192 | 27.00 | 21.81 | 3.10 |
| c02 | Binaural Reference Renderer (RoomIR) | 192 | 81.02 | 17.47 | 2.49 |
| c03 | IVAS Binaural (RoomIR) | 192 | 77.81 | 18.22 | 2.59 |
| c04 | IVAS Binaural (RoomReverb) | 192 | 85.84 | 16.32 | 2.32 |

Figure 6.1-1: results per condition for experiment ROOM-1

Figure 6.1-2: results per condition versus category (left) and sample (right) for experiment ROOM-1

Table 6.1-2: ANOVA results for experiment ROOM-1

| Category Variables | Sum of Squares | Degrees of Freedom | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| C(Sample) | 941.0 | 3 | 1.0100 | 0.3869 |
| C(Condition) | 434974.2 | 3 | 467.6100 | <0.0001 |
| C(Category) | 5045.3 | 3 | 5.4200 | 0.0011 |
| C(Category):C(Condition) | 19928.6 | 9 | 7.1400 | <0.0001 |
| C(Sample):C(Condition) | 8038.8 | 9 | 2.8800 | 0.0024 |
| Residual | 229452.8 | 740 |  |  |

Table 6.1-3: Tukey's HSD results for experiment ROOM-1

| Group 1 | Group 2 | Mean Difference | P-Value | Lower CI | Upper CI | Significant |
| --- | --- | --- | --- | --- | --- | --- |
| c01 | c02 | 54.02 | <0.0001 | 49.14 | 58.90 | True |
| c01 | c03 | 50.82 | <0.0001 | 45.94 | 55.70 | True |
| c01 | c04 | 58.85 | <0.0001 | 53.97 | 63.73 | True |
| c02 | c03 | -3.20 | 0.3300 | -8.08 | 1.68 | False |
| c02 | c04 | 4.83 | 0.0540 | -0.05 | 9.71 | False |
| c03 | c04 | 8.03 | <0.0001 | 3.15 | 12.91 | True |

## ROOM-2: FastConv Binaural Renderer (FOA, 96 kbps)

Experiment ROOM-2 was conducted by Philips International B.V..

The aggregated results per condition are listed in Table 6.2-1 and shown in Figure 6.2-1. The results separated by category and separated by sample are shown and Figure 6.2-2. All boxplots illustrate the distribution of votes by means of average ("x" in black), median (horizontal lines in red), inter-quartile-range (IQR, lower/upper box limit) and outliers (indicated by whiskers at ±1.5x IQR).

The ANOVA and Tukey's HSD analyses are provided in Table 6.2-2 and Table 6.2-3, respectively.

Table 6.2-1: results per condition for experiment ROOM-2

| Designator | Condition | COUNT | SCORE | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | IVAS Binaural (default) | 192 | 25.86 | 22.39 | 3.19 |
| c02 | Binaural Reference Renderer (RoomIR) | 192 | 81.28 | 19.52 | 2.78 |
| c03 | IVAS Binaural (RoomIR) | 192 | 80.08 | 17.45 | 2.48 |
| c04 | IVAS Binaural (RoomReverb) | 192 | 83.92 | 21.96 | 3.13 |

Figure 6.2-1: results per condition for experiment ROOM-2

Figure 6.2-2: results per condition versus category (left) and sample (right) for experiment ROOM-2

Table 6.2-2: ANOVA results for experiment ROOM-2

| Category Variables | Sum of Squares | Degrees of Freedom | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| C(Sample) | 594.2 | 3 | 0.5000 | 0.6824 |
| C(Condition) | 451330.8 | 3 | 379.7900 | <0.0001 |
| C(Category) | 3564.9 | 3 | 3.0000 | 0.0300 |
| C(Category):C(Condition) | 16800.1 | 9 | 4.7100 | <0.0001 |
| C(Sample):C(Condition) | 4671.8 | 9 | 1.3100 | 0.2274 |
| Residual | 293130.3 | 740 |  |  |

Table 6.2-3: Tukey's HSD results for experiment ROOM-2

| Group 1 | Group 2 | Mean Difference | P-Value | Lower CI | Upper CI | Significant |
| --- | --- | --- | --- | --- | --- | --- |
| c01 | c02 | 55.41 | <0.0001 | 50.04 | 60.78 | True |
| c01 | c03 | 54.21 | <0.0001 | 48.85 | 59.58 | True |
| c01 | c04 | 58.05 | <0.0001 | 52.68 | 63.42 | True |
| c02 | c03 | -1.20 | 0.9400 | -6.57 | 4.17 | False |
| c02 | c04 | 2.64 | 0.5840 | -2.73 | 8.01 | False |
| c03 | c04 | 3.84 | 0.2550 | -1.53 | 9.21 | False |

## ROOM-3: Time-domain object renderer / Crend binaural renderer (MC – 5.1, 96 kbps)

Experiment ROOM-3 was conducted by Philips International B.V..

The aggregated results per condition are listed in Table 6.3-1 and shown in Figure 6.3-1. The results separated by category and separated by sample are shown and Figure 6.3-2. All boxplots illustrate the distribution of votes by means of average ("x" in black), median (horizontal lines in red), inter-quartile-range (IQR, lower/upper box limit) and outliers (indicated by whiskers at ±1.5x IQR).

The ANOVA and Tukey's HSD analyses are provided in Table 6.3-2 and Table 6.3-3, respectively.

Table 6.3-1: results per condition for experiment ROOM-3

| Designator | Condition | COUNT | SCORE | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | IVAS Binaural (default) | 208 | 26.43 | 19.96 | 2.73 |
| c02 | Binaural Reference Renderer (RoomIR) | 208 | 85.01 | 13.48 | 1.84 |
| c03 | IVAS Binaural (RoomIR) | 208 | 84.71 | 14.11 | 1.93 |
| c04 | IVAS Binaural (RoomReverb) | 208 | 87.62 | 17.19 | 2.35 |

Figure 6.3-1: results per condition for experiment ROOM-3

Figure 6.3-2: results per condition versus category (left) and sample (right) for experiment ROOM-3

Table 6.3-2: ANOVA results for experiment ROOM-3

| Category Variables | Sum of Squares | Degrees of Freedom | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| C(Sample) | 627.0 | 3 | 0.7900 | 0.4969 |
| C(Condition) | 550555.6 | 3 | 697.9900 | <0.0001 |
| C(Category) | 4762.4 | 3 | 6.0400 | 0.0005 |
| C(Category):C(Condition) | 3602.0 | 9 | 1.5200 | 0.1356 |
| C(Sample):C(Condition) | 2125.4 | 9 | 0.9000 | 0.5263 |
| Residual | 211391.0 | 804 |  |  |

Table 6.3-3: Tukey's HSD results for experiment ROOM-3

| Group 1 | Group 2 | Mean Difference | P-Value | Lower CI | Upper CI | Significant |
| --- | --- | --- | --- | --- | --- | --- |
| c01 | c02 | 58.58 | <0.0001 | 54.44 | 62.72 | True |
| c01 | c03 | 58.27 | <0.0001 | 54.14 | 62.41 | True |
| c01 | c04 | 61.19 | <0.0001 | 57.05 | 65.33 | True |
| c02 | c03 | -0.31 | 0.9980 | -4.45 | 3.83 | False |
| c02 | c04 | 2.61 | 0.3660 | -1.53 | 6.75 | False |
| c03 | c04 | 2.92 | 0.2670 | -1.22 | 7.06 | False |

## ROOM-4: Crend binaural renderer (MC – 5.1.2, 128 kbps)

Experiment ROOM-4 was conducted by Philips International B.V..

The aggregated results per condition are listed in Table 6.4-1 and shown in Figure 6.4-1. The results separated by category and separated by sample are shown and Figure 6.4-2. All boxplots illustrate the distribution of votes by means of average ("x" in black), median (horizontal lines in red), inter-quartile-range (IQR, lower/upper box limit) and outliers (indicated by whiskers at ±1.5x IQR).

The ANOVA and Tukey's HSD analyses are provided in Table 6.4-2 and Table 6.4-3, respectively.

Table 6.4-1: results per condition for experiment ROOM-4

| Designator | Condition | COUNT | SCORE | STD | CI95 |
| --- | --- | --- | --- | --- | --- |
| c01 | IVAS Binaural (default) | 192 | 24.86 | 22.54 | 3.21 |
| c02 | Binaural Reference Renderer (RoomIR) | 192 | 83.49 | 13.34 | 1.90 |
| c03 | IVAS Binaural (RoomIR) | 192 | 82.25 | 15.83 | 2.25 |
| c04 | IVAS Binaural (RoomReverb) | 192 | 87.61 | 14.77 | 2.10 |

Figure 6.4-1: results per condition for experiment ROOM-4

Figure 6.4-2: results per condition versus category (left) and sample (right) for experiment ROOM-4

Table 6.4-2: ANOVA results for experiment ROOM-4

| Category Variables | Sum of Squares | Degrees of Freedom | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| C(Sample) | 1452.7 | 3 | 1.8400 | 0.1381 |
| C(Condition) | 514428.5 | 3 | 652.3300 | <0.0001 |
| C(Category) | 3513.4 | 3 | 4.4600 | 0.0041 |
| C(Category):C(Condition) | 17723.9 | 9 | 7.4900 | <0.0001 |
| C(Sample):C(Condition) | 3322.1 | 9 | 1.4000 | 0.1820 |
| Residual | 194523.1 | 740 |  |  |

Table 6.4-3: Tukey's HSD results for experiment ROOM-4

| Group 1 | Group 2 | Mean Difference | P-Value | Lower CI | Upper CI | Significant |
| --- | --- | --- | --- | --- | --- | --- |
| c01 | c02 | 58.63 | <0.0001 | 54.17 | 63.09 | True |
| c01 | c03 | 57.40 | <0.0001 | 52.93 | 61.86 | True |
| c01 | c04 | 62.76 | <0.0001 | 58.29 | 67.22 | True |
| c02 | c03 | -1.23 | 0.8920 | -5.70 | 3.23 | False |
| c02 | c04 | 4.12 | 0.0820 | -0.34 | 8.59 | False |
| c03 | c04 | 5.36 | 0.0110 | 0.90 | 9.82 | True |

# Conclusion

The results of the listening tests for the IVAS Characterization Phase were presented along with their statistical analyses. The raw voting data and further results are provided as an electronic attachment.

# References

- 3GPP S4-251565, "IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, Version v1.1.0," VoiceAge Corporation, July 2025.

- 3GPP Work Item Description SP-250262, "EVS Codec Extension for Immersive Voice and Audio Services, Phase 2," March 2025.

- Recommendation ITU-T P.800, "Methods for subjective determination of transmission quality," August 1996.

- Recommendation ITU-R BS.1534, "Method for the subjective assessment of intermediate quality level of audio systems," October 2015.

- Recommendation ITU-T P.1401, "Methods, metrics and procedures for statistical evaluation, qualification and comparison of objective quality prediction models," January 2020.

- 3GPP S4-251741, "Philips Listening Lab Report - IVAS Characterization Phase," Philips International B.V., November 2025.

Correspondance should be addressed to: Maximilian.Kentgens@head-acoustics.com, Jan.Reimes@head-acoustics.com