Source:	Philips International B.V., Qualcomm Incorporated

Title:	Updates to IVAS-8b Test Plan for Characterization Phase

Document for:	Approval

Agenda item:	7.5

# Introduction

During IVAS characterization, room acoustics tests ROOM-1 through ROOM-4 as specified in [2] have been completed and reported in [3]. As reported, before launching the tests it has been decided to use floating-point IVAS for room acoustics tests instead of fixed-point version due to limitations of the test infrastructure. Furthermore, the test plan does not contain detailed description of the acoustics environments used for these tests. This contribution proposes to update IVAS-8b: Test Plan for Characterization Phase to address these matters.

# Proposal

It is proposed to provide the following updates to the IVAS-8b permanent document.

## Use of IVAS floating-point decoder for room acoustic testing

Since the room acoustic tests were performed using floating-point IVAS version, tables I.1.1, I.2.1, I.3.1, and I.4.1 should be updated by replacing IVAS FX with IVAS FL.

## Addition of acoustic environments descriptions

The information regarding acoustic environments used in the experiments is currently missing. It is proposed to add clause I.5 to Annex I, containing the following text:

I.5. Acoustic Environments

For the room acoustics experiments ROOM-1 through ROOM-4, actual room acoustic environments were used. These environments were provided as impulse response recordings by Qualcomm and processed as specified in IVAS-7b Processing Plan for Characterization Phase. Additionally, 360-degree photos of the rooms were provided for the VR experience. The details of the impulse response recording setup and the acoustic environments are provided below.

SRIR measurements

2-point source measurements (stereo setup)
Test signal: 2 seconds sinesweep, 20-20000 Hz, 80dB headroom from noise floor, average of 4 recordings, 8 for Auditorium
Source: Genelec 8240A
Capture: EM32 Eigenmike with Eigenbox, Madiface XT
Software: RME Totalmix (monitoring & levels), MATLAB + AKTools
Setup: Source tweeter and mic horizon height = 128cm, source distance = 150cm
Output: deconvolved impulse response signal, converted to A-format.
360 photos: Ricoh Theta.

Acoustic environments

Auditorium

Dimensions: 20 × 16 × 7 m
Diffusers at front wall and on ceiling, mildly damped side and rear walls
Microphone behind front row

Kitchen corner

Dimensions: 6.5 × 4 × 3 m
No rugs, curtains open

Lounge

Dimensions: 8/6.5 x 6 x 3 m
Rigid drop ceiling
Doors to adjacent rooms open

Listening lab

Dimensions: 7 × 4 × 5 m
Meets acoustical requirements for ITU-R BS.1116-1

# References

- S4-251487 – IVAS Permanent Document IVAS-7b: Processing plan for characterization phase, v.1.0.0

- S4-251612 – IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.1.1.1

- S4-251741 – Philips Listening Lab Report – IVAS Characterization Phase