3GPP TSG-SA5 Meeting #163	S5-254524

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case to support network energy saving optimizations across operators

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for "Enhancements to support energy efficiency as a service criteria", related to WT-1.

This pCR introduces use case and potential requirements related to WT1.4 of the SI FS_Energy_Ph4_OAM. "

"

WT-1: Study 3GPP management system enhancements to support energy efficiency as a service criteria.

NOTE 1: This WT is to study the impacts on 3GPP management system to support the requirements specified by SA1 (in clause 6.15a TS 22.261) as part of 5GA Rel-20 WI EnergyServ_Ph2-REQ.

…

WT-1.4: Study management mechanisms to influence and/or control the network optimization actions targeting energy savings, within and across operators in a localized (i.e., geographically bound) and/or temporal (i.e., time bound) manner.

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 22.261: "Service requirements for the 5G system".

* * * Next Change * * * *

### 5.1.D	Use case #<D>: Management mechanisms to support network energy saving optimizations across operators

#### 5.1.D.1	Description

Network actions leveraging energy efficiency as a service criteria in clause 6.15a.6 of TS 22.261 [A] describes the use case to leverage energy-related information (e.g., energy consumption, energy efficiency), amongst others (e.g., network load), as criteria for network internal optimization actions targeting energy savings, within and across operators in a localized (i.e., geographically bound) and/or temporal (i.e., time bound) manner. The related requirements are specified in clause 6.15a.6.2 of TS 22.261 [A] to enable an operator to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators.

This use case is to study management mechanisms to enable the operator to configure the network to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators, including predefined times/locations, energy consumption/efficiency thresholds, etc.

#### 5.1.D.2	Potential requirements

PREQ-Energy_Opt_Cross_Operator-1: The 3GPP management system should enable the operator to configure the network to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators.

#### 5.1.D.3	Potential solutions

#### 5.1.D.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.1.D.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.1.D.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.1.D.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *