3GPP TSG-SA5 Meeting #164 	S5-255060

Dallas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0216 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 32.423 Trace Record Schema Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 3GPP TS 32.423 specification defines an optional attribute payloadSchemaURI in the Trace Record Header, intended to reference a schema URI that enables decoding of the Trace Record content. While this attribute provides a flexible way to associate a Trace Record with its corresponding schema definition, the current design exhibits several drawbacks that can negatively impact efficiency, scalability, and parser robustness, particularly when handling large-scale trace datasets. However, the current solution in 3GPP TS 32.423 has the some limitations: - Redundant Information in Every Record - Each Trace Record has the full payloadSchemaURI string included in its header—even when the consumer already possesses the schema. - This repetition adds unnecessary overhead and brings no added value once the schema is known to the decoder. - Fragmented URI Distribution - Schema URIs are distributed across individual Trace Records, making it difficult to centrally manage schema metadata. - From a Trace Record consumer perspective (e.g., decoders, analytics engines), this decentralized model hinders schema caching and optimization. - Trace Size Overhead - Including the payloadSchemaURI string in every single Trace Record leads to a non-negligible increase in trace data volume. - Given the potentially large number of Trace Records generated during normal network operations, this introduces inefficient use of storage and bandwidth resources, especially when traces are transmitted externally. - Parsing Complexity - The payloadSchemaURI location is not fixed, due to the variable-length Trace Record header. - This makes it tricky for decoders (especially schema-agnostic or minimal ones) to locate the URI without first knowing the exact header layout or receiving out-of-band header size hints. - In cases where a decoder lacks schema support, manual pre-processing or explicit offset configuration would be required, undermining self-descriptiveness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing a new administrative message which contains payload schema data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient Use of Network and Storage Resources |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.4.1, 5.2.4.x (new), G.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 5.2.2	Trace Record Header

The trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions.

Table 5.2.2.1 : Common fields in the trace record header

| Trace Record Header field name | Description |
| --- | --- |
| timeStamp (M) | Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer) |
| nfInstanceId (M) | Unique id of the Producer NF instance that produced this trace record represented by a (String) |
| nfType (M) | Type of the Producer NF that produced this trace record represented by a (String) |
| traceReference (M) | Trace Reference (see clause 5.6 of 3GPP TS 32.422 [3]) (represented by a 6 bytes octet string) See Note 6. |
| traceRecordingSessionReference (M) | Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [3]) represented by a (2 byte octet string. See Note 1.) |
| traceRecordTypeId (M) | Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL, TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT, TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, TRACE_FILE_OPEN, TRACE_FILE_CLOSE, TRACE_FILE_ABNORMAL_CLOSED, TRACE_RECORDING_SESSION_THROTTLED_START, TRACE_RECORDING_SESSION_THROTTLED_STOP, TRACE_SCHEMA_DATA. (See Note 2). |
| ranUeId (O) | RAN defined id to represent a UE (8 byte octet string. See Note 3.) |
| payloadSchemaURI (O) | URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.) |
| globalGnbId (CM) | Global gNB ID, as defined in subclause 9.3.1.6 of 3GPP TS 38.413 [23]. Applied for trace reported by gNB-CU-CP, gNB-CU-UP, gNB-DU. |
| vendorExtension (O) | Vendor-specific extension(s) (See Note 5.) |
| associatedTraceRecordingSessionReference (O) | Associated Trace Recording Session Reference used in continuous MDT represented by a 2 byte octet string. |
| CHOICE 1.1: traceRecordName (O) | A unique name string of a Trace Record. This attribute is not required for Administration events. |
| CHOICE 1.2: traceRecordId (O) | A unique integer ID of a Trace Record. |
| NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 37.483 [46] and 3GPP TS 38.473 [26]) has been captured in the traced signaling messages that value is used. NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). NOTE 5: The vendorExtension is typically a generic list of key-value pairs. NOTE 6: The encoding of the Trace Reference is a 6-byte Octet String in BCD format. The first 3-byte Octet String is the PLMN ID which consists of MCC and MNC. The next 3-byte Octet String is the Trace ID. The PLMN ID is encoded as specified in clause 9.3.3.5 of 3GPP TS 38.413 [23]. |  |

*** START OF NEXT CHANGE ***

#### 5.2.4.1	Introduction

The following administrative messages are defined for trace management purposes:

- Trace Session Start

- Trace Session Stop

- Trace Recording Session Start

- Trace Recording Session Stop

- Trace Stream Heartbeat (streaming only)

- Trace Session Not Started

- Trace Recording Session Not Started

- Trace Recording Session Dropped Events

- Trace File Open (file based only)

- Trace File Close (file based only)

- Trace File Abnormal Closed (file based only)

- Trace Recording Session Throttled Start

- Trace Recording Session Throttled Stop

- Trace Schema Data

*** START OF NEXT CHANGE ***

#### 5.2.4.x	Trace Schema Data administrative message

The Trace Schema Data administrative message contains the trace schema URI, which indicates the decoding schema of any subsequent Trace Records.

*** START OF NEXT CHANGE ***

# G.2	Trace Record Protocol Buffer (GPB) definitions

Normative GPB Trace Record schema, defined per clause 5.2:

syntax = “proto3”;

/* Trace Record per 3GPP 32.423 specification.

* v16

*/

enum TraceRecordType {

NORMAL = 0;

TRACE_SESSION_START = 1;

TRACE_SESSION_STOP = 2;

TRACE_RECORDING_SESSION_START = 3;

TRACE_RECORDING_SESSION_STOP = 4;

TRACE_STREAM_HEARTBEAT = 5;

TRACE_RECORDING_SESSION_DROPPED_EVENTS = 6;

TRACE_RECORDING_SESSION_NOT_STARTED = 7;

TRACE_FILE_OPEN = 8;

TRACE_FILE_CLOSE = 9;

TRACE_FILE_ABNORMAL_CLOSED = 10;

TRACE_RECORDING_SESSION_THROTTLED_START = 11;

TRACE_RECORDING_SESSION_THROTTLED_STOP = 12;

TRACE_SESSION_NOT_STARTED = 13;

TRACE_SCHEMA_DATA = 14;

}

message GlobalGnbId {

bytes plmn_identity = 1;

int64 gnb_id = 2;

}

message TraceRecordHeader {

int64  time_stamp = 1;

string nf_instance_id = 2;

string nf_type = 3;

bytes trace_reference = 4;

bytes trace_recording_session_ref = 5;

TraceRecordType trace_rec_type_id = 6;

optional bytes ran_ue_id = 7;

optional string payload_schema_uri = 8;

GlobalGnbId global_gnb_id = 9;

map<string, string> vendor_extension = 10;

optional bytes associated_trace_recording_session_ref = x;

}

message TraceSessionStart {

map<string, string> vendor_extension = 1;

}

message TraceSessionStop {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionStart {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionStop {

string reason = 2;

map<string, string> vendor_extension = 1;

}

message TraceStreamHeartbeat {

map<string, string> vendor_extension = 1;

}

message TraceRecordingSessionDroppedEvents {

int64 number_of_dropped_events = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionNotStarted {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceFileOpen {

map<string, string> vendor_extension = 1;

}

message TraceFileClose {

map<string, string> vendor_extension = 1;

}

message TraceFileAbnormalClosed {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionThrottledStart {

string reason = 1;

map<string, string> vendor_extension = 2;

}

message TraceRecordingSessionThrottledStop {

map<string, string> vendor_extension = 1;

}

message TraceSessionNotStarted {

string reason = 1;

map<string, string> vendor_extension = 2;

}

Message TraceSchemaData {

string URI = 1;

map<string, string> vendor_extension = 2;

}

message CommonTracePayload {

oneof record_payload {

TraceSessionStart trace_session_start = 1;

TraceSessionStop trace_session_stop = 2;

TraceRecordingSessionStart trace_recording_session_start = 3;

TraceRecordingSessionStop trace_recording_session_stop = 4;

TraceStreamHeartbeat trace_stream_heartbeat = 5;

TraceRecordingSessionDroppedEvents trace_recording_session_dropped_events = 6;

TraceRecordingSessionNotStarted trace_recording_session_not_started = 7;

TraceFileOpen trace_file_open = 8;

TraceFileClose trace_file_close = 9;

TraceFileAbnormalClosed trace_file_abnormal_closed = 10;

TraceRecordingSessionThrottledStart trace_recording_session_throttled_start = 11;

TraceRecordingSessionThrottledStop trace_recording_session_throttled_stop = 12;

TraceSessionNotStarted trace_session_not_started = 13;

TraceRecordSchemaData trace_schema_data = 14;

}

}

message TraceRecordPayload {

optional int64 payload_size = 1;

bytes binary_payload = 2;

}

message TraceRecord {

TraceRecordHeader header = 1;

TraceRecordPayload payload = 2;

}

message StreamingTraceRecord {

TraceRecord record = 1;

optional CommonTracePayload administrative_message = 2;

}

*** START OF NEXT CHANGE ***

Annex H (informative):
Examples of Protocol Buffer (GPB) encoded Streaming Trace administrative messages

The following examples illustrate the use of Prococol Buffer encoding for Streaming Trace administrative messages according to the definitions in clause 5.2.4.

The examples are in compact GPB format, using the schema defined in Annex G.

Example 1, Decoded Trace Session start message:

TraceRecord {
    header {
      time_stamp: 1584103023591,
      nf_instance_id: NETWORK_MANAGED_ELEMENT_ID,
      nf_type: RadioNode,
      trace_reference: ''H,
      trace_recording_session_reference: ''H,

trace_rec_type_id: TRACE_SESSION_START,

ran_ue_id: ''H,
    },
    payload: ''H
  },
  CommonTracePayload

}

Example 2, Decoded Trace Session stop message:

TraceRecord {
    header {
      time_stamp: 158415623591,
      nf_instance_id: NETWORK_MANAGED_ELEMENT_ID,
      nf_type: RadioNode,
      trace_reference: ''H,
      trace_recording_session_reference: ''H,

trace_rec_type_id: TRACE_SESSION_STOP,

ran_ue_id: ''H,
    },
    payload: '0A 01 09 11'H
  },
  CommonTracePayload {
    trace_session_stop {
    }
  }

Example 3, Decoded Trace Recording Session Dropped Events message:

TraceRecord {
    header {
      time_stamp: 1584103023591,
      nf_instance_id: NETWORK_MANAGED_ELEMENT_ID,
      nf_type: RadioNode,
      trace_reference: ''H,
      trace_recording_session_reference: ''H,

trace_rec_type_id: TRACE_RECORDING_SESSION_DROPPED_EVENTS,

ran_ue_id: ''H,
    },
    payload: '0A'H
  },
  CommonTracePayload {
    trace_recording_session_dropped_events {

number_of dropped_events: 6
    }
  }

Example y, Trace Schema Data message:

StreamingTraceRecord {

TraceRecord {
    header {
      time_stamp: 1584103023591,
      nf_instance_id: NETWORK_MANAGED_ELEMENT_ID,
      nf_type: RadioNode,
      trace_reference: ''H,
      trace_recording_session_reference: ''H,

trace_rec_type_id: TRACE_SCHEMA_DATA,

ran_ue_id: ''H,
    },
    payload: ''H
  },
  CommonTracePayload {
    trace_schema_data {

uri: 'www.schema.com/trace-schema-data-1'
    }
  }

}

*** END OF CHANGE ***