3GPP TSG-SA5 Meeting #164	S5-255107

Dallas, Texas, USA, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on CCL for Multi-domain ES Optimization

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.1.0

Work Item:	Closed Control Loop Management phase 2

Comments

This pCR is to add use case and requirements for CCL for energy saving control

- WT-3: Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.567: “Management and orchestration; Management Aspects of Closed Control Loops”

# 5. 			Use Cases

### 5.X	CCL for Multi-domain ES Optimization

5.X.1	Description

TS28.310 and 28.541 have specified capabilities for energy saving management where a distributed or centralized Energy Saving Function (DESF, CESF) decide whether to activate a cell (send a cell into energy saving state) or to deactivate a cell (wake up a cell from an energy saving state). The DESF and CESF are element management functionalities responsible or individual cells. At the domain level, a closed control loop can evaluate and control the ES actions for different elements: and for a scenario with several domains, e.g. several Ran domains or RAN and core domains, a closed control loop can evaluate and control the ES actions for several domains.

The closed control loop can composed to include existing capabilities. For example, TS 28.104 [X] has specified MDA reports on supporting analysis and predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands, energy saving analysis. The MnS consumer can compose MDA capabilities for service demands and energy savings analysis as part of a closed control loop for multi-domain energy saving optimization. The management system should enable the MnS consumer to compose or instantiate the energy savings closed control loop for multi-domain energy saving optimization.

#### 5.X.2	Potential requirements

REQ- CCL-ES-1: The management system should enable the MnS consumer to compose or instantiate the energy savings closed control loop for multi-domain energy saving optimization.

### 5.x.3	Possible solutions

This solution proposes to enhance the existing CCL information models defined in 3GPP TS 28.567 [2].

To support compose or instantiate the energy savings closed control loop for multi-domain energy saving optimization, the following can introduced:

-	an IOC for multi-domain energy savings optimization CCL.

-	The multi-domain energy savings optimization CCL inherits from a closed control loop

-	an IOC for a data collection component for energy savings data

-	IOCs each for service analytics and energy saving analysis to be applied as components of the multi-domain energy savings optimization CCL.

-	Introduce attributes on each component that take the analytics output of one component as input to the other components

-	A datatype and corresponding attribute indicating the actions of the multi-domain energy savings optimization CCL which can be taken towards the DESF or the CESF. The attributes of this dataType are the control parameters of the DESF and CESF

5.X.4			Evaluation of solutions

.

* * * End of Changes * * * *