3GPP TSG-SA5 Meeting #164	S5-255137

Dallas, USA, 17 - 21 November 2025

Source:	China Mobile

Title:	Pseudo-CR on TR 28.885 Add solution for renewable energy consumption

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	V0.1.0

Work Item:	FS_Energy_Ph4_OAM

Comments

This contribution proposes to add the solution on calculation of renewable energy related KPIs to TS 28.554 for renewable energy consumption per gNBs and UPFs to support to report the renewable energy consumption to 5G system for policy adjuctment.

Proposed Changes

* * * First Change * * * *

5.4.1	Use case #1: Renewable energy consumption

5.4.1.1	Description

The operators are interested in using renewable energy sources to reduce emissions and boost network efficiency. Therefore, it’s important to track and report the share of energy consumption from renewable sources in their networks.

Due to the highly variable and unpredictable nature of renewable energy sources, the supply of renewable energy varies substantially by time and location. There is a need for the 3GPP management system to support to report of renewable energy consumption on different granularities (e.g., NF, NE, Sub-network), and the total energy consumption considering energy consumption from energy supplies with renewable and non-renewable sources in order to estimate of renewable energy consumption, and to report the renewable energy consumption for different granularities.

Therefore, the 3GPP management system need to support to provide the renewable energy consumption based on energy related information obtained from various sources, or to provide the renewable energy consumption for different granularities, and support to report this information to authorized consumers, e.g., EIF.

5.4.1.2	Potential requirements

REQ-RECM-CON-1: The 3GPP management system shall be able to allow its authorized consumers to request reporting about renewable energy consumption at different granularities.

5.4.1.3	Potential solutions

5.4.1.3.1	Potential solution #1: Renewable energy ratio/factor

5.4.1.3.1.1	Potential solution #1.1: Renewable energy ratio/factor of non-split gNB

5.4.1.3.1.1.1	Introduction

In 3GPP TR 28.880[X], the existing solution for the Renewable energy factor of non-split gNB is defined, but it was omitted during the R19 normative phase. Upon re-evaluation, this solution in clause 5.3.3.2 of TR 28.880[X] can be reused to support the use case and requirements defined in 5.4.1. The applicable condition for this solution is: when operators can get the total energy consumption powered by renewable energy, the ratio/ factor can be obtained.

In clause 3.1 of TS 28.310[Y], the Renewable Energy Factor (REF) is defined as the ratio of the renewable energy to the total energy. This potential solution describes how the REF of a non-split gNB can be calculated by the 3GPP management system.

5.4.1.3.1.1.2	Description

Detailed solution sees 5.3.3.2.2.1 and 5.3.3.2.2.2 in 3GPP TR 28.880[X].

5.4.1.3.1.2	Potential solution #1.2: Renewable energy ratio/factor of the 5G NF

5.4.1.3.1.2.1	Introduction

In 3GPP TR 28.880[X], the existing solution for the Renewable energy factor of the 5G NF is defined, but it was omitted during the R19 normative phase. Upon re-evaluation, this solution in clause 5.3.3.3 of TR 28.880[X] can be reused to support the use case and requirements defined in 5.4.1. The applicable condition for this solution is: when operators can get the total energy consumption powered by renewable energy, the ratio/ factor can be obtained.

The power supply of network elements and network functions can change over different time periods. These power supply information includes grid electricity, Backup energy and locally generated energy (see clause 6 of ITU-T Recommendation L.1333 [20]). This information is not managed by 3GPP system but are defined by ETSI.  (ETSI ES 202 336-1 [21] to ETSI ES 202 336-12 [32]). Nevertheless, the 3GPP system requires this information which can be distributed and utilized within the 3GPP system (including 5GC and external 3rd parties).

This potential solution proposes a list of information elements to be associated with the network functions, for which the operator collects information and inputs into the 3GPP management system.

5.4.1.3.1.2.2	Description

Detailed solution see 5.3.3.3.2 in 3GPP TR 28.880[X].

5.4.1.3.2	Potential solution #2: Estimation of Renewable Energy Consumption (REC)

5.4.1.3.2.1	Potential solution #2.1: Estimation of Renewable Energy Consumption (REC) of UPF

5.4.1.3.2.1.1	Introduction

The applicable condition for this solution is: when operators can get the ratio/factor of different types of renewable energy powered the system, the total energy consumption powered by renewable energy can be obtained. This potential solution describes how the Renewable Energy Consumption (REC) of UPF can be estimated by the 3GPP management system.

5.4.1.3.2.1.2	Description

a) RECUPF

b)	This KPI provides the Estimation of Renewable Energy Consumption of a UPF (RECUPF) over a time period. The Estimation of Renewable Energy Consumption of a UPF (RECUPF) is the Energy Consumption of the UPF (ECUPF) multiplied by the Renewable Energy Factor (i.e., the ratio of renewable energy) of the energy source which powers the UPF  (REFUPF). The unit of RECUPF is J.

b-1)	Unit of RECUPF: Integer.

b-2)	Type of RECUPF: CUM

c)	This calculation formula is obtained as:

RECUPF=ECNF *  REFUPF

, where:

ECNF is the Energy Consumption of the UPF as defined in clause 6.7.3.1 in TS 28.554. Its unit is J;

REFUPF indicates Renewable Energy Factor, which is the ratio of the renewable energy to the total energy of the UPF. Its unit is percentage. This factor can be configured as operator specific.

d) ManagedElement

NOTE: 	This KPI is applicable for the UPFs that are powered using multiple energy supply including renewable energy supply and non-renewable energy supply.

Editor’s Note: how to obtain REFUPF is FFS.

5.4.1.3.2.2	Potential solution #2.2: Renewable energy ratio/factor of non-split gNB

5.4.1.3.2.2.1	Introduction

The applicable condition for this solution is: when operators can get the ratio/factor of different types of renewable energy powered the system, the total energy consumption powered by renewable energy can be obtained. This potential solution describes how the Renewable Energy Consumption (REC) of non-split gNB can be estimated by the 3GPP management system.

5.4.1.3.2.2.2	Description

a) RECgNB

b)	This KPI provides the Estimation of Renewable Energy Consumption of a gNB (RECgNB) over a time period. The Estimation of Renewable Energy Consumption of a gNB (RECgNB) is the Energy Consumption of the gNB (ECgNB) multiplied by the Renewable Energy Factor (i.e., the ratio of renewable energy) of the energy source which powers the gNB (REFgNB). The unit of RECgNB is J.

b-1)	Unit of RECgNB: Integer.

b-2)	Type of RECgNB: CUM

c)	This calculation formula is obtained as:

RECgNB=ECgNB *  REFgNB

, where:

ECgNB is the Energy Consumption of the gNB as defined in clause 6.7.3.4.2 in TS 28.554. Its unit is J;

REFgNB indicates Renewable Energy Factor, which is the ratio of the renewable energy to the total energy of the gNB. Its unit is percentage. This factor can be configured as operator specific.

d) ManagedElement

NOTE: 	This KPI is applicable for the gNBs that are powered using multiple energy supply including renewable energy supply and non-renewable energy supply.

Editor’s Note: how to obtain the REFgNB is FFS.

4.4.1.4  Evaluation of potential solutions

* * * End of Changes * * * *