3GPP TSG-SA5 Meeting #164	S5-255193

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0058 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.256 Clarification on CHF address and Charging Characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFSeg |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the description of TS 23.503 Policy and charging control, the "chfGroupId", "Charging address" and "Charging information" are different attributes in the policy information from PCF to SMF/AMF/SMSF and the "Charging address" is optionally associated with CHF instance and CHF set ID, which is not associated with "chfgroupid". According to the description of TS 29.503 Unified Data Management Services, "chfGroupId " and "3gppChargingCharacteristics" are different attributes in the "AccessAndMobilitySubscriptionData" of AMF, "SessionManagementSubscriptionData" of SMF and "SmsManagementSubscriptionData" of SMSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the CHF group id in the charging address and charging characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Charging Characteristics is inconsistent with the SA2/CT4 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3,A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-254832 |  |  |  |  |  |  |  |  |

First Change

### 5.1.3	CHF selection

The CHF Address(es) selection by the AMF is done during the Registration process, this selection may be based on the following options and with priority order depending on Operator's policies:

-	CHF Address(es) used for UE context transferred from old AMF.

-	CHF address(es) provided by the PCF as part of Access and mobility policy control information.

-	UDM provided charging characteristics.

-	NRF based discovery (Applicable for CHF selection by another CHF as well)

-	Locally provisioned charging characteristics (Applicable for CHF selection by another CHF as well)

The UDM provided charging characteristics (see annex A) may be used to indicate CHF instance ID(s), CHF set ID(s) ), CHF Group ID.

CHF instance ID(s), CHF set ID(s), CHF Group ID can be used for NRF based discovery, see TS 32.290 [57] clause 6.1.

Once selected, these CHF Address(es) shall be used as long as the UE is registered in the AMF.

Next Change

# A.1	General

A subscriber may have Charging Characteristics assigned to his subscription. Default Charging Characteristics may also be pre-provisioned on the AMF.

During UE Registration, the subscribed Charging Characteristics, if any, shall override the AMF pre-provisioned Charging Characteristics. The subscribed Charging Characteristics may be transferred from old AMF during UE context transfer or retrieved from UDM.

The Charging Characteristics parameter consists of a string of 16 bits designated as behaviours , freely defined by Operators. These behaviours may be things like:

-	CHF addresses: to be used by the AMF, optionally with associated CHF instance ID(s), CHF set ID(s), CHF Group ID associated with the subscription.

-	CHF selection method: this indicates how the AMF is to select the CHF instance e.g., local configuration, NRF (see clauses 5.1.3 and 5.1.5.2)

-	Charging scenario: indicates the charging scenario to be used i.e., ECUR, IEC, or PEC

-	Level: the default trigger levels for the Location reporting charging e.g., TAI, or Cell

One usage may be as a behaviour index associated to Registration, N2 connections and Location reporting in converged charging as described in the example in Table A.1.1:

Table A.1.1: Example of Charging Characteristics behaviours for AMF

|  |  |  | Registration charging | N2 connections charging | Location reporting charging |  |
| --- | --- | --- | --- | --- | --- | --- |
| Behaviour Index | CHF address | CHF selection method | Charging scenario | Charging scenario | Charging scenario | Level |
| 0 | CHF Set ID 1 | NRF | ECUR | - | - |  |
| 1 | Primary & secondary CHF addr. 1 & 2 | Local config. | IEC | PEC | PEC | TAI, Cell |
| 2 | CHF Group ID 1 | NRF | PEC | - | PEC | Cell |
| 3 | Primary & secondary CHF addr. 3 & 4 | Local config. | - | PEC | PEC | TAI |
| … | … |  | … | … | …… | … |

End of Change