3GPP TSG-SA5 Meeting #164	S5-255254

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0752 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add Number of UE related the SSB beam Index (Maximum) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current performance measurements in TS 28.552 only provide Number of UE related the SSB beam Index (mean). The proposed change would add a new measurement that number of UE related the SSB beam Index (Maximum) for each individual SSB within a cell. Adding this new measurement will bring following advantages: 1)Identifying if specific SSB beams within a cell are experiencing congestion due to an uneven distribution of User Equipments (UEs). 2)Quantifying the peak load on individual beams, which is a key factor for user experience in beam-centric deployments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed change would add a new measurement that number of UE related the SSB beam Index (Maximum) for each individual SSB within a cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of Number of UE related the SSB beam Index (Maximum) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.28.x(New clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.1.1.28.x	Number of UE related the SSB beam Index (Maximum)

a)	This measurement provides number of UE related the SSB beam index.

b)	CC.

c)	The measurement is obtained by sampling at a pre-defined interval, the number of UE related SSB beam index, and then taking the maximum. The UE related beam index which maintained by UE random access and handover and beam switch in case the beam switch function is enabled (see TS 38.331[20]).

d)	A single integer value.

e)	L1M.SSBBeamRelatedUeNbr.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	The main usage of this performance measurements is for performance assurance, and radio resource optimization such as updating the cell coverage shape. This measurement is only applicable when the beam switch function is activated.

==============End of change==============