3GPP TSG-SA5 Meeting #164	S5-255277

Dallas, USA, 17 - 21 November 2025

Source:	Ericsson

Title:	Pseudo-CR TR 28.888 Add background on authorization of external MnS consumer using CAPIF

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

The authorization of external MnS consumer using CAPIF is a Rel-19 leftover that was added to FS_EnExpo (Rel-20 SID) in SA5#163. This feature describes how external MnS consumer, using an access token emitted by CAPIF, can get access to service API(s) containing management service information.

This pCR aims to lay the foundation for this feature, by overviewing the capabilities of both CAPIF and MSED at the end of Rel-19, which constitute the starting point for discussing the use cases in scope of this feature. To that end, the pCR proposes to add a new subclause 4.x under the clause 4 (Concepts and background), consisting of the following sections:

- Clause 4.X.1 “Authorization in CAPIF Framework”: it provides an overview of current solutions across SA6/SA3/CT3. This includes the feature of fine-granularity of access control, developed in Rel-19.

- Clause 4.X.2 “The role of MSED”: it provides an overview of the MSED concept specified in TS 28.579, including a description of its functional scope.

- Clause 4.X.3: “Multi-AEF setup”: it describes how CAPIF works when two (or more) API provider domains are registered into the CCF, one of them being the 3GPP management system.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

[a]	3GPP TS 28.537: "Management and Orchestration; Management capabilities".

[b]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference point (IRP); Information Service (IS)"

[c]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[c]	3GPP TS 33.122: "Security aspects of Common API Framework for 3GPP Northbound APIs "

[d]	3GPP TS 29.222: " Common API Framework for 3GPP Northbound APIs".

[e]	RFC 6749: "The OAuth 2.0 Authorization Framework".

[f]	RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[g]	RFC 9068: "JSON Web Token (JWT) Profile for OAuth 2.0 Access Tokens".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

## 4.X	Authorization of external MnS consumers using CAPIF

An operator can decide to expose 3GPP management system capabilities to external MnS consumers either directly (using existing MnS interfaces) or indirectly (using CAPIF), as noted in TS 28.533 [2].

When CAPIF is not used, an external MnS consumer gets access to MnS APIs using 3GPP management system mechanisms defined for discovery (see TS 28.537 [a] and TS 28.622 [b]) and access control (see clause 4.9 of TS 28.533 [2]), in the same way as an internal MnS consumer. The access control capability is provided by authentication and authorization service producers, which are responsible for authenticating and authorizing MnS consumers (see Annex D of TS 28.533 [2]), based on operator-configured information. The operator can configure this information using the MSAC framework defined in TS 28.319 [4], by associating Identity resources with Role and AccessRule resources, and allocating separate Identities to individual (or group of) MnS consumers.

When CAPIF is used, an external MnS consumers gets access to MnS APIs using CAPIF mechanisms defined for discovery (see TS 23.222 [c]) and access control (see TS 33.122 [d]), in the same way as an API invoker. To expose capabilities using CAPIF, the 3GPP management system needs to be registered as an CAPIF API provider domain. To that end, the 3GPP management system provides required API provider domain functionalities (AMF, APF, AEF) in a framework called Management Service Exposure Domain (MSED).

In this clause the focus is on the second case. Section 4.x.1 describes the CAPIF mechanisms defined for access control as of Release 19. Section 4.x.2 describes the role of MSED.

### 4.X.1	Authorization in CAPIF Framework

3GPP TS 33.122 [d] specifies security aspects of the CAPIF Framework. These aspects include defining how API invokers can get authorization to access one or more service APIs exposed by an API Exposing Function (AEF). This access can be authenticated and protected using one of the following security methods: TLS-PSK (TS 33.122, clause 6.5.2.1), PKI (TS 33.122, clause 6.5.2.2) and TLS with OAuth token (TS 33.122, clause 6.5.2.3). Which specific security method is to be used between an API invoker and the AEF is negotiated between the API invoker and the CCF (TS 33.122, clause 6.3.1.2), using CAPIF_Security_API (TS 29.222 [e], clause 8.5).

In the context of the present study, the relevant security method is TLS with OAuth token. This method makes use of the OAuth2.0 framework (RFC 6749 [f]), where API invoker performs the role of resource owner, CCF performs the role of authorization server, and AEF performs the role of resource server.

#### 4.X.1.1	API invoker’s authorization using TLS with OAuth token

As noted in clause 6.5.2.3 of TS 33.122 [d], this selection method leads to the authorization workflow pictured in Figure 4.X.1.1-1.

Figure 4.X.1.1-1: API Invoker’s authorization workflow when security method is TLS with OAuth token (see Figure 6.5.2.3-1 in TS 33.122 [3]).

This workflow consists of two stages: Service API authorization request, over CAPIF-1e interface (steps 1-4); and Service API invocation request, over CAPIF-2e interface (steps 5-8). Details on these steps are explained below:

Step 1. The API invoker establishes a TLS session with CCF based on mutual authentication. Here, the server certificate is CCF’s Root CA (sent to the API invoker after enrolment), and the client certificate is API invoker’s certificate in PEM format (sent to the API invoker during the onboarding).

Step 2. To invoke one or more service APIs, the API invoker needs to get an access token. To that end, the API invoker sends an access token request to the CCF, consuming CAPIF_Security_API (clause 8.5 of TS 29.222) over CAPIF-1e interface. The resource URI is {apiRoot}/capif-security/<api-version>/securities/{securityId}/token. The request body carries information described by the AccessTokenReq datatype (clause 8.5.3.2.6 of TS 29.222), which includes:

- client_id, which uniquely identifies an OAuth2.0 client. Since API invoker is OAuth2.0 client, client_id is set to “apiInvokerId”. “apiInvokerId” is generated by the CCF during the API invoker’s onboarding procedure (see clause 8.4 of TS 29.222).

- grant_type, which is set to “client-credentials”

- (optional) client_cred, which is set to onboarding secret.

- (optional) scope, which represents the OAuth2.0 requested scope, i.e. the list of service APIs for which the API invoker requests access to. If not provided, the CCF understands that the token is requested to all the service APIs the invoker is subscribed to.

Step 3. Upon receiving the access token request, the CCF uses the authorization function (performs the role of OAuth2.0 server) to authorize the API invoker, by verifying whether the client_id match with the API invoker’s certificate (step 1), and by checking the list of service APIs allowed for API invoker, which is captured in the API Invoker Profile (see TS 23.222 [c]). Following this authorization, the authorization function generates an access token and digitally signs it (based on the private key).

NOTE: The authorization function is an internal entity of the CCF (see clause 6.3.7 in TS 23.222 [c]).

Step 4. The CCF issues the access token to the API invoker. The response payload is represented with AccessTokenRsp (clause 8.5.4.2.7 of TS 29.222), which includes:

-  access_token: digitally signed access token containing access token’s claims (i.e. “iss”, “scope”, “exp”).

- expires_in: expiration time (number of seconds after which the token expires). It is the same as the token’s claim “exp”.

- (optional)scope, which represents the OAuth2.0 granted scope, i.e. the list of service APIs that the API invoker has granted access to. The granted scope can be the same or a subset of the requested scope (step 1). It is the same as the token’s claim “scope”.

After Step 4, the TLS session is torn down and API invoker is ready to reach out to the AEF.

Step 5. The API invoker establishes a TLS session with the AEF based on mutual authentication.

NOTE: In this step, the AEF communicates with CCF over CAPIF-3 interface, fetching relevant security information to perform authentication and secure interface establishment with the API invoker.

Step 6. The API invoker sends an API invocation request to the AEF, which includes:

- Service API call: {<service API name>, < resource>, <operation>}

- Requestor’s identity: “apiInvokerId”

- Requestor’s access token: the access token received in step 4. This access token is included in the request as an HTTP header filed as specified in clause 2.1 of RFC 6750 [g].

Step 7. The AEF receives the API invocation request and processes it, thus performing the role of OAuth2.0 resource server. First, it validates the integrity and authenticity of the token (see RFC 9068 [h]) and that the token has not expired. If this validation is successful, next the AEF verifies the service API call against the authorization claims in the access token, ensuring that the service API call parameters (i.e. <service API name>, <resource>, <operation>) are within the token’s scope. If within, the AEF executes the service API call. Otherwise, it generates an error response.

Step 8. The AEF sends response back to the API invoker, which includes the result of service API call execution or an error response

#### 4.X.1.2	Access token format

During Rel-19 normative work, CAPIF authorization functionality was enhanced with two novel features: “CAPIF_Ext1” and “CAPIF_Ext2” (see clause 8.5.6 in TS 29.222). “CAPIF_Ext1” enabled support for fine-granularity API access control, with resource and/or operation-level access control within a given service API. “CAPIF_Ext2” enabled support for generic string-based OAuth scope encoding based on clause 3.3 of RFC 6749 [f].

Based on the support of these features, the OAuth2.0 scope of an access token in CAPIF framework can take three possible formats (see clause 8.5.4.2.6, 8.5.4.2.7 and 8.5.4.2.8 of TS 29.222), as described in Table 4.X.1.2-1.

Table 4.X.1.2-1: Formats of access token scopes in CAPIF Rel-19.

| Format | Example | Support |
| --- | --- | --- |
| Format 0 | '3gpp#aef-jiangsu-nanjing: 3gpp-monitoring-event,3gpp-as-session-with-qos; aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' | Neither the “CAPIF_Ext1” feature nor the “CAPIF_Ext2” feature are supported. |
| Format 1 | 3gpp#aef1: 3gpp-monitoring-event:res.subscriptions,3gpp-as-session-with-qos: res.subscriptions:op.create; aef-zhejiang-hangzhou: 3gpp-cp-parameter-provisioning,3gpp-pfd-management:res.transactions:op.read' | The “CAPIF_Ext1” is supported and the “CAPIF_Ext2" feature is not supported. |
| Format 2 | List of space-delimited strings, as defined in clause 3.3 of RFC 6749 [e]. | The “CAPIF_Ext2” is supported and the “CAPIF_Ext1" feature is not supported. |

As seen, Format 0 corresponds to legacy solution, where the granularity of the (requested/granted) scope is a list of service API names. Format 1 enriches Format 0, as it allows to provide resource and/or operation level authorization information per service API name. Finally, Format 2 represents a list of strings whose content and granularity require negotiation between the AEF and CCF; this negotiation occurs out-of-band, according to the business relationships between CCF provider and API provider domain provider.

### 4.X.2	The role of MSED

The details of MSED are specified in 3GPP TS 28.579 [3]. Figure 4.X.2-1 illustrates the role of MSED in CAPIF framework.

Figure 4.X.2-1: 3GPP management system used with CAPIF framework. Blue interfaces are 3GPP management system interfaces, and black interfaces are CAPIF interfaces.

As seen, the MSED provides needed mediation to communicate API provider domain (3GPP management system) with CCF (CAPIF-3/4/5 interfaces) and API invoker (CAPIF-2e interface). Within the API provider domain, the MnS producers offer MnSs and authentication and authorization service producers provide access control capability. There’s also a system admin (operator) who is responsible for setting up the environment in the design-time, with offline activities that include:

- Registering MSED into CAPIF. The operator can do this with the CAPIF_API_Provider_Management API (see clause 8.9 of TS 29.222).

- Deciding which management service information will be published into CAPIF. In particular, the operator decides the specific management services (among all the management services supported in 3GPP management system) to be published, and for each one, the specific set of resources and operations (MnS component A and B) and optionally the performance/fault data to be published (MnS component C). The operator configures these decisions on the authentication and authorization service producer, by defining access rules using the MSAC framework [4].

- Deciding how to publish the selected management service information, i.e. the name and number of service APIs that will offer this information. The operator configures these decisions on the MSED’s APF, which will proceed with the publishing of these service APIs using the CAPIF_Publish_Service_API (see clause 8.2 of TS 29.222).

- Ensuring MSED’s AEF and CCF can interpret the same scope’s format for the access token.

- Enrolling API invokers. When an API invoker enrols into the CAPIF, a subscription for this invoker is created. This subscription defines the list of published service APIs that the API invoker has subscribed to, together with SLA parameters related to service API invocation (e.g., quota, throttling). Once enrolled, the API invoker can later proceed with the onboarding, by using CAPIF_API_Invoker_Management API (see clause 8.4 of TS 29.222), or the open discovery, by using CAPIF_Open_Discover_Service_API (see clause 8.11 of TS 29.222).

- Define discovery policies for API invokers, e.g. to restrict discovery to category of APIs. These discovery policies, which are outside the scope of standardization, can be applied to individual (or a group of) API invokers.

It is worth noting that the four activities are agnostic to existing/upcoming API invokers. The last two activities allow differentiation of API invokers in terms of what they can discover and consume.

With these offline activities, the environment is operational and API invokers can start applying the standardized procedures over CAPIF-1e interface (e.g. onboarding, (open) discovery, Service API authorization request) and CAPIF-2e interface (e.g., Service API invocation request).

To ensure that the API invoker’s access token issued by the CCF can be later interpreted by the MSED’s AEF, it is required that CCF and API provider domain agree on the format of access token’s scope, as explained in clause 4.X.1.2. The selected format shall be supported by the authorization function that both CCF and AEF use for token generation and validation, respectively.

### 4.X.3	Multi-AEF setup

CAPIF framework allows the registration of multiple API provider domains, each hosting a different AEF. For example, in case 5GC and 3GPP management system register as API provider domains, it is possible to publish service APIs providing 5GC functionality (exposed by NEF’s AEF) and OAM functionality (exposed by MSED’s AEF). In this multi-AEF context, where one single API invoker can enrol to service APIs offered by different AEFs, the API invoker onboarding procedure remains the same as in single-AEF context; the only difference is that the API Invoker Profile (see TS 23.222 [c]), created and guarded by the CCF after successful onboarding, now contains service APIs from different AEFs.

NOTE: The API Invoker Profile contains all the details of an onboarded API invoker, and is used by the CCF for discovery and authorization procedures over CAPIF-1e interface.

When the API invoker requests authorization for service APIs in multi-AEF context, two situations can happen. On the one hand, the API invoker can issue a single authorization request for service APIs from different AEFs; in this case, the CCF issues a single access token, with a scope’s format that the CCF and all the serving AEFs can interpret. On the other hand, the API invoker can issue multiple authorization requests, each containing service APIs from a different AEF; in this case, the CCF issues multiple access tokens, each with a scope’s format that both CCF and the serving AEF support. In the previous example, with NEF’s AEF and MSED’s AEF offering service APIs, the API invoker can have a single access token (whose scope’s format requires support by the CCF, NEF’s AEF and MSED’s AEF) or two access tokens, i.e. one to access service APIs providing 5GC functionality (whose scope’s format requires support by the CCF and NEF’s AEF) and another to access service APIs providing OAM functionality (whose scope’s format requires support by the CCF and MSED’s AEF).

* * * End of Changes * * * *