3GPP TSG-SA5 Meeting #164	S5-255289

Dallas, USA 17 - 21 November 2025

Source:	L.M. Ericsson Limited

Title:	Rel-20 pCR TR 28.883 Retrieval of NDTReports associated with deleted NDTJobs

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.1.0

Work Item:	FS_NDT_Ph2

Comments

Proposes studying the ability to fetch reports produced by deleted/removed NDTJobs.
The rationale is to compare old instances of job invocations with new job iterations, where the same simulation is performed but the underlying network digital twin/configuration has been changed.

Proposed Changes

* * * First Change * * * *

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

# 5	Use cases

## 5.X	Use Case #<X>: Retrieval of NDTReports associated with deleted NDTJobs

### 5.X.1	Description

The MnS Consumer requires the capability to retrieve NDTReports that were generated by NDTJobs which have subsequently been deleted and no longer present in 3GPP management system.
In the current specification, the ndtJobRef attribute of the NDTReport refers to the Distinguished Name (DN) of the NDTJob instance. Once the corresponding NDTJob is removed, the ndtJobRef becomes invalid, preventing access to the NDTReport via the MnS interface.

This limitation makes it difficult for the MnS Consumer to perform comparisons between previous and current simulation results, such as intent evaluation or performance verification over time.

It is therefore desirable to allow the retrieval of NDTReports even after the associated NDTJob has been deleted, for example by enabling:

- persistent storage of NDTReports independent of NDTJob lifecycle, or

- a query method for archived NDTReports that references metadata rather than the live DN of the deleted NDTJob.

Table 6.2.1.3.8.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTJobOutputData | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | F | F | T |
| ndtFunctionRef | M | T | F | F | T |

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 5.X.2	Potential requirements

NDT-LCM-XX: The MnS Producer should provide the capability for the MnS Consumer to retrieve NDTReports which have been produced by historical NDTJobs.

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

# 6	Conclusions and Recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *