3GPP TSG-SA5 Meeting #164	S5-255365

# Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Enhancements on ES Scenarios and actors

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.1.0

Work Item:	FS_Energy_Ph4_OAM

Comments

TS 28.310 defines solutions for ES including a number of actors. The specification refers to many terms which are more than the number of actors. The relationship among the actors is as such not clearly understandable and some are completely confusing. Hence it is important to identify and fix the related inconsistencies in the existing specifications to enable support for any enhancements or new use cases to improve energy saving, energy efficiency and reducing carbon footprint of 5G network.

This pCR introduces the analysis showing the concern and the aspects that need to be improved.

Relates to:

WT-3: Study use case(s), requirement(s) and solution(s) to improve energy saving, energy efficiency and reducing carbon footprint of 5G network.

Proposed Changes

* * * First Change * * * *

### 5.3.A	Use case #A: Enhancements on ES Scenarios and actors

#### 5.3.A.1	Description

TS 28.310 [2] defines solutions for ES. It introduces a number of actors (see clause 6.2) including:

-	Domain-Centralized ES management

-	MnS Producer of Domain-Centralized ES

-	MnS Producer of Distributed ES management

-	Distributed ES function

-	Domain-SON ES functionality

-	Domain-centralized SON ES

-	Distributed SON ES functionality

-	Domain Centralized SON ES management

-	CESManagementFunction (IOC defined in TS 28.541 [3])

-	DESManagementFunction (IOC defined in TS 28.541 [3])

The relations among these are unclear. In some cases the functionalities are over lapping. The unclarities are as follows:

Issue 1: Scenarios addressed for ES and entity names

Based on the descriptions and illustrations in TS28.310, there are three scenarios are addressed for ES but not fully listed out as such. These include (see figure 5.2.C.1-1):

-	Scenario 1: A domain centralized entity configures the NR cells directly:

-	Scenario 2: A domain distributed entity configures the NR cells

-	Scenario 3: A distributed entity local to the gNBs that is configured on how to control ES in NR cells

These variations of scenarios for how ES states may be configured should be explicitly clarified and the related requirements updated.

Based on these three scenarios, there are four (4) functional entities involved in ES but several names are used for these entities.

- The NR cells that are configured to enter or exit energySaving state. Only 1 name is used for this, there are no concerns with this.

- A domain centralized entity that configures the NR cells directly. Two names are used for this -  CES Management Function (CESManagementFunction IOC) and “Domain centralized SON ES management".

- A domain distributed entity that configures the NR cells. Two names are used for this -  DES Management Function (DESManagementFunction IOC ) and “Distributed SON ES management".

- A distributed entity in the RAN that internally configures the NR cells states. The name for this entity is not explicitly described but is showed in illustration as "Distributed Energy saving Function". The name is however too close to another that is used for a different purpose, i.e. "producer of Distributed ES ".

Figure 5.3.A.1-1: Scenarios for ES that can be concluded form the descriptions and illustrations in TS28.310.

Issue 2: Centralized ES activation and Deactivation

TS 28.310 [2] seems to indicate that Prov. MnS producer should have the intelligence to decide how to configure the other cells when only given a configuration for the NR capacity booster cell (Step 3 in both Figure 6.2.2.1.1-1 6.2.2.1.2-1 in TS 28.310). This is not always true. It should be clarified that the producer of Centralized ES takes responsibility of deriving all the require attributes and configures them to the respective cells.

Issue 3: Centralized ES activation and Deactivation

The distributed ES activation and Deactivation mixes the scenarios that are related to distributed ES (scenarios 2 and 3 in issue 1 above). It mixes the functionalities of the domain distributed function and the gNB local distributed ES. It should be made clear that the Distributed ES function is responsible for controlling ES states but can be configured by the domain centralized or distributed ES management functions.

REQ-ESCOL-FUN-1: The management service producer responsible for energy saving should have the capability allowing its authorized consumer to configure the cell overlaid relations, and energy saving policies, and to enable or disable the Distributed SON ES function for a NR capacity booster cell to enter energy saving mode.l

No new requirement is expected for issue 3 but existing requirement REQ-ESCOL-FUN-1: needs to be revised, e.g., as follows

Figure 5.3.A.1-2: Potential revision to requirement REQ-ESCOL-FUN-1 in TS28.310.

#### 5.3.A.2	Potential requirements

ES-enh-1: The 3GPP management system should support a capability enabling an MnS consumer to configure a CES Management Function, a DES Management Function or a Distributed-SON Energy Saving Function.

ES-enh-2: The 3GPP management system should support a capability enabling for a centralized ES to derive and execute ES-related configurations on NR coverage and capacity booster cells.

#### 5.3.A.3	Potential solutions

Issue 1: Scenarios addressed for ES and entity names

To clarify the scenario and naming

-	Introduce a description of the three candidate scenarios with their illustration, e.g. similar to Figure 3.2.2-1.

-	Apply only 3 names for the functional actors in ES and the related service producers. Potential names that also align with TS 28.541 could be:

-	"Distributed-SON Energy Saving Function": A distributed entity in the RAN that internally configures the NR cells states given some configurations by an operator or another ES related function.

-	"CES Management Function" (CESManagementFunction IOC):- The domain centralized entity that can either directly configure NR cells or can configure "Distributed Energy saving Functions" local to the gNBs. As such, the CES Management Function is the "MnS producer of CES Management" exposing a CES MnS which enables an MnS consumer to configure different aspects expressed as attributes of the CESManagementFunction IOC.

-	"DES Management Function" (DESManagementFunction IOC ): The domain distributed entity that configures can either directly configure NR cells or can configure "Distributed Energy saving Functions" local to the gNBs. As such, the DES Management Function is the "MnS producer of DES Management" exposing a DES MnS which enables an MnS consumer to configure different aspects expressed as attributes of the DESManagementFunction IOC.

Issue 2: Centralized ES activation and Deactivation

@startuml MnS producer of Domain Centralized SON ES management makes the NR capacity booster cell enter the energySaving state

skinparam Shadowing false

autonumber

'skinparam maxMessageSize 200

participant "MnS producer of \nCentralized ES Management" as CESMP

participant "Performance Assurance \nMnS producer" as PerfP

participant "NR capacity Booster cell\n(exposing NF Prov MnS)" as NRCapCell

collections "Other NR cells\n(exposing NF Prov MnS)" as NRCells

Note over PerfP,NRCells: Collect traffic load performance measurements

PerfP->CESMP: Collect traffic load \n performance measurements

CESMP-> CESMP: Analyses traffic load \nperformance measurements

Opt Decision is taken that the capacity booster cell should enter energy saving state

CESMP->NRCapCell: ModifyMOIAttributes(energy saving, Oldvalue = "off", new value = "on"

CESMP->NRCells: Configure candidate cells

NRCapCell->CESMP: notifyMOIAttributesChanges(energy saving, Oldvalue = "off", new value = "on")

end

@enduml

Figure 6.2.2.1.1-1: Centralized energy saving activation

To address issue 2, update Figures 6.2.2.1.1-1 and 6.2.2.1.2-1 as follows:

Figure 5.3.A.1-1: Proposed revision of Figure 6.2.2.1.1-1 on centralized energy saving activation in TS28.310.

@startuml MnS producer of Domain Centralized SON ES management makes the NR capacity booster cell enter the energySaving state

skinparam Shadowing false

autonumber

'skinparam maxMessageSize 200

participant "MnS producer of \nCentralized ES Management" as CESMP

participant "Performance Assurance \nMnS producer" as PerfP

participant "NR capacity Booster cell\n(exposing NF Prov MnS)" as NRCapCell

collections "Other NR cells\n(exposing NF Prov MnS)" as NRCells

Note over PerfP,NRCells: Collect traffic load performance measurements

PerfP->CESMP: Collect traffic load \n performance measurements

CESMP-> CESMP: Analyses traffic load \nperformance measurements

Opt Decision is taken to reactivate NR capacity booster cell

CESMP->NRCapCell: ModifyMOIAttributes(energy saving, Oldvalue = "on", new value = "off"

CESMP->NRCells: Configure candidate cells

NRCapCell->CESMP: notifyMOIAttributesChanges(energy saving, Oldvalue = "on", new value = "off")

end

@enduml

Figure 6.2.2.1.2-1: Centralized energy saving deactivation

Figure 5.3.A.1-1: Proposed revision of Figure 6.2.2.1.1-1 on centralized energy saving deactivation in TS28.310.

Issue 3: Centralized ES activation and Deactivation

@startuml MnS producer of Domain Centralized SON ES management makes the NR capacity booster cell enter the energySaving state

skinparam Shadowing false

autonumber

'skinparam maxMessageSize 200

participant "MnS producer of \nDistributed ES Management" as DESMP

participant "D-SON ES Function\n(exposing NF Prov MnS)" as DESSON

participant "NR capacity Booster cell" as NRCapCell

collections "Other NR cells" as NRCells

DESSON->DESSON: Monitor traffic load of the candidate cell

DESSON->DESSON: Detectes that additional capacity is needed

Opt Decisions is taken to reactivate NR capacity booster cell

DESSON->NRCapCell: Reactivate NR capacity booster cell

DESSON->NRCells: Configure other NR cells

DESSON->DESMP: notifyMOIAttributesChanges(energy saving, Oldvalue = "on, new value = "off")

end

@enduml

Figure 6.2.3.2.2-1: Distributed energy saving deactivation

To address issue 3, update Figures 6.2.3.2.1-1 and 6.2.3.2.2-1 as follows:

Figure 5.3.A.1-1: Proposed revision of Figure 6.2.3.2.1-1 on Distributed energy saving activation in TS28.310.

@startuml MnS producer of Domain Centralized SON ES management makes the NR capacity booster cell enter the energySaving state

skinparam Shadowing false

autonumber

'skinparam maxMessageSize 200

participant "MnS producer of \nDistributed ES Management" as DESMP

participant "D-SON ES Function\n(exposing NF Prov MnS)" as DESSON

participant "NR capacity Booster cell" as NRCapCell

collections "Other NR cells" as NRCells

DESMP->DESSON: ModifyMOIAttributes(cell overlaid relations)

DESSON->DESSON: configure cell overlaid relations

DESSON->DESMP: notifyMOIAttributesChanges (cell overlaid relations)

DESMP->DESSON: ModifyMOIAttributes(ES policy (incl. threshold))

DESSON->DESSON: configure ES policy (incl. threshold)

DESSON->DESMP: notifyMOIAttributesChanges (ES policy (incl. threshold))

DESMP->DESSON: ModifyMOIAttributes(ESSwitch to "ON")

DESSON->DESSON: Switch on energy saving functionality

DESSON->DESMP: notifyMOIAttributesChanges (ESSwitch to "ON")

DESSON->DESSON: Analyses traffic load \nperformance measurements

Opt Decision is taken to deactivate NR capacity booster cell

DESSON->NRCapCell: Deactivate NR capacity booster cell

DESSON->NRCells: Configure other NR cells

DESSON->DESMP: notifyMOIAttributesChanges(energy saving, Oldvalue = "off, new value = "on")

end

@enduml

Figure 6.2.3.2.1-1: Distributed energy saving activation

Figure 5.3.A.1-1: Proposed revision of Figure 6.2.3.2.2-1 on Distributed energy saving deactivation in TS28.310.

* * * End of Changes * * * *