3GPP TSG-SA5 Meeting #162	S5-255378

Dallas, USA, 17 - 21 November 2025

Source:	Ericsson Korea Partners Co Ltd

Title:	Rel-20 pCR TR 28.883 Add use case for job execution control

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.1.0

Work Item:	FS_NDT_Ph2

Comments

This pCR adds a use case and requirement for NDTJob execution control based on convergence criteria.

This pCR is related to WT-4.

Proposed Changes

* * * First Change * * * *

# 5	Use cases

## 5.X	Use Case #X: Control job execution

### 5.X.1	 Description

Currently, NdtJobExecutionReqts only supports the maxRuntime attribute to constrain the execution of NDT jobs. While useful, this is insufficient for scenarios where operators need more precise control over scheduling, stopping rules, and resource usage. Realistic network operations demand the ability to (a) prioritize jobs, (b) stop based on convergence criteria instead of arbitrary timeouts, and (c) specify the amount of compute resources to balance accuracy against cost.

In current NDT job execution, simulations may run until a fixed time limit (maxRuntime) expires. This approach does not always reflect how real operators or automation systems evaluate network performance. Many NDT jobs are iterative by design. For example, each iteration produces KPI measurements (e.g., throughput, latency, BLER) that gradually stabilize as the simulated network reaches equilibrium.

However, because the existing execution model lacks a convergence-based stopping rule, operators face several issues:

- Inefficient resource utilization: Jobs continue consuming compute resources even after key KPIs have stabilized, wasting energy and processing time.

- Inconsistent results: Using arbitrary time limits leads to results that may depend on the chosen runtime rather than on actual network behavior.

- Limited determinism: Although “maxRuntime” defines an upper bound, the actual point when KPIs stabilize is unknown. Without a convergence rule, jobs may finish either too early or continue unnecessarily, reducing efficiency and responsiveness in automated loops.

- Unnecessary delay: For large-scale what-if analyses, over-running jobs delay decision making and increase operational cost.

A convergence-based stopping mechanism allows an NDT job to automatically terminate once monitored KPIs meet defined stability criteria. For example, when the change between successive KPI values remains below a threshold for a specified number of iterations.

This capability ensures that NDT executions stop precisely when the simulated network behavior has meaningfully converged, rather than when an arbitrary time limit is reached. As a result, operators gain faster, deterministic, and cost-efficient NDT evaluations aligned with their accuracy targets and automation timelines.

### 5.X.2	Potential requirements

REQ-NDT-ExecControl-1- The 3GPP management system should support a capability enabling authorized MnS Consumer to specify a convergence-based stopping criteria for the execution of NDTJobs.

* * * End of Changes * * * *