3GPP TSG-SA5 Meeting #163	S5-254829rev1

Wu Han, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0609 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.255 Clarify the CHF selection method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFSeg |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reason is that if you have a CHF Set or Group you will have to use NRF while for CHF instance addr there is no real need to do a NRF lookup. This means that the address type will more or less dictate if you should do NRF lookup or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the CHF selection method in Charging Characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CHF selection method in Charging Characteristics is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# A.1	General

A subscriber may have Charging Characteristics assigned to the subscription and/or the subscribed DNNs. Default Charging Characteristics may also be pre-provisioned on the SMF.

During PDU session establishment, when the SMF retrieves subscription data from the UDM, if a subscribed Charging Characteristics for the requested DNN is identified, it may override the SMF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as behaviours, freely defined by Operators.

These behaviours may be things like:

-	CHF addresses: to be used by the SMF, optionally with associated CHF instance ID(s), CHF set ID(s), CHF Group ID

-	CHF selection method: this indicates how the SMF is to select the CHF instance i.e., local configuration, NRF (see clauses 5.1.8 for the priority order of CHF selection and 5.1.9.2 for CHF selection in Roaming)

-	Default charging method: indicates what charging method to be used for every PCC rule of the PDU Session, i.e., "online", "offline", or “offline only”

-	Charging service: indicates if converged charging or offline only charging service is to be used

-	Triggers: the default triggers for the PDU session e.g., tariff time, time limit, volume limit, number of charging condition changes

One usage may be as a behaviour index associated to the PDU session in converged charging as described by the example in the Table A.1.

Table A.1: Example of Charging Characteristics behaviours for SMF

| Behaviour index | 5G data connectivity domain charging behaviours |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CHF address | CHF selection method | Default charging method | Charging service | Time Limit | Volume Limit | Cond. changes | Tariff time | … |
| 0 | CHF Set ID 1 | NRF | Online | Converged charging | 30 min | 2 MB | 2 | - | ... |
| 1 | Primary & secondary CHF addr. 1 & 2 | Local config. | Offline only | Converged charging | 15 min | 5 MB | 3 | 00:00 | … |
| 2 | CHF Group ID 1 | NRF | Offline | Converged charging | 30 min | 1 MB | 1 | - | ... |
| 3 | Primary & secondary CHF addr. 3 & 4 | Local config. | Offline only | Offline only charging | 15 min | 10 MB | 3 | 00:00 | ... |
| 4 | CHF Group ID 2 | NRF | Online | Converged charging | 15 min | 3 MB | 1 | - | … |
| … | … | … | … | … | … | … | … | … | … |

The "Default charging method", PDU session charging method and CHF addresses with possible associated CHF instance ID(s) and/or CHF set ID(s) configured in the applicable Charging Characteristics behaviour, are superseded by "Default charging method", PDU session charging method and CHF addresses with possible associated CHF instance ID(s) and/or CHF set ID(s) supplied by the PCF if any, during SMF interaction with the PCF at PDU session establishment, as described in TS 23.503 [202].

End of Change