3GPP TSG-SA5 Meeting #163	S5-254831rev2

Wu Han, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0092 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.274 Clarification on CHF address and Charging Characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFSeg |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the description of TS 23.503 Policy and charging control, the "chfGroupId", "Charging address" and "Charging information" are different attributes in the policy information from PCF to SMF/AMF/SMSF and the "Charging address" is optionally associated with CHF instance and CHF set ID, which is not associated with "chfgroupid". According to the description of TS 29.503 Unified Data Management Services, "chfGroupId " and "3gppChargingCharacteristics" are different attributes in the "AccessAndMobilitySubscriptionData" of AMF, "SessionManagementSubscriptionData" of SMF and "SmsManagementSubscriptionData" of SMSF. In that way, the chfgroupid can not include the charging characteristics of SMF, AMF and SMSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the CHF group id from the charging address and charging characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Charging Characteristics is inconsistent with the SA2/CT4 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.3, B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

5.4.1.3	CHF selection

The CHF to be used by the SMSF can be:

-	Discovered via NRF upon SMS activation from AMF to SMSF.

-	UDM provided charging characteristics.

-	Locally provisioned charging characteristics.

The option depends on Operator's policies.

The UDM provided charging characteristics (see Annex B) may be used to indicate CHF instance ID(s), CHF set ID(s), CHF Group ID.

CHF instance ID(s), CHF set ID(s), CHF Group ID can be used for NRF based discovery, see TS 32.290 [57] clause 6.1.

In the case the charging characteristics is not provided by UDM, then the CHF selection by SMSF is performed via NRF based discovery:

-	For UE determined by SMSF as served by the same PLMN as the SMSF (i.e. non-roaming scenario), the CHF of the PLMN can be discovered based on the UE identifier.

-	For UE determined by SMSF in V-PLMN as an in-bound roamer (i.e. roaming scenario), the CHF of the V-PLMN can be discovered based on the PLMN Id of the UE H-PLMN.

Next Change

Annex B (normative):
Charging Characteristics

B.1	General

A subscriber may have Charging Characteristics assigned to his subscription. Default Charging Characteristics may also be pre-provisioned on the SMSF.

The Charging Characteristics from UDM, if any, shall override the SMSF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as behaviours, freely defined by Operators. These behaviours may be things like:

-	CHF addresses: to be used by the SMSF, optionally with associated CHF instance ID(s), CHF set ID(s), CHF Group ID

-	CHF Group ID: associated with the subscription, which may be used by the SMSF to discover the CHF.

-	CHF selection method: this indicates how the SMSF is to select the CHF instance e.g., local configuration, NRF (see clause 5.4.1.3)

-	Charging scenario: indicates the charging scenario to be used i.e., ECUR, IEC, or PEC

One usage may be as a behaviour index associated to SMS Submit, SMS to deliver, SMS Submit Answer and SMS Deliver Answer in converged charging as described in the example in Table B.1.1:

Table B.1.1: Example of Charging Characteristics behaviours for SMSF

|  |  |  | SMS Submit | SMS to deliver | SMS Submit Answer | SMS Deliver Answer |
| --- | --- | --- | --- | --- | --- | --- |
| Behaviour Index | CHF address | CHF selection method | Charging scenario | Charging scenario | Charging scenario | Charging scenario |
| 0 | CHF Set ID 1 | NRF | ECUR | ECUR- | - |  |
| 1 | Primary & secondary CHF addr. 1 & 2 | Local config. | - | - | PEC | PEC |
| 2 | CHF Group ID 1 | NRF | IEC | IEC | PEC | PEC |
| 3 | Primary & secondary CHF addr. 3 & 4 | Local config. | ECUR | ECUR- | - |  |
| … | … |  | … | … | …… | … |

End of Change