3GPP TSG SA WG5 Meeting #163 
S5-254809
Wuhan, China 13 - 17 October 2025
Source:
Nokia

Title:
pCR TR 28.891 General Description
Document for:
Approval

Agenda Item:
7.5.1
1
Decision/action requested

This is a pCR to TR 28.891 General Description.
2
References

[1]
3GPP TR 28.891 " Study on Charging Aspects of CAPIF phase 3".

[2]
3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs"
3
Rationale

This pCR proposes to introduce the update of the scope in TR 28.891.
4
Detailed proposal

Propose to include the following change into the TR 28.891 [1].   
	First changes


4.1
General Description

The CAPIF core function provides the centralized CAPIF APIs that enable onboarding, discovery, security, and monitoring of exposed APIs. API invokers connect to the CAPIF core via CAPIF-1 and CAPIF-2 interfaces for onboarding and discovery of APIs, while CAPIF-1e extend these capabilities by enabling the authorization and authentication, service API discovery by an API Invoker outside the PLMN Trust domain, CAPIF-2e enabke the API Invoker to communicate to the service APIS which belong to different trust domains.
The inter-operability of different CAPIF Providers is supported by the CAPIF-6e reference point, which enables integration with CCF of a trusted domain (i.e. 3rd Party, PLMN). This ensures that API invokers from one trust domain can securely discover and consume APIs exposed in another trust domain, preserving interoperability and trust management. 
Figure 5.1-1 shows the reference point based functional model for the CAPIF.

[image: image1.emf]Trust domain of CAPIF provider A

API invoker

CAPIF-1e

CAPIF core function 

API exposing function

Service APIs

Service APIs Service APIs

CAPIF-3

CAPIF-2e

API invoker

CAPIF-1

CAPIF-2

CAPIF-4

API publishing function

API provider domain

CAPIF APIs

API management function

CAPIF-5

Trust domain of CAPIF provider B

API invoker

CAPIF-1e

CAPIF core function 

API exposing function

Service APIs

Service APIs Service APIs

CAPIF-3

CAPIF-2e

API invoker

CAPIF-1

CAPIF-2

CAPIF-4

API publishing function

API provider domain

CAPIF APIs

API management function

CAPIF-5

CAPIF-2e

CAPIF-2e

CAPIF-6e


Figure x.y-1: CAPIF interconnection with multiple CAPIF provider domains
	End of changes


_1615623219.vsd
�

Trust domain of CAPIF provider A


�

API invoker


CAPIF-1e


CAPIF core function 


API exposing function


Service APIs


Service APIs


Service APIs


CAPIF-3


CAPIF-2e


API invoker


CAPIF-1


CAPIF-2


CAPIF-4


API publishing function


�

API provider domain


CAPIF APIs


API management function


CAPIF-5


�

Trust domain of CAPIF provider B


�

API invoker


CAPIF-1e


CAPIF core function 


API exposing function


Service APIs


Service APIs


Service APIs


CAPIF-3


CAPIF-2e


API invoker


CAPIF-1


CAPIF-2


CAPIF-4


API publishing function


�

API provider domain


CAPIF APIs


API management function


CAPIF-5



