3GPP TSG-SA5 Meeting #163	S5-254311

Wuhan, China, 13 – 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on CCL for network slice resource optimization

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.0.0

Work Item:	Closed Control Loop Management phase2

Comments

This pCR is to add use case and requirements for CCL for network slice resource optimization

- WT-3: Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 5. 			Use Cases

### 5.X	Dynamic composition of CCL for network slice resource optimization

5.X.1	Description

3GPP TS 28.104 [X], specified MDA reports on root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands. MDA reports include descriptive, diagnostic, predictive and prescriptive analysis for network slicing.

Descriptive analytics provide a description of the events in the network, e.g., statistics on such events. Diagnostic analytics provide descriptions what could have happened including what could be the causes. Predictive analytics provide information on what might happen, e.g. in future if no change is done in the network; Prescriptive analysis provide recommendations what should happen, e.g., candidate actions that could be taken to achieve certain objectives.

The MnS consumer may want to compose MDA capabilities for descriptive, diagnostic, predictive and prescriptive network slicing analysis as part of a closed control loop for network slice resource optimization. The management system should enable the MnS consumer to compose the network slice resource optimization closed control loop.

Note: relation of this use case with the assurance closed control if FFS.

#### 5.X.2	Potential requirements

REQ- CCL-ES-1: The management system should enable the MnS consumer to compose a network slice resource optimization closed control loop from descriptive, diagnostic, predictive and prescriptive analytics capabilities on network slicing.

### 5.x.3	Possible solutions

To support the composition of a CCL on network slicing resource optimization, the following can be introduced:

-	an IOC for network slice resource optimization closed control loop.

-	The network slice resource optimization closed control loop inherits from a closed control loop

-	an IOC for a data collection component for network slice resource optimization data

-	IOCs for descriptive, diagnostic, predictive and prescriptive network slice resource optimization analytics components

-	The attributes on each component that take the analytics output of one component as input to the other components

Note: the attributes to capture the input-output relations of the network slice resource optimization CCL components are FFS.

5.X.4			Evaluation of solutions

.

* * * End of Changes * * * *