3GPP TSG-SA5 Meeting #163	S5-254636

WUHAN, CHINA, 13 - 17 October 2025

Source:	Nokia, NTT Docomo

Title:	Pseudo-CR on Rel-19 pCR TR 28.869 Add conclusions and recommendations to the use of VNF generic OAM functions

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.6.0

Work Item:	FS_Cloud_OAM

Comments

The pCR proposes to add conclusions and recommendations to the use of VNF generic OAM functions defined in clause 5.1 of TR 28.869

Proposed Changes

* * * First Change * * * *

# 6 			Conclusions and recommendations

## 6.Y Use of VNF generic OAM functions

The study has identified use cases that show how VNF generic OAM functions and ETSI NFV MANO can be used to support cloud-native VNFs in terms of non-application level configuration, policy management and upgrade operations.  The VNF generic OAM functions and corresponding interfaces specified in ETSI GS NFV-IFA 049 [2]. Their specification is out of scope of 3GPP. The ETSI NFV MANO and corresponding interfaces specified in ETSI GS NFV-IFA013 [8] and ETSI GS NFV-IFA008 [9]. Their specification is out of scope of 3GPP.

The application-level configuration, policy management and upgrade operations are performed by the 3GPP management system and the corresponding 3GPP MnSs.

The study also identified a use case and corresponding solution for cloud-native VNF traffic management relying on the traffic enforcer VNF generic OAM function (see clause 5.1.3). However, it is not clear what cloud-native VNF traffic means and the relation with 3GPP signalling traffic. Accordingly, based on the contents in the present document, it is concluded that the traffic management use case is out of scope for 3GPP SA5.

No conclusion and recommendation could be reached regarding the use of VNF generic OAM functions.

* * * End of Changes * * * *