3GPP TSG-SA5 Meeting #163	S5-254767

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0041 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.537 Add missing functionality and solution description for managing external management data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The solution reference description for serveral capabillities in TS 28.537 was added in previous meeting. However, the soluton description for Managing external management data in clause 6.4 in TS 28.537 is missing. 2. The use cases and requirements for Managing external management data are described in clause 6.4 in TS 28.537, and corresponding NRM fragment is defined in TS 28.622. However, the detailed implementation description (RESTful HTTP-based solution set) for each functionality is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add explicit description for functionalities provided for managing external management data. 2. Add the detailed implementation description (RESTful HTTP-based solution set) for each functionality |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for the functionalities provided by for managing external management data is missing in the published TS 28.537. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-254280 is the revision of S5-253888 - S5-254767 is the revision of S5-254280 |  |  |  |  |  |  |  |  |

1st Change

6.4.X	Solutions

6.4.X.1	Stage 2 definition

Following are the stage 2 definition for management of external management data:

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

- The External data type NRM fragment (including ExternalDataType IOC) defined in clause 4.3.73 in TS 28.622 [9].

6.4.X.2	Stage 3 definition

Following are the stage 3 definition for managing external management data:

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document "TS28623_ExternalDataMgmtNrm.yaml" in clause 4.3 in TS 28.623 [10].

Following are the detailed SS to support managing external management data based on Table 12.1.1.1.1-1 and Table 12.1.1.2.1-1 in TS 28.532 [2].

Table 6.4.X.2-1 SS to support managing external management data

|  | Functionality | IS operations | HTTP Method | Resource URI |
| --- | --- | --- | --- | --- |
| Discovery of external management data types | Query available external management data types | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/SubNetwork={id} |
|  | Subscribe changes on available external management data types | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/SubNetwork={id} |
|  | Unsubscribe changes on available external management data types | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/SubNetwork={id}/NtfSubscriptionControl={id} |
|  | Notify changes on available external management data types | notifyMOIAttributeValueChanges notification | POST | {notificationTarget} |
| Request and retrieve external management data | Request external management data | getMOIAttributes operation | GET | MnSRoot}/ProvMnS/{MnSVersion}/SubNetwork={id}/{ExternalDataType}=id |

Note: The ExternalDataType resource is defined in "TS28623_ExternalDataMgmtNrm.yaml", the NtfSubscriptionControl resource is defined in "TS28623_SubscriptionControlNrm.yaml".

End of Changes