3GPP TSG-SA5 Meeting #163	S5-254659

Wuhan, China, 13. – 17. October 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Add solution for use case#3

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides a potential solution for Use case#3 Assisting and reporting intent decomposition across intent handling functions.

Proposed Changes

* * * First Change * * * *

## 4.3 Use case #3:  Assisting and reporting intent decomposition across intent handling functions

### 4.3.1	Description

An intent-driven MnS producer can translate an intent and decompose it into one or more new intents. If these newly generated intent(s) cannot be handled by the intent-driven MnS producer, it can submit them to another intent-driven MnS producer(s). In this case, the former intent-driven MnS producer acts as the consumer of the latter intent driven MnS producer(s), to which the newly generated intents are submitted.

The issue arises when the decomposition of an intent into new intents is carried out by an intent driven MnS producer, these decomposed intents may be submitted to intent-driven MnS producers of managed domains that are not desired by the original intent-driven MnS consumer. The intent-driven MnS consumer needs to be able to assist the intent driven MnS producer regarding the decomposition of intents to other MnS producers. Furthermore, the intent driven MnS consumer needs to receive a report regarding this decomposition. This is also important for troubleshooting an intent-driven MnS producer, to check whether it is acting as expected.

### 4.3.2	Potential requirements

REQ-Intent_DCMP-1: The intent driven MnS producer should have the capability to allow an MnS consumer to specify a context to be able to assist the intent-driven MnS producer regarding the decomposition of an intent to other MnS producers.

REQ-Intent_DCMP-2: The intent driven MnS producer should have the capability to allow an intent-driven MnS consumer to receive a report regarding the decomposition of an intent to other MnS producers.

### 4.3.3	Potential solutions

#### 4.3.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing intent expectations and intent report information models defined in 3GPP TS 28.312 [1].

Intent expectations are enhanced with:

- A new expectation context that specifies the constraints applicable in the intent decomposition performed by the intent-diven MnS producer, such as a list of Intent Handling Functions identity among those under MnS consumer’s authorization to which the intent decomposition is not recommended by the intent-driven MnS consumer.

The IntentReport IOC in 3GPP TS 28.312 [1] is enhanced with following information:

- The identity of Intent Handling Functions to which the decomposed intents are sent.

- The identity of each intent and expectation resulting from the decomposition.

* * * End of Changes * * * *