3GPP TSG-SA5 Meeting #164	S5-255370

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0150 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.158 Fix status code table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The error code “409 Conflict” is referenced in the design pattern for deleting resource (clause 5.4). The code is however missing from the HTTP Error codes table (clause 6.6.2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing error table to the definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential implementation errors due to incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 6.6.2-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.6.2	HTTP error codes

A status code of the classes 4xx (Client Error) or 5xx (Server Error) is returned to the MnS Consumer in the error response status line. A complete list of error status codes is maintained by IANA.

Tables 6.6.2-1 and 6.6.2-2 list the status codes that shall be supported by MnS Producer and MnS Consumer implementations compliant to this specification.

Table 6.6.2-1: Supported 4xx client error status codes

| Error status code | Reference | Description |
| --- | --- | --- |
| 400 Bad Request | IETF RFC 7231 [2] | indicates that the server cannot or will not process the request due to something that is perceived to be a client error (e.g., malformed request syntax, invalid request message framing, or deceptive request routing). |
| 403 Forbidden | IETF RFC 7231 [2] | indicates that the server understood the request but refuses to authorize it. |
| 404 Not Found | IETF RFC 7231 [2] | indicates that the origin server did not find a current representation for the target resource or is not willing to disclose that one exists. |
| 405 Method Not Allowed | IETF RFC 7231 [2] | indicates that the method received in the request-line is known by the origin server but not supported by the target resource. |
| 406 Not Acceptable | IETF RFC 7231 [2] | indicates that the target resource does not have a current representation that would be acceptable to the user agent, according to the proactive negotiation header fields received in the request (Section 5.3), and the server is unwilling to supply a default representation. |
| 408 Request Timeout | IETF RFC 7231 [2] | indicates that the server did not receive a complete request message within the time that it was prepared to wait. |
| 409 Conflict | IETF RFC 7231 [2] | indicates that the request could not be completed due to a conflict with the current state of the target resource. |
| 410 Gone | IETF RFC 7231 [2] | indicates that access to the target resource is no longer available at the origin server and that this condition is likely to be permanent. |
| 411 Length Required | IETF RFC 7231 [2] | indicates that the server refuses to accept the request without a defined Content-Length field containing the length of the message body in the request message. |
| 413 Payload Too Large | IETF RFC 7231 [2] | indicates that the server is refusing to process a request because the request payload is larger than the server is willing or able to process. |
| 414 URI Too Long | IETF RFC 7231 [2] | indicates that the server is refusing to service the request because the request-target is longer than the server is willing to interpret. |
| 415 Unsupported Media Type | IETF RFC 7231 [2] | indicates that the origin server is refusing to service the request because the payload is in a format not supported by this method on the target resource. |
| 422 Unprocessable Entity | IETF RFC 4918 [17] | indicates the server understands the content type of the request entity (hence a 415(Unsupported Media Type) status code is inappropriate), and the syntax of the request entity is correct (thus a 400 (Bad Request) status code is inappropriate) but was unable to process the contained instructions. |
| 426 Upgrade Required | IETF RFC 7231 [2] | indicates that the server refuses to perform the request using the current protocol but might be willing to do so after the client upgrades to a different protocol. |
| 429 Too Many Requests | IETF RFC 6585 [18] | indicates that the user has sent too many requests in a given amount of time ("rate limiting"). |
| 451 Unavailable For Legal Reasons | IETF RFC 7725 [20] | Identifies the entity that blocks access to a resource following receipt of a legal demand. |

Table 6.6.2-2: Supported 5xx server error status codes

| Error status code | Reference | Description |
| --- | --- | --- |
| 500 Internal Server Error | IETF RFC 7231 [2] | Indicates that the server encountered an unexpected condition that prevented it from fulfilling the request. |
| 501 Not Implemented | IETF RFC 7231 [2] | indicates that the server does not support the functionality required to fulfill the request. |
| 502 Bad Gateway | IETF RFC 7231 [2] | indicates that the server, while acting as a gateway or proxy, received an invalid response from an inbound server it accessed while attempting to fulfill the request. |
| 503 Service Unavailable | IETF RFC 7231 [2] | indicates that the server is currently unable to handle the request due to a temporary overload or scheduled maintenance, which will likely be alleviated after some delay. |
| 504 Gateway Timeout | IETF RFC 7231 [2] | indicates that the server, while acting as a gateway or proxy, did not receive a timely response from an upstream server it needed to access in order to complete the request. |
| 505 HTTP Version Not Supported | IETF RFC 7231 [2] | indicates that the server does not support, or refuses to support, the major version of HTTP that was used in the request message. |

A MnS Producer may use other error response codes as well. However, there is no guarantee that a MnS Consumer understands the semantics beyond what is specified in clause 6 of IETF RFC 7231 [2]: "A client MUST understand the class of any status code, as indicated by the first digit, and treat an unrecognized status code as being equivalent to the x00 status code of that class".