3GPP TSG-SA5 Meeting #165	S5-260646

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0079 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.313 enhance MRO use case to support CHO and LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | According to clause 15.5.2.2.2 In TS 38.300, for intra-system mobility, it states that: In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. But in current type C definition of CHO and LTM, the corresponding performance measurements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add corresponding performance measurements in type C definition to support MRO for CHO and LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MRO for CHO and LTM solution can not implement correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.3.1, 7.1.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.1.6	MRO for Conditional Handover (CHO)

7.1.6.1	MnS component type A

MRO for CHO re-uses the component A for MRO, see clause 7.1.2.1.

7.1.6.2	MnS Component Type B definition

7.1.6.2.1	Control information

These parameters are used to control the CHO function.

Table 7.1.6.2.1-1: MRO fro CHO control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| CHO function control | This attribute allows the operator to enable/disable the CHO functionality. See attribute dCHOControl in TS 28.541 [13]. | Boolean On, off |

7.1.6.2.2	Parameters to be updated

MRO for CHO re-uses the same parameters to be updated as MRO, see clause 7.1.2.2.3.

7.1.6.3	MnS Component Type C definition

7.1.6.3.1	Performance measurements

Performance measurements related to MRO for CHO are captured in Table 7.1.6.3.1.-1:

Table 7.1.6.3.1-1. MRO for CHO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested conditional handover preparations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 [5] clause 5.1.1.6.6.1) |  |
| Number of successful conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 [5] clause 5.1.1.6.6.2) |  |
| Number of failed conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 [5] clause 5.1.1.6.6.3) |  |
| Number of requested conditional handover resource allocations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations (see TS 28.552 [5] clause 5.1.1.6.6.4) |  |
| Number of successful conditional handover resource allocations | Counts the number of successful inter-gNB conditional handover preparations (see TS 28.552 [5] clause 5.1.1.6.6.5) |  |
| Number of failed conditional handover resource allocations | Counts the number of unsuccessful inter-gNB conditional handover preparations (see TS 28.552 [5] clause 5.1.1.6.6.6) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing inter-gNB conditional handover candidates requested (see TS 28.552 [5] clause 5.1.1.6.6.7) |  |
| Number of UEs configured with conditional handover. | Counts the number of UEs that has been configured with inter-gNB conditional handover (see TS 28.552 [5] clause 5.1.1.6.6.8) |  |
| Number of successful conditional handover executions | Counts the number of successful inter-gNB conditional handover executions received (see TS 28.552 [5] clause 5.1.1.6.6.9) |  |
| Mean Time of requested conditional handover executions | Counts the mean time of inter-gNB conditional handover executions (see TS 28.552 [5] clause 5.1.1.6.6.11) |  |
| Max Time of requested conditional handover executions | Counts the max time of inter-gNB conditional handover executions (see TS 28.552 [5] clause 5.1.1.6.6.12) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing intra-gNB conditional handover candidates requested (see TS 28.552 [5] clause 5.1.1.6.7.1 |  |
| Number of UEs configured with conditional handover | Countes the the number of UEs that has been configured with conditional handover (see TS 28.552 [5] clause 5.1.1.6.7.2) |  |
| Number of successful conditional handover executions | Counts the number of successful intra-gNB conditional handover executions received (see TS 28.552 [5] clause 5.1.1.6.7.3) |  |
| Number of requested conditional handover preparations | Counts the number of outgoing intra-gNB conditional handover preparations requested, for a split gNB deployment (see TS 28.552 [5] clause 5.1.3.7.1.3) |  |
| Number of successful conditional handover preparations | Countes the number of successful intra-gNB conditional handover preparations, for a split gNB deployment (see TS 28.552 [5] clause 5.1.3.7.1.4) |  |
| Number of intra-system too late CHO execution | Counts the number of Too Late CHO Execution. (see TS 28.552 [5] clause 5.1.1.25.x1). |  |
| Number of intra-system too early CHO execution | Counts the number of Too Early CHO Execution (see TS 28.552 [5] clause 5.1.1.25.x1). |  |
| Number of intra-system CHO execution to wrong cell | Counts the number of CHO Execution to Wrong Cell. (see TS 28.552 [5] clause 5.1.1.25.x1). |  |

Next changes

7.1.8	MRO for Lower-layer Triggered Mobility (LTM)

7.1.8.1	MnS component type A

MRO for LTM re-uses the component A for MRO, see clause 7.1.2.1.

7.1.8.2	MnS Component Type B definition

7.1.8.2.1	Control information

These parameters are used to control the LTM function.

Table 7.1.8.2.1-1: MRO for LTM control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| LTM function control | This attribute allows the operator to enable/disable the LTM functionality. See attribute dLTMControl in TS 28.541 [13]. | Boolean |
| Control function for conditional LTM | This attribute allows the operator to enable/disable the conditional LTM functionality. See attribute dCLTMControl in TS 28.541 [13]. | Boolean |

7.1.8.2.2	Parameters to be updated

MRO for LTM re-uses the same parameters to be updated as MRO, see clause 7.1.2.2.3.

7.1.8.3	MnS Component Type C definition

7.1.8.3.1	Performance measurements

Performance measurements related to MRO for LTM are captured in Table 7.1.8.3.1-1:

Table 7.1.8.3.1-1. MRO for LTM related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of configured cell switch candidates | Counts the number of outgoing intra-gNB cell switch candidates requested (see TS 28.552 [5] clause 5.1.1.6.14.1) |  |
| Number of UEs configured with cell switch | Counts the the number of UEs that has been configured with intra-gNB cell switch (see TS 28.552 [5] clause 5.1.1.6.14.2) |  |
| Number of successful cell switch executions | Counts the number of successful intra-gNB cell switch executions received (see TS 28.552 [5] clause 5.1.1.6.14.3) |  |
| Number of requested cell switch preparations | Counts the number of outgoing intra-gNB cell switch preparations requested, for a split gNB deployment (see TS 28.552 [5] clause 5.1.3.7.1.9) |  |
| Number of successful cell switch preparations | Counts the number of successful intra-gNB cell switch preparations, for a split gNB deployment (see TS 28.552 [5] clause 5.1.3.7.1.10) |  |
| Number of intra-system too late LTM cell switch | Counts the number ofs Too Late LTM cell switch (see TS 28.552 [5] clause 5.1.1.25.x2). |  |
| Number of intra-system too early LTM cell switch | Counts the number of Too Early LTM cell switch. (see TS 28.552 [5] clause 5.1.1.25.x2). |  |
| Number of intra-system LTM cell switch to wrong cell | Counts the number of LTM cell switch to Wrong Cell. (see TS 28.552 [5] clause 5.1.1.25.x2). |  |

End of changes