3GPP TSG-SA5 Meeting #164	S5-260656

Goa, India, 9-13 February 2026

Source:	Ericsson

Title:	Rel-20 pCR TR 28.881 Add solution, evaluation and conclusion to UC#18 (IHF and NDTFunction)

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

In this pCR it is proposed to add a potential solution and conclusion to the use case #18 on the relation and interactions between intent handling functions and NDTFunctions.

This pCR is related to WT-5.1.

Proposed Changes

* * * First Change * * * *

## 4.18	Use case #18: The relation and the interactions between intent handling function and NDTFunction

### 4.18.1	Description

This use case proposes a scenario where intent-driven management services (IDMS) are enabled through the Network Digital Twin (NDT). A Network Digital Twin, as referenced in 3GPP TS 28.561 [12], is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any impacts to the real network. This capability is crucial for enabling intent handling function to validate intent feasibility and explore the best values for corresponding targets.

For example, during the pre-evaluation phase, as specified in 3GPP TS 28.312 [1], when received an intent from the MnS consumer, the intent handling function can invoke NDT's simulation capabilities to perform an intent feasibility check or explore best values for intent targets, validating the impact of these intents. This helps validate whether an intent (e.g., "assuring 500 concurrent UEs in a stadium during a concert") can be fulfilled before initiating intent fulfilment or explore the best values for intent targets within the optimal network parameters (e.g., adjusting UE group priorities to avoid resource conflicts) in the NDT.

### 4.18.2	Potential requirements

REQ-Intent_NDT-1: The intent driven MnS producer should have the capability to interact with NDTFunction to validate the feasibility of intents or explore the best values of intent targets in intent pre-evaluation phase.

REQ-Intent_NDT-2: The intent driven MnS producer should have the capability to allow MnS consumer to obtain the information of NDTFunctions as enabler information used for intent feasibility check and intent exploration.

### 4.18.3	Potential solutions

#### 4.18.3.1 Potential solution #2

This potential solution considers enabling an intent-driven MnS producer to interact with NDTFunction MnS producers. The solution does not require updating the NRM or already specified interfaces, and it only necessitates of updating the 3GPP TS 28.312 [1] to describe that the intent driven MnS producer, if necessary, can interact with NDTFunction MnS producers in the intent pre-evaluation phase, with reference to 3GPP TS 28.561 [12].

* * End of Changes * * * *