3GPP TSG SA5 Meeting #165	S5-260730

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Pseudo-CR on enhancement of MgmtDataInfo to reuse the supportedPerfMetricGroups and supportedTraceMetrics

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.3.0

Work Item:	FS_MADCOL_Ph3

Comments

This contribution proposes to add use case for the enhancement of MmgmtDataInfo.

Proposed Changes

* * * First Change * * * *

4.4 	Clarification of Mechanisms to Discover, Request and Retrieve Management Data

4.4.X	Use Case#4-X: Clarification on supportedManagementData in MgmtDataInfo IOC

4.4.X.1      Description

In TS 28.622 [2], the SupportedPerfMetricGroup and supportedTraceMetric are defined as attributes of several Managed Objects (including SubNetwork, ManagedElement, ManagedFunction and EP_RP) to represent performance metrics and trace metrics supported for corresponding managed objects. MnS consumer can request to query ManagedObject instance to obtain the supported performance metrics and trace metrics.

- SupportedPerfMetricGroup <<dataType>> is defined to captures a group of supported performance metrics, and associated (production and monitoring) granularity periods and reporting methods that are supported for the specified performance metric group.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceMetrics | M | T | F | F | T |
| granularityPeriods | M | T | F | F | T |
| reportingMethods | M | T | F | F | T |
| reportingPeriods | M | T | F | F | T |

- SupportedTraceMetrics is defined to capture for the trace metrics supported for this object.

In TS 28622 [2], the MgmtDataInfo IOC is defined to represent the management capabilities for a set of management data, which can be used for management data registry and discovery. The MgmtDataInfo IOC includes following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedManagementData | M | T | T/F | F | T |
| supportedGranularityPeriods | CM | T | T/F | F | T |
| supportedReportingPeriods | CM | T | T/F | F | T |
| historicalDataPeriod | CM | T | T/F | F | T |
| supportedReportingMethod | M | T | T/F | F | T |
| supportedDataScope | O | T | T/F | F | T |
| Atttibutes related to roles |  |  |  |  |  |
| supportedDataRequestMnSRef | M | T | T/F | F | T |
| supportedDataReportingMnSRef | M | T | T/F | F | T |
| Note: For all attributes of MgmtDataInfo, the property “isWritable=T” for the Separate MnSRegistry deployment scenario described in TS 28.537, and “isWritable=F” for the Embedded MnSRegistry deployment scenario. |  |  |  |  |  |

The attribute “supportedManagementData” is a choice between:

- a list of data categories (attribute mgtDataCategory)

- a list of management data identified with their name (attribute "mgtDataName"), a list of performance metric name and trace metric name.

4.4.X.2      Problem Statement

1. The definition for supported performance/trace metrics in MgmtDataInfo (Name contained by MnSRegistry) is different with the definitions for supported performance/trace metrics in concrete ManagedObjects (including SubNetwork, ManagedElement, ManagedFunction and EP_RP).

2. Attribute “mgtDataName” is defined as string to represent performance metric name and trace metric name, however, if a string read by consumer, it is not clear whether this is a performance metric or trace metric.

3. It is not clear how to use the MmgtDataInfo IOC to support the extension for new types of management data (which can be future standardized management date type or non-standardized management data type).

4.4.X.3      Potential Requirements

No new requirements identified

4.4.X.4      Potential Solution

It proposes to enhance the MgmtDataInfo IOC definition to reuse the supportedPerfMetricGroups and supportedTraceMetric as follows:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedManagementData | M | T | T/F | F | T |
| supportedGranularityPeriods | CM | T | T/F | F | T |
| supportedReportingPeriods | CM | T | T/F | F | T |
| historicalDataPeriod | CM | T | T/F | F | T |
| supportedReportingMethod | M | T | T/F | F | T |
| supportedDataScope | O | T | T/F | F | T |
| Atttibutes related to roles |  |  |  |  |  |
| supportedDataRequestMnSRef | M | T | T/F | F | T |
| supportedDataReportingMnSRef | M | T | T/F | F | T |
| Note: For all attributes of MgmtDataInfo, the property “isWritable=T” for the Separate MnSRegistry deployment scenario described in TS 28.537, and “isWritable=F” for the Embedded MnSRegistry deployment scenario. |  |  |  |  |  |

The attribute “supportedManagementData” is defined as SupportedManagementData <<CHOICE>> as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 supportedPerfMetricGroups | M | T | T/F | F | T |
| CHOICE_2.1 supportedTraceMetrics | M | T | T/F | F | T |
| CHOICE_3.1 supportedMgtDataCategory | M | T | T/F | F | T |

Note: The CHOICE can be further extended by the implementations, as appropriate.

* * * End of Changes * * * *