3GPP TSG-SA5 Meeting #163	S5-254223

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0733 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add UPF event exposure related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An NF (e.g., NEF or AF (when in trusted domain) or NWDAF) may need to subscribe and get notified about UPF events for UPF data collection. Therefore, it is necessary to add the UPF event exposure related measurement to support UPF enhancement for exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UPF event exposure related measurements: number of event exposure subscriptions/unsubscriptions/notifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor measurements related to UPF event exposure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.4.X	UPF event exposure related measurements

5.4.X.1 Number of event exposure subscriptions

a)	This measurement provides the number of UPF event exposure subscriptions received by the UPF.

b)	CC.

c)	Receipt by the UPF of an Nupf_EventExposure_Subscribe message from a UPF event consumer (e.g. AF (when in trusted domain) or NEF) (see TS 23.502 [7]).

d)	A single integer value.

e)	UPF.EventExpSubs

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X.2 Number of successful event exposure subscriptions

a)	This measurement provides the number of successful event exposure subscriptions at the UPF.

b)	CC.

c)	Transmission of an Nupf_EventExposure_Subscribe response by the UPF to a UPF event consumer (e.g. AF (when in trusted domain) or NEF) indicating a successful event exposure subscription.

d)	A single integer value.

e)	UPF.EventExpSubsSucc

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X.3 Number of event exposure notifications

a)	This measurement provides the number of event exposure notifications sent by the UPF.

b)	CC

c)	Transmission of an Nupf_EventExposure_Notify message by the PCF to a UPF event consumer (e.g. AF (when in trusted domain) or NEF) (see TS 23.502 [7]).

d)	A single integer value.

e)	UPF.EventExpNotify

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X.4 Number of event exposure unsubscriptions

a)	This measurement provides the number of event exposure unsubscriptions received by the UPF.

b)	CC

c)	Receipt of an Nupf_EventExposure_UnSubscribe (Subscription Correlation ID) message by the UPF from an NF consumer or the SMF on behalf of other NF (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	UPF.EventExpUnsubs

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

next change

A.X Monitoring of UPF event exposure for UPF Data Collection

To get exposure data from UPF, NF consumer may use UPF event exposure service (Notify, Subscribe, UnSubscribe) for UPF data collection targets one UE, a group of UE or Any UE.

The NF consumer may use the events exposed by UPF for QoS Monitoring, User DataUsage Measures and User DataUsage Trends. Measurements of UPF event exposure can help operators for enhancing network operation and maintenance efficiency, optimizing service quality, and promoting business innovation, hence the performance of UPF event exposure needs to be monitored.

End of changes