3GPP TSG-SA5 Meeting #163	S5-254261

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.882 Add Management support to NW-Side model training use case

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

The new SID SP-2501197 on study on MDA was approved at the SA #108 meeting. The objectives including:

WT-1: AI/ML Lifecycle Management Enhancements

WT-1.1: Investigate enhancements of AI/ML management capabilities throughout the AI/ML lifecycle in 5GS, including training, testing, emulation, deployment, inference, to support AI/ML-enabled features in the 5GS. This includes:

- ML model transfer/delivery as defined by RAN for Solution 4b: OAM can transfer/delivery AI/ML model(s) to UE.

- NG-RAN use cases including QoE optimization, network energy saving, and mobility use case(as defined in RP-250812).

- 5GC Analytics: Encompasses new 5GC analytics use cases currently under study under WT#2 (see SP-250413) and investigates OAM support for provisioning ML models to relevant 5GC functions to enable AI/ML-based analytics.

- LMF-based AI/ML Positioning including data collection and ML model training by the OAM for UE positioning.

- Study feasibility and potential requirements for data collection for (e.g., UE-side and Network-side) to enable model training.

The bullet 5 is to study feasibility and potential requirements for data collection to enable model training.

The approved reply LS S5-252842 confirm that the current Management based immediate MDT procedure defined in TS 32.422 can be reused to support RAN2 OAM-centric NW-side data collection.

According to the agreed CR RP- 252772 in TS 38.300, for AI/ML-based beam management, both NW-side model and UE-side model are supported.

This contribution proposes to add Management support to beam management use case for management support to NW-Side model training.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x1]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2"

[x2]	3GPP TR 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[x3]	3GPP TS 28.532: "Management and orchestration; Generic management services".

* * * Next Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

* * * Next Change * * * *

X	Use cases

X.Y	Management support to NW-Side model training

X.Y.Z	Management support to AI/ML-based beam management

X.Y.Z.1	Description

To support AI/ML-based beam management defined in TS 38.300 [x1], UE will provide data of beam management to 3GPP management system via gNB. 3GPP management system needs to collect data from gNB for NW-side model training.

Figure X.Y-1: Management of NW-side data collection and reporting for NW-side model training

The 3GPP management system request gNB(s) to collect the NW-side data of beam management and provides the configuration to gNB(s), then gNB(s) shall send configuration information to the UE(s) for NW-side data collection. After the NW-side data are produced, UE(s) shall report the NW-side data to gNB(s), and 3GPP management system shall collect data from the gNB(s). 3GPP management system collect data should depend on access control information specified by operator.

X.Y.Z.2	Potential requirements

REQ-ML_NWSIDE-x1: The 3GPP management system shall have a capability to configure the NW-side data collection for NW-side model training.

REQ-ML_NWSIDE-x2: The 3GPP management system shall have a capability to obtain the NW-side data of beam management for NW-side model training.

REQ-ML_NWSIDE-x3: The 3GPP management system shall have a capability to allowing the operator to specify access control information.

X.Y.Z.3	Potential solutions

Figure X.Y.Z.3-1: Management of NW-side data collection and reporting for NW-side model training

Following are the potential management capabilities which can be used for NW-side data collection control and data report:

-	For NW-side data collection control mechanism, following potential management capabilities can be used:

- CRUD operations/notifications with ManagementDataCollection IOC defined in clause 4.3.47 in TS 28.622[x2].

- CRUD operations/notifications with TraceJob defined in clause 4.3.30 in TS 28.622[x2].

-	For NW-side data reporting mechanism, following existing management capabilities can be used:

- Streaming data reporting service defined in clause 11.5 in TS 28.532[x3].

- File data reporting service in clause 11.6 in TS 28.532[x3].

Following are the potential enhancements to support NW-side data collection control and data reporting

-	Enhance the existing trace metrics to include the NW-side data.

NOTE: the definition of NW-side data depending on RAN2.

X.Y.Z.4	Evaluation of solutions

It is feasible to reuse the trace mechanism and the only enhancement is to add new NW-side data to the existing trace metrics to support AI/ML-based beam management.

* * * End of Changes * * * *